3GPP TSG RAN Meeting #105 	RP- 241765

Melbourne, Australia, September 9-12, 2024

Agenda item:	9.3.1.1

Source: 	Qualcomm Incorporated

Title: 	Views on the scope of Rel-19 MIMO WI

Document for:	Discussion/Decision

# Introduction

In this contribution, we express Qualcomm’s view on the following five Rel-19 MIMO topics recently proposed and discussed in RAN1, which are not in the scope of Rel-19 MIMO WID [2].

- SRS antenna switching for 3T6R, 3T3R and 4T6R

- Multi-TA for asymmetric DL sTRP / UL mTRP

- Partially coherent codebook for UL 3Tx

- Reduced complexity MIMO for 6Rx/8Rx UE

- UL precoder cycling

# SRS antenna switching for 3Tx

SRS antenna switching, which is an important component to complete the functionality of 3 Tx UE, is missing in the current scope of Rel-19 MIMO for 3 Tx UE.

Furthermore, there is a RAN4 LS to request RAN1 introduce 3T6R and 4T6R based on RAN-P WID. Therefore, we support to specify at least 3T6R and 3T3R in Rel-19. For 4T6R, given the larger specification effort to support it, it can be pursued in future release.

Proposal 1: support to specify 3T6R and 3T3R SRS antenna switching in Rel-19.

# Multi-TA for asymmetric DL sTRP / UL mTRP

During the Rel-19 MIMO scoping discussion in RAN plenary last year, the following two objectives were proposed and discussed for asymmetric DL sTRP/UL mTRP deployment scenarios [4]:

- Objective 1: Extension of Rel-18 2TA mDCI to sDCI assuming legacy PRACH resources

- Objective 2: Separate UL power control for SRS only to downlink S-TRP from SRS to UL M-TRP and introduce path loss measurement to uplink M-TRP.

Proposal 2: Supporting of two TAs as part of enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios can be considered in an updated Rel-19 MIMO WID.

# Partial-coherent 3 Tx precoders

For 3Tx UE, in Rel-19, 3GPP specified only non-coherent precoders. It is straightforward to extend the support of 3 Tx MIMO with partial-coherent precoders. The specification effort for it is mainly on design 3 Tx partial-coherent codebook, which can be handled by viewing partial-coherent 3Tx as full-coherent 2Tx plus a standalone 1-Tx and reuse the full-coherent 2Tx codebook as much as possible.

Proposal 3: In an updated Rel-19 MIMO WID, support 3-Tx partial-coherent codebook for rank 1, 2, and 3.

# Reduced-complexity MIMO receiver for 6/8 Rx UE

Topic 6: 6Rx/8Rx UE enhancements with lower complexity utilizing two segments of 3/4 Rx antenna units up to 8-layer DL Tx based on legacy codebook and legacy codeword to layer mapping

- Objective 1: SRS antenna port grouping, CSI and codeword association to the segments of receive antenna.

Topic 6: 6Rx/8Rx UE enhancements with lower complexity utilizing two segments of 3/4 Rx antenna units up to 8-layer DL Tx based on legacy codebook and legacy codeword to layer mapping

- Objective 1: SRS antenna port grouping, CSI and codeword association to the segments of receive antenna.

During the scoping of Rel-19 MIMO in RAN last year (2023), reduced MIMO complexity for 6Rx/8Rx UE was discussed as topic 6 in [5].

This topic was removed in the final WID of Rel-19 MIMO [2].

Figure 1. Reduced complexity MIMO via two Rx groups: (a) 2-CW case; (b) single-CW case with LLR combining

In RAN1#117 meeting, the following proposal to revive the above topic is proposed in [3] and treated with the following documented in [4].

Proposal 1.A.2: For a UE configured with a total of PSRS=6 or 8 ports across ≥1 SRS resources for antenna switching intended for xT6R or xT8R, respectively, support the following fixed SRS port grouping where (with the PSRS ports indexed in an ascending order according to SRS resource ID and port number within each SRS resource): - SRS port group 0, corresponding to CW0, comprises the first PSRS/2 out of PSRS ports; and - SRS port group 1, corresponding to CW1, comprises the second PSRS/2 out of PSRS ports No other spec enhancement is introduced, e.g. new CW-to-layer mapping, DL resource allocation, DCI format Note: The above grouping assumption is to align NW and UE on the association between SRS ports and reported CQIs for the two CWs when reportQuantity = ‘cri-RI-CQI’. Note: different SRS ports are associated with different UE antenna ports. Note: if one single CW is scheduled, both SRS port groups can correspond to the same CW Note: This feature is a separate UE capability and, for UEs supporting this capability, configured via RRC (FFS details on the extend of RRC configuration) Companies are encouraged to evaluate for further discussion in RAN1#118.

## Discussion of open issues with the simplified receiver

While we acknowledge that the currently proposed topic is lesser in scope than that originally proposed (and not adopted), many of our concerns regarding the original proposal equally apply to the new proposal.

First of all, a feasibility study is needed to evaluate whether a UE can support this Rx/SRS ports grouping, because of the following reasons.

- Rx port grouping may depend on the UE antenna correlation, imbalance etc.

- UE may not be able to always group the ports by fixedly mapping the antennas to the SRS resources for antenna switch, e.g. due to the RF switching circuitry implementation.

Secondly, this topic needs a careful study of performance degradation vs complexity reduction to understand the cost-benefit tradeoff of this simplified MIMO receiver. It is premature to agreeing on specify this simplified MIMO receiver scheme without a study phase. The performance study should consider the following aspects.

- Intercell interference: One issue of the SRS port grouping scheme is that there is no PMI feedback from UE, i.e., the scheme fully depends on gNB to select PMI that can null the interference between the two port groups. However, gNB does not see interference received at UE side. The precoder gNB selected will cause leakage between the two groups hence further performance loss.

- Imperfect ZF at gNB due to SRS aging and estimation error: Since the SRS port grouping scheme is sensitive to precoder selection to null interference between two groups, performance evaluation should be based on realistic SRS channel modelling and channel estimation.

- Precoding assumption mismatch between UE and gNB: in a real system with this SRS port grouping scheme, a UE has to calculate and feedback CQI/RI based on certain precoder assumption. However, due to the port grouping, the UE cannot perform interference suppressing base precoding in CQI/RI calculation, while gNB will perform interference suppressing such as ZF in precoder determination. Therefore, there is a precoding assumption mismatch between UE and gNB in this scheme, which should be modelled in the performance study.

- Uneven rank between 2 CWs: with existing CW-to-layer mapping, if sum rank of two CWs is odd, Rx group 0 always has one less layer than Rx group 1, How much would such rank restriction impact performance?

Thirdly, from RAN4 perspective, new demodulation requirements are needed for this simplified MIMO receiver. In addition, new REFSENS requirements and new CQI reporting performance requirements may be needed. RAN4 is needed to be involved in the study and (if justified) standardization of this scheme.

Lastly, some open technical issues of the proposal are listed below. They need to be addressed/clarified by the proponents of the proposal.

- Whether/what information exchange between the two component receivers (one for each CW)? For rank<=4, it is unclear how the receiver is assumed to operate.  In order for the gNB to select PMI and MCS, it needs to know which of the following option is used at a UE.

  - One component receiver (fixed which one) is turned off

  - One component receiver (adaptive which one) is turned off

  - Both component receivers attempt to decode independent (diversity)

  - Both component receivers operate and some combining happens before decoding

- How is the RI determined in the 2 CWs case. Does each component receiver come up with a rank independent from the other? What happens if the two ranks are dissimilar and incompatible?

- Does CSI processing timeline extend with the simplified receiver?

- Whether there is more impact to SRS sounding procedure beside the SRS port indices grouping?

- For PDCCH decoding, whether a legacy 6/8 Rx receiver is used or a similar simplified receiver is used?

## Performance study of the simplified receiver

In the following, a preliminary study is performed to compare the performance of the above proposal (8 Rx with SRS port grouping) with the existing Rel-15 MIMO scheme (8Rx without SRS port). Key simulation assumptions are listed as below.

| Parameters | Values |
| --- | --- |
| Simulation BW | 64 RBs |
| Channel | CDL-B |
| Delay Spread | 100 ns |
| Doppler Shift | 11, 66 Hz |
| gNb TxRUs | 64 |
| Layers | 8 |
| SRS Periodicity | 10, 20, 40, 80 Subframes |
| SRS Channel Estimation | Genie/Practical |
| DMRS Channel Estimation | Practical |
| Link Adaptation | On |
| PDSCH retransmissions | Enabled |
| gNb Precoder selection | SVD based on 8Tx SRS for the existing Rel-15 MIMO scheme SVD with ZF based on 2 x 4 SRS for port grouping |
| NumRbsWithSamePrecoding | 4 RB |
| Intercell interference | Not modelled |

One should notice that the above simulation assumptions are in favour of the proposal of port grouping, because of the following three “shortcuts” took in the preliminary study.

- Intercell interference is not modelled. One issue of the SRS port grouping scheme is that there is no PMI feedback from UE, i.e., the scheme fully depends on gNB to select PMI that can null the interference between the two port groups. However, gNB does not see interference received at UE side. The precoder gNB selected will cause leakage between the two groups hence further performance loss, which is not captured in this preliminary study.

- CQI/RI feedback is not modelled. In a real system with this SRS port grouping scheme, a UE has to calculate and feedback CQI/RI based on certain precoder assumption. However, due to the port grouping, the UE cannot perform interference suppressing base precoding in CQI/RI calculation, while gNB will perform interference suppressing such as ZF in precoder determination. Therefore, there is a precoding assumption mismatch between UE and gNB in this scheme, which is not modelled in this preliminary simulation.

In the following figure, we observed that even with simulation assumptions in favour of SRS port grouping, port grouping generates near 3~4dB performance loss comparing with the existing Rel-15 MIMO scheme, with practical SRS channel estimation, 10 slots SRS periodicity, and 11Hz Doppler. In terms of peak throughput at high SNR, the loss is about 18%.

Fig 2: Comparison between simplified receiver vs Rel-15 baseline receiver, left subfigure with genie SRS channel estimation, right subfigure with practical SRS channel estimation.

The above observation implies that the simplified receiver performance is sensitive to SRS aging and SRS channel estimation error, which will determine the ZF beamforming quality at gNB. The following simulation results (with 66Hz Doppler, realistic SRS channel estimation, fixed MCS 10) further confirmed this observation. With larger SRS periodicity, the performance degradation is even larger. With 80 slots SRS periodicity, the simplified receiver can barely work.

Fig 3: Simplified receiver performance under realistic SRS channel estimation with different SRS periodicity.      Left-top subfigure with SRS periodicity 10 slots, right-top subfigure with SRS periodicity 20 slots, left-bottom subfigure with SRS periodicity 40 slots, right-bottom subfigure with SRS periodicity 80 slots.

With the above analysis of the proposal and the observation from preliminary simulation, we have the following proposal.

Proposal 4: Don’t support SRS port grouping scheme for simplified 6Rx/8Rx MIMO receiver without a study phase being added in an updated Rel-19 MIMO WID to studying the following aspects:

- Performance degradation vs complexity reduction to understand the cost-benefit tradeoff of the simplified MIMO receiver.

- RAN4 specification impact including at least new performance requirements of PDSCH demodulation, REFSENS, and CSI reporting, for rank<=4 and rank >4 respectively.

- RAN1 specification impact including at least the following

  - CQI reporting procedure for rank<=4

  - RI reporting procedure for rank >4

  - CSI processing timeline extension

  - SRS sounding procedure with SRS grouping

  - PDCCH decoding procedure

# UL precoder cycling

Open-loop based UL precoder cycling was briefly mentioned in [1] but not included in the Rel-19 MIMO WID. In RAN1 # 117, a proposal is proposed to define UL PRG to enabling open loop UL precoding cycling.

Similar to the previous topic as discussed in Section 4, a study phase is needed to evaluate the UL precoder cycling scheme to understand its performance benefit and specification impact, before agreeing on specifying it. Specifically, we think the study at least need to address the following questions.

- What is the performance difference between UL precoder cycling and S-CDD for PUSCH.

- What SRS enhancement is needed for precoder cycling, e.g., whether introduce new SRS port(s) with precoder cycling to match the channel between SRS and PUSCH.

- How to perform RAN4 testing if the precoding cycling is transparent to standards.

- What are the details of PRG configuration? How about PRG configurability?

Proposal 5: A study phase is needed for UL precoding cycling scheme if it is added in an updated Rel-19 MIMO WID.

# Conclusions

In this contribution, we gave our views on the following Rel-19 MIMO topics.

- SRS antenna switching for 3T6R, 3T3R and 4T6R

- Multi-TA for asymmetric DL sTRP / UL mTRP

- Partial-coherent 3Tx precoders

- Reduced complexity MIMO for 6Rx/8Rx UE

- UL precoder cycling

Subsequently, we made the following proposals:

Proposal 1: support to specify at least 3T6R and 3T3R SRS antenna switching in Rel-19.

Proposal 2: Supporting of two TAs as part of enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios can be considered in an updated Rel-19 MIMO WID.

Proposal 3: In an updated Rel-19 MIMO WID, support 3-Tx partial-coherent codebook for rank 1, 2, and 3.

Proposal 4: Don’t support SRS port grouping scheme for simplified 6Rx/8Rx MIMO receiver without a study phase being added in an updated Rel-19 MIMO WID to studying the following aspects.

- Performance degradation vs complexity reduction to understand the cost-benefit tradeoff of the simplified MIMO receiver.

- RAN4 specification impact including at least new performance requirements of PDSCH demodulation, REFSENS, and CSI reporting, for rank<=4 and rank >4 respectively.

- RAN1 specification impact including at least the following

  - CQI reporting procedure for rank<=4

  - RI reporting procedure for rank >4

  - CSI processing timeline extension

  - SRS sounding procedure with SRS grouping

  - PDCCH decoding procedure

Proposal 5: A study phase is needed for UL precoding cycling scheme if it is added in an updated Rel-19 MIMO WID.

# References

- RP-233089, Moderator's summary on new WI NR MIMO Phase 5, Samsung (Moderator), RAN#102, Dec. 11 – 14, 2023.

- RP-234007, New WID: NR MIMO Phase 5, Samsung (Moderator), RAN#102, Dec. 11 – 14, 2023.

- R1-2403945, On 128 CSI-RS ports and UE reporting enhancement, Huawei, HiSilicon, 3GPP TSG RAN WG1 #117, Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024.

- Chairman’s Notes RAN1#117, May 20 – 24, 2024.

- RP-232745, Summary for RAN Rel-19 Package: RAN1/2/3-led, RAN Chair, RAN#102, Dec.11-14,2023.