3GPP TSG RAN Meeting #105	RP-241766

Melbourne, Australia, September 9-12, 2024

Source:	Qualcomm Incorporated

Title:	Views on the scope of Rel-19 LP-WUS/WUR WI

Agenda Item:	9.3.1.4

Document for: 	Discussion and Decision

# Background

In this contribution, we provide our views on the Rel-19 LP-WUS [RAN1 WI: NR_LPWUS].

# Potential RAN2 TU adjustment for LP-WUS procedure in connected mode

LP-WUS WID has following checkpoint in RAN#105 [1]:

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR … - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

In RAN1, multiple options of LP-WUS procedure in connected mode have been discussed and following agreements & working assumption were achieved.

RAN1 agreements & working assumption on the options from RAN1#116bis and RAN1#118:

Agreement Update the following agreement in RAN1#116 in red: Agreement - For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring: - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - This option may replace DCP functionality - Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - PDCCH monitoring possibly irrespective of drx-onDurationTimer - Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration - FFS duty-cycled and/or continuous LP-WUS monitoring - Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded should be considered. - RAN1 does not discuss C-DRX related timers other than drx-onDurationTimer, this topic is up to RAN2 - Note: Above does not preclude to support fallback mechanism to trigger PDCCH monitoring, if any Agreement For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. - LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - UE is configured with legacy C-DRX configurations as Rel-18 - UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration) - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration - LP-WUS triggers the start of a timer during which UE monitors PDCCH - FFS the timer is existing timer and/or new timer - UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected - drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL - No impact on RRM/RLM/BFD measurement requirements is assumed - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1) - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not - UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions.

RAN2 has also discussed the spec impacts of the identified options and achieved following agreements.

RAN2 agreements from RAN2#126 and RAN2#127:

In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1. For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message. For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

As excerpted in the above, option 1-1, from RAN2 point of view, can be based on Rel-16 DCP. Therefore, it must be possible to specify LP-WUS in connected mode, at least based on option 1-1, without increasing RAN2 TU.

For option 1-2, RAN1 achieved an agreement that summarizes its framework. Since option 1-2 has a large commonality with option 1-1 according to it, support of option 1-2 should not require substantial work for optimization that requires RAN2 TU increase. RAN2 should confirm to support option 1-1, discuss details of option 1-2 further, and decide whether to conform RAN1’s working assumption for support of option 1-2, depending on the work-load necessary for option 1-2.

Proposal 1:

- RAN2 TU is kept unchanged.

- For LP-WUS procedure in connected mode, following are clarified for RAN2:

  - Option 1-1 should be supported based on legacy DCP mechanism.

  - Option 1-2 can be supported by leveraging the RAN1#118 agreement, if it fits within the RAN2 TU budget.

# Clarification of LP-WUS scope for frequency ranges

RAN1 has an FFS on the following RAN1 agreement achieved at RAN1#117 meeting:

Agreement Update agreement in last meeting as below: From RAN1 perspective, support X PRBs for LP-WUS and LP-SS with SCS 30kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5MHz - X to be down-selected between = 11 and 12 PRBs - FFS if other number of PRBs needed, for LP-SS and LP-WUS with a channel bandwidth equal or less than 5MHz FFS: Whether the above is applicable to FR2

When the above FFS (whether X = 11 PRBs is applicable to FR2 that uses FR2-specific SCS, e.g., 120kHz) was discussed in RAN1, there was a confusion on whether or not FR2 is in the scope of the work item. Further, it was mentioned that LP-WUS and LP-SS for FR2 would need dedicated study and specification work. Similar confusion was observed also in RAN4. However, we would like to point out the following:

- The WID [1] does not have a limit on the frequency range where LP-WUS and LP-SS are specified. Rather, the WID captures both TS38.101-1 and TS38.101-2 in the list of “Impacted existing TS/TR” in Section 5. This indicates that the WID, approved in RAN, already endorses support of LP-WUS and LP-SS on both FR1 and FR2.

- Once RAN1 agrees “X = 11 PRBs is applicable to FR2 that uses SCS 120kHz”, no other dedicated work is necessary for FR2 in RAN1. The LP-WUS and LP-SS designs, including waveform generation, agreed for FR1 with SCS 30kHz, can simply be re-used for FR2 that uses SCS 120kHz. RAN2 must not need a dedicated work for FR2 as well.

  - The design target of LP-WUS/LP-SS is to achieve the coverage of Msg.3 PUSCH. In [2], we have shown that if a LP-WUS/LP-SS with X = 11 PRBs with SCS 30kHz is designed to achieve the coverage as Msg.3 PUSCH on an FR1 carrier, the LP-WUS/LP-SS with X = 11 PRBs with SCS 120kHz can also achieve the coverage of Msg.3 PUSCH on an FR2 carrier. Therefore, no optimization is necessary.

- RAN4#112 has achieved following agreement. The agreed sentence essentially means RAN4 considers FR2 is in the scope.

Issue 1-1-6: Specify FR2 LP-WUS RF requirements Agreements: - Wait to RAN1 progress on FR2 LP-WUS work.

From RAN1/RAN2 point of view, the issue of FR2 is simple – it is just necessary to confirm the value of X for FR2. On the other hand, RAN4 needs to specify relevant requirements for an FR2 band based on the value of X. For work item completion, it is not constructive for WGs to discuss whether FR2 is in scope or not. Instead, we propose in RAN (1) to clarify/confirm that Rel-19 LP-WUS work item scopes both FR1 and FR2 and (2) to request RAN1 to make a progress on the FFS “Whether the above is applicable to FR2” at the earliest possible time (in RAN1#118bis).

Proposal 2:

- Confirm in RAN#105 that Rel-19 LP-WUS work item scopes both FR1 and FR2.

- Request RAN1 to make a progress on the FFS “Whether the above is applicable to FR2” in RAN1#118bis.

# Conclusion

In this contribution, we shared our views on Rel-19 LP-WUS scopes and proposed the following.

Proposal 1:

- RAN2 TU is kept unchanged.

- For LP-WUS procedure in connected mode, following are clarified for RAN2:

  - Option 1-1 should be supported based on legacy DCP mechanism.

  - Option 1-2 can be supported by leveraging the RAN1#118 agreement, if it fits within the RAN2 TU budget.

Proposal 2:

- Confirm in RAN#105 that Rel-19 LP-WUS work item scopes both FR1 and FR2.

- Request RAN1 to make a progress on the FFS “Whether the above is applicable to FR2” in RAN1#118bis.

# Reference

- RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony

- R1-2407040, LP-WUS and LP-SS design, Qualcomm Incorporated

- R4-2414309, WF on LP-WUS UE RF requirements, vivo