| 3GPP TR 38.743 V2.0.0 (2024-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group RAN; Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions of terms, symbols and abbreviations	5

3.1	Terms	5

3.2	Symbols	5

3.3	Abbreviations	5

4	Use cases and Solutions	6

4.1	AI/ML based Network Slicing	6

4.1.1	Use case description	6

4.1.2	Solutions and standard impacts	6

4.2	AI/ML based Coverage and Capacity Optimization	7

4.2.1	Use case description	7

4.2.2	Solutions and standard impacts	7

5	Rel-18 Leftovers and solutions	9

5.1	Mobility optimization for NR-DC	9

5.1.1	Use case description	9

5.1.2	Potential Standard impacts	9

5.2	Split architecture support for Rel-18 use cases	9

5.2.1	Use case description	9

5.2.2	Potential Standard impacts	10

5.3	Energy saving enhancements	10

5.3.1	Use case description	10

5.3.2	Solutions and standard impacts	10

5.4	Continuous MDT collection targeting the same UE across RRC states	10

5.4.1	Use Case Description	10

5.4.2	Potential Standard impacts	11

5.5	Multiple-hop UE trajectory across gNBs	11

5.5.1	Use case description	11

6	Conclusion	11

Annex <A> (informative): Change history	12

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

1	Scope

The present document provides the description and investigation of new AI/ML based use cases, i.e., Network Slicing and Coverage and Capacity Optimization, and its corresponding solutions, and initial analysis of Rel-18 leftovers.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description".

[3]	3GPP TS 38.401: "NG-RAN; Architecture description".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

<Void>

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<Void>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<Void>

4	Use cases and Solutions

4.1	AI/ML based Network Slicing

4.1.1	Use case description

Support of network slicing in NG-RAN is defined in TS 38.300 [2].

The NG-RAN plays a key role in taking mobility, load balancing and Radio Resources Management decisions for the purpose of meeting target requirements derived from the SLA of each supported network slice.

AI/ML function can analyze metrics related to network and UE level performance related to perform optimal resource management and mobility decisions for network slicing to meet the requirements.

4.1.2	Solutions and standard impacts

4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based network slicing:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

4.1.2.2	Input data of AI/ML based Network Slicing

To predict the optimized network slicing decisions, a gNB may need the following information as input data for AI/ML-based network slicing:

From local node:

-	Measured/Predicted radio resource status per slice

-	Measured/Predicted slice available capacity

-	Legacy predicted UE trajectory

From neighbouring gNBs:

-	Measured/Predicted radio resource status per slice

-	Measured/Predicted slice available capacity

From the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

4.1.2.3	Output data of AI/ML based Network Slicing

AI/ML-based network slicing model in a gNB can generate the following information as output:

-	Predicted radio resource status per slice

-	Predicted slice available capacity

-	Resource management decisions for resources within RRM policies (used by gNB internally)-	Slice aware mobility decisions (used by gNB internally)

4.1.2.4	Feedback of AI/ML based Network Slicing

To optimize the performance of AI/ML-based network slicing model, the following feedback can be considered to be collected from gNBs:

-	Measured Radio resource status per slice

-	Measured Slice available capacity

-	Legacy UE performance feedback for those UEs handed over from the source gNB

-	Finer granularity UE performance feedback for those UEs handed over from the source gNB to determine UE performance for a certain slice in use by a certain UE.

NOTE:	Exact level of finer granularity to be defined in normative phase.

4.1.2.5	Potential standard impacts

Following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

Xn interface:

-	Enhanced existing procedure to collect predicted information between gNBs:

■	Predicted radio resource status per slice

■	Predicted slice available capacity

4.2	AI/ML based Coverage and Capacity Optimization

4.2.1	Use case description

The objective of NR Coverage and Capacity Optimization (CCO) function is to detect and resolve or mitigate CCO issues. An NG-RAN node may autonomously adjust within and switch among coverage configurations. When a change is executed, a NG-RAN node may notify its neighbour NG-RAN nodes with the list of cells and SSBs with modified coverage included.

In the legacy CCO solution, a reactive approach is used: when the gNB (gNB-CU in case of CU-DU split architecture) detects a CCO issue which negatively impacts network and UE performance after it has already occurred, the gNB (gNB-DU in case of CU-DU split architecture) attempts to resolve or mitigate it.

With an AI/ML based CCO, a more proactive approach is used to prevent (or limiting at an early stage) the rise of a CCO issue with the consequent degradation of network (and UE) performance.

4.2.2	Solutions and standard impacts

4.2.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based CCO:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

4.2.2.2		Input data of AI/ML based CCO

For a proactive prediction and resolution of a CCO issue, a gNB may need the following information as input data for AI/ML-based CCO:

From local node:

-	Measured/Predicted radio resource status

-	Current CCO State

From neighbouring gNBs:

-	Measured/Predicted radio resource status

From the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

-	SON Reports (e.g., RLF, CEF, RA)

4.2.2.3	Output data of AI/ML based CCO

AI/ML-based CCO model in a gNB can generate following information as output:

-	Predicted CCO issue (including predicted affected cells/beams)

-	Future CCO State

NOTE 1:	Future CCO State can also be derived by legacy means. Signalling future CCO state will not be described as prediction over Xn.

NOTE 2:	Future CCO state and predicted CCO issue will be provided together with timing information.

NOTE 3:	Future CCO state can be derived based on the predicted CCO issue and other information.

4.2.2.4	Feedback of AI/ML based CCO

To optimize the performance of AI/ML-based CCO model, following feedback can be considered to be collected from gNBs:

-	Measured radio resource status

-	Legacy UE performance feedback for those UEs handed over from the source gNB

SON Reports (e.g., RLF, CEF, RA)

4.2.2.5	Potential standard impacts

Following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18 based on the CCO framework described in TS 38.300 [2] and TS 38.401 [3].

Xn interface:

Enhance existing procedure to collect information between gNBs:

-	Future CCO State together with an associated coverage modification cause

-	Timing Information

F1 interface:

-	Predicted CCO Issue (including predicted affected cells/beams) from gNB-CU to gNB-DU.

-	Future CCO State from gNB-DU to gNB-CU

-	Timing Information

NOTE 4:	Whether gNB-CU generates the suggested future CCO state and sends it to gNB-DU as assistance information is left to normative discussion.

NOTE 5:	For split architecture, future CCO State is derived based on the predicted CCO issue signaled from gNB-CU and other information in the gNB-DU.

5	Rel-18 Leftovers and solutions

5.1	Mobility optimization for NR-DC

5.1.1	Use case description

Mobility in NR-DC can be optimized by means of AI/ML.

Mobility Optimization for NR-DC is studied by assuming inference at the MN only. The main use case is limited to Dual Connectivity only and Conditional Dual Connectivity procedures are out of scope.

5.1.2	Potential Standard impacts

The Dual Connectivity procedures (e.g., SN Addition, MN-initiated SN Change) are enhanced to trigger the collection of measured UE performance.

5.2	Split architecture support for Rel-18 use cases

5.2.1	Use case description

The split architecture should be enhanced to support the Rel-18 use cases, e.g, Load Balancing, Energy Saving, and Mobility Optimization.

In case of CU-DU architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU(-CP);

-	AI/ML Model Training and Model Inference are both located in the gNB-CU(-CP).

5.2.2	Potential Standard impacts

The following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18:

-	The details of signaling measured UE performance metrics from gNB-DU to gNB-CU-CP and/or from gNB-CU-UP to gNB-CU-CP need further discussion during normative phase.

-	Measured Energy Cost from gNB-DU to gNB-CU.

5.3	Energy saving enhancements

5.3.1	Use case description

A description of the AI/ML-based Network Energy Saving use case as such is presented in TR 37.817. Based on the conclusions in TR 37.817, a metric called Energy Cost was introduced during Rel-18 normative work; this metric is defined as an index representing the energy consumption at the NG-RAN node and can be exchanged between NG-RAN nodes over the Xn interface upon request with a per NG-RAN node granularity. However, a node-level Energy Cost metric might be too coarse to provide a good understanding of the energy impact of AI/ML-based Network Energy Saving actions in NG-RAN nodes handling multiple cells, so possible solutions to improve Energy Cost granularity could be beneficial.

5.3.2	Solutions and standard impacts

During Rel-18 normative work, the Energy Cost (EC) metric was defined on an NG-RAN node-level granularity even though the AI/ML Energy Saving actions that we assumed in Rel-18 were on a cell-level granularity, e.g., cell switch-off actions. Enhancements to the node-level EC to a finer granularity could be beneficial, assuming that such enhancements enable network-level energy saving.

The following approaches to improve the EC granularity were discussed:

a.	EC per group of Cells based on energy consumption measurements associated to the hardware serving the group of cells

b.	EC per Cell based on energy consumption estimations

c.	EC per one or more HO event based on energy consumption estimations

d.	EC per gNB-DU based on energy consumption measurements associated to the hardware serving the gNB-DU

5.4	Continuous MDT collection targeting the same UE across RRC states

5.4.1	Use Case Description

The problem of continuous data collection for management-based MDT can be described as follows: a UE in the NG-RAN can be configured with management-based Logged MDT when in RRC_Idle and RRC_Inactive states and with management-based Immediate MDT when in RRC_Connected state. Differently from signalling-based MDT, in management-based MDT, a UE is not uniquely identified in the MDT activation. Therefore, when a UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive (during which Logged MDT data have been collected) or when a UE is handed over between gNBs, the network does not have standardized means to select again the same UE for continuous MDT for subsequent MDT data collection.

The Data Collection continuity in this scenario can be split into two tasks as below:

-	Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

-	Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

5.4.2	Potential Standard impacts

Potential solutions addressing the problems above can be discussed during normative phase.

5.5	Multiple-hop UE trajectory across gNBs

5.5.1	Use case description

In Rel-18, the cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node. Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to one or more gNBs where the UE is expected to connect and these cells are listed in chronological order.

6	Conclusion

The new use cases and the Rel-18 leftover cases follow Rel-18 AI/ML framework.

The following new use cases are recommended by RAN3 to be specified in the Rel-19 normative phase:

-	AI/ML-based Network Slicing

-	AI/ML-based Coverage and Capacity Optimization

Recommended solutions and standard impacts for each use case are based on section 4.1.2 and section 4.2.2.

For each use case above, it is recommended to take the corresponding section “Solutions and standard impacts” (section 4.1.2 and 4.2.2) as basis during the normative work.

The following Rel-18 leftovers are recommended by RAN3 to be specified in Rel-19 normative phase:

-	Mobility Optimization for NR-DC

-	Split architecture support for Rel-18 use cases

-	Continuous MDT collection targeting the same UE across RRC states

The corresponding descriptions and potential standard impacts for each Rel-18 leftovers above shall be taken as baseline during the normative phase.

Annex <A> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-03 | RAN3#123bis | R3-241865 |  |  |  | Skeleton for TR38.743 v0.0.0 | 0.0.0 |
| 2024-05 | RAN3#124 | R3-243170 |  |  |  | Including text proposals R3-242231 and R3-242233 agreed in RAN3#123bis meeting. | 0.0.1 |
| 2024-06 | RAN3#124 | R3-243971 |  |  |  | Including text proposals R3-243918, R3-243951 and R3-243952 agreed in RAN3#124 meeting. Submit to RAN#104 for information | 1.0.0 |
| 2024-08 | RAN3#125 | R3-244028 |  |  |  | Editorial corrections for following drafting rules. Resubmit to RAN3#125 | 1.1.0 |
| 2024-08 | RAN3#125 | R3-244848 |  |  |  | Including text proposals: R3-244810, R3-244804, R3-244843, R3-244841, R3-244842 | 1.2.0 |