3GPP TSG RAN Meeting #105                                RP-241803

Melbourne, Australia, September 9-12, 2024

Source:	Spreadtrum Communications

Title:	Discussion on Rel-19 XR WID

Document for:	Discussion and decision

Agenda Item:	9.3.2.3

- Introduction

In RAN#103 meeting, the WID of “XR (eXtended Reality) for NR Phase 3” was revised [1]. However, there are some checkpoints in the latest WID for RAN#105.

In this contribution, we will discuss the issues about these checkpoints and provide a revised WID for R19 XR.

- Discussion

In the RAN2#127 meeting [2], RAN2 has studied the possible enhancements for multi-modality in RAN including multi-modal awareness, RAN enhancements based on multi-modal information, traffic synchronization and PDU Set discarding. The related agreements are listed in the following:

Working assumption: Regardless of SA2 decision, RAN2 can extend the UAI for multi-modal awareness at least for uplink QoS flows in Rel-19 XR, by having the UE report existence of multi-modality application and association information among QFIs to gNB.

FFS whether this can be applied to DL.

RAN2 considers that based on multi-modal information:

- The gNB may perform joint admission control. Details can be left up to RAN3 in potential WI phase. FFS if MMSID can be used for this purpose.

- The gNB may consider this information during QoS flow to DRB mapping (up to gNB implementation)

For UL, RAN2 does not intend to perform LCP enhancements due to complexity vs gains concerns.

For DL, whether traffic synchronization (on a per packet basis) can be achieved depends on whether packet level synchronization information can be provided from CN to RAN.

RAN2 thinks PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the multi-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4.

RAN2 thinks in case this is feasible, it should be limited to intra-DRB case.

Besides, SA2 has discussed the feasibility of sending the Multi-modal Service ID to RAN and agreed that forwarding MMSID to RAN via control plan is in principle technically feasible [3]. Whether and what other Multi-modal info is needed can be studied further as RAN2 also made a working assumption that RAN can extend the UAI for multi-modal awareness including UL and/or DL. Some enhancements for multi-modality in RAN can be specified based on the multi-modal awareness as RAN2 has identified. Therefore, we propose to change the SI to WI for the multi-modality. The objective can consist of the multi-modal awareness, UL/DL traffic synchronization, admission control and PDU Set discarding across QoS flows of the same multi-modal service.

Proposal 1: Change the SI for multi-modality to WI.

For the objective of Enhancements for Scheduling, WID has a note that LCP implementation complexity need to be taken into account when evaluating solutions and need check in RAN#105. In the past RAN2 meetings, we discussed the LCP restrictions based solutions and adjust the priority of LCH based solutions. The LCP restrictions based solutions were excluded due to the large complexity. In addition, LCP prioritization within a logical channel will not be considered in RAN2 which also avoids extra complexity in UE. The DSR enhancements were discussed to report multiple pairs of remaining time and buffer sizes without further complexity introduced. In our understanding, the enhancements for UL Scheduling can focus on the using additional priority configured to LCH and DSR enhancements without introducing large complexity.

So we propose to revise the enhancements for Scheduling according to RAN2 agreements.

Proposal 2: Update the objective of Enhancements for Scheduling according to RAN2 progress.

For the last note that Whether / to what extent network exposure / RAN awareness / e.g. RAN involved rate control, possibly additional info for DL scheduling, parallel with SA2 work, shall be covered in this WI is TBD. In R18, RAN can provide the congestion information (i.e. a percentage of packets that UPF uses for ECN marking for L4S) of the QoS Flow via GTP-U header extension to PSA UPF which will bring large delay for UL rate control. In order to speed the UL rate control for XR, RAN can provide the UL congestion info via air interface directly.

Proposal 3: Specify the UL congestion signaling in Uu for XR rate control.

Based on the above discussions, we will provide a revised WID for Rel-19 XR for further discussion.

Proposal 4: A revised WID for Rel-19 XR is provided below for discussion.

- Conclusion

In this contribution, we provided our views on the details of checkpoints in R19 XR WID. The following proposals are proposed:

Proposal 1: Change the SI for multi-modality to WI.

Proposal 2: Update the objective of Enhancements for Scheduling according to RAN2 progress.

Proposal 3: Specify the UL congestion signaling in Uu for XR rate control.

Proposal 4: A revised WID for Rel-19 XR is provided below for discussion.

We also provide the revised WID for Rel-19 XR:

The Rel-19 XR Phase 3 objectives are as follows: - Specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2, RAN3]. - Multi-modal awareness in RAN. [RAN2] - UL/DL traffic synchronization. [RAN2, RAN3] - Admission control enhancement. [RAN3] - PDU Set discarding across QoS flows of the same multi-modal service based on the dependency information between the multi-modal flows in intra-DRB case. [RAN2] - Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4] - Specify Enhancements for Scheduling, as follows: - For the UL, Specify enhancements using additional priority configured to LCHs with delay-critical data and enhancements of DSR with multiple pairs of remaining time and buffer sizes, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. - Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. - Specify Core requirements related to the above objectives as necessary [RAN4] - Extend Release 18 standalone mechanism to support NR-NR dual connectivity as follows [RAN3] - PDU set based handling - ECN marking - Burst Arrival Time reporting, if needed - PSI Discard coordination, if needed - Note: No RAN2 impact from above items - Specify the UL congestion signalling for XR rate control per QoS flow/per DRB [RAN2]

- Reference

- RP-240791, Revised WID on XR (eXtended Reality) for NR Phase 3.

- 3GPP, RAN2#127 Chair’s Notes.

- S2-2409444.