3GPP TSG RAN Meeting #105                                RP-241805

Melbourne, Australia, September 9-12, 2024

Source:	Spreadtrum Communications

Title:	Discussion on R19 network energy savings

Document for:	Discussion and decision

Agenda Item:	9.3.1.5

- Introduction

In RAN#104 meeting, the WID of ‘Enhancements of network energy savings for NR’ was revised [1]. The corresponding objective is copied below.

4.1 Objective of SI or Core part WI or Testing part WI The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105 - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

As shown in WID, whether to covert on-demand SIB1 from study into normative work should be determined in RAN#105. In this meeting, we will share our view on on-demand SIB1.

- Discussion

In RAN1 #117 [2], companies presented evaluations showing that the energy saving gain for NES cell is promising in most cases (e.g., SIB1 periodicity is 20ms or 40ms, SSB and SIB1 PDSCH are TDMed), and the energy saving loss for cell A is trivial in most case (e.g., on-demand SIB1 rate is larger than 30%). And the feasibility of Case 2 and Case 3 are agreed in RAN1 #117 and #118 respectively [3]. Hence, we support to covert on-demand SIB1 from study into normative work.

Agreement@117 For SIB1 in idle/inactive mode, prioritize RAN1 discussions on Case 2 and Case 3 - Case 2 (Option 1+B+X) is feasible from RAN1 perspective. - Further study Case 3, focusing on the additional NES benefits over Case 2, feasibility, complexity, and spec impact.

Agreement@118 For on-demand SIB1 in idle/inactive mode, Case 3 (Option 2+B+Y) is feasible from RAN1 perspective for some scenarios. Case 3 (Option 2+B+Y) is lower priority compared to Case 2 from RAN1 perspective. - RAN1 is inconclusive on whether the required specification support is justified by the observed NES gain for Case 3 Agreement@118 RAN1 recommends specifying on-demand SIB1 only for Case 2 (Option 1+B+X) in Rel-19. - Note: RAN1 strive to minimize impact to legacy UE. Note: RAN1 specification impact to support this feature should be minimized.

Also, agreements on OD-SIB1 was reached in RAN2 #127 as follow [4]:

Agreements on OD-SIB1@127 Scenarios: - No consensus for RAN1 case 3 in RAN2. OD-SIB1 SI conclusion: - RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

Based on RAN1 and RAN2 agreements, we have the following proposals:

Proposal 1: Specify at least Case 2 to support on-demand SIB1 in Rel-19.

Proposal 2: Support to covert on-demand SIB1 from study into normative work and the WID can be revised as follow:

<<----- unchanged text omitted ----->> 2. Study Specify procedures and signaling method(s) at least for Case 2 to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105 <<----- unchanged text omitted ----->>

- Conclusion

In this contribution, we provided our views on on-demand SIB1 with the following proposals:

Proposal 1: Specify at least Case 2 to support on-demand SIB1 in Rel-19.

Proposal 2: Support to covert on-demand SIB1 from study into normative work and the WID can be revised as follow:

<<----- unchanged text omitted ----->> 2. Study Specify procedures and signaling method(s) at least for Case 2 to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105 <<----- unchanged text omitted ----->>

- Reference

- RP-241650, Revised WID: Enhancements of network energy savings for NR, June 17-20, 2024.

- 3GPP, RAN1#117 Chair’s Notes, May 20th – 24th, 2024.

- 3GPP, RAN1#118 Chair’s Notes, August 19th – 23rd, 2024.

- 3GPP, RAN2#127 Chair’s Notes, August 19th – 23rd, 2024.