3GPP TSG RAN Meeting #105 	          	      		            	 RP-241810

Melbourne, Australia, September 9-12, 2024

Agenda Item:   	9.2.3

Source:   	LG Electronics

Title: 	Discussion on ISAC channel modeling SI scope in Rel-19

Document for:   	Discussion and decision

- Introduction

There is a checkpoint marked as yellow below regarding whether or not to include additional study beyond channel modelling for ISAC in Rel-19 ISAC channel modelling SID [1].

The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: - UAVs - Humans indoors and outdoors - Automotive vehicles (at least outdoors) - Automated guided vehicles (e.g. in indoor factories) - Objects creating hazards on roads/railways, with a minimum size dependent on frequency All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). Frequencies from 0.5 to 52.6 GHz are the primary focus, with the assumption that the modelling approach should scale to 100 GHz. (If significant problems are identified with scaling above 52.6 GHz, the range above 52.6 GHz can be deprioritized.) For the above use cases, sensing modes and frequencies: - Identify details of the deployment scenarios corresponding to the above use cases. - Define channel modelling details for sensing using 38.901 as a starting point, and taking into account relevant measurements, including: - modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns; - spatial consistency. It will be discussed at RAN#105 whether to include additional study beyond channel modelling for ISAC.

In this contribution, we provide our view on the additional study beyond channel modelling for ISAC, which requires the plenary guidance.

- Discussion

For the topic of “Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR”, the following agreements/conclusions were made in RAN1#118 meetings.

# 9.7.1 ISAC deployment scenarios ## RAN1#118 Conclusion RAN1 will consider the recommendations for the physical characteristics (e.g., sizes, shapes, materials, velocities, etc.) of sensing targets and objects provided in 5GAA LS (R1-2405964), along with the relevant characteristics defined in 3GPP TRs, within the scope of the Rel-19 study item. - No LS response from RAN1 to 5GAA is necessary. - R1-2405964 is proposed to be NOTED. Agreement General principles for all sensing target deployment scenarios should consider the following: - “Sensing mode” is removed in the scenario tables, but may be included in the evaluation/calibration phase. Per the SI, all sensing modes are possible for the deployment scenarios. - “Sensing area” may be addressed as part of the sensing target distribution and/or Tx/Rx characteristics and/or cell layout. Agreement For UAV sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#117 as a baseline. Note: Additional parameters, value/value ranges are not precluded. The detailed scenario description in this clause can be used for channel model calibration. ISAC-UAV Details on ISAC-UAV scenarios are listed in Table x. Table x. Evaluation parameters for UAV sensing scenarios Parameters; Value. Applicable communication scenarios; UMi, UMa, RMa [38.901] UMi-AV, UMa-AV, RMa-AV. Sensing transmitters and receivers properties; Rx/Tx Locations; Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenarios. Note1: This may include aerial UEs for UMi-AV, UMa-AV, RMa-AV communication scenarios. In this case, other Rx/Tx properties (e.g. mobility) are also taken from the corresponding communication scenario.. Sensing target; Outdoor/indoor; Outdoor. 3D mobility; Horizontal velocity: uniform distribution between 0 and 180km/h, if horizontal velocity is not fixed to 0. Vertical velocity: 0km/h, optional {20, 40} km/h NOTE2: 3D mobility can be horizontal only or vertical only or a combination for each sensing target FFS: time-varying velocity.. 3D distribution; Horizontal plane: Option A: N targets uniformly distributed within one cell. Option B: N targets uniformly distributed per cell. Option C: N targets uniformly distributed within an area not necessarily determined by cell boundaries. FFS: Value of N, defined area, and other distributions Vertical plane: Option A: Uniform between 1.5m and 300m. Option B: Fixed height value chosen from {25, 50, 100, 200, 300} m assuming vertical velocity is equal to 0. FFS Other options are not precluded. Note2: target(s) are outside the minimum distance to the Tx/Rx. Orientation; Random in horizontal domain. Physical characteristics (e.g., size); Size: - Option 1: 1.6m x 1.5m x 0.7m - Option 2: 0.3m x 0.4m x 0.2m FFS: Material(s), Structure, Other size(s). Minimum 3D distances between pairs of Tx/Rx and sensing target; Option B: Min distances based on min. TRP/UE distances defined in TR36.777 as a starting point. Option C: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx. Minimum 3D distance between sensing targets; Option 1: At least larger than the physical size of a target Option 2: 10 meters. [Unintended/Environment objects, e.g., types, characteristics, mobility, distribution, etc.]; FFS. Note: further down-selection between the options in the table is not precluded. # 9.7.2 ISAC channel modeling ## RAN1#118 Agreement if RCS related coefficient of a scattering point is included in small scale, the RCS related coefficients are separately determined for different pairs of incident/scattered ray(s) at the scattering point. Agreement For radio propagation Case 1, for modelling the target channel of a target with single scattering point, - To model a direct path, a single LOS ray from Tx to target and a single LOS ray from target to Rx are generated - AoA/ZoA of the direct path at Rx, AoD/ZoD of the direct path at target are generated at least based on the 3D location of target and Rx in the global coordinate system - AoD/ZoD of the direct path at Tx, AoA/ZoA of the direct path at target are generated at least based on the 3D location of Tx and target in the global coordinate system - The Delay of the direct path = (d3D_tx_target + d3D_target_rx)/c - The Doppler of the direct path is generated by spherical unit vectors by AoD/ZoD at Tx, by spherical unit vectors by AoA/ZoA at Rx, and velocity of Tx, target and Rx - The power of the direct path is generated as the product of the power of the LOS ray from Tx to target, the power of the LOS ray from target to Rx, and the effect of RCS - FFS initial phase - FFS how to model RCS, polarization of target - FFS number of direct paths - FFS on detailed modelling of indirect path(s) - FFS applicability of direct path generation to each scattering point when the target is modelled as multiple scattering points Agreement For the target channel of a target with single scattering point, when stochastic cluster is used to model an indirect path in the target channel, - An indirect path in small scale is modelled by concatenation of path(s) from Tx to target and from target to Rx, i.e., Option 1 in the agreement of RAN1 #117 - AoD/ZoD/AoA/ZoA from Tx to target or from target to Rx are - generated for a LOS ray at least based on the 3D location of Tx/Rx and target in the global coordinate system - stochastically generated for a NLOS ray using section 7, 38.901 as starting point - Delay is sum of delay of LOS/NLOS ray from Tx to target and the LOS/NLOS ray from target to Rx - Doppler is generated by spherical unit vector by AoD/ZoD at Tx and velocity of Tx, by spherical unit vector by AoA/ZoA at Rx and velocity of Rx, and by spherical unit vectors by AoA/ZoA/AoD/ZoD at target and velocity of target - FFS The mobility of stochastic clutter - The power of the indirect path is generated as the product of the power of the LOS/NLOS ray from Tx to target, the power of the LOS/NLOS ray from target to Rx and the effect of RCS - FFS initial phase - FFS how to model effect of RCS at target - FFS whether/how to model polarization at target - FFS How to reduce complexity - FFS applicability of the indirect path generation to each/a single scattering point when the target is modelled as multiple scattering points Working assumption The RCS related coefficient of a scattering point can be modelled with two components, i.e., linear value RCS = A*B - A first RCS component A (m2) is included in large scale - FFS the first RCS component is deterministic or stochastic - FFS The first component is dependent on incident and scattered directions at target - A second RCS component B (unit ratio) is included in small scale - FFS The second component is dependent on incident and scattered directions at target - FFS the second RCS component is deterministic or stochastic or combination - Note: RCS component A or B can be disabled by setting its linear value to 1 - Whether to disable a component can be discussed per target type - FFS how to determine A and B for each target - FFS whether/how to model polarization impact at target - FFS whether/how to normalize power accounting for target channel and background channel Agreement The impact of a scattering point of the target in the target channel is modelled by a scalar RCS value times a complex-valued 2x2 polarization matrix CPMst, i.e., RCS∙CPMst - FFS whether CPMst is angular/ray-dependent or independent - FFS whether polarization matrix CPMstis modelled assuming specular reflection or random coefficient for diffraction or scattering - FFS whether polarization matrix CPMst is explicitly modelled or merged with other polarization matrixes from Tx to target and/or from target to Rx Note: subscript is changed from st to sp. Agreement For modeling stochastic cluster in background channel, in order to define the background channel for TRP-UE and UE-TRP bistatic sensing mode, - The large scale and small scale parameters defined in TR 38.901, TR 37.885, TR 36.777 are used as start point In order to define the background channel for TRP-TRP and UE-UE bistatic sensing mode, - RAN1 to study how to model background channel - Option 1: The large scale and small scale parameters defined in TR 38.901, TR 38.858, 37.885, 38.859 are used as start point - Option 2: New channel model based on measurement results or ray-tracing model validated by experimental results FFS whether/how to do power normalization between target channel and background channel Agreement In order to define the background channel for TRP and UE mono-static sensing mode, - RAN1 to study how to model the background channel - Option 1: randomly drop at least one virtual Rx, and then the background channel is generated based on the channel generated as TR 38.901 between the real Tx and each virtual Rx for a scenario - FFS EO is modelled in the background channel - Option 2: New channel model based on measurement results or ray-tracing model validated by experimental results or radar literatures - FFS EO is modelled in the background channel - Option 3: the locations of clusters in the target channel are deterministically generated, then the background channel is generated using the clusters with the determined locations. - Other options are not precluded Agreement When EO type-2 is modelled, specular reflection is considered to model EO type-2 using section 7.6.8 of TR 38.901 as reference - As starting point, the effect of type-2 EO (i.e., in the path node1-EO-node2) is modelled as cR∥specular00-R⊥specular, c is a scaling factor (e.g., c equals to d3DdGR relative to LOS ray in section 7.6.8 in TR 38.901) - FFS any update to c,R∥specular,R⊥specular - FFS any update taking EO orientation into account - FFS any changes based on section 7.6.8 of TR 38.901 if EO type-2 has finite size - FFS whether diffraction and scattering can be considered in addition to specular reflection - EO type-2 is an optional modelling component if supported in a sensing scenario - FFS which deployment scenario(s) EO type-2 will apply Note: change parameter name c to b

There are still many remaining open issues for decision in both agendas of ISAC deployment scenarios and ISAC channel modelling, including:

- ISAC deployment scenarios

  - Channel model calibration/evaluation parameters for sensing target scenarios other than UAV, i.e., Automotive, Humans (indoor/outdoor), AGVs, and objects creating hazards on roads/railways.

  - Whether/how to parameterize unintended/environment objects in the deployment scenarios.

- ISAC channel modelling

  - Physical object modelling

    - Exact RCS value/pattern for each target type

    - RCS for bistatic

    - Details on modelling object with multiple scattering points

  - Target channel

    - Details on concatenation of Tx-target link and target-Rx link, and complexity reduction

    - Polarization matrix of direct/indirect path

    - Details on generating power of direct/indirect path

    - Number of direct paths for a target with single scattering point

    - Target channel modelling for target with multiple scattering point

    - Remaining details on EO type-2

    - Whether to model interaction between two targets

    - Details on Doppler modelling

  - Background channel

    - Remaining details on background channel for bistatic/monostatic sensing modes

  - Combining target channel and background channel

    - With or without power normalization

  - Further details on spatial consistency

  - Calibration of the ISAC channel model

Any additional study beyond channel modelling can only be possible after completing the ISAC channel modelling. Given the open issues above and the progress so far, we can only use the remaining time during Rel-19 SI for specifying every details of the ISAC channel modelling. In this regard, we do not think we can start any additional study in Rel.-19.

In addition, we did not finalize yet the UAV deployment scenario, which is just one of the five deployment scenarios required by ISAC SID. As any details of the remaining four deployment scenarios were not discussed yet, we do not even expect whether and what issues there will be regarding those four remaining scenarios.

Considering all these aspects, we do not think that including additional study beyond channel modelling for ISAC is feasible in Rel-19.

Proposal 1: Do not include additional study beyond channel modelling for ISAC in Rel-19.

- Conclusion

In this contribution, we discussed the checkpoint in Rel-19 ISAC SID that requires plenary guidance in the upcoming plenary meeting. The following proposal is given:

Proposal 1: Do not include additional study beyond channel modelling for ISAC in Rel-19.

- Reference

[1] RP- RP-240799, “Revised SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR,” Xiaomi, AT&T