3GPP TSG RAN Meeting #105								              RP-241822

Melbourne, Australia, September 9-12, 2024

Status Report to TSG

Agenda item:			9.3.1.4

| WI / SI Name | Low-power wake-up signal and receiver for NR (LP-WUS/WUR) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_LPWUS |  |  |  |  |
| Unique ID | 1020094 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-240801 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2025 | Performance part: 03/2026 | Testing part: N/A |  |
| Overall Completion level | Study Item: n/a | Core part: 30% | Performance Part: 0% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN 1 |
| --- | --- | --- |
| Rapporteur (primary) | Name | Xueming Pan |
|  | Company | vivo |
|  | Email | panxueming@vivo.com |
| Rapporteur | Name | Shinya Kumagai |
|  | Company | NTT DOCOMO |
|  | Email | shinya.kumagai.yw@nttdocomo.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

RAN1#118: LP-WUS and LP-SS design

Agreement

For RRC idle/inactive state, support the following option for at least indicating subgroup information using LP-WUS:

- Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs

  - UE monitoring one or more MOs (up to X MOs) for the same beam within an LO is supported

    - Value of X is larger than 1. FFS on additional details of X.

Agreement

[H][FL1] Proposal 3.1-1: Confirm the Working Assumption that OOK-4 with M=4 for 15KHz SCS is supported for LP-WUS.

Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - M=4 for 15KHz SCS - M=4 for 30KHz SCS (working assumption) - FFS M=1 for OOK-4

Agreement

Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs (for non-CA case), select at least one from the following

- Option 1: A bitmap with each bit corresponding to [one or more] UEs

- Option 3: A codepoint value corresponding to [one or more] UEs

- FFS details for extension of option 1 and/or 3 when UE is configured with CA

Agreement

For overlaid OFDM sequences for LP-WUS, support option 1-1 for OOK-4 M>1.

- Option 1-1: overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing

Agreement

For overlaid OFDM sequences for LP-WUS, further down-selection between following two options for OOK-1 and OOK-4 with M=1(if supported)

- Option 1-1: Overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing

- Option 2: Overlaid sequence(s) are the sequence(s) of an OFDM symbol before IFFT processing

- Note: Different options for OOK-1 and OOK-4 with M=1 (if supported) is not precluded – in which case, it should be deemed necessary

Agreement

The number of binary LP-SS sequences is 4.

Agreement

To determine the binary sequences for LP-SS, the evaluation assumes the following:

- Sync accuracy: timing estimation error smaller than T us for P=90 % of the time for at least SNR=-3dB and SNR=-6dB (lower priority),

  - T=2us(optional), 5us for OOK-1

  - T=1us(optional), 2us for OOK-4 with M=2

  - T=0.5us(optional), 1us for OOK-4 with M=4

  - Other values of SNR, T, M are up to company report.

  - Additionally, companies can submit results on SINR with details on how the interference was modelled.

  - Assume one-shot estimation

  - The time error is based on 20ppm maximum frequency error for the detection of the first LP-SS

  - Note: Companies can assume other values inside of 20ppm maximum as long as the values are justified

- RRM measurement accuracy: measurement accuracy within range ± XdB for Q=90% measurements based on Y LP-SS samples within a period comparable to Z=the length of I-DRX cycle that is larger or equal to 1.28s for at least SNR=-3dB and SNR=-6dB (lower priority), X, Y, and Z is up to company report, other values of SNR are up to company report.

  - As a starting point, the time error is based on 5-10ppm residual frequency error.

  - Note: 5-10ppm assumes frequency error correction is performed, e.g., RTC calibration and/or MR assistance

  - Note: Companies can assume other values outside of 5-10ppm as long as the values are justified

- The frequency error: up to company report

- Sampling rate: 3.84MHz (optional) or 7.68MHz assumed for 30kHz SCS.

- Channel: AWGN and TDL-C 300ns is assumed.

- Consider cross-correlation for 4 binary sequences under time and frequency error

- Companies are encouraged to provide details on other additional simulation assumptions, high level description of their receiver algorithm, and assessment on power consumption

Agreement

For OOK-based LP-WUR, the LP-WUS design assumes the following:

- As starting point, the time error is based on the 5-10ppm residual frequency error after frequency error correction without considering impact of drift.

- The frequency error: up to company report.

- Note: 5-10ppm assumes frequency error correction is performed, e.g., RTC calibration and/or MR assistance

- Note: Companies can assume other values outside of 5-10ppm as long as the values are justified

Agreement

As a starting point, for both time error and frequency error, the overlaid OFDM sequence design of LP-WUS assumes that the residual frequency error is 0.1-5ppm for OFDM-based LP-WUR after frequency error correction without considering impact of drift.

- Note: Companies can use any value within the above range with justification on the value (including impact on power consumption)

Agreement

Support Manchester coding for LP-WUS

- FFS other coding schemes

RAN1#118: LP-WUS operation in IDLE/INACTIVE modes

Agreement

The definitions of LP-RSRP and LP-RSSI for LP-RSRQ are updated as follows:

LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols over the frequency resources defined by the number of REs that carry LP-SS.

LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols over the frequency resources defined by the number of REs that carry LP-SS.

Agreement

At least support 1:1 association between LP-WUS MO(s)/LP-SS transmissions and SSB beams.

Agreement

For the wake-up delay, consider the following options:

Option 2: UE reports one value from X candidate values for the wake-up delay via UE capability reporting.

- FFS: X is 2, 3, 4

Above applies for IDLE/INACTIVE mode.

Definition of wake-up delay: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring

Agreement

For the maximum number of subgroups per PO (X), down-select from the following options in RAN1#118bis:

- Option 1: X = 8

- Option 2: X = 16

- Option 3: X = 32

- Option 4: X = 64

- Option 5: X = 128

- Option 6: X = 256

For decision in RAN1#118bis, companies are encouraged to provide the maximum number of information bits per LP-WUS (Z), the number of OFDM symbols occupied by LP-WUS per MO, the number of MOs for their preferred option.

Conclusion

There is no consensus in RAN1 on the support of dynamic PO.

Agreement

On the LO configuration for iDRX, offset value(s) between a LO and a reference PO/PF is/are configured.

- FFS: one or multiple offset values, and if multiple offset values are supported, how a UE decides which value to use

- FFS: the exact definition of the reference PF/PO and the detailed procedure for UE to determine the LO(s) corresponding to Option 1, 2, or 3 (if supported)

- (Working Assumption) For each UE, the periodicity of LO is the same as its iDRX cycle.

- If a UE receives a wake-up indication in a LP-WUS, consider the following alternatives

  - Alt 1: it monitors the PO associated with the offset.

  - Alt 2: it monitors the first PO after its reported wake-up delay.

  - Other alternatives are not precluded

Note: The PO mentioned above refers to legacy PO configured for the UE.

Agreement

Send an LS to RAN2 and RAN4 to convey the following

In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. Request RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

Final LS in R1-2407559.

Conclusion

RAN1 will not initiate work on entry/exit conditions based on RRM measurement and RRM measurement offloading/relaxation conditions unless triggered by RAN2 and RAN4

RAN1#118: LP-WUS operation in CONNECTED modes

Agreement

For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective.

- LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

- UE is configured with legacy C-DRX configurations as Rel-18

- UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration)

  - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration

- LP-WUS triggers the start of a timer during which UE monitors PDCCH

  - FFS the timer is existing timer and/or new timer

- UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected

  - drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL

- No impact on RRM/RLM/BFD measurement requirements is assumed

- For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1)

  - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up

  - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not

- UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

Working Assumption

From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration

- Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

- Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

- FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE

Note: Above can be revisited considering RAN2 decisions.

Agreement

Select one of the following alternatives in RAN1#118bis

Alt 1: For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting.

  - FFS: X

  - FFS: definition of reported value

Alt2: For RRC CONNECTED mode, UE reports multiple values for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting.

  - FFS: X

  - FFS: definition of reported value

  - Different minimum time gaps correspond to different sleep states

  - Companies are encouraged to details on how the reported values are to be used by the network

Agreement

LP-WUS is at least supported for the case where a UE is configured with CA in RRC CONNECTED mode

- FFS: DC

Agreement

For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS

- FFS applicable QCL type(s)

2.1.2	Remaining Open issues

- Remaining details of an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes

  - Remaining details of OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol

- For IDLE/INACTIVE modes

  - Remaining details of procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring

  - Remaining details of LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell.

    - Remaining details of LP-SS based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences.

- For CONNECTED mode, remaining details of procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring

2.2	RAN2

2.2.1	Agreements

RAN2#127: Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

Baseline: The network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE

Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS.

Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ.

The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

RAN2#127: RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs.

For serving cell measurement offloading (i.e., there is no serving cell measurement by MR):

    - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

    - The exit condition is based on the LR measurement results.

RAN2#127: Procedures for LP-WUS in RRC_CONNECTED

For option 1-2,

    - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.)

    - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

2.2.2	Remaining Open issues

- For IDLE/INACTIVE modes

  - Remaining details of procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring

  - Remaining details of RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions

- For CONNECTED mode, remaining details of procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring

2.3	RAN3

2.3.1	Agreements

New IEs should be introduced to support LP-WUS on NG, Xn and F1.

2.3.2	Remaining Open issues

- For IDLE/INACTIVE modes

  - Remaining details of procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring

2.4	RAN4

2.4.1	Agreements

RAN4#112 RRM core requirements for LP-WUS/WUR

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

Agreement:

- Support case #3

  - For serving cell measurement, further discuss:

    - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR

    - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR

    - Option 3: It is up to UE implementation whether to combine the measurement across the two radios.

    - Other options are not precluded.

| RRM measurement case index | MR serving cell measurement | MR neighbouring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

  - Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility.

RAN4#112 Simulation assumptions and results

Issue 2-1-1: SINR setting

Agreement:

  - As starting point for RAN4 RRM simulation purpose only, -3dB, 0.5dB, 2dB dB Ês/Iot value is used for serving cell for both OOK based and OFDM based LP-WUR.

    - The SINR value can be updated if any update on the noise figure value in RF session.

  - FFS the side condition for RAN4 requirement.

Issue 2-1-1-1: SNR setting for serving and interference cell derivation from SINR setting

Agreement:

- Consider two cells for RAN4 simulation: one serving cell and one interference cell.

Agreement:

- To derive SNR for serving cell and interference cell from serving cell Ês/Iot, a relationship for the SNR or transmission power between serving cell and interference cell need be pre-defined.

- Consider the SNR/transmission power of the interference cell is 9 dB or 6 dB lower compared with that of the serving cell. Other value can be considered.

Issue 2-1-3: Time/frequency error

Agreement:

Residual frequency error:

OFDM based receiver [5] ppm

OOK based receiver [ 10 20] ppm

Issue 2-1-5: Measurement interval

Agreement:

Use 320ms for SSB based LP-WUR firstly.

Other agreements for simulation assumptions

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | Agreement: 2.6 GHz initially |
| Subcarrier spacing | 30KHz initially TBD for 15 KHz |
| LP-SS/SSB measurement interval | LP-SS: 320 ms SSB: 320 ms |
| LP-SS BW | 132 subcarriers for SCS=30kHz for LP-SS initially TBD for 15KHz SCS |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS, NR-SSS (OFDM based LP-WUR) | - | To be indicated by companies | To be indicated by companies |
| LP-SS |  | OOK-1; or OOK-4 with M = [2,4] Note: M value [2 4] are up to company selection | when Cell 1 uses OOK-1; OOK-1 or NR signal is used for Cell 2 when Cell 1 uses OOK-4, OOK-4 with same M value as cell 1 or NR signal is used for Cell 2 |
| LP-SS pattern |  | [M sequence] [Golden sequence] [Computer search sequence] Note: Company can simulate one or all of them | [M sequence] [Golden sequence] [Computer search sequence] Note: Company can simulate one or all of them |
| Frequency offset relative to UE frequency reference | Hz | OFDM based receiver [5] ppm OOK based receiver [ 10 20] ppm | N/A |
| SNR | dB | SNR setting for serving and interference cell are derived based on agreement of Issue 2-1-1-1 |  |
|  |  | When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-2.7] - When SNR of cell 2 is 6 dB lower compared with cell 2 SNR = [-2.4] | When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-11.7] - When SNR of cell 2 is 6 dB lower compared with cell 2; SNR = [-8.4] |
|  |  | When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 | When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 |
|  |  | When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 | When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 |
| Ês/Iot | dB | -3; -0.5dB; 2dB | N/A |

RAN4#112: LP-WUS UE RF

Sub-topic 1-1 General

Issue 1-1-1: New RAN4 TR to capture simulation and analysis of LP-WUS receiver and requirements

Agreement:

- RAN4 suggest having a new RAN4 TR to capture the simulations and analysis for LP-WUS RF.

Issue 1-1-2: Performance metric for Rx RF requirements

Agreement:

- 1% MDR for evaluation [and core requirements]

- FFS on the testability issue.

Issue 1-1-3: Performance metric for Demodulation requirements

Agreement:

- [1% or 0.1%] FAR as assumption for MDR result calibration.

Issue 1-1-6: Specify FR2 LP-WUS RF requirements

Agreements:

- Wait to RAN1 progress on FR2 LP-WUS work.

Sub-topic 1-2 System parameters

Issue 1-2-1: Channel raster for LP-WUR

Agreements:

  - RAN4 can further discuss and align understandings on whether new channel raster is needed for LP-WUS.

  - RAN1 and RAN2 status on how to indicate the centre operation frequency of LP-WUS will be helpful.

Issue 1-2-3: number of RBs for LP-WUS with 15kHz SCS

Agreements:

  - Leave RAN1 to make decision.

  - RAN4 can discuss assumption of RBs under 15kHz SCS for simulation

Issue 1-2-5: Whether need to define CBW for LP-WUR?

Agreements:

  - No need to define BW for LP-WUR

Sub-topic 2-1 SNR simulation and values

Issue 2-1-1: Channel model to specify LP-WUS RF requirements

Agreement:

  - Follow typical RF requirements approach, AWGN should be selected.

Issue 2-1-2: Target SNR simulation condition

Agreement:

  - Follow similar approach of MR NR, no repetition should be used when simulate target SNR.

Issue 2-1-3: Whether RAN4 should conclude a target SNR first

Agreement:

- Target SNR is the basis for many Rx requirements discussion. Group should conclude SNR definition and SNR values first.

Issue 2-1-4: Target SNR value

Agreements:

  - RAN4 further discuss target SNR value for LP-WUS

  - FFS single SNR for OOK-based and OFDM-based receiver

Sub-topic 2-2 NF and REFSENS requirements

Issue 2-2-2: Baseline architecture for OOK-based LP-WUS

Agreement:

  - use zero-IF receiver as a baseline architecture for envelop based LP_WUR

Issue 2-2-3: Baseline architecture for OFDM-based LP-WUS

Agreements:

  - Wait for RAN1’s conclusion on the OFDM-based LP-WUR.

Issue 2-2-4: whether different NF for OOK-based and OFDM-based

Agreement:

  - Differentiate NF for OOK-based and OFDM-based receiver.

Issue 2-2-9: MDR value for REFSENS test case

Agreements:

  - FFS the MDR for conformance testing should be aligned with the value for RF requirement.

Sub-topic 2-3 ASCS simulation and requirements

Issue 2-3-1: ASCS requirements value

Agreements:

  - RAN4 further discuss how to specify ASCS requirement based on simulation outcome.

Issue 2-3-2: Required number of guard RB for ASCS

Agreements:

  - RAN4 further discuss the required number of guard RBs for ASCS requirement based on simulation outcome.

Issue 2-3-3: Test parameters for ASCS

Agreements:

  - RAN4 further discuss the test parameters for ASCS requirement.

Sub-topic 2-4 ACS simulation and requirements

Issue 2-4-2: ACS requirements value

Agreements:

  - RAN4 further discuss how to specify ACS requirements based on simulation outcome.

  - FFS same as MR, or further relaxed

Issue 2-4-3: Required guard RB for ACS requirements

Agreements:

  - RAN4 further discuss the number of guard RBs for ACS requirement based on simulation outcome.

Issue 2-4-4: Test parameters for LP-WUR ACS case

Agreements:

  - RAN4 further discuss the test parameters for ACS requirement.

Sub-topic 4-1 Testability for UE RF requirements

Issue 4-1-1: General framework on LP-WUS testing

Agreements:

  - RAN4 further discuss the high-level approach on how to achieve LP-WUS performance verification.

RAN4#112: LP-WUS BS RF

Issue 1-1: Manufacture declaration on LP-WUS power boosting

- WF

  - FFS on following options:

    - Option 1: Set the LP-WUS power boosting a complete manufacture declaration feature, including whether supporting LP-WUS power boosting and the supported boosting level.

    - Option 2: Minimum power boosting level in core specification together with manufacturer declaration in the conformance test specification.

Issue 1-2: Concept of LP-WUS power boosting

- WF

  - FFS on following options:

    - Introduce EPRE ratio to define LP-WUS power boosting, since it has the merit that power boosting does not vary with different CBWs.

      - Option 1: The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power NR REs (the NR carrier excluding the LP-WUS REs).

      - Option 2: The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of an LP-WUS RE (averaged power per RE based on all LP-WUS REs within a NR transmission bandwidth configuration) and the average power of a non-LP-WUS RE (averaged power from all REs from the NR carrier excluding the LP-WUS REs).

    - Adopt following definition rather than introduce EPRE ratio.

      - Option 3: The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs).

Issue 1-3: Whether to preclude small CBW for consideration of LP-WUS power boosting

- WF

  - FFS on following options:

    - Option 1: Preclusion on small CBW respective to different power boosting level and/or power degradation on NR signal can be considered for LP-WUS power boosting.

      - Option 1-1: Also introduce the maximum allowed power degradation as one of the core requirements for LP-WUS power boosting.

      - Option 1-2: Operator inputs on the tolerable power degradation can be considered for the determination on the CBW to be precluded.

      - Option 1-3: other option does not excluded.

    - Option 2: No need to preclude any CBW, only if the LP-WUS power boosting can be defined as dynamic range where the level is up to manufacturer declaration and 0dB as minimum core requirement.

Issue 1-4: On applicable BS type for LP-WUS

- WF

  - FFS whether all available BS types can be considered for LP-WUS due to the support of LP-WUS is optional and the power boosting level could be up to manufacturer declaration.

  - FFS whether only conductive based BS RF requirements should be defined for LP-WUS:

Issue 1-5: On minimum value for LP-WUS power boosting

- WF

  - FFS on the following options:

    - Option 1: If power boosting will be defined as EPRE ratio, use 3dB as minimum requirement.

    - Option 2: If power boosting will be defined as dynamic range, based on existing specification for OOK-1 waveform, the core requirement for the dynamic range should be limited to 3dB.

    - Option 3: Not to introduce minimum value for LP-WUS power boosting.

Issue 1-6: Whether a cap for LP-WUS power boosting should be considered

- WF

  - FFS on the following options:

    - Option 1: It should be considered as 3dB.

    - Option 2: It should be considered as 6dB

    - Option 3: Not to introduce maximum value for LP-WUS power boosting.

Issue 1-7: Unwanted emissions

- WF

  - The legacy BS unwanted emission requirements should apply to LP-WUS.

Issue 1-8: Transmitted signal quality

- WF

  - Option 1: Legacy transmitted signal quality requirements apply, no impact when BS transmitting LP-WUS signal.

  - Option 2: The OOK-based EVM should be defined for LP-WUS. FFS on how to specify this requirement with consideration of RAN1 waveform design.

Issue 1-9: Multi-band operation

- WF

  - FFS on multi-band operation.

2.4.2	Remaining Open issues

- RRM：For IDLE/INACTIVE modes

  - Remaining details of procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring.

  - Remaining details of LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell.

  - Remaining details of requirements for further RRM relaxation of UE MR for both serving and neighbor cell measurements, and requirements for UE serving cell RRM measurement offloaded from MR to LP-WUR.

- RF：

  - Remaining details of UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

- Demod：

  - Not started

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

### RAN1#118

- R1-2405806	Discussion on LP-WUS and LP-SS Design	FUTUREWEI

- R1-2405867	Signal Design of LP-WUS and LP-SS	Huawei, HiSilicon

- R1-2405919	Discussion on LP-WUS and LP-SS design	Spreadtrum Communications

- R1-2405966	Preliminary Assessment on Low-Power Wake-Up Receiver	Tejas Networks Limited

- R1-2405996	Discussion on LP-WUS and LP-SS design	CMCC

- R1-2406083	Discussion on LP-WUS and LP-SS Design	EURECOM

- R1-2406104	Discussion on LP-WUS and LP-SS Design	TCL

- R1-2406193	Discussion on LP-WUS and LP-SS design	vivo

- R1-2406222	Signal design for LP-WUS and LP-SS	OPPO

- R1-2406295	Discussion on LP-WUS and LP-SS design	Xiaomi

- R1-2406379	Design of LP-WUS and LP-SS	CATT

- R1-2406412	Discussion on LP-WUS design	ZTE Corporation, Sanechips

- R1-2406422	LP-WUS and LP-SS design	Nokia

- R1-2406480	LP-WUS and LP-SS design	Sony

- R1-2406498	Discussion on LP-WUS and LP-SS design framework for Low power WUS	InterDigital, Inc.

- R1-2406504	Discussion on LP-WUS and LP-SS design	Everactive

- R1-2406537	Discussion on LP-WUS and LP-SS design	NEC

- R1-2406583	Discussion on LP-WUS and LP-SS design	HONOR

- R1-2406597	Discussions on LP-WUS and LP-SS design	Ruijie Networks Co. Ltd

- R1-2406611	Discussion on LP-WUS and LP-SS design	LG Electronics

- R1-2406661	Discussion on LP-WUS and LP-SS design	Samsung

- R1-2406762	LP-WUS and LP-SS design	MediaTek Inc.

- R1-2406785	Discussion on the LP-WUS and LP-SS design	Panasonic

- R1-2406814	Discussion on LP-WUS and LP-SS design	Lenovo

- R1-2406850	LP-WUS and LP-SS design	Apple

- R1-2406881	Discussion on LP-WUS and LP-SS design	Sharp

- R1-2406941	Discussion on LP-WUS and LP-SS design	NTT DOCOMO, INC.

- R1-2407040	LP-WUS and LP-SS design	Qualcomm Incorporated

- R1-2407059	LP-WUS and LP-SS design	Ericsson

- R1-2407136	On LP-WUS and LP-SS design	Nordic Semiconductor ASA

- R1-2407287	Summary #1 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2407356	Summary #2 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2407420	Summary #3 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2407491	Summary #4 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2405807	Discussion on LP-WUS Operation in IDLE/INACTIVE Modes	FUTUREWEI

- R1-2405868	Procedures and functionalities of LP-WUS in IDLE/INACTIVE mode	Huawei, HiSilicon

- R1-2405920	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Spreadtrum Communications

- R1-2405997	Discussion on LP-WUS operation in IDLE/INACTIVE modes	CMCC

- R1-2406105	Discussion on LP-WUS Operation in IDLE/INACTIVE modes	TCL

- R1-2406194	Discussion on LP-WUS operation in IDLE/INACTIVE modes	vivo

- R1-2406223	Further consideration on LP-WUS operation in RRC_IDLE/INACTIVE modes	OPPO

- R1-2406296	Discussion on LP-WUS operation in Idle/Inactive modes	Xiaomi

- R1-2406380	System design and procedure of LP-WUS operation for UE in IDLE/Inactive Modes	CATT

- R1-2406413	Discussion on LP-WUS operation in IDLE/INACTIVE mode	ZTE Corporation, Sanechips

- R1-2406423	LP-WUS operation in IDLE/INACTIVE mode	Nokia

- R1-2406481	LP-WUS operation in IDLE / INACTIVE modes	Sony

- R1-2406499	Discussion on LP-WUS operation in IDLE/INACTIVE modes	InterDigital, Inc.

- R1-2406519	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Fujitsu

- R1-2406538	Discussion on LP-WUS operation in RRC IDLE/INACTIVE mode	NEC

- R1-2406567	Discussion on LP-WUS operation in IDLE/INACTIVE modes	KT Corp.

- R1-2406612	Discussion on LP-WUS operation in IDLE/INACTIVE modes	LG Electronics

- R1-2406662	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Samsung

- R1-2406735	Discussion on LP-WUS operation in IDLE/INACTIVE modes	ETRI

- R1-2406763	LP-WUS operation in IDLE INACTIVE modes	MediaTek Inc.

- R1-2406786	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Panasonic

- R1-2406815	Discussion on LP-WUS operation in Idle/Inactive modes	Lenovo

- R1-2406851	LP-WUS operation in IDLE/INACTIVE modes	Apple

- R1-2406853	Summary #1 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2406854	Summary #2 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2406855	Summary #3 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2407506	Summary #4 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2405869	Procedures and functionalities of LP-WUS in CONNECTED mode	Huawei, HiSilicon

- R1-2405921	Discussion on LP-WUS operation in CONNECTED modes	Spreadtrum Communications

- R1-2405998	Discussion on LP-WUS operation in CONNECTED mode	CMCC

- R1-2406195	Discussion on LP-WUS operation in CONNECTED modes	vivo

- R1-2406224	Further consideration on LP-WUS operation in connected mode	OPPO

- R1-2406297	Discussion on LP-WUS operation in Connected mode	Xiaomi

- R1-2406381	System design and procedure of LP-WUS operation for UE in CONNECTED Modes	CATT

- R1-2406390	Discussion on LP-WUS operation in CONNECTED mode	Panasonic

- R1-2406414	Discussion on LP-WUS operation in CONNECTED mode	ZTE Corporation, Sanechips

- R1-2406424	LP-WUS operation in CONNECTED mode	Nokia

- R1-2406482	LP-WUS operation in CONNECTED mode	Sony

- R1-2406500	Discussion on RRC CONNECTED mode LP-WUS monitoring	InterDigital, Inc.

- R1-2406510	Discussion on LP-WUS operation in CONNECTED modes	Lenovo

- R1-2406539	Discussion on LP-WUS operation in RRC CONNECTED mode	NEC

- R1-2406613	Discussion on LP-WUS operation in CONNECTED modes	LG Electronics

- R1-2406663	Discussion on LP-WUS operation in CONNECTED modes	Samsung

- R1-2406736	Discussion on LP-WUS operation in CONNECTED modes	ETRI

- R1-2406764	LP-WUS operation in CONNECTED modes	MediaTek Inc.

- R1-2406852	LP-WUS operation in CONNECTED modes	Apple

- R1-2406883	Discussion on LP-WUS operation in CONNECTED modes	Sharp

- R1-2406943	Discussion on LP-WUS operation in CONNECTED mode	NTT DOCOMO, INC.

- R1-2407042	LP-WUR operation in connected mode	Qualcomm Incorporated

- R1-2407061	LP-WUS operation in CONNECTED mode	Ericsson

- R1-2407104	Discussion on LP-WUS procedures in Connected mode	TCL  Late submission

- R1-2407301	FL summary #1 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

- R1-2407378	FL summary #2 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

- R1-2407516	FL summary #3 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

### RAN2#127

- R2-2406787	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Huawei, HiSilicon

- R2-2407240	Discussion on LP-WUS operation in RRC_IDLE/INACTIVE modes	InterDigital, Inc.

- R2-2407156	LP-WUS operation in RRC_IDLE and RRC_INACTIVE	LG Electronics Inc.

- R2-2407096	LP-WUS operation in IDLE/Inactive state	 Qualcomm Incorporated

- R2-2407396	LP-WUS in Idle and Inactive	Ericsson

- R2-2407013	LP-WUS in IDLE and INACTIVE	Nokia

- R2-2406730	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Apple

- R2-2406427	Discussion on LP-WUS WUR in RRC_IDLE INACTIVE	vivo

- R2-2407543	Discussion on Procedure and configuration in RRC_IDLE-INACTIVE	NTT DOCOMO INC..

- R2-2406772	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	OPPO

- R2-2406447	Procedure and configuration of LP-WUS for IDLE and INACTIVE mode ZTE Corporation, Sanechips

- R2-2406495	LP-WUS procedure in RRC_IDLE INACTIVE 	NEC

- R2-2406575	LP-WUS in RRC_IDLE/INACTIVE	CATT

- R2-2406585	General considerations on the procedure for RRC_IDLE_INACTIVE	Xiaomi Communications

- R2-2406617	RAN2 aspects on LP-WUS/WUR in RRC Idle/Inactive mode	Sony

- R2-2406753	Discussion on LP-WUS operation in IDLE/INACTIVE mode	Spreadtrum Communications

- R2-2406802	Discussion on entry exit conditions for LP-WUS monitoring	Sharp

- R2-2406900	LP-WUS Operation in RRC_IDLE/INACTIVE	China Telecom

- R2-2406985	LP-WUS operation in IDLE/INACTIVE modes	CMCC

- R2-2407127	Procedure and Configuration of LP-WUS in RRC Idle/ Inactive	Lenovo

- R2-2407310	Procedure and Configuration of LP-WUS in RRC Idle Inactive Mode	Samsung

- R2-2407357	Procedure of LP-WUS in RRC_IDLE and INACTIVE	HONOR

- R2-2406496	LP-WUS RRM measurement 	NEC

- R2-2406754	Discussion on RRM measurement relaxation and offloading in IDLE/INACTIVE mode	Spreadtrum Communications

- R2-2406803	Discussion on RRM measurement offloading and relaxation	Sharp

- R2-2406970	Discussion on RRM measurement relaxation and offloading in RRC_IDLE INACTIVE	CMCC

- R2-2406285	RRM measurement relaxation and offloading in RRC_IDLE or RRC_INACTIVE	Huawei, HiSilicon

- R2-2406428	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	vivo

- R2-2406448	RRM measurement relaxation and offloading in RRC_IDLE and RRC_INACTIVE mode	ZTE Corporation, Sanechips

- R2-2406576	RRM Relaxation and Offloading in RRC_IDLE/INACTIVE	CATT

- R2-2406586	Discussion on RRM measurement relaxation for RRC_IDLE_INACTIVE	Xiaomi Communications

- R2-2406618	Discussion on RRM aspects for  LP-WUS/WUR	Sony

- R2-2406731	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Apple

- R2-2406739	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE mode	China Telecom

- R2-2406767	Discussion on RRM measurement in RRC IDLE and INACTIVE	OPPO

- R2-2406882	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Lenovo

- R2-2407014	RRM measurement relaxation in RRC_IDLE/INACTIVE	Nokia

- R2-2407098	LP-WUS RRM measurement relaxation and offloading	Qualcomm Incorporated

- R2-2407157	RRM relaxation and RRM offloading	LG Electronics Inc.

- R2-2407241	Discussion on RRM measurement relaxation and offloading	InterDigital, Inc.

- R2-2407311	RRM measurement relaxation and offloading in RRC Idle Inactive Mode	Samsung

- R2-2407397	LP-WUS and RRM measurements	Ericsson

- R2-2406587	Discussing on LP-WUS monitoring for RRC_Connected	Xiaomi Communications

- R2-2406449	Procedures for LP-WUS in RRC_CONNECTED	ZTE Corporation, Sanechips

- R2-2406577	Analysis on LP-WUS for RRC_CONNECTED Mode	CATT

- R2-2406429	Discussion on LP-WUS WUR in RRC_Connected	vivo

- R2-2406497	LP-WUS procedure in RRC_CONNECTED 	NEC

- R2-2406619	Considerations on LP-WUS/WUR in RRC Connected mode	Sony

- R2-2406717	Discussion on LP-WUS for RRC_CONNECTED mode	Huawei, HiSilicon

- R2-2406732	Procedures for LP-WUS in RRC_CONNECTED	Apple

- R2-2406768	Discussion on LP-WUS in RRC_CONNECTED	OPPO

- R2-2406901	Discussion on LP-WUS in RRC_CONNECTED	China Telecom

- R2-2406978	Discussion on LP-WUS operation in CONNECTED mode	CMCC

- R2-2407097	LP-WUS operation in CONNECTED state	Qualcomm Incorporated

- R2-2407134	LP-WUS in RRC Connected Mode	Lenovo

- R2-2407242	Discussion on LP-WUS operation in RRC_CONNECTED mode	InterDigital, Inc.

- R2-2407286	Discussion on Procedures for UE MR PDCCH monitoring triggered by LP-WUS in RRC_CONNECTED Mode	LG Electronics Inc.

- R2-2407312	Procedures for LP-WUS in RRC Connected Mode	Samsung

- R2-2407358	Discussion on LP-WUS in RRC_CONNECTED	HONOR

- R2-2407398	LP-WUS in Connected	Ericsson

- R2-2407406	LP-WUS in RRC_CONNECTED	Noki

- R2-2407512	Discussion on LP-WUS in RRC_CONNECTED	NTT DOCOMO INC..

### RAN3#125

- R3-244069	Work plan for Rel-19 WI on LP-WUSWUR (vivo)

- R3-244070	Discussion on signaling design for LP-WUS subgrouping (vivo, NTT DOCOMO INC.)

- R3-244513	Discussion on potential RAN3 impacts of LP-WUS (CATT)

- R3-244424	Overview of RAN3 impacts of LP-WUS (Nokia )

- R3-244095	(TP to TS 38.413, 38.473, 38.423) Discussion on LP-WUS subgrouping (ZTE Corporation)

- R3-244605	Discussion on LP-WUS subgrouping (Samsung)

- R3-244409	Discussion of LPWUS impacts on NG-RAN architecture (Ericsson)

- R3-244076	(TP for TS 38.413 and TS 38.423) Discussion on Low-power wake-up signal and receiver for NR (Huawei)

- R3-244096	[draft] LS on LP-WUS subgrouping (ZTE Corporation)

- R3-244147	Support LP-WUS subgrouping (Qualcomm Inc.)

- R3-244199	Initial discussion on LP-WUS subgrouping (NTT DOCOMO INC..)

- R3-244425	Support of LP-WUS (Nokia )

- R3-244514	(TP on BLCR for TS 38.300)Support of LP-WUS (CATT)

- R3-244626	BLCR for TS 38.300 on support of LP-WUS (vivo)

- R3-244410	Introduction of LP-WUS paging monitoring (Ericsson)

- R3-244077	Introduction of low-power wake-up signal and receiver for NR (Huawei)

- R3-244606	(TP to BLCR for TS 38.413, 38.423, 38.473) LP-WUS Assistance Information to support subgrouping (Samsung)

- R3-244611	Discussion on support LP-WUS indicating paging monitoring (CMCC)

### RAN4#112

- R4-2411094	Further discussion on BS RF requirements for LP-WUS	CATT

- R4-2411095	Further discussion on system parameters for LP-WUS	CATT

- R4-2411227	Further consideration on general aspects for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2411228	Further consideration on UE RF REFSENS, ACS, ASCS requirements for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2411229	Further consideration on UE RF other Rx requirements for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2411230	Further consideration on UE RF testability issue for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2411231	Further consideration on BS RF for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2411361	Discussion on simulation assumptions for LP-WUR measurement	CATT

- R4-2411362	Discussion on RRM requirements for LP-WUS/WUR	CATT

- R4-2411449	On simulation assumption for LP-WUR based measurement	Apple

- R4-2411450	On RRM core requirements for LP-WUR	Apple

- R4-2411493	Discussion on core requirements for LP-WUS WUR	OPPO

- R4-2411494	Discussion on RX requirements of REFSENS for LP-WUR	Spreadtrum Communications

- R4-2411495	Discussion on systems parameters for LP-WUR	Spreadtrum Communications

- R4-2411538	views on requirements of the low-power wake-up receiver	Sony

- R4-2411616	Simulation assumptions and results for LP-WUS/WUR	Xiaomi

- R4-2411617	Discussion on RRM core requirements for LP-WUS/WUR	Xiaomi

- R4-2411645	On Low-power Wake-up Receiver for NR	Apple

- R4-2411653	System parameters for LP-WUR	Nokia

- R4-2411654	Fundamental RX requirements for LP-WUR	Nokia

- R4-2411655	Other RX requirements for LP-WUR	Nokia

- R4-2411656	Testability aspects of LP-WUR	Nokia

- R4-2411683	Discussions on RRM core requirements for LP-WUS/WUR	NTT DOCOMO, INC.

- R4-2411694	Discussion on UE RF requirements for LP-WUR	Samsung

- R4-2411730	(NR_LPWUS-Core) Discussion on LP-WUS UE system parameters requirements	CMCC

- R4-2411731	(NR_LPWUS-Core) Discussion on LP-WUS UE RF Rx requirements of REFSENS, ASCS and ACS	CMCC

- R4-2411732	(NR_LPWUS-Core) Discussion on LP-WUS UE RF Rx requirements of IBB, OBB, intermodulation, spurious emissions and others	CMCC

- R4-2411733	(NR_LPWUS-Core) Discussion on LP-WUS BS RF requirements	CMCC

- R4-2411762	(NR_LPWUS-Core) Discussion on RRM impact of LP-WUR	CMCC

- R4-2411818	Topic summary for [112][223] NR_LPWUS	Moderator (vivo)

- R4-2411894	Discussion on BS RF requirements for LP-WUS/WUR	ZTE Corporation, Sanechips

- R4-2411895	Discussion on system parameters for LP-WUS/WUR	ZTE Corporation, Sanechips

- R4-2411896	Discussion on REFSENS, ASCS, ACS for LP-WUR	ZTE Corporation, Sanechips

- R4-2411897	Discussion on receiver characteristics for LP-WUR	ZTE Corporation, Sanechips

- R4-2411898	Discussion on testability for UE RF requirements	ZTE Corporation, Sanechips

- R4-2412041	Discussion on RRM requirements for LP-WUS/WUR	LG Electronics Inc.

- R4-2412057	Discussion on LP-WUS general	vivo

- R4-2412058	Discussions on LP-WUS system parameters	vivo

- R4-2412059	Discussions on LP-WUS REFSENS, ASCS and ACS	vivo

- R4-2412061	Discussions on LP-WUS Testability	vivo

- R4-2412062	Discussions on LP-WUS BS RF requirements	vivo

- R4-2412121	Discussion on RRM core requirements for LP-WUS/WUR	China Telecom

- R4-2412275	Discussion on IBB and OBB requirements for LP-WUS	LG Electronics France

- R4-2412276	Discussion on REFSENS and ASCS for the LP-WUS/WUR	LG Electronics France

- R4-2412290	Consideration on simulation assumptions and results for LP-WUR	vivo

- R4-2412291	Consideration on RRM requirements for LP-WUR	vivo

- R4-2412441	Simulation assumptions and results for LP-WUS/WUR measurement	OPPO

- R4-2412505	Discussion on LP-WUS RRM requirement	Ericsson

- R4-2412506	Simulation on LP-WUS RRM measurement	Ericsson

- R4-2412531	RRM impact for Rel-19 LP-WUS/WUR	Samsung

- R4-2412595	BS RF requirements for low-power wake-up signal for NR	Nokia

- R4-2412669	Simulation assumption for LP-WUR measurement	Huawei, HiSilicon

- R4-2412670	Discussion on RRM requirements for LP-WUR	Huawei, HiSilicon

- R4-2412801	Discussion on LP-WUS RRM simulation assumptions and results	Nokia

- R4-2412802	Discussion on LP-WUS RRM core requirements	Nokia

- R4-2412835	Topic summary for [112][133] NR_LPWUS_UERF	Moderator(VIVO)

- R4-2412974	BS RF requirement overview for LP-WUS	Ericsson

- R4-2412975	On general issues for WUR	Ericsson

- R4-2412976	On system paramter for WUR	Ericsson

- R4-2412977	On WUR RF requirement testability	Ericsson

- R4-2412978	WUR RF requirement other than REFSENS	Ericsson

- R4-2412979	WUR RF requirement REFSESN ASC ASCS	Ericsson

- R4-2413041	Discussion on LP-WUS for core part	ZTECorporation,Sanechips

- R4-2413223	On UE Rx requirements for the LPWUR	Qualcomm Incorporated

- R4-2413324	Simulation assumptions for R19 LP-WUS	MediaTek inc.

- R4-2413325	Discussion on the RRM core requirements for LP-WUS	MediaTek inc.

- R4-2413409	Topic summary for [112][309] NR_LPWUS	Moderator (Huawei)

- R4-2413452	RRM requirements for LP-WUR	Qualcomm Incorporated

- R4-2413511	Draft LS to RAN1 on power boosting for LPWUS	Huawei

- R4-2413512	Way Forward for [112][309] NR_LPWUS	Huawei

- R4-2413898	Coffee break discussion minutes for [112][223] NR_LPWUS	Vivo

- R4-2413899	WF for RRM core requirements for LP-WUSWUR	Vivo

- R4-2414309	WF on LP-WUS UE RF requirements	Vivo

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template