3GPP TSG RAN Meeting #105								RP-241834

Melbourne, Australia, September 9-12, 2024

Agenda Item:	9.3.1.5

Source:	OPPO

Title:	Discussion on WID Scope for R19 network energy saving enhancement

Document for:	Discussion & Decision

Introduction

According to the work plan assigned by R19 NES WID,  it is at RAN#105 to check the study phase outcome for objective#2 about enhancement realizing on-demand SIB1, and RAN is expected to decide whether a normative work on on-demand SIB enhancement is supported in R19. In this paper, we provide our views and the reasoning on this topic.

Discussion

RAN plenary assigned WGs to conduct a study phase for the objective #2. In RAN1, the NES gains were evaluated according to different cases to check whether the on-demand SIB1 can provide effective energy saving for the network. The evaluated cases include

Case A: SSB period and SIB1 period are both set to 20 ms, which is also a case closer to the real deployment in the current market;

Case C: SSB period is 20 ms while SIB1 period is 160 ms, which reflects the maximum SIB period being supported by the current specification;

Case D: SSB period is 20 ms and SIB1 period is 40 ms.

After extensive simulations, RAN1 has drawn evaluation observations for these three cases c.f., Appendix. The NES gain for Case A is 39.37% - 47.69% for empty load, 30.56% - 41.8% for low load and 9.88% - 14.4% for medium load. This shows a significant NES gain compared with the most realistic deployment. Naturally, when Case C and Case D are considered, the NES gain decreases, for example with Case C, the NES gain is down to 2.21 – 10.6% for empty load , 1.2 – 8.3% for low load and 0.3 – 2.44% for medium load. After all, in all evaluated cases, the NES gain can be confirmed.

Observation 1: RAN1 confirmed the NES gain with on-demand SIB1.

On the other hand, WG1 and WG2 have also discussed the possible mechanisms for on-demand SIB1. Among them, the different mechanisms can be categorized as:

Case 1: UE obtains WUS configuration from NES cell, UE transmits WUS to NES cell and UE obtains SIB1 from NES cell.

Case 2: UE obtains WUS configuration from Cell A, UE transmits WUS to NES cell and UE obtains SIB1 from NES cell.

Case 3: UE obtains WUS configuration from Cell A, UE transmits WUS to Cell A and UE obtains SIB1 from Cell A.

By the end of the study phase, RAN1 has concluded that Case 2 and Case 3 are feasible but Case 3 is lower prioritized than Case 2; however, regarding Case 1, RAN1 concludes that it is only technically possible for R19. Therefore, RAN1 has recommended that normative work in R19 only focuses on Case 2, c.f., Appendix.

Observation 2: RAN1 has recommended a normative work in R19 focusing on Case 2.

With the above reasoning, we support RAN1’s recommendation and believe that on-demand SIB1 can bring effective energy saving benefits to network. Moreover, we suggest that R19 RRC_IDLE/RRC_INACTIVE UE as well as RRC_CONNECTED UE can all be considered when designing enhancement for on-demand SIB1. Finally, we suggest to avoid impact to the on-demand SIB1 non-capable UE including idle/inactive/connected modes.

Proposal 1: Support on-demand SIB1 normative work in R19 with the focus on Case 2

Proposal 2: Both RRC_IDLE/RRC_INACTIVE UE and RRC_CONNECTED UE are considered for this normative work.

Proposal 3: The enhancement should avoid impact to on-demand SIB1 non-capable UE in both RRC_IDLE/RRC_INACTIVE and RRC_CONNECTED state.

Conclusion

In this document, we discussed the outcome from the study phase for on-demand SIB1 and based on our analysis we provide our observations and proposal below:

Observation 1: RAN1 confirmed the NES gain with on-demand SIB1.

Observation 2: RAN1 has recommended a normative work in R19 to focusing on Case 2.

Proposal 1: Support on-demand SIB1 normative work in R19 with the focus on Case 2

Proposal 2: Both RRC_IDLE/RRC_INACTIVE UE and RRC_CONNECTED UE are considered for this normative work.

Proposal 3: The enhancement should avoid impact to on-demand SIB1 non-capable UE in both RRC_IDLE/RRC_INACTIVE and RRC_CONNECTED state.

Appendix

Agreements in RAN#117

Agreement

For SIB1 in idle/inactive mode, prioritize RAN1 discussions on Case 2 and Case 3

- Case 2 (Option 1+B+X) is feasible from RAN1 perspective.

Further study Case 3, focusing on the additional NES benefits over Case 2, feasibility, complexity, and spec impact.

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 20ms SIB1 period (Case A), FR1, empty load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 39.37% - 47.69% from the following 7 sources

    - R1-2404463, R1-2405595, R1-2405341, R1-2404859, R1-2404507, R1-2405363, R1-2404625

  - One source (R1-2405162) reports 25% NES gain with PRACH configuration index = 152

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 24.6% with one beam of SIB1 PDSCH transmitted in each slot

    - 17.4% with two beams of SIB1 PDSCH transmitted in each slot

  - One source (R1-2404122) reports the following with FDMed SSB and PDSCH of SIB1

    - 9.27%~18.94% NES gain with UL WUS configuration transmitted in legacy SIB on Cell A

    - 0%~9.66% NES gain with UL WUS configuration transmitted in separated SIB on Cell A. The separated SIB is TDMed with legacy SIB and transmitted in a 20ms period using dedicated resource.

    - 0% NES gain with Case 1 (Options 1+A+X) and WUS configuration transmitted in separate SIB on the NES cell

    - 13.25% NES gain with Case 1 (Options 1+A+X) and pre-configured for fixed WUS configuration

      - NES cell does not transmit the WUS configuration

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 22.1% - 42.6% from the following 5 sources

    - R1-2404408, R1-2405341, R1-2404758, R1-2405363, R1-2404625

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 16.8% with one beam of SIB1 PDSCH transmitted in each slot

    - 10.9% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 30.43% - 45.79% from the following 4 sources

    - R1-2404463, R1-2404859, R1-2405363, R1-2404625

  - One source (R1-2405162) reports the following NES gain with PRACH configuration index = 152

    - 25% NES gain with Case 3 (Options 2+B+Y) assuming the UE can camp on the NES cell and the SIB1 is only transmitted in a single beam corresponding to the UL WUS beam

    - 23% with Case 2 (Options 1+B+X)

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 21.79% - 33.11% from the following 2 sources

    - R1-2405363, R1-2404625

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 20.5% - 39.34% from the following 8 sources

    - R1-2404463, R1-2403942, R1-2405341, R1-2404224, R1-2404859, R1-2404507, R1-2404561, R1-2405595

  - One source (R1-2404224) reports 11.9% NES gain with medium cell load on Cell A

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 19.31 – 19.39% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 29.19% - 36.87% from the following 3 sources

    - R1-2404463, R1-2403942, R1-2404859

  - One source (R1-2404561) reports 15.87%~20.18% NES gain with PRACH configuration index = 148

- For Cat 1 BS, 4/8 beams, 4% < on-demand SIB1 transmission rate < 24%, the NES gain is 17.98%~24.48% from one source (R1-2404408).

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 160ms SIB1 period (Case C), FR1, empty load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 2.21 – 10.6% from the following 10 sources

  - R1-2405595, R1-2403979, R1-2405341, R1-2404859, R1-2405162, R1-2404122, R1-2404507, R1-2405363, R1-2404625, R1-2404561

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 1.7% - 6.23% from the following 5 sources

    - R1-2403979, R1-2405341, R1-2405363, R1-2404625, R1-2404758

  - One source (R1-2404758) reports 28.9% NES gain with Case 3 where no PRACH and paging is activated for the NES cell

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.75% - 8.48% from the following 6 sources

  - R1-2405106, R1-2405162, R1-2404859, R1-2405363, R1-2404625, R1-2404561

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.01% - 5.8% from the following 3 sources

  - R1-2405106, R1-2405363, R1-2404625

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 3.24% - 7.76% from the following 5 sources

  - R1-2403942, R1-2405341, R1-2404859, R1-2404507, R1-2404561

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 2.96% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 2.5% - 7.44% from the following 3 sources

  - R1-2404561, R1-2403942, R1-2404859

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 20ms SIB1 period (Case A), FR1, low load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 30.56% - 41.8% from the following 5 sources

    - R1-2405595, R1-2405341, R1-2404859, R1-2405363, R1-2404625

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 13.9% with one beam of SIB1 PDSCH transmitted in each slot

    - 7.9% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 16% - 36.2% from the following 5 sources

    - R1-2405247, R1-2405341, R1-2404033, R1-2405363, R1-2404625

  - One source (R1-2404033) reports 10% NES gain with FDMed SSB and PDSCH of SIB1

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 5.2% with one beam of SIB1 PDSCH transmitted in each slot

    - 1.9% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 22.44% - 37.64% from the following 3 sources

    - R1-2404859, R1-2405363, R1-2404625

  - One source (R1-2404122) reports the following with FDMed SSB and PDSCH of SIB1

    - 4.73%~9.64% NES gain with UL WUS configuration transmitted in legacy SIB on Cell A

    - 0%~4.8% NES gain with UL WUS configuration transmitted in separated SIB on Cell A. The separated SIB is TDMed with legacy SIB and transmitted in a 20ms period using dedicated resource.

    - 0% NES gain with Case 1 (Options 1+A+X) and WUS configuration transmitted in separate SIB on the NES cell

    - 7% NES gain with Case 1 (Options 1+A+X) and pre-configured for fixed WUS configuration

      - NES cell does not transmit the WUS configuration

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 13.17% - 36.2% from the following 3 sources

    - R1-2404625, R1-2405247, R1-2405363

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 16.8% - 28.77% from the following 6 sources

    - R1-2403942, R1-2405341, R1-2404224, R1-2404859, R1-2404561, R1-2405595

  - One source (R1-2404224) reports 10.5% NES gain with medium cell load on Cell A

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 12.71% – 12.88% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 14.43% - 27.47% from the following 3 sources

  - R1-2404561, R1-2403942, R1-2404859

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 160ms SIB1 period (Case C), FR1, low load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 1.2 – 8.3% from the following 7 sources

    - R1-2405595, R1-2403979, R1-2405341, R1-2404859, R1-2405363, R1-2404625, R1-2404561

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.2% - 5.46% from the following 4 sources

  - R1-2403979, R1-2405341, R1-2405363, R1-2404625

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 2.87% - 7.38% from the following 4 sources

  - R1-2404859, R1-2405363, R1-2404625, R1-2404561

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.97% - 4.48% from the following 2 sources

  - R1-2405363, R1-2404625

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 3.04% - 5.57% from the following 4 sources

  - R1-2403942, R1-2405341, R1-2404859, R1-2404561

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 1.79% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 2.12% - 5.26% from the following 3 sources

  - R1-2404561, R1-2403942, R1-2404859

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 20ms SIB1 period (Case A), FR1, medium load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 9.88% - 14.4% from the following 2 sources

    - R1-2405341, R1-2405363

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 4.5% with one beam of SIB1 PDSCH transmitted in each slot

    - 2.3% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 4.92% - 8.4% from the following 2 sources

    - R1-2405341, R1-2405363

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 1.5% with one beam of SIB1 PDSCH transmitted in each slot

    - 0.5% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 11.5% - 12.88% from one source (R1-2405363)

  - One source (R1-2404122) reports the following with FDMed SSB and PDSCH of SIB1

    - 1.94%~4% NES gain with UL WUS configuration transmitted in legacy SIB on Cell A

    - 0%~1.94% NES gain with UL WUS configuration transmitted in separated SIB on Cell A. The separated SIB is TDMed with legacy SIB and transmitted in a 20ms period using dedicated resource.

    - 0% NES gain with Case 1 (Options 1+A+X) and WUS configuration transmitted in separate SIB on the NES cell

    - 2.64% NES gain with Case 1 (Options 1+A+X) and pre-configured for fixed WUS configuration

      - NES cell does not transmit the WUS configuration

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 6%~7.53% from one source (R1-2405363)

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 8.08% - 15.47% from the following 4 sources

  - R1-2405341, R1-2404224, R1-2404561, R1-2403942

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 4.09% – 4.19% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 10.78% - 15.3% from the following 2 sources

  - R1-2404561, R1-2403942

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 160ms SIB1 period (Case C), FR1, medium load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.3 – 2.44% from the following 4 sources

  - R1-2403979, R1-2405341, R1-2405363, R1-2404561

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.1% - 1.3% from the following 3 sources

  - R1-2403979, R1-2405341, R1-2405363

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.24% - 2.26% from the following 2 sources

  - R1-2405363, R1-2404561

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 0.67% - 1.6% from one source (R1-2405363)

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.95% - 4.13% from the following 3 sources

  - R1-2403942, R1-2405341, R1-2404561

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.48% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.44% - 4.02% from the following 2 sources

  - R1-2404561, R1-2403942

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 40ms SIB1 period (Case D), FR1, empty load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 24.51% - 31.3% from the following 6 sources

    - R1-2405595, R1-2405341, R1-2404859, R1-2404507, R1-2405363, R1-2404625

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 16.3% with one beam of SIB1 PDSCH transmitted in each slot

    - 10.6% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 12.4% - 35.5% from the following 5 sources

    - R1-2404408, R1-2405341, R1-2404758, R1-2405363, R1-2404625

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 10.1% with one beam of SIB1 PDSCH transmitted in each slot

    - 6.1% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 19.44% - 28.81% from the following 3 sources

  - R1-2404859, R1-2405363, R1-2404625

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 12.87% - 20.09% from the following 2 sources

  - R1-2405363, R1-2404625

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 11.77% - 25.46% from the following 5 sources

  - R1-2403942, R1-2405341, R1-2404859, R1-2404507, R1-2404561

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 10.69 – 10.74% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 16.46% - 24.24% from the following 2 sources

    - R1-2403942, R1-2404859

  - One source (R1-2404561) reports 8.92%~10.37% NES gain with PRACH configuration index = 148

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 40ms SIB1 period (Case D), FR1, low load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 18.38% - 26.4% from the following 5 sources

    - R1-2405595, R1-2405341, R1-2404859, R1-2405363, R1-2404625

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 8.1% with one beam of SIB1 PDSCH transmitted in each slot

    - 4.3% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 9.84% - 17.41% from the following 3 sources

    - R1-2405341, R1-2405363, R1-2404625

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 2.8% with one beam of SIB1 PDSCH transmitted in each slot

    - 1% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 13.36% - 23.68% from the following 3 sources

  - R1-2404859, R1-2405363, R1-2404625

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 7.27% - 15.59% from the following 2 sources

  - R1-2405363, R1-2404625

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 10.48% - 17.72% from the following 4 sources

  - R1-2403942, R1-2405341, R1-2404859, R1-2404561

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 7.01% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 7.7% - 17.05% from the following 3 sources

  - R1-2403942, R1-2404859, R1-2404561

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 40ms SIB1 period (Case D), FR1, medium load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 5.07% - 7.8% from the following 2 sources

    - R1-2405341, R1-2405363

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 2.4% with one beam of SIB1 PDSCH transmitted in each slot

    - 1.2% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 2.46% - 4.4% from the following 2 sources

    - R1-2405341, R1-2405363

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 0.7% with one beam of SIB1 PDSCH transmitted in each slot

    - 0.3% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 5.62~7.01% from one source (R1-2405363)

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 2.77% - 3.85% from one source (R1-2405363)

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 4.08%~10.1% from the following 3 sources

  - R1-2403942, R1-2405341, R1-2404561

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 2.02%~2.09% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 5.61% - 9.94% from the following 2 sources

  - R1-2404561, R1-2403942

Agreements in RAN#118

Agreement

For on-demand SIB1 in idle/inactive mode, Case 3 (Option 2+B+Y) is feasible from RAN1 perspective for some scenarios. Case 3 (Option 2+B+Y) is lower priority compared to Case 2 from RAN1 perspective.

RAN1 is inconclusive on whether the required specification support is justified by the observed NES gain for Case 3

Conclusion

For on-demand SIB1 in idle/inactive mode, RAN1 was not able to achieve consensus to support Case 1 (Option 1+A+X) in Rel-19, while Case 1 is technically possible from RAN1’s perspective.

Agreement

RAN1 recommends specifying on-demand SIB1 only for Case 2 (Option 1+B+X) in Rel-19.

- Note: RAN1 strive to minimize impact to legacy UE.

- Note: RAN1 specification impact to support this feature should be minimized.