3GPP TSG RAN Meeting #105					     RP-241835

Melbourne, Australia, September 9-12, 2024

Source:	OPPO

Title:	Discussion on Rel-19 ISAC channel model

Agenda Item:	9.2.3

Document for:	Discussion and Decision

Introduction

In this document, we provide our views on the check-point in SID [1], which decides whether to include additional study beyond channel modelling for ISAC.

Discussion

The SID [1] sets a check-point as following:

“It will be discussed at RAN#105 whether to include additional study beyond channel modelling for ISAC.”

Although RAN1 made good progresses in the past four meetings, the overall progress seems to be still lagging behind. The details of RAN1 progresses are listed as below.

For deployment scenarios considered for ISAC channel model, RAN1 spent 4 meetings to just make UAV scenario nearly stable (there are still FFS’s in UAV scenario table), and haven’t started the detailed discussions for all other scenarios such as scenario with automotive sensing target, scenario with human sensing target, scenario with AGV sensing target and scenario with hazard object sensing target. In other words, the completion level of deployment scenario studies is just roughly 20% (if hazard object scenario is eventually included in the study) to 25% (if hazard object scenario is eventually not included). It would be a concern that, if RAN1 shift some TU’s to perform certain ISAC study beyond channel model, RAN1 may end up with a support of only a limited number of deployment scenarios in Rel-19 ISAC channel model.

For the ISAC channel model methodology, most of the agreement achieved so far are just the agreements for setting up modeling directions and/or starting points, with quite a lot details being missing in place, for both large-scale fading and small-scale fading. For example,

- For LOS/NLOS probability, although it is agreed in RAN1 #117 to take LOS probability schemes in existing 3GPP TRs (e.g., 38.901, 36.777, 37.885 etc) as “starting point”, it is known that these existing TR’s may not define a LOS probability between a sensing Tx/Rx and a sensing target, for example, the LOS probability for a link between UAV-UE and UAV target in UAV-related scenarios and LOS probability for a link between a UE and an automotive target in automotive-related scenarios. Even more general, if RAN1 agrees there is no power normalization to keep (target channel + background channel) having the same/similar large-scale fading as in legacy 38.901 channel model, above-mentioned LOS probabilities are still needed for a normal UE/UAV-UE that does not even involve any sensing functionality but just doing the communication with interference reflected/scattered from the targets.

- For direct/indirect paths in target channel, although most of steps could follow the existing TR’s under the umbrella of concatenation, there is something that all existing TR’s have missed, such as

  - the LOS scattering: RAN1 already sets a FFS task for LOS scattering modeling.

  - the concatenation modeling method may need additional RAN1 discussion upon the balance between complexity reduction and modeling accuracy.

  - the correlation between LOS probability of Tx-target link and LOS probability of target-Rx link [and maybe also the LOS probability of Tx-Rx link], i.e., whether such correlation should be a part of spatial consistency or should be separately determined in a new way.

  - Whether/how to model per-cluster shadowing and per-link shadowing, and whether to make the overall shadowing of Tx-target-Rx match the one in 38.901.

  - There are proposals in RAN1 to use deterministic free-space pathloss formula to model the LOS rays. So far there is no RAN1 discussion on this matter.

- For channel impulse response generation, most of issues are still open except that RAN1 agreed in RAN1 #117 that the step 11 in section 7.5 of 38.901 is used as a starting point.

- For RCS modeling, RAN1 reached a working assumption that the RCS of a target can be modeled as RCS=A*B where A in unit of square meter is in large-scale and B is unit-less in small-scale. All the other details including how to determine A and B are FFS.

- For environmental object (EO) modeling, RAN1 agreed to follow the principle of ground reflection model in TR38.901 as “starting point” for EO Type-2 modeling, which may, however, need the math derivation from scratch because the ground reflection has horizontal reflection surface with infinite size while the EO Type-2 gives vertical reflection surface with finite size.

- For background channel modeling, this might be the modeling component that some people think the easiest at the beginning of SI but in fact one of the most critical modeling parts when it comes to the impacts to the communication-purpose evaluation. So far RAN1 had no discussion upon the following for bi-static sensing mode:

  - Whether the background channel should be power-normalized, either alone or together with target channel, to keep the (target channel + background channel) have the same large scale fading as in legacy channel model. Because RAN1 did not make the decision at the beginning of the SI discussion, the current RAN1 progresses give a strong implication that the large-scale fading may be different between (target channel + background channel) and legacy channel model, which may unfortunately introduce target presences into a Tx-Rx channel even if neither of the Tx and the Rx performs sensing but just in a joint sensing-communication evaluation with another pair of sensing Tx/Rx.

  - Whether the Ricean-K factor in the background channel (if in LOS condition) should count the ray powers in the target channel as well, or only the rays in background channel.

RAN1 has almost zero progress on background channel in mono-static sensing mode.

- For spatial consistency, RAN1 just agreed in RAN1 #117 to support spatial consistency in ISAC channel model, especially in case any of Tx/Rx/Target has movement. But there is no further discussion to follow up this high-level agreement. Meanwhile, it is recognized in RAN1 that the spatial consistency in ISAC channel model can be more challenging than the one in legacy channel model because spatial consistency in at least UAV scenario needs to go up to 3D.

- For modeling validation, it is supposed to be undertaken after the channel model gets stable, either directly against the measurement data or indirectly against a ray-tracing model that is validated by the measurement data.

- For link-level channel model, RAN1 has not started any discussion yet. It might not be a quick work given it will be RAN1’s first time to discuss the “link-level concatenation”.

Observation 1: The current RAN1 progress on ISAC channel model does not justify additional study beyond channel modeling for ISAC, given that

- The completion level of deployment scenario discussion is around 20~25%.

- For the modeling methodology, the most of agreements still stay at the phase of “just having the starting point”, and a lot of modeling details are missing.

Proposal 1: For the RAN#105 check-point on whether to include additional ISAC study beyond channel modeling for ISAC, RANP chooses from the following two options:

- Option-1: Confirm that any additional ISAC study beyond ISAC channel model is out of Rel-19 scope.

- Option-2: Postpone the RAN #105 check-point to future RAN meetings that fits with RAN1 progress.

Conclusions

In this contribution, we discuss channel model for sensing with the following observations and proposals:

Observation 1: The current RAN1 progress on ISAC channel model does not justify additional study beyond channel modeling for ISAC, given that

- The completion level of deployment scenario discussion is around 20~25%.

- For the modeling methodology, the most of agreements still stay at the phase of “just having the starting point”, and a lot of modeling details are missing.

Proposal 1: For the RAN#105 check-point on whether to include additional ISAC study beyond channel modeling for ISAC, RANP chooses from the following two options:

- Option-1: Confirm that any additional ISAC study beyond ISAC channel model is out of Rel-19 scope.

- Option-2: Postpone the RAN #105 check-point to future RAN meetings that fits with RAN1 progress.

Reference

- RP-240799, SID: Study on channel modelling for Integrated Sensing and Communication (ISAC) for NR, Xiaomi, AT&T

- 3GPP TR 38.901 V17.0.0