3GPP TSG RAN Meeting #105			RP-241838

Melbourne, Australia, September 9-12, 2024

Agenda item:	9.3.1.3

Title:	Discussion on AI/ML for NR air interface

Source:	OPPO

Document for:	Discussion and Decision

1. Introduction

In RAN#102, the WI on AI/ML for NR air interface [1] was established on beam management and positioning. It also includes the continuous study at least on a) CSI feedback enhancement, b) model transfer/delivery and c) collection of UE-side model training data highlighted as yellow.

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. - For CSI prediction (one-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843 (e.g., cell/site specific model could be considered to improve performance gain). - Necessity and details of model Identification concept and procedure in the context of LCM [RAN2/RAN1] - CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods - Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis

It is time to check the progress of the study in RAN#105 and make decisions for the remaining of Rel-19 WI/SI phase. In this contribution, we analyze the current status of each study and propose to specify CSI prediction and to extend the study of CSI compression, UE-sided model training data collection and model transfer/delivery.

- Normative work for CSI prediction in Rel-19

In RAN1#118, numerous evaluation results are provided by companies and finally summarized into observations. The following observation briefly introduce the study of CSI prediction with UE-side model. RAN1 has conducted the study from the aspects of performance gain with proper metrics and benchmarks, generalization and model monitoring mechanism.

Observation The following aspects have been studied for CSI prediction using UE-sided model: - From the perspective of basic performance gain over non-AI/ML benchmark (without considering generalization), - It has been studied with corresponding observations on: - the metrics of SGCS, mean UPT, 5% UPT; - the benchmarks of nearest historical CSI and non-AI/ML based CSI prediction. - The impact of channel estimation error, phase discontinuity, spatial consistency, UE Speed, observation window, prediction window, CSI-RS periodicity. - It has been studied with corresponding observations on complexity for both AI/ML based CSI prediction and non-AI/ML based CSI prediction. - It has been studied on localized model including evaluation methodology, but is lack of observations. - From the perspective of generalization over various scenarios, - It has been studied with corresponding observations on (with the metric of SGCS): - the scenario including various UE speeds, deployment scenarios, carrier frequency - From the perspective of performance monitoring, it has been studied on boundary between Type 1 and 3 performance monitoring, and potential specification impact for each performance monitoring type 1,2 and 3.

With considerable performance evaluations from the efforts of companies, high level observations are given as in following agreement. Not only the performance gain over the intermediate metric of SGCS, but also the mean and 5% UPT from system level are provided. One may notice that for cell-edge UE, the performance gain on UPT can be significantly up to 20%. On complexity perspective, the ratio of FLOPs (AI/ML over benchmark 2) ranges from 1 to 35 by majority companies as expected. On robust aspect, AIML can achieve higher gain when channel estimation error and phase discontinuity exist in reality.

Observation Based on the evaluation for CSI prediction, the following high-level observations are provided: - From the perspective of basic performance gain over benchmark of non-AI/ML based CSI prediction, under the same UE speed for training and inference - for AI/ML based CSI prediction over non-AI/ML based CSI prediction, 0%~7.8% gain depending on traffic model, in terms of mean UPT, is observed by 7 sources - for AI/ML based CSI prediction over non-AI/ML based CSI prediction, 3.8%~20.7% gain depending on traffic model, in terms of 5% UE UPT, is observed by 7 sources. - From a perspective of AI/ML complexity, 19 sources adopt the model subject to the computational complexity in units of FLOPs from 0.05M to 3000M. For complexity of non-AI/ML benchmark, 16 sources adopt the algorithm (e.g., Kalman filter, Auto-regression, Wiener filter) subject to the computational complexity in units of FLOPs from 0.14M to 107M. The ratio of FLOPs (AI/ML over benchmark 2) ranges from 1 to 35, which is used by majority sources. For non-AI/ML benchmark, main computation complexity is dominated by filter updates, which may not be need to be updated per inference at the expense of performance loss. - From the perspective of performance impact on channel estimation error and phase discontinuity, compared to non-AI/ML CSI prediction, higher gain is observed by 10 sources for AI/ML based CSI prediction in the presence of channel estimation error or phase discontinuity. - From the perspective of generalization over various scenarios/configurations (e.g., various UE speed, deployment scenario, carrier frequency) that have been evaluated, compared to generalization Case 1 where the AI/ML model is trained with dataset subject to a certain scenario#B/configuration#B and applied for inference with a same scenario#B/configuration#B - For generalization Case 2 where the AI/ML model is trained with dataset from a different scenario#A/configuration#A, generalized performance may be achieved for some certain combinations of scenario#A/configuration#A and scenario#B/configuration#B but not for others. - For generalization Case 3 where the training dataset is constructed with data samples subject to multiple scenarios/configurations including scenario#B/configuration#B, generalized performance of the AI/ML model can be achieved.

From RAN1 recommendation listed as below, the study of AI/ML-based CSI prediction has been completed with observable performance gain and increased complexity. Moreover, the specification impact of supporting the AI/ML-based CSI prediction can be minimum in our understanding. Two existing WIs can be learnt. One is the CSI prediction introduced in Rel-19 MIMO which is selected as benchmark scheme during evaluation. The other one is the ongoing AI/ML for beam management, which also is applicable to the UE-sided model.

Agreement From RAN1 perspective, study of CSI prediction has been completed and performance improvement is observed with increased complexity.

With above being said, we would like to share the following observation and proposal for CSI prediction.

- AI/ML-based CSI prediction achieves observable performance gain in the metrics of SGCS, 5% UPT and mean UPT with acceptably increased complexity

- It performs better over non-AI/ML benchmarks when channel estimation error and phase discontinuity are considered

- It introduces minimum specification impact which can be handled within Rel-19.

- Support AI/ML-based CSI prediction as normative work in Rel-19.

- Extended study for CSI compression

For CSI compression with two-side model, RAN1 has recommended to extend the study due to the fact that the study is technically not completed yet.

Agreement According to the Rel-19 study objective, - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.. RAN1 is recommending extending the study of the AI/ML CSI compression based on RAN1 understanding the study on CSI compression is not completed.

A few of pending issues should be addressed before stepping into normative work. For instance, to make sure that the two-side model can be trained properly, options/sub-options of cross-vendor collaboration should be further studied and down selected in upcoming RAN1 meeting(s). Then how to monitor the performance of two-sided model of CSI compression is next key issue to be addressed. Last but not least, data collection for training should be comprehensively investigated as well.

Regarding the duration of the extended study, we are not confident that RAN1 can complete it within two additional meetings before RAN#106 (Dec. 2024). It seems sufficient to carry on the study until the end of Rel-19. With above being mentioned, we would like to have the following proposal.

- Extend the study of AI/ML-based CSI compression with two-sided model until the end of Rel-19.

- Prioritize the study of cross-vendor collaboration for model training

- Extended study for model identification, transfer and delivery

Similar to the case of CSI compression, RAN1 also made the following recommendation on both model identification and model transfer/delivery in RAN1#118.

Agreement RAN1 is recommending extending the study of the Model identification, and Model transfer/Model delivery based on RAN1 understanding the study is not completed.

On model identification (MI), RAN1 have conducted study on five different approaches and decided to deprioritize certain MI-Options. For instance, MI-Option2 (based on dataset transfer) for one-sided model is not be pursued in Rel-19 normative work (if there is), due to large payload of dataset transfer from NW to UE.

On model transfer/delivery, it highly couples with the two-sided model use case, i.e. CSI compression. For instance, Case z4 depends on the known model structure(s) in the aspects of model type/backbone and tremendous parameters. However, assume typical neural network as the model, next level details, e.g. number of layers, layer type/size, has not been fully touched until RAN1#118.

One more pending issue is to ensure consistency across training and inference. Particularly, UE may assume NW-side additional conditions are consistent with the same associated ID within a cell. In RAN1, whether associated ID can be applicable across multiple cells or globally is still a valid question. That implies the same associated ID from different cells may not guarantee consistency on NW-side additional condition.

From the reasons and recommendation, we would like to have the following proposal.

- Extend the study of model identification and model transfer/delivery until the end of Rel-19.

- Data collection for UE-sided model training

In recent RAN2#126 and RAN2#127, data collections for UE-side model training has been updated in TR 38.843 [2] as follow.

7.2.1.3.2	Data collection for UE-side model training

The following options were discussed in RAN2:

1a.	UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected.

1b.	UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO).

2.	UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server.

3.	UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server.

RAN2 have conducted comprehensive analysis on the meaning of each option, especially on controllability and visibility aspects. From our understanding, it is very difficult for RAN2 to further down select those options for UE-sided model training. Clearly RAN’s guidance to RAN2 is necessary, otherwise, RAN2 may have to extend the study of all options until the end of Rel-19.

As for the involvement of SA group, it is welcomed to further study how those options work from SA perspective. However, we believe the down selection among those options should not be done in SA.

- Clear RAN’s guidance to RAN2 is necessary, otherwise, RAN2 will extend the study of all options by the end of Rel-19.

6. Conclusion

To draw attention from readers, allow us to repeat observations and proposals in what follows.

- AI/ML-based CSI prediction achieves observable performance gain in the metrics of SGCS, 5% UPT and mean UPT with acceptably increased complexity

- It performs even better over non-AI/ML benchmarks when channel estimation error and phase discontinuity are considered

- It introduces minimum specification impact which can be handled within Rel-19.

- Support AI/ML-based CSI prediction as normative work in Rel-19.

- Extend the study of AI/ML-based CSI compression with two-sided model until the end of Rel-19.

- Prioritize the study of cross-vendor collaboration for model training

- Extend the study of model identification and model transfer/delivery until the end of Rel-19.

- Clear RAN’s guidance to RAN2 is necessary, otherwise, RAN2 will extend the study of all options by the end of Rel-19.

7. References

[1] RP-234039, “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”

[2] R2-2407807, “CR on RAN2 inputs to TR 38.843”