3GPP TSG RAN meeting #105										RP-241840

Melbourne, Australia, September 9-12, 2024

## Status Report to TSG

Agenda item:			9.2.9

| WI / SI Name | Study on IMT parameters for NR for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_IMT_4400_7125_14800MHz |  |  |  |  |
| Unique ID | 1030075 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-241326 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 12/2024 | Core part: | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: 50 % | Core part: | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Thomas Chapman |
|  | Company | Ericsson |
|  | Email | Thomas [dot] chapman [at] Ericsson [dot] com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

The 4400 – 4800 MHz range was agreed during Q2 2024 and a reply LS sent to ITU-R. During RAN4#112, two maintenance TPs were agreed for the 4400 – 4800 MHz range in R4-2412587 and R4-2413278.

For the 7400 – 8125MHz range, all parameters were agreed. Agreements during RAN4#112 were agreed in the following WF:

- R4-2414282	WF on 8GHz UE parameters

- R4-2414281	WF on 8GHz BS parameters

The full set of agreements for the 7125 – 8400 MHz range were documented in the technical report using the following text proposals:

- R4-2414300	TP for 38.922 on UE IMT parameters for 7125-8400MHz

- R4-2414301	TP for BS IMT parameters  for range 7125 to 8400 MHz

The 7125 – 8400 MHz LS to ITU was approved in R4-2414302 (LS Reply on Parameters for 7125 to 8400 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27). With this LS and TP, the 7125 – 8400 MHz part of the SI is completed.

For the 14800 – 15350 MHz range, simulation assumptions were updated using the following TP to the TR: R4-2414303 (TP to TR 38.922: Revisions of system level simulation assumptions for study on IMT parameters for 14800 to 15350 MHz frequency range)

Some further agreements and options for the BS and UE requirements for the 14800 – 15350 MHz range were documented in the following:

- R4-2414284	WF on 15GHz UE parameters

- R4-2414283	WF on 15GHz BS parameters

During RAN4#112, there was extensive offline discussion on the other questions, in particular MIMO modelling, but no WF was agreed.

#### 2.4.2	Remaining Open issues

- Study the IMT parameters relevant for sharing and compatibility for 14800 to 15350 MHz frequency range.

- Send LS reply to ITU-R WP 5D (CC to TSG RAN) regarding NR in 14800 to 15350 MHz.

- IMT technology related parameters (template as attached in Table 1).

- Beamforming antenna characteristics for IMT (template as attached in Table 2).

- As part of additional information requested by ITU-R WP 5D establish a model for array correlation factor () roll-off to describe how the array factor collapses moving away from the centre frequency. If the beamforming performance degrades towards a single element radiation pattern, then the frequency when the continuity of array features is regarded as degraded, will also need to be determined.

- Considering the system-level simulation work typically conducted in ITU-R WP 5D for sharing and compatibility studies using 3D modelling of antenna pattern, ITU-R WP 5D is seeking information on whether for implementation of IMT BS multiuser spatial beamforming techniques, such as zero-forcing (ZF) or minimum mean-square error (MMSE) based schemes, would be necessary to improve the assessment of interference and the accuracy of studies. If considered necessary, guidance on the process of deriving the necessary beamforming weights for the IMT AAS BS to compute its radiation pattern.

- Provide guidance on if UE array antenna is likely or not. If UE array antenna is considered provide relevant parameters.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

The following is a list of contributions submitted to and treated during RAN4#112

| R4-2411021 | Text Proposals on MIMO models and PA nonlinearity impacts and ACLR | Spark NZ Ltd |
| --- | --- | --- |
| R4-2411090 | Further discussion on IMT parameters for NR in 7125 to 8400MHz | CATT |
| R4-2411091 | Further discussion on IMT parameters for NR in 14.8GHz to 15.35GHz | CATT |
| R4-2411092 | TP to TR 38.922 for 15GHz BS RF parameters | CATT |
| R4-2411093 | TP for other issues (Adjacent channel modelling) | CATT |
| R4-2411141 | TP for 38.922 on UE IMT parameters for 7125-8400MHz | Apple |
| R4-2411142 | On UE antenna parameters for 14800-15350MHz | Apple |
| R4-2411143 | TP on UE antenna parameters for 14800-15350MHz | Apple |
| R4-2411193 | IMT parameters relevant for sharing and compatibility for 7125 to 8400 MHz frequency range | Ericsson |
| R4-2411194 | LS Reply on Parameters for 7125 to 8400 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27 | Ericsson |
| R4-2411307 | Discussion on Frequency allocation in Korea | Korea Testing Laboratory |
| R4-2411518 | Views on IMT parameters for 7125 - 8400 MHz | Qualcomm Germany |
| R4-2411519 | Text proposal on 7125 – 8400 MHz IMT parameters in TR 38.922 | Qualcomm Germany |
| R4-2411520 | Views on Additional AAS aspects | Qualcomm Germany |
| R4-2411521 | Views on Radio parameters for 14800 - 15350 MHz | Qualcomm Germany |
| R4-2411522 | Views on co-existence parameters for 14800 - 15350 MHz | Qualcomm Germany |
| R4-2411720 | Discussion on ACLR Calculation Procedure for 14800 - 15350 MHz with Beamforming and MIMO | ISSDU |
| R4-2411775 | Coexistence study for 14800 to 15350 MHz | MediaTek inc. |
| R4-2411776 | Coverage study for 14800 to 15350 MHz | MediaTek inc. |
| R4-2411873 | On the topic of additional information requested by ITU-R WP 5D | Ericsson |
| R4-2411874 | On general aspects related to IMT parameter SI | Ericsson |
| R4-2411948 | IMT parameters: General system and UE aspects | Nokia |
| R4-2412068 | Discussion on the UE parameter for 7125 to 8400 MHz | vivo |
| R4-2412069 | Discussion on the coexistence for 14800 to 15350 MHz | vivo |
| R4-2412070 | Discussion on the UE parameter for 14800 to 15350 MHz | vivo |
| R4-2412126 | On Co-existence simulation assumptions for 14800 to 15350 MHz frequency range | Ericsson |
| R4-2412127 | On Urban Macro and Indoor Co-existence simulation results for 14800 to 15350 MHz frequency range | Ericsson |
| R4-2412128 | On Antenna feasibility and RF parameters for 14800 to 15350 MHz frequency range | Ericsson |
| R4-2412137 | Views on IMT parameters for 7125 to 8400 MHz frequency range | Samsung |
| R4-2412138 | Initial simulation results of co-existence scenarios for 14800 – 15350 MHz | Samsung |
| R4-2412139 | Views on UE type for 14800 to 15350 MHz frequency range | Samsung |
| R4-2412565 | Discussion on IMT UE parameters for 7125 to 8400 MHz | MediaTek (Wuhan) Inc. |
| R4-2412587 | TP to TR 38.922: Corrections and clarifications on IMT parameters for 4400 to 4800 MHz frequency range | Nokia |
| R4-2412588 | BS parameters for 7125 to 8400 MHz frequency range | Nokia |
| R4-2412589 | Urban macro coexistence simulation results for 14800 to 15350 MHz frequency range | Nokia |
| R4-2412590 | TP to TR 38.922: Revisions of system level simulation assumptions for study on IMT parameters for 14800 to 15350 MHz frequency range | Nokia, Fujitsu |
| R4-2412591 | BS antenna and simulation parameters for 14800 to 15350 MHz frequency range | Nokia |
| R4-2412592 | Study of AAS performance in adjacent bands | Nokia |
| R4-2412593 | Impact of beamforming schemes on the coexistence of IMT with other services | Nokia |
| R4-2412608 | TR 38.922 version 0.2.0 | Ericsson |
| R4-2412710 | Discussion on IMT parameters for 7125-8400MHz | ZTE Corporation, Sanechips |
| R4-2412711 | Discussion on co-existence evaluation for 14800 to 15350 MHz | ZTE Corporation, Sanechips |
| R4-2412712 | Discussion on radio and antenna parameters for 14800 to 15350 MHz | ZTE Corporation, Sanechips |
| R4-2412713 | Discussion on other issues in ITU-R LS | ZTE Corporation, Sanechips |
| R4-2412821 | Topic summary for [112][119] FS_NR_IMT_part1 | Moderator(Ericsson) |
| R4-2412822 | Topic summary for [112][120] FS_NR_IMT_part2 | Moderator(Nokia) |
| R4-2412967 | Discussion on IMT parameters for 7125 to 8400MHz | Google |
| R4-2413278 | TP for IMT technology related parameters for 4400 to 4800 MHz | Huawei, HiSilicon |
| R4-2413279 | Remaining issues for IMT parameters for range 7125 to 8400 MHz | Huawei, HiSilicon |
| R4-2413280 | TP for BS IMT parameters for range 7125 to 8400 MHz | Huawei, HiSilicon |
| R4-2413281 | AAS modelling considerations for IMT base stations | Huawei, HiSilicon |
| R4-2413368 | Impact of beamforming schemes on the coexistence of IMT with other services | Nokia |
| R4-2414279 | Ad hoc minutes on study of IMT parameters | Ericsson |
| R4-2414281 | WF on 8GHz BS parameters | Huawei |
| R4-2414282 | WF on 8GHz UE parameters | Vivo |
| R4-2414283 | WF on 15GHz BS parameters | ZTE |
| R4-2414284 | WF on 15GHz UE parameters | Qualcomm |
| R4-2414299 | WF on other issues (MIMO) | Nokia |
| R4-2414300 | TP for 38.922 on UE IMT parameters for 7125-8400MHz | Apple |
| R4-2414301 | TP for BS IMT parameters for range 7125 to 8400 MHz | Huawei, HiSilicon |
| R4-2414302 | LS Reply on Parameters for 7125 to 8400 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27 | Ericsson |
| R4-2414303 | TP to TR 38.922: Revisions of system level simulation assumptions for study on IMT parameters for 14800 to 15350 MHz frequency range | Nokia, Fujitsu |
| R4-2414449 | LS Reply on Parameters for 7125 to 8400 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27 | Ericsson |

12.08.2024		minor adaptations for RAN #105

30.04.2024		minor adaptations for RAN #104

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template