3GPP TSG RAN Meeting #105	　　   　　                                                                  RP-241843

Melbourne, Australia, September 9-12, 2024

Status Report to TSG

Agenda item:			9.3.4.7

| WI / SI Name | # NR Sidelink: Intra-band Carrier Aggregation in ITS band |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_SL_intraB_CA_ITS-Core |  |  |  |  |
| Unique ID | 1030084 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-240840 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: 09/2025 | Performance part: | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 25% | Performance Part: | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Xing, Jinqiang, Yang, Yoonoh |
|  | Company | OPPO, LGE |
|  | Email | xingjinqiang@oppo.com, yoonoh.yang@lge.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4#112

- Approved WF (R4-2414312 WF on NR sidelink for intra-band non-contiguous CA), main agreements as below.

Issue 2-1-1: Operating band

Agreement: Merge two tables.

Table 5.1.2-1 Intra-band non-contiguous SL CA operating bands in FR1

| NR intra-band non-contiguous SL CA operating Band | NR SL Operating Band | Interface |
| --- | --- | --- |
| SL_n47(2A) | n47 | PC5 |
| NOTE 1: The minimum requirements only apply for non simultaneous Tx/Rx between all carriers in n47 band. |  |  |

Table 5.1.1-1: Intra-band contiguous CA operating bands for SL CA in FR1

| NR SL CA Band | NR Band | Interface |
| --- | --- | --- |
| SL_n47 | n47 | PC5 |

Issue 2-1-2: Channel bandwidth

Agreement:

| Sidelink CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sidelink CA configuration | Sidelink CA configuration for TX | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| SL_n47(2A) | SL_n47(2A) | 10 | 10, 20 |  |  | 30 | 0 |

Issue 2-1-2: How to define the requirement

Agreement:

Define MPR to meet -13dBm/MHz and MPR to meet -30dBm/MHz as NR intra-band non-contiguous CA

Consider B =  (LCRB1* 12* SCS1 + LCRB2 * 12 * SCS2)/1,000 for PSSCH as NR intra-band non-contiguous CA

PSSCH/PSCCH with 2x20dBm PA + 1LO or 2x20dBm PA + 2LO

Agreement:

use PSSCH/PSCCH with 2x20dBm PA for PC3 sidelink non-contiguous CA evaluation.

RAN4#111

- Approved WF (R4-2410586 WF on NR_SL_ intraB_CA_ITS_part1), main agreements as below.

Issue 2-1-1: How to capture the operating band

Agreement:

To capture the intra-band contiguous and non-contiguous SL CA bands under the same sub-clause.

Issue 2-1-3: SCS

Agreement:

- All the requirements defined for PC2/PC3 sidelink intra-band non-contiguous CA shall apply under the assumption of the same subcarrier spacing for SL CA.

Sub-topic 2-1 MPR/A-MPR simulation

Issue 2-1-1: Architecture

Agreement

- Do evaluations based on assumptions of 1LO and 2LO

- Consider the following architecture for evaluation

| Arch | description | Remarks |
| --- | --- | --- |
| #1-1 | 1x26dBm PA + 1LO | Single–Tx |
| #1-2 | 2x23dBm PA + 1LO | dual-Tx or txDiversity |
| #2-2 | 2x23dBm PA + 2LO | dualPA-Architecture |

Issue 2-1-2: Frequency gap

Agreement:

- Consider the worst case for BW gap size between sub-block #1 and #2 for the MPR requirement.

- For evaluations, consider all the gap size.

Issue 2-1-4: Requirement for simulation

Agreement: For FCC regulation, work is based on current NS_52 first and wait for FCC update.

Sub-topic 2-2 TX Requirements

Agreement:  Use proposal 1 as the starting point.

Issue 3-1-1: RX requriements

Agreement: Use proposal 1 as the starting point.

- Approved WF (R4-2410587 WF on NR_SL_ intraB_CA_ITS_part2), main agreements as below.

Issue 1: Tx UE RF requirements for PC2 SL intra-band contiguous CA

Issue 1-1: MOP

Agreement:

- For PC2 intra-band contiguous SL CA, the following two aspects should be specified in Rel-19

  - Minimum requirements for Power Class 2 should be applicable for SL intra-band contiguous CA combination configuration, and update the maximum output power for sidelink CA

- Define SL intra-band contiguous CA PC2 MOP as total transmitted power (Per UE) for PC2 SL CA UE

Issue 1-2: MPR

Agreement:

- MPR/A-MPR requirements for intra-band contiguous SL CA with PC2 should be specified in Rel-19 referring to the methodology of that with PC3 in Rel-18 including, PSSCH/PSCCH, PSFCH and S-SSB

  - For PSSCH/PSCCH: specify MPR for power class 2 contiguous CA with contiguous/non-contiguous RB allocation, including inner and outer.

  - For PSFCH: specify MPR for PC2 contiguous CA based on different range of R, R is the ratio of the gap bandwidth between the two PSFCH transmitted on the two intra-band carrier by the total bandwidth of the two carrier

  - For S-SSB: specify MPR for PC2 contiguous CA by single S-SSB and two S-SSB, separately.

Issue 1-3: Simulation Assumption for MPR

<Way forward>: Keep 25dBc carrier leakage as agreed WF(R4-2406611) for PC2 intra-band SL contiguous CA.

Issue 1-4: UE RF architecture for MPR evaluation

Agreement:

- The PC2 UE RF architecture of NR intra-band contiguous CA for MPR can be found as below for information (1 PA, dual Tx(Tx diversity), and dualPA)

  - High priority for MPR evaluations

    - 1 PA : 1x26dBm + 1LO

    - Dual Tx : 2x23dBm + 1LO

  - Second priority for MPR evaluations

    - dualPA : 2x23dBm + 2LO

Issue 1-6: ACLR

Agreement: Reuse 31dB ACLR for intra-band contiguous PC2 CA

Issue 1-7: Whether to introduce ΔPPowerClass,SL CA

Agreement: Not to introduce ΔPPowerClass,SL CA due to no SAR related issues for sidelink CA on the ITS band

Issue 1-8-1: A-MPR for EU regulation

Agreement: No A-MPR requirement based on EU regulation is needed for sidelink intra-band contiguous CA.

Issue 1-8-2: A-MPR for US regulation

Agreement: Study and specify the related A-MPR requirements with NS_52 after final FCC announcement for the additional emission limits in US

Issue 1-9: Other Tx requirements than MOP, MPR and A-MPR

<Way forward>: Reuse the requirements of PC3 SL intra-band contiguous CA

- Minimum output / transmit OFF power /transmit ON/OFF time mask

- Power control

- Transmit signal quality

- SEM/

- General spurious emissions/UE-to-UE coexistence SE

- Transmit intermodulation

Issue 2: Rx requirements for SL intra-band contiguous CA

Sub-topic 2-1 : Rx UE RF requirements for PC2 SL intra-band contiguous CA

Issue 2-1: Rx UE RF requirements

Agreement: The Rx requirements including REFSENS, ACS, maximum input power and blockings for intra-band contiguous SL CA with PC3 in current Spec shall apply to the intra-band contiguous SL CA with PC2 directly

Issue 3: System Parameter

Agreement: For PC2 intra-band contiguous CA, reuse the same channel bandwidth of PC3 intra-band contiguous CA.

RAN4#110bis

- Approved WF (R4-2410587 WF on NR_SL_ intraB_CA_ITS_part2), main agreements as below.

Issue 2-2-1: UE RF architecture for non-contiguous CA

WF:

  - For PC3 and PC2 intra-band non-contiguous CA, it is proposed to use Option1 and Option 2 as a starting point assumption for MPR/A-MPR simulation.

  - RAN4 can consider all candidate RF architectures to derive PC3/PC2 MPR/A-MPR requirements for intra-band non-contiguous CA.

Issue 2-2-2: Simulation assumption

WF:

  - Agree on below table for simulation assumption

  - The carrier leakage can be further discussed

Issue 2-2-3: Frequency gap

WF:

  - for the intra-band non-contiguous SL CA, the frequency gap should be determined based on the real deployment scenarios in global countries. The detail frequency gap is FFS.

For PC2 intra-band contiguous CA:

WF:

  - It is proposed to use option 1 and option 3 as the starting point assumption for PC2 intra-band contiguous CA.

  - RAN4 can consider all candidate RF architectures to derivePC2 MPR/A-MPR requirements for intra-band contiguous CA.

Issue 3-2-2: Simulation assumption

WF:

  - Agree on below table for simulation assumption

  - The carrier leakage can be further discussed

2.4.2	Remaining Open issues

- For intra-band contiguous CA PC2

  - Do MPR/A-MPR simulation and define requirements

  - UE TX requirements

- Intra-band non-contiguous CA PC3/PC2

  - Do MPR/A-MPR simulation and define requirements

  - UE TX requirements

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #112 contributions

- R4-2411650	TP on TR38.787 Updated Reference and Objectives for SL Intra-band CA in ITS spectrum	Meta Ireland

- R4-2411651	TP on TR38.787: Operating bands and UE RF requirements for intra-band non-contiguous SL CA UE	Meta Ireland

- R4-2411652	TP on TR38.787: Operating bands and UE RF requirements for intra-band contiguous SL CA UE	Meta Ireland

- R4-2411871	Tx requirements for intra-band contiguous CA	LG Electronics

- R4-2412014	NR side link requirements and European regulation for band n47	LG Electronics Finland

- R4-2412017	Tx requirements for SL intra-band non-contiguous CA	LG Electronics Finland

- R4-2412731	SL Intra-band contiguous CA simulation results	OPPO

- R4-2412732	SL Intra-band non-contiguous CA RX requirements	OPPO

- R4-2412733	SL Intra-band non-contiguous CA simulation results	OPPO

- R4-2412734	TP on TR38.787 on RX requirement for intra-band contiguous SL CA	OPPO

- R4-2412735	TP on TR38.787 on RX requirement for intra-band non-contiguous SL CA	OPPO

- R4-2412736	TP on TR38.787 on system parameter for intra-band contiguous SL CA	OPPO

- R4-2412737	TP on TR38.787 on system parameter for intra-band non-contiguous SL CA	OPPO

- R4-2412738	TP on TR38.787 to capture the intra-band contiguous SL CA simulation result	OPPO

- R4-2412739	TP on TR38.787 to capture the intra-band non-contiguous SL CA simulation result	OPPO

- R4-2412823	Topic summary for [112][121] NR_SL_ intraB_CA_ITS_part1	Moderator(OPPO)

- R4-2412824	Topic summary for [112][122] NR_SL_ intraB_CA_ITS_part2	Moderator(LGE)

- R4-2414312	WF on NR sidelink for intra-band non-contiguous CA	OPPO

- R4-2414313	TP on TR38.787: Operating bands and UE RF requirements for intra-band non-contiguous SL CA UE	Meta Ireland

- R4-2414314	TP on TR38.787 on RX requirement for intra-band non-contiguous SL CA	OPPO

- R4-2414315	TP on TR38.787: Operating bands and UE RF requirements for intra-band contiguous SL CA UE	Meta Ireland

- R4-2414424	TP for TR 38.787 V0.0.1	OPPO

RAN4 #111 contributions

- R4-2407612	On Tx requirements for sidelink intra-band contiguous CA in PC2 in Rel-19	Huawei, HiSilicon

- R4-2407613	On Tx requirements for sidelink intra-band non-contiguous CA in PC3 and PC2 in Rel-19	Huawei, HiSilicon

- R4-2407614	On Rx requirements for sidelink intra-band non-contiguous CA in PC3 and PC2 in Rel-19	Huawei, HiSilicon

- R4-2407817	Discussion on Tx requirements for sidelink intra-band non-contiguous CA  in n47	Xiaomi

- R4-2407818	Discussion on Rx requirements for sidelink intra-band non-contiguous CA  in n47	Xiaomi

- R4-2407819	Discussion on Tx requirements for sidelink intra-band contiguous CA  in n47	Xiaomi

- R4-2407820	Discussion on Rx requirements for sidelink intra-band contiguous CA  in n47	Xiaomi

- R4-2407983	Tx requirements for intra-band non-contiguous CA	LG Electronics

- R4-2407984	Tx requirements for intra-band contiguous CA	LG Electronics

- R4-2408045	UE RF requirements for NR SL Intra-band non-contiguous CA	Facebook Japan G.K.

- R4-2408048	UE RF requirements for NR SL Intra-band contiguous CA	Facebook Japan G.K.

- R4-2408129	Discussion on Tx RF requirements for PC2 SL intra-band contiguous CA	vivo

- R4-2408831	SL Intra-band contiguous TX and RX requirement	OPPO

- R4-2408832	SL Intra-band non-contiguous RX requirement	OPPO

- R4-2408833	SL Intra-band non-contiguous TX requirement	OPPO

- R4-2408834	SL on system parameters	OPPO

- R4-2408941	Topic summary for [111][130] NR_SL_ intraB_CA_ITS_part1	Moderator(OPPO)

- R4-2408942	Topic summary for [111][131] NR_SL_ intraB_CA_ITS_part2	Moderator(LGE)

- R4-2408952	Discussion on system parameter for intra-band non-contiguous SL CA in n47	Xiaomi

- R4-2410586	WF on NR_SL_ intraB_CA_ITS_part1	OPPO

- R4-2410587	WF on NR_SL_ intraB_CA_ITS_part2	Huawei

RAN4 #110bis contributions

- R4-2404505	On sidelink intra-band contiguous CA in PC2	Huawei, HiSilicon

- R4-2404506	On sidelink intra-band contiguous/non-contiguous CA in PC3 and PC2	Huawei, HiSilicon

- R4-2404542	Consideration of leftover issue of NR sidelink WI in Rel-18 and scope in Rel-19	Meta Ireland

- R4-2404547	Discussion on sidelink intra-band non-contiguous CA with PC3 and PC 2 in n47	Xiaomi

- R4-2404548	Discussion on sidelink intra-band contiguous CA with power class 2 in n47	Xiaomi

- R4-2404603	Discussion on UE RF requirements for SL intra-band non-contiguous CA	LG Electronics

- R4-2404604	Discussion on UE RF requirements for SL intra-band contiguous CA	LG Electronics

- R4-2404667	Discussion on UE RF requirements for intra-band non-contiguous CA	vivo

- R4-2404668	Discussion on UE RF requirements for intra-band contiguous CA	vivo

- R4-2405286	Topic summary for [110bis][133] NR_SL_ intraB_CA_ITS	Moderator(OPPO)

- R4-2405386	Initial discussion on Sidelink intra-band contiguous CA work	OPPO

- R4-2405387	Initial discussion on Sidelink intra-band non-contiguous CA work	OPPO

- R4-2405388	TR skeleton for intra-band sidelink CA	OPPO

- R4-2405389	Work Plan for Rel-19 sidelink	OPPO, LGE

- R4-2406599	TR skeleton for intra-band sidelink CA	OPPO

- R4-2406611	WF on NR_SL_ intraB_CA_ITS	OPPO, Meta