3GPP TSG RAN Meeting #105								                    RP-241845

Melbourne, Australia, September 9-12, 2024

## Status Report to TSG

Agenda item:		9.2.8

| WI / SI Name | Study on NR frequency range 2 (FR2) OTA (Over the Air) testing Phase 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_FR2_OTA_Ph3 |  |  |  |  |
| Unique ID | 1030074 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-241158 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: Sep/2025 | Core part: | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: 70% | Core part: | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Han, Bin |
|  | Company | Qualcomm Incorporated |
|  | Email | binhan@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#112

Issue 1-1-1: Signal level condition for measuring/distinguishing EIRP per TCI

- Agreement

  - RAN4 confirms the feasibility of measuring EIRP per TCI with the configuration of rank 2 PUSCH transmission under SDM scheme.

    - Up to a power difference of [40dB] is assumed.

    - Detailed MU analysis can be left to RAN5 if needed, taking into account the exact Test Case parameters.

Issue 1-2-1: AoA separation and UE orientation for EIRP PUMAX,f,c,k testing

- Agreement

  - Take below proposal as the starting point:

    - For the selection of AoA pair, one of AoA is at the beam peak direction of single CC operation without STxMP enabled (i.e., from Beam Peak Search for MOP in 6.2 of TS 38.101-2, the other AoA is at the direction decided by UE declared orientation (listed in TS38.101-2) and AoA pair selecting from set of {30deg, 60deg, 90deg, 120deg, 150deg}.

      - FFS whether a test direction other than the beam peak direction of single CC operation without STxMP enabled can be used for first AoA

    - Due to lack of absolute probe location definition of the multi-AoA UE RF test system, defined in Clause 5.2.2 of TR38.871, FFS on whether or how such test system can guarantee the same relative angular orientations between specific UE test directions and probes.

Issue 1-2-2: Applicability of ΔMPRSTxMP, MPR and A-MPR for EIRP PUMAX,f,c,k testing

- Agreement

  - The MPR/A-MPR does not need to be verified in STxMP testing as ΔMPRSTxMP = 3dB applies on top of legacy MRR/A-MPR for single panel.

Issue 1-2-3: Measurement grid for EIRP PUMAX,f,c,k testing

- Agreement

  - Re-use the legacy measurement grid in Annex M.2 of TS38.521-2 for the beam peak search of single CC operation without STxMP enabled; FFS whether a grid relaxation for this TC could be considered.

Issue 1-3-1: AoA separation and UE orientation EIRPmax testing

- Agreement

  - The test directions selected in EIRP PUMAX,f,c,k testing can be reused if the feasibility of the solution in Issue 1-2-1 is confirmed.

Issue 1-3-2: Additional margin for skipping rule for EIRPmax testing

- Agreement

  - Further discuss additional margin considering the range of [0.5dB, 2dB].

  - Encourage companies to provide technical Analysis and/or simulation results.

Issue 1-4-1: AoA separation and UE orientation TRPmax testing

- Agreement

  - The test directions selected in EIRP PUMAX,f,c,k testing can be reused if the feasibility of the solution in Issue 1-2-1 is confirmed.

Issue 1-4-2: Additional margin for skipping rule for TRPmax testing

- Agreement

  - Further discuss additional margin considering the range of [0.5dB, 2dB].

  - Encourage companies to provide technical Analysis and/or simulation results.

Issue 1-4-3: Measurement grid for TRPmax testing

- Agreement

  - RAN4 consider the legacy PC1 and PC5 measurement grid for TRP defined in Annex M.4 of TS38.521-2 as the baseline; FFS whether a grid relaxation for this TC could be considered.

#### 2.4.2	Remaining Open issues

The remaining issues need to be solved:

- Confirm the baseline solution for EIRP PUMAX,f,c,k testing, EIRPmax testing and TRPmax testing

- Confirm the measurement grid based on the baseline solution

- Define additional margin for skipping the rule

- Test procedure

- Preliminary uncertainty assessments

## 4.	References

RAN4 #112

- R4-2413229	VSAT testability SID proposal							     Eutelsat Group

- R4-2413263	Skeleton of TR 38.xyz: Study on NR frequency range 2 (FR2) OTA (Over the Air) testing Phase 3												            Qualcomm Incorporated

- R4-2411531	Discussion on STxMP measurement limitations				Rohde & Schwarz

- R4-2411698	On FR2 OTA testing of STxMP							               Samsung

- R4-2412072	Discussion on the test method for sTxMP						          vivo

- R4-2412919	Discussion on FR2 OTA test method on STxMP			 Qualcomm Incorporated

- R4-2413198	RF testing methodology for FR2 non-handheld UE that can transmit simultaneously with multi-pane														                    Nokia

- R4-2413434	Topic summary for [112][334] NR_FR2_OTA			             Qualcomm Incorporated

- R4-2413534	Way Forward for [112][334] NR_FR2_OTA				  Qualcomm Incorporated

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template