3GPP TSG RAN Meeting #105								RP- 241852

Melbourne, Australia, September 9-12, 2024

Agenda Item:		9.3.1.5

Source:			Nokia

Title:			Updates on Enhancements of Network energy savings for NR

Document for:		Discussion and Decision

# 1	Introduction

In RAN#102 the work item on Enhancements of Network energy savings for NR has been approved [1], which included a checkpoint for decision in RAN#105 regarding the study on-demand SIB1 (OD-SIB1) for UEs in RRC idle/inactive mode. In this contribution we present our views on the conclusions from the study and the proposed way forward.

# 2	On-demand SIB1 for UEs in RRC idle/inactive mode

The objective on-demand SIB1 (OD-SIB1) for UEs in idle/inactive mode is defined as [1]:

2.	Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3]

•	Triggering method by uplink wake-up-signal using an existing signal/channel.

•	Wake-up-signal configuration provisioning to UE

-	Note: No modification of SSB will be discussed under this objective

•	Information exchange between gNBs at least for the configuration of wake-up signal, if necessary.

•	Checkpoint for normative work in RAN#105

The work has been carried out in RAN1 and RAN2 assuming three cases, as described below and illustrated in Figure 1:

- Case 1: also called standalone case, where UE receives WUS configuration from NES cell, following the UL WUS transmission for OD-SIB1 requesting to NES cell, and receives the OD-SIB1 also from NES cell.

- Case 2: UE receives WUS configuration from Cell A, but UE transmits UL WUS signal to NES cell for OD-SIB1 request and receives the OD-SIB1 also from NES cell.

- Case 3: similar to Case 2, but here the Cell A (normally the coverage cell) apart from providing the UL configuration to the UE, it also receives the UL WUS requesting of OD-SIB1 from the UE, and in addition Cell A will be responsible for providing the OD-SIB1 of NES cell to the UE.

| - Case 1 | - Case 2 | - Case 3 |
| --- | --- | --- |

Figure 1: Potential cases for Network Energy Savings, as studied by RAN WGs.

In RAN1#118 (August 2024) the following agreements and conclusions on these cases [2]:

Agreement

For on-demand SIB1 in idle/inactive mode, Case 3 (Option 2+B+Y) is feasible from RAN1 perspective for some scenarios. Case 3 (Option 2+B+Y) is lower priority compared to Case 2 from RAN1 perspective.

- RAN1 is inconclusive on whether the required specification support is justified by the observed NES gain for Case 3

Conclusion

For on-demand SIB1 in idle/inactive mode, RAN1 was not able to achieve consensus to support Case 1 (Option 1+A+X) in Rel-19, while Case 1 is technically possible from RAN1’s perspective.

Finally, RAN1 was able to make the following agreement regarding the recommendation for specification work in Rel-19 on OD-SIB1:

Agreement

RAN1 recommends specifying on-demand SIB1 only for Case 2 (Option 1+B+X) in Rel-19.

- Note: RAN1 strive to minimize impact to legacy UE.

- Note: RAN1 specification impact to support this feature should be minimized.

Similarly, RAN2#127 (Aug. 2024) reached the following agreement regarding OD-SIB1 [3]:

OD-SIB1 SI conclusion:

- RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

RAN3#125 (Aug. 2024) reached the following agreement regarding OD-SIB1 [4]:

On-demand SIB1:

- Case 2 can be supported with the below details:

-UL WUS configuration is decided by NES Cell DU.

-UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface.

-Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). FFS if Cell A broadcasts it right away or not.

Hence, given the common agreement from RAN1, RAN2 and RAN3 regarding normative work of Case 2 for on-demand SIB1, and relatively lower priority, lower depth of analysis, and/or lack of consensus for the other cases, we make the following proposal:

Proposal: Proceed with normative work on Case 2 only for on-demand SIB1 for UEs in RRC idle/inactive mode in Rel-19.

# Conclusion

In this contribution we have discussed RAN1’s conclusion and propose the following:

Proposal: Proceed with normative work on Case 2 only for on-demand SIB1 for UEs in RRC idle/inactive mode in Rel-19.

# References

[1] RP-241650, “Revised WID for Enhancements of network energy savings for NR,” June 2024.

[2] “Chair notes RAN1#118”, Aug. 2024.

[3] “Report from session on V2X/SL, R18/19 MOB, and R19 NES”, Vice Chairman (Samsung), Aug. 2024

[4] RP-241701, Status Report RAN WG3, RAN3 chair