3GPP TSG RAN Meeting #105	RP-241855

Melbourne, Australia, September 9-12, 2024

Agenda item:	9.3.2.1

Source:	Nokia

Title:	On the Revisions to NR Mobility Enhancements Phase 4 Work Item

Document for:	Discussion and Decision

# 1	Introduction

In Rel-18, lower layer-triggered mobility (LTM) was introduced for intra-CU mobility scenarios, with the aim of reducing handover delay and interruption time by enabling early sync to candidate cells, L1 measurement-based cell switch and subsequent cell changes without RRC signalling from the network to the UE. The Rel-19 work item (WI) on NR mobility enhancements, as defined in [1], focuses on lifting certain limitations of the Rel-18 LTM feature by:

- Extending the feature to inter-CU scenarios with single and dual connectivity

- Enhancing the measurement and reporting framework through:

  - Enabling event-triggered L1 measurement reporting to reduce the reporting overhead, as Rel-18 LTM is based on periodic L1 measurement reporting

  - Supporting CSI-RS measurements, in addition to the already supported SSB measurements

- Improving robustness of LTM cell switch by supporting conditional LTM, where the network configures the UE with a condition, which the UE evaluates and if the condition is met, the UE can switch to a target cell.

For the latter enhancement, i.e., support of conditional LTM, a checkpoint to review this objective at RAN#105 has been set and no RAN WG work has been done till this point. More specifically, the agreed objective reads as follows [1]:

- Specify support of conditional LTM [RAN2, RAN3, RAN1]

  - Specify UE evaluated conditions for triggering LTM

  - Aim to support conditional LTM including subsequent LTM

  - Prioritise intra-CU LTM

  - Checkpoint to review objective at RAN#105. RAN WG work to not start before this checkpoint

In this paper, we advocate for the support of conditional LTM and provide details of the foreseen related work within the mobility enhancements WI.

Secondly, in RAN1#118, it became apparent that companies have differing views on the aspects that should be covered under the objective for CSI-RS-based measurements. Therefore, it is crucial to clearly define the scope of this objective during RAN#105. In this paper, we present our perspective on the aspects to be included for this objective.

# 2	Discussion

## 2.1	Motivation for Conditional LTM

In Rel-18 LTM, which, as described above, focuses on reducing the handover delay and interruption time, the LTM cell switch is a network-triggered procedure. More specifically, based on the measurement reports provided from the UE to the network, the source DU sends a cell switch command (CSC) MAC CE to the UE to instruct it to perform a cell switch to a specific (already prepared during the LTM preparation phase) candidate/target cell. Depending on UE capability support, Rel-18 LTM procedure also includes early uplink synchronisation and downlink synchronisation procedure that helps in reducing the interruption time for switching.

Nevertheless, the reliability of L2 based switching command under rapidly degrading source cell is not addressed in Rel-18 LTM. In such cases, if UE cannot receive the CSC, it may lead to RLF and further recovery procedure via another LTM attempt or re-establishment procedure. The mobility robustness introduced by Rel-16 conditional handover (CHO) can mitigate this problem. In CHO, the UE is configured with execution condition at early stages of mobility when the source cell is in good radio condition and UE evaluates and executes the handover based on the conditions provided by the network being met.

A similar mechanism can be applied to LTM, resulting in the so-called conditional LTM. By configuring the UE with conditional execution conditions for the candidate cells during the LTM preparation phase, the UE can still execute a cell switch to a candidate cell for which the conditional execution condition has been satisfied, even if the quality of the link to source gNB is too low for a successful reception of a CSC MAC CE. This enhancement would enable the LTM feature to provide not only reduced interruption time, but also improved robustness, combining the benefits of existing mobility mechanisms. Taking into consideration that LTM will be also extended to inter-CU scenarios (also a part of Rel-19 WI), this will result in a single powerful mobility solution that will be applicable to all deployment scenarios.

### 2.1.1 Detailed conditional LTM objectives

The WID already describes the objectives of the conditional LTM work. Here, we discuss these objectives in detail and provide our proposals for the required work and extensions:

- Specify UE evaluated conditions for triggering LTM taking the early UL and DL synchronisation into account.

The events to be configured to the UE for conditional LTM cell switch must be defined. To this end, the reference signals to be measured must be determined. More specifically, apart from SSBs, the use of CSI-RSs, if concluded to be supported in Rel-19 in a timely manner, should be considered for conditional cell switch triggering. Furthermore, we should consider if any other measurement quantities, apart from RSRP, i.e., RSRQ, SINR, should be used for triggering. Another important aspect is to ensure that the events are defined so that we avoid an unfavourable situation where the UE autonomously “jumps” from cell to cell with too short stays at each cell. Hence, we should consider whether L1 and/or L3 measurements should be used and the corresponding filtering, if any, as well as an appropriate time to trigger (TTT). To avoid duplication and potential conflicts, we note that the work on event definition should follow and take as baseline the work done under work on event-triggered measurement reporting within the Rel-19 work item.

A further aspect to be considered regarding triggering conditional LTM is the early synchronization status of the UE. Ideally, to achieve reduced interruption via RACH-less cell switch, the UE should be already in sync with the target cell when the cell switch decision is made, but this may not always be possible. Therefore, necessary work on how the early sync status of the UE with the target cell impacts the conditional LTM cell switch is foreseen.

Given that reduced interruption time is a core benefit of the LTM feature, it is our view that RACH-less conditional LTM should be supported. A solution for early TA acquisition that is suitable for conditional LTM must be specified. In Rel-18 LTM, the TA computed at the candidate target DU based on PDCCH ordered random access preamble transmission from the UE is provided to the source DU via the CU and the source DU provides it to the UE with the CSC. For conditional LTM, the legacy procedure is not directly applicable, as the UE does not receive a CSC to perform a cell switch, but rather autonomously triggers a cell switch when a network-configured condition is met. Therefore, a solution to provide the TA to the UE before the cell switch must be specified. Relevant solutions considering TA provision from the source DU or from the candidate target DU were already studied for Rel-18 LTM and can be used as a baseline. A decision on the preferrable solution is required, taking into account inter-CU-related aspects.

In addition, in Rel-18 LTM, maintenance of the RACH-based TA was naturally left to network implementation, as only the network was aware of the estimated TA value before the cell switch. Since the TA is expected to be available at the UE before the cell switch, considerations on acquired TA value are warranted. Furthermore, the triggering mechanism for both early DL and UL synchronization must be specified. More specifically, we should analyse mobility robustness benefit any new solution brings to the UE, compared to the legacy Rel-18 procedure.

Regarding resource provision, we should consider whether enhancement of the legacy Rel-18 procedures is required for provision of PRACH resources, as well as PUSCH resources for the first UL transmission to the target cell in RACH-less conditional LTM.

For all the above points, including conditional cell switch triggering conditions, early sync and resource provision, a design that allows for subsequent cell switches without the need for reconfiguration of the UE by the network is necessary.

Finally, as for the rest of the objectives of the WI, RAN4 work to specify RRM requirements related to conditional LTM is to be conducted. For conditional LTM this would mean at least conditional cell switch delay requirements and, depending on the early UL/DL synchronization design, RRM requirements for those procedures.

- Do not support subsequent conditional LTM but support subsequent LTM after conditional LTM.

Subsequent LTM working together with conditional LTM may create more harm than benefits for the network to track the UE. Especially, for inter-CU scenario when the UE may do two subsequent conditional inter-CU LTM executions, the network may not be able to complete the handover completion, e.g., path switch in time. Similarly for intra-CU scenario before a handover success like message goes to source DU and completes the LTM procedure the UE may move on to another DU. The case where subsequent conditional LTM will be beneficial is the case where the network will be late to send two MAC CEs consequently. For that reason, we think it is beneficial not to support subsequent conditional LTM but to support subsequent LTM after conditional LTM.

- Prioritise intra-CU LTM

The WID states that intra-CU LTM should be prioritized to ensure proper specification of the feature within the limited available time. We should analyse and evaluate the specification effort required to support inter-CU conditional LTM in addition to intra-CU conditional LTM, taking also into account the potential consequences, issues and specification effort of coexistence with inter-CU (network-controlled) LTM, if only intra-CU conditional LTM is supported.

- Prioritise conditional MCG LTM, conditional SCG LTM only if it is limited to intra-CU LTM.

We think that supporting conditional LTM for both MCG and SCG LTM may be achieved if the conditional LTM is limited to intra-CU LTM. Inter-CU SCG LTM may be left out of the scope of this WI to keep the workload at a manageable level.

Proposal 1: Agree to include support of conditional LTM in Rel-19 WI on NR mobility enhancements Phase 4 with following sub-objectives:

- Specify UE evaluated conditions for triggering conditional LTM including the support for early UL and DL synchronization

- Support conditional LTM including subsequent LTM

  - Subsequent conditional LTM is not supported, but subsequent LTM after conditional LTM is supported

- Prioritise intra-CU LTM

- Prioritise conditional LTM for MCG

- Conditional LTM for SCG, if defined, is restricted to intra-CU scenario

## 2.2 Objective for CSI-RS based Measurements

Although the current WID objective includes specifying support for CSI-RS measurements for LTM procedures, CSI-RS-based beam management, and other necessary physical layer operations on candidate cells before LTM, discussions in RAN1#118 revealed divergent views among companies regarding the aspects that should be addressed under this objective. Therefore, it is crucial to clearly define the scope of this objective in RAN#105.

It’s important to note that in Rel-18, SSB-based measurements on candidate cells could be sufficient for handing over the UE to another cell using coarse DL synchronization, lowest MCS selection, and a RACH-less switch. Features like fine DL synchronization, appropriate link adaptation, and narrow beam selection can be enabled later in the target cell through additional measurements and reporting from the UE. Therefore, in Rel-19, the primary motivation for enabling CSI-RS-based measurements is to further enhance LTM procedures with advanced features such as fine DL synchronization, narrow beam selection, and CSI-based link adaptation, immediately after the cell switch, which are only achievable with CSI-RS measurements on candidate cells. Implementing CSI-RS-based measurements will, however, introduce additional configuration overhead to provide CSI-RSs from multiple candidate cells, as well as increased measurement overhead on the UE side. If the objective is restricted to a limited set of features, it may undermine the potential benefits of CSI-RS-based enhancements and decrease the likelihood of widespread adoption in actual deployments.

Therefore, we support the following enhancements with CSI-RS-based measurements and reporting:

- CSI-RS-Based DL Synchronization: Enables finer and faster DL synchronization due to the larger bandwidth and more frequent (lower periodicity) transmissions of CSI-RSs compared to SSBs. If time permits, the impact of UL synchronization (TA acquisition) can be studied.

- CSI-RS-Based Beam Management: Facilitates narrow beam selection, particularly for DL and UL transmit beams.

  - Note: Since UE Rx beam selection using CSI-RSs with repetition may require support for aperiodic CSI-RSs from a candidate cell, further discussion is needed on whether this support is necessary.

- CSI Acquisition: Supports the acquisition of CSI information (e.g., CQI, PMI, RI) for the target cell’s channel, enabling appropriate link adaptation immediately after the cell switch.

Observation 1: In RAN1#118, companies had divergent views regarding the aspects that should be addressed under the objective for CSI-RS based measurements; therefore, it is crucial to clearly define the scope of this objective in RAN#105.

Observation 2: SSB-based measurements on candidate cells could be sufficient for handover to another cell using coarse DL synchronization and RACH-less switch. Therefore, limiting the objective for CSI-RS-based measurements to a few features could reduce the likelihood of this feature being adopted for deployment.

Proposal 2: To ensure comprehensive support for CSI-RS-based measurements and reporting in LTM, the following aspects should be included to study and specify in the objective:

- CSI-RS based measurements and reporting, both periodic and event-triggered L1 measurement reporting

- CSI-RS based DL synchronization including the study on the impact on UL synchronization

- CSI-RS based beam management, at least for DL and UL Tx beam selection

- Acquisition of CSI information (e.g. CQI, PMI, RI) for the target cell to enable appropriate link adaptation directly after LTM cell switch

# 3	Conclusions

In this contribution, we discussed the motivation for supporting conditional LTM as a part of Mobility Enhancements in Rel-19 and provided details of the foreseen required work. We also discussed the aspects that should be addressed under the objective of CSI-RS based measurements. The following is proposed:

Proposal 1: Agree to include support of conditional LTM in Rel-19 WI on NR mobility enhancements Phase 4 with following sub-objectives:

- Specify UE evaluated conditions for triggering conditional LTM including the support for early UL and DL synchronization

- Support conditional LTM including subsequent LTM

  - Subsequent conditional LTM is not supported, but subsequent LTM after conditional LTM is supported

- Prioritise intra-CU LTM

- Prioritise conditional LTM for MCG

- Conditional LTM for SCG, if defined, is restricted to intra-CU scenario

Proposal 2: To ensure comprehensive support for CSI-RS-based measurements and reporting in LTM, the following aspects should be included to study and specify in the objective:

- CSI-RS based measurements and reporting, both periodic and event-triggered L1 measurement reporting

- CSI-RS based DL synchronization including the study on the impact on UL synchronization

- CSI-RS based beam management, at least for DL and UL Tx beam selection

- Acquisition of CSI information (e.g. CQI, PMI, RI) for the target cell to enable appropriate link adaptation directly after LTM cell switch

Proposal 3: Adopt the WID changes shown in the Annex.

# References

- RP-241515, Revised Work Item: NR mobility enhancements Phase 4, 3GPP TSG RAN Meeting #104, June 17-20, 2024

# Annex – Revised WID

We suggest the following changes to the Objective section of the WID (modifications based on RP-241515):

# 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI - Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1] - CSI-RS based measurements and reporting, both periodic and event-triggered L1 measurement reporting - CSI-RS based DL synchronization including the study on the impact on UL synchronization - CSI-RS based beam management, at least for DL and UL Tx beam selection - Acquisition of CSI information (e.g. CQI, PMI, RI) for the target cell to enable appropriate link adaptation directly after LTM cell switch - Specify support of conditional LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering conditional LTM including the support for early UL and DL synchronization - Support conditional LTM including subsequent LTM - Subsequent conditional LTM is not supported, but subsequent LTM after conditional LTM is supported - Prioritise intra-CU LTM - Prioritise conditional LTM for MCG - Conditional LTM for SCG, if defined, is restricted to intra-CU scenario - Specify RRM requirements related to the above objectives as necessary [RAN4] ### 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. - Specify RRM performance requirements related to the core objectives as necessary [RAN4] ### 4.3 RAN time budget request (not applicable to RAN5 WIs/SIs) NOTE: For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings. The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI. One time unit (TU) corresponds to ~ 2 hours in the meeting. If no TU is needed, then leave the field empty otherwise enter a number >0 in the field. For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID. If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table. additional comments to the time budget request in the attached Excel table: