3GPP TSG-RAN3 Meeting #125	R3-244802

# Maastricht, Netherlands, 19-23 Aug, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0419 | rev | 3 | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of IAB-node authorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, ZTE, Samsung, Nokia, Nokia Shanghai Bell, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2024-08-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - For an IAB-node, the IAB donor-CU receives the authorizations from 5GC or from another IAB donor CU, when the authorization status is received for the first time and “not authorized”, the IAB-donor-CU neither establishes the backhaul resources nor allocates any BAP address, TNL address or default BAP configuration for this IAB-node. However, when the authorization status is not received for the first time, different action shall be taken, e.g. the IAB-donor-CU attempts to hand over the UEs served by the IAB-node to other cell(s), releases the F1 interface towards the IAB-DU, and releases all backhaul resources (including the BAP address, TNL address and default BAP configuration) for this IAB-node. As we can see, different actions are taken by the IAB donor CU for the two cases. However, the two cases when the authorization status is received are not distinguished in the current description in 38.401. - The current description for the case when MT is dual-connected with two donors is incomplete and incorrect. Based on current specification, when the IAB-MT is NR dual-connected, two cases are described separately, i.e. case 1: the non-F1-terminating IAB-donor-CU is the MN, case 2: the F1-terminating IAB-donor-CU is the MN. For case 2 (i.e. the MN is the F1-terminating IAB-donor-CU), it’s specified “upon reception of the authorization status, the IAB-node’s authorization procedure follows the same steps as described in clause 8.9.17.2.1.”. However, clause 8.9.17.2 and even 8.9.17.1 which is referred by clause 8.9.17.2 only describes the case when IAB-node connects to a single IAB-donor wherein some step (e.g., F1-terminating donor informing the non-F1 terminating donor that the corresponding operation (e.g. handover of UEs) has completed) is missing. Actually, these operations applies to all scenarios when the IAB node is served by two IAB-donors, not only to part of scenarios as described currently in 38.401. - It’s impossible for an IAB-node to connect with a gNB incapable of serving the IAB-MT. Thus, it’s not correct to use the term “non-IAB-capable gNB” in clause 8.9.17.2.2. - For the scenario of NR-DC with single IAB-donor, it is not appropriate to say the MN/SN is the IAB-donor-CU since the MN/SN comprises both CU and DU functionality. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that if the authorization status is received for the first time and the status is “not authorized”, the IAB-donor-CU neither establishes the backhaul resources nor allocates any BAP address, TNL address or default BAP configuration for this IAB-node. - Update the description for the case when the IAB-MT is NR dual-connected with two IAB-donors. - Remove the term “non-IAB-capable” in clause 8.9.17.2.2. - Replace “IAB-donor-CU” with “IAB-donor” in IAB authorization chapters. Impact analysis Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). This CR has impact on the functional point of view, the impact can be considered isolated because it only impacts the IAB node authorization. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 2 description of IAB node authorization is incorrect and incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.9.17.2.1, 8.9.17.2.2, 8.9.17.3. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - |  |  |  |  |  |  |  |  |

| This CR's revision history: | - Rev 1 Merge the corresponding changes for IAB authorization to TS 38.401. - Rev 2 Remove the definition of “Non-IAB-donor gNB”. - Rev 3 Replace “IAB-donor-CU” with “IAB-donor” for authorization chapters. |
| --- | --- |

-------------------------------------------Start of change-------------------------------------------

### 8.9.17	IAB-node authorization

#### 8.9.17.1	IAB-node in NSA

During the IAB-node integration procedure, the eNB receives the authorization status of the IAB-node from the EPC. The eNB forwards the authorization status to the IAB-donor in the SGNB ADDITION REQUEST message. If the authorization status is “not authorized”, the IAB-donor neither establishes the backhaul resources nor allocates any BAP address, TNL address or default BAP configuration for this IAB-node.

When the authorization status for the IAB-node changes, the EPC sends an updated authorization status to the IAB-MT’s eNB. The eNB forwards the authorization status to the IAB-donor in the SGNB ADDITION REQUEST message or SGNB MODIFICATION REQUEST message.

In case the authorization status is “not authorized”, the IAB-donor performs the following actions in this order: it attempts to hand over the UEs served by the IAB-node to other cell(s), releases the F1 interface towards the IAB-DU, and releases all backhaul resources (including the BAP address, TNL address and default BAP configuration) for this IAB-node. In case the authorization status is changed back from “not authorized” to “authorized”, the phase 2 and phase 3 of the IAB-node integration procedure as defined in clause 8.12 are carried out.

8.9.17.2	IAB-node with single IAB-donor in SA

8.9.17.2.1	IAB-node is single-connected

During the IAB-node network integration or RLF recovery, the IAB-donor receives the authorization status of the IAB-node from the 5GC. Also, during the Inter-CU topology adaptation procedure, the target IAB-donor receives the authorization status of the IAB-node from the source IAB-donor as well as from the 5GC when performing the Path Switch Request procedure. If the authorization status is received for the first time by the IAB-donor and if the status is “not authorized”, the IAB-donor neither establishes the backhaul resources nor allocates any BAP address, TNL address or default BAP configuration for this IAB-node. When the authorization status for the IAB-node changes, the 5GC sends an updated authorization status to the IAB-donor. When the authorization status received by the IAB-donor changes, the IAB-donor performs the SA equivalent of the steps described for NSA in clause 8.9.17.1.

8.9.17.2.2	IAB-node is NR dual-connected

In case the IAB-node is dual-connected and the two parent nodes connect to the same IAB-donor, the IAB-donor receives the authorization status of the IAB-node from the 5GC. Upon reception of the authorization status, the IAB-donor performs the equivalent of the steps described for NSA in clause 8.9.17.1.

In case the IAB-node is dual-connected to a gNB and to an IAB-donor, the MN receives the authorization status of the IAB-node from the 5GC. If the MN is the gNB and the SN is the IAB-donor, the MN forwards the authorization status to the SN in the S-NODE ADDITION REQUEST message or S-NODE MODIFICATION REQUEST message. Upon reception of the authorization status, the IAB-donor performs the SA equivalent of steps described for NSA in clause 8.9.17.1. If the MN is the IAB-donor and the SN is the gNB, the IAB-donor performs the SA equivalent of the steps described for NSA in clause 8.9.17.1.

8.9.17.3	IAB-node is served by two IAB-donors in SA

In case the IAB-MT only connects to the non-F1-terminating IAB-donor or in case the IAB-MT is NR dual-connected with the non-F1-terminating IAB-donor as the MN, the non-F1-terminating IAB-donor sends the authorization status received from the 5GC to the F1-terminating IAB-donor in the IAB-TRANPORT MIGRATION MODIFICATION REQUEST message.

In case the IAB-MT only connects to the non-F1-terminating IAB-donor or in case the IAB-MT is NR dual-connected, upon reception of the authorization status, the F1-terminating IAB-donor performs the equivalent of steps described for NSA in clause 8.9.17.1. If the authorization status is “not authorized”, the F1-terminating IAB-donor sends to the non-F1-terminating IAB-donor an IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message requesting the release of all offloaded traffic, after which the non-F1-terminating IAB-donor releases the offloaded traffic and all backhaul resources, BAP address, TNL address and default BAP configuration for the IAB-node.

-------------------------------------------End of change------------------------------------------