3GPP TSG-RAN3 Meeting # 125 	R3-244670

# Maastricht, Netherlands, 19th – 23rd Aug. 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1460 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on configuration update for an inactive Multicast session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, CATT, Ericsson, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_enh-Core |  |  |  |  |  | Date: |  |  | 2024-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on functions specified in TS 38.300 (16.10.5.2) and corresponding RRC protocol support in TS 38.331, a gNB is able to configure UEs with PTM configuration via the RRCRelease message, and UE is able to apply the configuration upon session activation (e.g., through enhanced group paging). Information necessary for this “pre-configuration” is partly provided by the gNB-DU and may be updated by gNB-DU through Multicast Context Notification. However in current specification, there is a restriction on when this procedure can be applied, i.e., only “during an active multicast MBS session”, which is not necessary. Given the above mentioned specificaton status in TSs 38.300 and 38.331, there is no need to for such limitation in F1AP, i.e., network should be able to configure UE with the latest configuration, e.g., whenever network finds the previous pre-configuration is not valid anymore, besides, it is probably more adviseable to not perform such configuration change during an active multicast session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the restriction on when Multicast Context Notification applies (currently limited to active session only). Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary protocol limitation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.14.12.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev. 1: update coversheet on typos. |  |  |  |  |  |  |  |  |

// change starts.

### 8.14.12	Multicast Context Notification

#### 8.14.12.1	General

The purpose of the Multicast Context Notification is to inform the gNB-CU about changes in the multicast context configuration.

The procedure uses MBS-associated signalling.

// change ends.