3GPP TSG RAN WG1 #105		                                       RP-241902
Melbourne, Australia, September 9-12, 2024

Agenda Item:	9.3.1.3

Source:	InterDigital, Inc.

Title:	On the checkpoints for Rel-19 AI/ML for NR Air Interface

Document for:	Discussion/Decision

# Introduction

The Rel-19 WID on AI/ML for NR Air Interface [1] has the following study objective on CSI feedback enhancement, UE data collection, and model transfer/delivery with checkpoints in RAN#105:

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. - For CSI prediction (one-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843 (e.g., cell/site specific model could be considered to improve performance gain). - Necessity and details of model Identification concept and procedure in the context of LCM [RAN2/RAN1] - CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods - Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis NOTE: offline training is assumed for the purpose of this project. NOTE: the outcome of the study objectives should be captured in TR 38.843 for future reference. NOTE: Coordination with SA/SA WGs of the ongoing study/work as it may relate to their required work.

In this contribution, we share our views on the study objectives with checkpoints based on the observations from the additional study and the next step of the study objectives for the rest of Rel-19.

# Discussion

## CSI compression

The study of AI/ML CSI compression use case has been extended from Rel-18 due to the lack of performance gain, feasibility/complexity of the inter-vendor collaboration, as well as RAN4 testability issue. RAN1 has studied further on AI/ML CSI compression including improving trade-off between performance and complexity/overhead with new compression schemes (e.g., spatial/temporal/frequency compression), investigating feasible inter-vendor training collaboration options in real deployment scenario. Due to the limited progress and lack of observations, RAN1 has agreed to continue the study AI/ML CSI compression as following [2]:

Agreement According to the Rel-19 study objective, - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.. RAN1 is recommending extending the study of the AI/ML CSI compression based on RAN1 understanding the study on CSI compression is not completed.

Considering that there are still several remaining issues to finish the study and draw conclusions such as CSI compression schemes (e.g., Case 2 and Case 3), further details of inter-vendor training collaboration options, and required specification impacts, it is recommended to continue the study on AI/ML CSI compression for the rest of Rel-19 and no scope change is required for the CSI compression part.

Proposal 1: 	Continue study AI/ML CSI compression for the rest of Rel-19 and no scope change is required.

## CSI prediction

The study of AI/ML CSI prediction use case has been extended from Rel-18 as well since RAN1 couldn’t make a meaningful observation for the AI/ML CSI prediction in terms of performance gain over non-AI/ML CSI prediction scheme. Only few companies provided evaluation results on that case and the results were quite diverging to make observations and draw conclusions.

In the Rel-19 study phase, additional evaluation results were collected and the following observations have been made [2]:

Observation Based on the evaluation for CSI prediction, the following high-level observations are provided: - From the perspective of basic performance gain over benchmark of non-AI/ML based CSI prediction, under the same UE speed for training and inference - for AI/ML based CSI prediction over non-AI/ML based CSI prediction, 0%~7.8% gain depending on traffic model, in terms of mean UPT, is observed by 7 sources - for AI/ML based CSI prediction over non-AI/ML based CSI prediction, 3.8%~20.7% gain depending on traffic model, in terms of 5% UE UPT, is observed by 7 sources. - From a perspective of AI/ML complexity, 19 sources adopt the model subject to the computational complexity in units of FLOPs from 0.05M to 3000M. For complexity of non-AI/ML benchmark, 16 sources adopt the algorithm (e.g., Kalman filter, Auto-regression, Wiener filter) subject to the computational complexity in units of FLOPs from 0.14M to 107M. The ratio of FLOPs (AI/ML over benchmark 2) ranges from 1 to 35, which is used by majority sources. For non-AI/ML benchmark, main computation complexity is dominated by filter updates, which may not be need to be updated per inference at the expense of performance loss. - From the perspective of performance impact on channel estimation error and phase discontinuity, compared to non-AI/ML CSI prediction, higher gain is observed by 10 sources for AI/ML based CSI prediction in the presence of channel estimation error or phase discontinuity. - From the perspective of generalization over various scenarios/configurations (e.g., various UE speed, deployment scenario, carrier frequency) that have been evaluated, compared to generalization Case 1 where the AI/ML model is trained with dataset subject to a certain scenario#B/configuration#B and applied for inference with a same scenario#B/configuration#B - For generalization Case 2 where the AI/ML model is trained with dataset from a different scenario#A/configuration#A, generalized performance may be achieved for some certain combinations of scenario#A/configuration#A and scenario#B/configuration#B but not for others. - For generalization Case 3 where the training dataset is constructed with data samples subject to multiple scenarios/configurations including scenario#B/configuration#B, generalized performance of the AI/ML model can be achieved.

From the additional study in Rel-19, although the evaluation results are provided with very limited number of companies and the results are diverging, it has been observed that the performance gain from AI/ML CSI prediction is marginal (i.e., 0%~7.8% for mean UPT) while it requires up to 35 times higher UE complexity compared with the benchmark 2 (i.e., non-AI/ML CSI prediction) under the scenario where AI/ML CSI prediction shows the best performance over the benchmark 2.

Observation 1: the performance gain from AI/ML CSI prediction is marginal while it requires excessive UE complexity (i.e., up to 35 times) compared to non-AI/ML CSI prediction.

The non-AI/ML based CSI prediction has been introduced in Rel-18 under MIMO topic with its reporting schemes and CSI-RS enhancement. Given that CSI prediction scheme itself is up to UE implementation, using implementation-based AI/ML CSI prediction should be considered as a benchmark that needs to be compared to justify the standards efforts/specification impacts for AI/ML CSI prediction. The AI/ML model generalization study can be considered as an investigation to see if implementation-based AI/ML CSI prediction could work properly or not. As seen by the observation, AI/ML model for CSI prediction generalized very well across multiple scenarios/configuration with very marginal loss, which implies that a UE vendor can use a generalized AI/ML CSI prediction model instead of non-AI/ML CSI prediction scheme without any additional specification impact which will provide almost similar performance with the standardized AI/ML CSI prediction scheme.

Observation 2: UE implementation based AI/ML CSI prediction scheme can be used instead of non-AI/ML CSI prediction scheme with the CSI prediction reporting scheme specified in Rel-18.

RAN1 currently has two use cases in normative phase such as AI/ML positioning and beam management but the progress is behind the schedule and RAN2 couldn’t even start some of LCM discussion for AI/ML positioning due to the lack of inputs from RAN1. As such, having additional use case for normative work in Rel-19 make even worse to complete the WI on time.

Considering that the performance gain from AI/ML CSI prediction scheme is marginal while requiring significantly increased UE complexity, UE implementation based AI/ML CSI prediction is also possible without specification efforts, and current progress on normative work for two use cases (positioning and BM) is behind the schedule, it is difficult to justify the additional normative work for CSI prediction use case.

Proposal 2: 	AI/ML CSI prediction is not pursued for normative work in Rel-19.

# Summary

In this contribution, the following observations were made:

Observation 1: the performance gain from AI/ML CSI prediction is marginal while it requires excessive UE complexity (i.e., up to 35 times) compared to non-AI/ML CSI prediction.

Observation 2: UE implementation based AI/ML CSI prediction scheme can be used instead of non-AI/ML CSI prediction scheme with the CSI prediction reporting scheme specified in Rel-18.

Based on the observations, the following proposal is made:

Proposal 1: 	Continue study AI/ML CSI compression for the rest of Rel-19 and no scope change is required.

Proposal 2: 	AI/ML CSI prediction is not pursued for normative work in Rel-19.

# References

RP-234039, New WID on Aritificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, December 2023

Chair notes, RAN1#118, Aug 2024