3GPP TSG RAN Meeting #105																				RP-241903

Melbourne, Australia, September 9-12, 2024

Agenda Item:		9.3.1.1

Source:				InterDigital, Inc.

Title:					On the Scope of Rel-19 NR MIMO Ph5

Document for:	Discussion and Decision

# Introduction

For NR Rel-19, the MIMO WID describes five main objectives as the main enhancements for DL/UL MIMO operation [1]. Given the progress made by the RAN1, companies have proposed additional topics for potential consideration for the remaining time left in Rel-19.

In this contribution, we discuss and share our views on the proposed topics for potential scope expansion of NR MIMO in Rel-19.

# Background and Motivation

Based on our understanding, there are 6 additional topics that are being considered for inclusion in Rel-19 MIMO. Table 1 captures the proposed additional topics under each of the existing objectives. Inclusion of all proposed topics may require an increase in TUs, hence some discussion for justification and prioritization of each topic may be needed.

Table 1 – Proposed topics for scope expansion for Rel-19 NR MIMO Ph5

| Sub agenda | Proposed topics for scope expansion |
| --- | --- |
| Objective 1: Enhancements for UE-initiated/event-driven beam management | - NA |
| Objective 2 & 3: CSI enhancements | - SRS port grouping for TDD low-complexity 6/8RX receiver |
| Objective 4: Support for 3-antenna-port codebook-based transmissions | - 3T6R SRS antenna switching - Non-codebook-based transmission - Partial-coherent codebook transmission - UL PRG / Precoder cycling |
| Objective 5: Enhancement for asymmetric DL sTRP/UL mTRP scenarios | - 2TA without CoresetPoolIdx association for asymmetric DL sTRP/UL mTRP |

CSI Enhancement

Low complexity 6/8 RX for TDD-based transmission has been proposed, wherein the transmission of each CW is according to an independent CSI determined per SRS port group [2]. In our view, if there exist a strong market demand for such category of UEs, then it would be worthwhile to specify enabling mechanisms, including, SRS port grouping.

Proposal 1: Support SRS port grouping for TDD low-complexity 6/8RX receiver, if it is justified by market needs.

Support for 3-antenna-port codebook-based transmissions

For the recently introduced 3TX UE, there are four main proposals for potential consideration. In our view, while all the proposed topics are beneficial, given the limited time left for Rel-19, some may have a higher priority than others. For example, SRS antenna switching, and non-codebook-based uplink transmission are among basic UE functionalities that is supported by every other UE category, i.e., 2/4/8 TX UEs, and hence they should be also supported by a 3TX UE [3].

Proposal 2: For a 3TX UE, support following basic UE functionalities,

- antenna switching cases 3T3R and 3T6R

- non-codebook-based uplink transmission

The main motivation for introducing the class of 3TX UE was to create a low-cost UE category, therefore the initial focus was solely on non-coherent UE architectures. However, as shown in Figure 1, there are different ways that a 3 TX UE could be implemented. For a cross-polarized antenna setup, the UE could be considered partially coherent for which a more performing codebook should be considered [3]. However, to assure the timely completion of the related specification work, some design restriction may be needed. For example, as the case of non-coherent codebook, the TPMI field should not exceed 3 bits.

Proposal 3: For a 3TX UE, support partial-coherent codebook for UL MIMO, with the assumption that the TPMI field does not exceed 3 bits.

Figure 1 - Co- and cross-polarized antennas

To enhance UL performance at medium and high speed, subband-based open loop precoder cycling is proposed [4]. To enable subband-based precoding, gNB should be aware of the subband size of the precoding, so that the phase continuity of channel estimation can be maintained. Therefore, the focus of this work would be on defining the number of precoders for the scheduled band which could be through configuration of UL PRG (precoding granularity). Since UL subband precoding is a new functionality, it may require some adjustment in the required TUs.

Proposal 4: If time allows, for a 3TX UE, support UL subband precoder cycling.

Enhancement for asymmetric DL sTRP/UL mTRP scenarios

A typical deployment of asymmetric DL sTRP/UL mTRP consists of at least one TRP with both downlink and uplink transmission capabilities, and at least one receive-only point (ROP). Extending the functionality of Rel-18 two TA features in this area will expand the feature applicability to more deployments. In our opinion, having this feature considered as an extension to mTRP intra-cell with two TAs will be beneficial.

Proposal 5: Support extending Rel-18 mTRP feature with two TAs for the asymmetric UL TRP operation in Rel-19.

# Conclusions

In this contribution, we discuss and share our views on the proposed topics for potential scope expansion of NR MIMO in Rel-19. Based on the discussion, we make the following proposals,

Proposal 1: Support SRS port grouping for TDD low-complexity 6/8RX receiver, if it is justified by market needs.

Proposal 2: For a 3TX UE, support following basic UE functionalities,

- antenna switching cases 3T3R and 3T6R

- non-codebook-based uplink transmission

Proposal 3: For a 3TX UE, support partial-coherent codebook for UL MIMO, with the assumption that the TPMI field does not exceed 3 bits.

Proposal 4: If time allows, for a 3TX UE, support UL subband precoder cycling.

Proposal 5: Support extending Rel-18 mTRP feature with two TAs for the asymmetric UL TRP operation in Rel-19.

# References

- RP-234007, “New WID: NR MIMO Phase 5”, 3GPP TSG RAN Meeting #102, December 2023

- RP-233660, “MIMO evolution in Rel-19”, Huawei, 3GPP TSG RAN Meeting #102, December 2023

- R1-2403850, “Summary of Offline Discussions on 3TX CB-based Uplink”, Moderator (InterDigital, Inc.), 3GPP WG1 Meeting #117, May 2024

- RP-233049, “Views on Rel-19 MIMO evoluation, Definition of PRG for PUSCH, Vivo, Deutsche Telekom, Spreadtrum , Lenovo, 3GPP TSG RAN Meeting #102, December 2023