3GPP TSG RAN WG1 #105		                                	      RP-2401905
Melbourne, Australia, September 9-12, 2024

Agenda Item:	9.3.1.5

Source:	InterDigital, Inc.

Title:	Discussion on on-demand SIB1 for UEs in idle/inactive mode

Document for:	Discussion/Decision

# Introduction

The Rel-19 WID on enhancements of network energy savings for NR [1] has the following study objective on on-demand SIB1 (OD-SIB1) for UEs in idle/inactive mode:

- Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105

In RAN1#118 [2], the following agreement was made recommending specifying OD-SIB1:

Agreement RAN1 recommends specifying on-demand SIB1 only for Case 2 (Option 1+B+X) in Rel-19. - Note: RAN1 strive to minimize impact to legacy UE. - Note: RAN1 specification impact to support this feature should be minimized.

In RAN2#127 [3], the following conclusion was made on OD-SIB1:

RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

In this contribution, we discuss the observations from the study and the potential scope for normative work in Rel-19 on OD-SIB1 for UEs in idle/inactive mode.

# Discussion

## Evaluations of NES gains on NES Cell with OD-SIB1

In the study phase, companies provided evaluation results showing considerable NES gains on NES cell for different cell load conditions and SIB1 periodicities. For example, in RAN1#117 [4], when using 20ms periodicity for SSB and SIB1 in low load scenario, Cat 2 BS and 8 beams, the range of NES gain observed by 6 sources is 16.8% - 28.77%. For the same setting and when using 160ms periodicity for SIB1, the range of NES gain observed by 4 sources is reduced to 3.04% - 5.57%. The full evaluation results provided by companies are listed in the Appendix [4].

From the analysis of the evaluation results, clearly the achievable NES gains are due to not transmitting SIB1 on the NES cell, where higher gains are possible when using a shorter SIB1 periodicitity (e.g. 20ms). For energy savings, the NES cell can resort to transmitting SIB1 only when there is need, where the need can be signalled by the UE in a UL WUS indication to request for SIB1 on on-demand basis.

Observation 1: 	Significant amount of NES gains (e.g. up to 28.77% for 20ms SIB1 periodicity and up to 5.57% for 160ms SIB1 periodicity) are achievable on NES Cell when supporting OD-SIB1 for UEs in idle/inactive mode.

Additionally, considerable progress was made in RAN1 and RAN2 on the on different aspects of OD-SIB1 transmision including the deployment options, UL WUS configuration provisioning procedure and the design options for OD-SIB1 [2][3][4][5].

## Deployment cases for OD-SIB1 transmisison

In the study phase, different deployment cases for OD-SIB1 transmission were discussed and the following agreement was made in RAN1#116bis [5]:

Agreement For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 focuses its studies on the following cases: - Case 1: Option 1+A+X - Case 2: Option 1+B+X - Case 3: Option 2+B+Y Where the options 1/2/A/B/X/Y are defined below: - On target cell of UL WUS transmission: - Option 1: UE transmits UL WUS to NES Cell - Option 2: UE transmits UL WUS to Cell A - On configuration provision for UL WUS transmission - Option A: UE obtains the UL WUS configuration from NES Cell - Option B: UE obtains the UL WUS configuration from Cell A - On receiving of SIB1 - Option X: UE receives on-demand SIB1 from NES Cell - Option Y: UE receives on-demand SIB1 from Cell A

In Case 1 (corresponding to standalone case for NES cell), the UE in idle/inactive mode may receive SSB from the NES cell that it wants to camp on but the cell may not be broadcasting SIB1. The UE needs to receive SIB1 to be able to camp on or perform random access on this NES cell. In such case, the UE may receive OD-SIB1 from the NES cell by transmitting the UL-WUS to the NES cell.

In Case 1, the NES cell is unable to provide the UL WUS configuration to UE via legacy means due to lack of always-on SIB1. To address this, different approaches were discussed such as predefining UL WUS configuration or defining a new MIB. Although some NES gains are possible with Case 1, most companies raised concerns regarding the potentially large spec impact for provisioning the UL WUS configuration. Based on the discussion, the following conclusion was made in RAN1#118 regarding Case 1:

Conclusion For on-demand SIB1 in idle/inactive mode, RAN1 was not able to achieve consensus to support Case 1 (Option 1+A+X) in Rel-19, while Case 1 is technically possible from RAN1’s perspective.

Observation 2: 	In Case 1, large spec impact is expected for provisioning UL WUS configuration directly on NES cell

In Case 2 and Case 3, the idle/inactive UEs may be in coverage of at least one anchor cell (Cell A) and one or many NES cells. The functionalities related to camping and initial access for the idle/inactive UEs can be supported with the assistance from Cell A, which can continue to transmit common signals/channels, including SIB1.

In Case 2, the UE transmits UL WUS to the NES cell and receives OD-SIB1 from the NES cell. The UL WUS configuration, including PRACH preamble and time/frequency resources, are provided by the Cell A. In this case, minimal impact at Cell A is expected for provisioning the UL WUS configuration by reusing aspects of PRACH resource provisioning procedure. Similarly, minimal impact is expected at the NES cell for receiving UL WUS and transmitting OD-SIB1 by resuing aspects of the RACH procedure.

Observation 3: 	In Case 2, the potential spec impact is minimized based on the following:

  - At Cell A, the procedure for provisioning UL WUS configuration reuses aspects of legacy PRACH resource provisioning procedure

  - At NES Cell, the UL WUS reception and OD-SIB1 transmission reuses aspects of legacy PRACH procedure

Based on discussion regarding feasibility, practicalty and spec impact on Case 2, the following agreement was made in RAN1#117 [4]:

Agreement For SIB1 in idle/inactive mode, prioritize RAN1 discussions on Case 2 and Case 3 - Case 2 (Option 1+B+X) is feasible from RAN1 perspective. - Further study Case 3, focusing on the additional NES benefits over Case 2, feasibility, complexity, and spec impact.

In Case 3, only Cell A is used for the transmission of UL WUS and reception of OD-SIB1. Although in Case 3 the energy savings may be slightly higher compared to Case 2 at the NES Cell (due to not transmitting OD-SIB1), the usability of Case 3 may be limited only to deployments where both Cell A and NES Cell may be collocated. When Cell A and NES Cell are non-collocated, for example, the idle/inactive UE located in the coverage of the NES Cell needs to transmit UL WUS with higher Tx power towards a far located Cell A. Due to possibly worse UL coverage conditions to Cell A, higher probability of preamble failures are more likely in Case 3 when transmitting UL-WUS.

Additionally, in deployments where the Cell A caters for multiple NES Cells, SIB1 and PRACH overhead/capacity is a concern, as each SIB can be large (e.g. up to 3 Mbits) and when multiple NES cells are supported, such overhead is scaled linearly. Further, the energy consumption at the Cell A can be much higher due to monitoring of UL-WUS from all UEs and for transmitting OD-SIB1s of multiple NES Cells. To ensure that all Cell As surrounding the NES cell have the same NES OD-SIB1 configuration, there is a requirement for inter-gNB signalling to exchange the same full SIB1 contents, which requires RAN3 support.

Also, if the UE is camped over the NES cell and needs to re-acquire SIB1 or when the UL WUS config is no longer valid, the UE may need to re-select to Cell A, acquire SIB1 and then camp on NES cell again. This will result in multiple rounds of switching and increased UE power consumption. As such, transmitting UL WUS directly to and receiving OD-SIB1 from the NES cell as in Case 2 is more practical than in Case 3.

Observation 4: 	In Case 3, large spec impact is expected for addressing the following issues:

  - High preamble failures and high UE Tx power during UL WUS transmission to Cell A due to potentially worse UL coverage when the UE is in better coverage of the NES cell

  - High SIB overhead and energy consumption at the Cell A due to transmitting OD-SIB1s of multiple NES Cells

  - SIB1 content exchange over the Xn interface is needed to ensure all Cell As provide the same SIB1 for a given NES cell, thus requiring RAN3 impacts

  - High energy consumption at UE due to multiple rounds of switching and cell (re)selection between Cell A and NES cell during SIB1/UL WUS update

Based on the discussion related to the NES gains and potential spec impacts for Case 2 and Case 3, the following agreement was made in RAN1#118:

Agreement For on-demand SIB1 in idle/inactive mode, Case 3 (Option 2+B+Y) is feasible from RAN1 perspective for some scenarios. Case 3 (Option 2+B+Y) is lower priority compared to Case 2 from RAN1 perspective. - RAN1 is inconclusive on whether the required specification support is justified by the observed NES gain for Case 3

## Potential scope of normative work for OD-SIB1

In RAN1#118, a recommendation was made for specifying OD-SIB1 only for Case 2 (Option 1+B+X) in Rel-19. This was based on the study on the NES gains achievable and the analysis of pros and cons of all deployment cases [6], where it was found that Case 2 is preferred by most companies and incurs the least spec effort (per observations in earlier section). In this regard, by adopting only Case 2 without the other additional cases for normative work the potential spec impact is minimized (as per the note in RAN1 recommendation).

In addition to minimizing the spec impact, another important aspect to focus is to minimize the impact to legacy idle/inactive UE when supporting OD-SIB1 (as per the first note in RAN1 recommendation). In this regard, multiple options were considered including, for example, NES cell that uses certain SSB types (e.g. SSBs not on sync raster) or MIB-based cell barring to prevent legacy UEs from performing initial cell selection on NES cell. Such info on NES cell can be provided by Cell A only to Rel-19 UEs (e.g. in the UL WUS configuration) to enable requesting OD-SIB1 when detecting the NES cell. The impacts on legacy UEs are expected to be minimal in Case 2 since the UEs are either barred or unable to detect the NES cell. Other approaches to minimize the impact to legacy UEs can be discussed further in the normative phase.

In the study of OD-SIB1 for Case 2, all sub-objectives under the existing study objective are found to be relevant and applicable for normative. Based on this reasoning, and the recommendations from RAN1/RAN2, we propose the following scope for normative work.

Proposal 1: 	Support the following scope for normative work on OD-SIB1 for UEs in idle/inactive mode:

- Study Specify procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode only for Case 2, including: [RAN1/2/3]

  - Triggering method by uplink wake-up-signal using an existing signal/channel.

  - Wake-up-signal configuration provisioning to UE

    - Note: No modification of SSB will be discussed under this objective

  - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary.

  - Note: In Case 2 UE obtains the UL WUS configuration from Cell A, UE transmits UL WUS to NES Cell and UE receives on-demand SIB1 from NES Cell

# Summary

In this contribution, the following observations were made:

Observation 1: 	Significant amount of NES gains (e.g. up to 28.77% for 20ms SIB1 periodicity and up to 5.57% for 160ms SIB1 periodicity) are achievable on NES Cell when supporting OD-SIB1 for UEs in idle/inactive mode.

Observation 2: 	In Case 1, large spec impact is expected for provisioning UL WUS configuration directly on NES cell

Observation 3: 	In Case 2, the potential spec impact is minimized based on the following:

  - At Cell A, the procedure for provisioning UL WUS configuration reuses aspects of legacy PRACH resource provisioning procedure

  - At NES Cell, the UL WUS reception and OD-SIB1 transmission reuses aspects of legacy PRACH procedure

Observation 4: 	In Case 3, large spec impact is expected for addressing the following issues:

  - High preamble failures and high UE Tx power during UL WUS transmission to Cell A due to potentially worse UL coverage when the UE is in better coverage of the NES cell

  - High SIB overhead and energy consumption at the Cell A due to transmitting OD-SIB1s of multiple NES Cells

  - SIB1 content exchange over the Xn interface is needed to ensure all Cell As provide the same SIB1 for a given NES cell, thus requiring RAN3 impacts

  - High energy consumption at UE due to multiple rounds of switching and cell (re)selection between Cell A and NES cell during SIB1/UL WUS update

Based on the observations, the following proposal is made:

Proposal 1: 	Support the following scope for normative work on OD-SIB1 for UEs in idle/inactive mode:

- Study Specify procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode only for Case 2, including: [RAN1/2/3]

  - Triggering method by uplink wake-up-signal using an existing signal/channel.

  - Wake-up-signal configuration provisioning to UE

    - Note: No modification of SSB will be discussed under this objective

  - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary.

  - Note: In Case 2 UE obtains the UL WUS configuration from Cell A, UE transmits UL WUS to NES Cell and UE receives on-demand SIB1 from NES Cell

# Appendix: Agreeements on evaluations of NES gains with OD-SIB1

Agreement For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 20ms SIB1 period (Case A), FR1, empty load - For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 39.37% - 47.69% from the following 7 sources - R1-2404463, R1-2405595, R1-2405341, R1-2404859, R1-2404507, R1-2405363, R1-2404625 - One source (R1-2405162) reports 25% NES gain with PRACH configuration index = 152 - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98 - 24.6% with one beam of SIB1 PDSCH transmitted in each slot - 17.4% with two beams of SIB1 PDSCH transmitted in each slot - One source (R1-2404122) reports the following with FDMed SSB and PDSCH of SIB1 - 9.27%~18.94% NES gain with UL WUS configuration transmitted in legacy SIB on Cell A - 0%~9.66% NES gain with UL WUS configuration transmitted in separated SIB on Cell A. The separated SIB is TDMed with legacy SIB and transmitted in a 20ms period using dedicated resource. - 0% NES gain with Case 1 (Options 1+A+X) and WUS configuration transmitted in separate SIB on the NES cell - 13.25% NES gain with Case 1 (Options 1+A+X) and pre-configured for fixed WUS configuration - NES cell does not transmit the WUS configuration - For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 22.1% - 42.6% from the following 5 sources - R1-2404408, R1-2405341, R1-2404758, R1-2405363, R1-2404625 - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98 - 16.8% with one beam of SIB1 PDSCH transmitted in each slot - 10.9% with two beams of SIB1 PDSCH transmitted in each slot - For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, - the NES gain is 30.43% - 45.79% from the following 4 sources - R1-2404463, R1-2404859, R1-2405363, R1-2404625 - One source (R1-2405162) reports the following NES gain with PRACH configuration index = 152 - 25% NES gain with Case 3 (Options 2+B+Y) assuming the UE can camp on the NES cell and the SIB1 is only transmitted in a single beam corresponding to the UL WUS beam - 23% with Case 2 (Options 1+B+X) - For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, - the NES gain is 21.79% - 33.11% from the following 2 sources - R1-2405363, R1-2404625 - For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 20.5% - 39.34% from the following 8 sources - R1-2404463, R1-2403942, R1-2405341, R1-2404224, R1-2404859, R1-2404507, R1-2404561, R1-2405595 - One source (R1-2404224) reports 11.9% NES gain with medium cell load on Cell A - For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 19.31 – 19.39% from one source (R1-2405341) - For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, - the NES gain is 29.19% - 36.87% from the following 3 sources - R1-2404463, R1-2403942, R1-2404859 - One source (R1-2404561) reports 15.87%~20.18% NES gain with PRACH configuration index = 148 - For Cat 1 BS, 4/8 beams, 4% < on-demand SIB1 transmission rate < 24%, the NES gain is 17.98%~24.48% from one source (R1-2404408). Agreement For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 160ms SIB1 period (Case C), FR1, empty load - For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 2.21 – 10.6% from the following 10 sources - R1-2405595, R1-2403979, R1-2405341, R1-2404859, R1-2405162, R1-2404122, R1-2404507, R1-2405363, R1-2404625, R1-2404561 - For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 1.7% - 6.23% from the following 5 sources - R1-2403979, R1-2405341, R1-2405363, R1-2404625, R1-2404758 - One source (R1-2404758) reports 28.9% NES gain with Case 3 where no PRACH and paging is activated for the NES cell - For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.75% - 8.48% from the following 6 sources - R1-2405106, R1-2405162, R1-2404859, R1-2405363, R1-2404625, R1-2404561 - For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.01% - 5.8% from the following 3 sources - R1-2405106, R1-2405363, R1-2404625 - For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 3.24% - 7.76% from the following 5 sources - R1-2403942, R1-2405341, R1-2404859, R1-2404507, R1-2404561 - For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 2.96% from one source (R1-2405341) - For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 2.5% - 7.44% from the following 3 sources - R1-2404561, R1-2403942, R1-2404859 Agreement For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 20ms SIB1 period (Case A), FR1, low load - For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 30.56% - 41.8% from the following 5 sources - R1-2405595, R1-2405341, R1-2404859, R1-2405363, R1-2404625 - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98 - 13.9% with one beam of SIB1 PDSCH transmitted in each slot - 7.9% with two beams of SIB1 PDSCH transmitted in each slot - For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 16% - 36.2% from the following 5 sources - R1-2405247, R1-2405341, R1-2404033, R1-2405363, R1-2404625 - One source (R1-2404033) reports 10% NES gain with FDMed SSB and PDSCH of SIB1 - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98 - 5.2% with one beam of SIB1 PDSCH transmitted in each slot - 1.9% with two beams of SIB1 PDSCH transmitted in each slot - For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, - the NES gain is 22.44% - 37.64% from the following 3 sources - R1-2404859, R1-2405363, R1-2404625 - One source (R1-2404122) reports the following with FDMed SSB and PDSCH of SIB1 - 4.73%~9.64% NES gain with UL WUS configuration transmitted in legacy SIB on Cell A - 0%~4.8% NES gain with UL WUS configuration transmitted in separated SIB on Cell A. The separated SIB is TDMed with legacy SIB and transmitted in a 20ms period using dedicated resource. - 0% NES gain with Case 1 (Options 1+A+X) and WUS configuration transmitted in separate SIB on the NES cell - 7% NES gain with Case 1 (Options 1+A+X) and pre-configured for fixed WUS configuration - NES cell does not transmit the WUS configuration - For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, - the NES gain is 13.17% - 36.2% from the following 3 sources - R1-2404625, R1-2405247, R1-2405363 - For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 16.8% - 28.77% from the following 6 sources - R1-2403942, R1-2405341, R1-2404224, R1-2404859, R1-2404561, R1-2405595 - One source (R1-2404224) reports 10.5% NES gain with medium cell load on Cell A - For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 12.71% – 12.88% from one source (R1-2405341) - For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 14.43% - 27.47% from the following 3 sources - R1-2404561, R1-2403942, R1-2404859 Agreement For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 160ms SIB1 period (Case C), FR1, low load - For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 1.2 – 8.3% from the following 7 sources - R1-2405595, R1-2403979, R1-2405341, R1-2404859, R1-2405363, R1-2404625, R1-2404561 - For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.2% - 5.46% from the following 4 sources - R1-2403979, R1-2405341, R1-2405363, R1-2404625 - For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 2.87% - 7.38% from the following 4 sources - R1-2404859, R1-2405363, R1-2404625, R1-2404561 - For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.97% - 4.48% from the following 2 sources - R1-2405363, R1-2404625 - For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 3.04% - 5.57% from the following 4 sources - R1-2403942, R1-2405341, R1-2404859, R1-2404561 - For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 1.79% from one source (R1-2405341) - For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 2.12% - 5.26% from the following 3 sources - R1-2404561, R1-2403942, R1-2404859 Agreement For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 20ms SIB1 period (Case A), FR1, medium load - For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 9.88% - 14.4% from the following 2 sources - R1-2405341, R1-2405363 - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98 - 4.5% with one beam of SIB1 PDSCH transmitted in each slot - 2.3% with two beams of SIB1 PDSCH transmitted in each slot - For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 4.92% - 8.4% from the following 2 sources - R1-2405341, R1-2405363 - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98 - 1.5% with one beam of SIB1 PDSCH transmitted in each slot - 0.5% with two beams of SIB1 PDSCH transmitted in each slot - For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, - the NES gain is 11.5% - 12.88% from one source (R1-2405363) - One source (R1-2404122) reports the following with FDMed SSB and PDSCH of SIB1 - 1.94%~4% NES gain with UL WUS configuration transmitted in legacy SIB on Cell A - 0%~1.94% NES gain with UL WUS configuration transmitted in separated SIB on Cell A. The separated SIB is TDMed with legacy SIB and transmitted in a 20ms period using dedicated resource. - 0% NES gain with Case 1 (Options 1+A+X) and WUS configuration transmitted in separate SIB on the NES cell - 2.64% NES gain with Case 1 (Options 1+A+X) and pre-configured for fixed WUS configuration - NES cell does not transmit the WUS configuration - For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 6%~7.53% from one source (R1-2405363) - For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 8.08% - 15.47% from the following 4 sources - R1-2405341, R1-2404224, R1-2404561, R1-2403942 - For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 4.09% – 4.19% from one source (R1-2405341) - For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 10.78% - 15.3% from the following 2 sources - R1-2404561, R1-2403942 Agreement For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 160ms SIB1 period (Case C), FR1, medium load - For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.3 – 2.44% from the following 4 sources - R1-2403979, R1-2405341, R1-2405363, R1-2404561 - For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.1% - 1.3% from the following 3 sources - R1-2403979, R1-2405341, R1-2405363 - For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.24% - 2.26% from the following 2 sources - R1-2405363, R1-2404561 - For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 0.67% - 1.6% from one source (R1-2405363) - For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.95% - 4.13% from the following 3 sources - R1-2403942, R1-2405341, R1-2404561 - For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.48% from one source (R1-2405341) - For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.44% - 4.02% from the following 2 sources - R1-2404561, R1-2403942 Agreement For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 40ms SIB1 period (Case D), FR1, empty load - For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 24.51% - 31.3% from the following 6 sources - R1-2405595, R1-2405341, R1-2404859, R1-2404507, R1-2405363, R1-2404625 - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98 - 16.3% with one beam of SIB1 PDSCH transmitted in each slot - 10.6% with two beams of SIB1 PDSCH transmitted in each slot - For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 12.4% - 35.5% from the following 5 sources - R1-2404408, R1-2405341, R1-2404758, R1-2405363, R1-2404625 - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98 - 10.1% with one beam of SIB1 PDSCH transmitted in each slot - 6.1% with two beams of SIB1 PDSCH transmitted in each slot - For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 19.44% - 28.81% from the following 3 sources - R1-2404859, R1-2405363, R1-2404625 - For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 12.87% - 20.09% from the following 2 sources - R1-2405363, R1-2404625 - For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 11.77% - 25.46% from the following 5 sources - R1-2403942, R1-2405341, R1-2404859, R1-2404507, R1-2404561 - For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 10.69 – 10.74% from one source (R1-2405341) - For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, - the NES gain is 16.46% - 24.24% from the following 2 sources - R1-2403942, R1-2404859 - One source (R1-2404561) reports 8.92%~10.37% NES gain with PRACH configuration index = 148 Agreement For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 40ms SIB1 period (Case D), FR1, low load - For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 18.38% - 26.4% from the following 5 sources - R1-2405595, R1-2405341, R1-2404859, R1-2405363, R1-2404625 - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98 - 8.1% with one beam of SIB1 PDSCH transmitted in each slot - 4.3% with two beams of SIB1 PDSCH transmitted in each slot - For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 9.84% - 17.41% from the following 3 sources - R1-2405341, R1-2405363, R1-2404625 - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98 - 2.8% with one beam of SIB1 PDSCH transmitted in each slot - 1% with two beams of SIB1 PDSCH transmitted in each slot - For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 13.36% - 23.68% from the following 3 sources - R1-2404859, R1-2405363, R1-2404625 - For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 7.27% - 15.59% from the following 2 sources - R1-2405363, R1-2404625 - For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 10.48% - 17.72% from the following 4 sources - R1-2403942, R1-2405341, R1-2404859, R1-2404561 - For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 7.01% from one source (R1-2405341) - For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 7.7% - 17.05% from the following 3 sources - R1-2403942, R1-2404859, R1-2404561 Agreement For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 40ms SIB1 period (Case D), FR1, medium load - For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 5.07% - 7.8% from the following 2 sources - R1-2405341, R1-2405363 - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98 - 2.4% with one beam of SIB1 PDSCH transmitted in each slot - 1.2% with two beams of SIB1 PDSCH transmitted in each slot - For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 2.46% - 4.4% from the following 2 sources - R1-2405341, R1-2405363 - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98 - 0.7% with one beam of SIB1 PDSCH transmitted in each slot - 0.3% with two beams of SIB1 PDSCH transmitted in each slot - For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 5.62~7.01% from one source (R1-2405363) - For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 2.77% - 3.85% from one source (R1-2405363) - For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 4.08%~10.1% from the following 3 sources - R1-2403942, R1-2405341, R1-2404561 - For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 2.02%~2.09% from one source (R1-2405341) - For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 5.61% - 9.94% from the following 2 sources - R1-2404561, R1-2403942 Agreement For the evaluation of NES loss on cell A, the following is observed - NES loss of 0%~0.98% from the following 15 sources with UL WUS configuration transmitted in legacy SIB on Cell A (with different assumptions on SIB periodicity and cell loading): - R1-2404408, R1-2404463, R1-2403942, R1-2403979, R1-2405341, R1-2404224, R1-2404859, R1-2404758, R1-2405162, R1-2404122, R1-2404507, R1-2404033, R1-2405363, R1-2404625, R1-2404561 (for Case 2) - One source (R1-2405106) reports for the case with UL WUS configuration transmitted in separated SIB on Cell A where the separated SIB is TDMed with legacy SIB and transmitted in a 320ms period using dedicated resource. - 0.62%~2.65% NES loss for Case 2 (Options 1+B+X) with empty load - 0.73%~3.20% NES loss for Case 3 (Options 2+B+Y) with empty load - One source (R1-2404122) reports - 2.95%(medium load)~15.49%(empty load) NES loss with UL WUS configuration transmitted in separated SIB on Cell A - The separated SIB is TDMed with legacy SIB and transmitted in a 20ms period using dedicated resource. - 1.28%(medium load)~3.62%(empty load) NES loss with UL WUS configuration transmitted in legacy SIB on Cell A Agreement For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 20ms SIB1 period (Case A), FR1, 8 beams, on-demand SIB1 transmission rate > 30% - For Cat 1 BS, empty load, the NES gain is 37.77% from one source (R1-2404463) - For Cat 2 BS, empty load, - the NES gain is 8.9% - 28.71% from the following 3 sources - R1-2404224, R1-2404561, R1-2404463 - The NES gain is 1.47% from one source (R1-2404561) with 75% on-demand SIB1 transmission rate - For Cat 2 BS, low load, - the NES gain is 7.9% - 14.65% from the following 2 sources - R1-2404224, R1-2404561 - The NES gain is 0.42% from one source (R1-2404561) with 75% on-demand SIB1 transmission rate - For Cat 2 BS, medium load, - the NES gain is 6.2% - 10.4% from the following 2 sources - R1-2404224, R1-2404561 - The NES gain is 0.12% from one source (R1-2404561) with 75% on-demand SIB1 transmission rate Agreement For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 40ms SIB1 period (Case D), FR1, 8 beams, on-demand SIB1 transmission rate > 30% - For Cat 2 BS, empty/low/medium load, the NES gain is 3.77% - 5.97% from one source (R1-2404561) Agreement For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 160ms SIB1 period (Case C), FR1, 8 beams, on-demand SIB1 transmission rate > 30% - For Cat 1/Cat 2 BS, empty/low/medium load, the NES gain is 0.31%~3.33% from the following 2 sources - R1-2404561, R1-2404122

# References

RP-241650, Revised WID: Enhancements of network energy savings for NR, June 2024

Chair notes, RAN1#118, Aug 2024

Chair notes, RAN2#127, Aug 2024

Chair notes, RAN1#117, May 2024

Chair notes, RAN1#116bis, Apr 2024

R1-24xxxxx, FL summary for on-demand SIB1 in idle/inactive mode, MTK, Aug 2024