3GPP TSG-RAN Meeting #105	RP-241907

Melbourne, Australia, September 9th–12th, 2024

Agenda item:	9.3.2.3

Source:	Interdigital

Title:	Scoping the Rel-19 XR Enhancements for NR Ph3

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The NR XR Ph3 WID [1] describes checkpoints for RAN#105 based on the study and work done by the WGs:

- Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2]. NOTE: Check in RAN#105 (check also other WG involvement if needed). - Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. NOTE: LCP implementation complexity need to be taken into account when evaluating solutions. NOTE: Check in RAN#105

This contribution discusses the above topics for potential inclusion/scope expansion of NR XR Ph3 in Rel-19.

# 2	Discussion

## 2.1	Multi-modality support

According to SA1 [2], for immersive multi-modal VR applications, synchronization between different media components is critical in order to avoid having a negative impact on the user experience (i.e., viewers detecting lack of synchronization), particularly when the synchronization threshold between two or more modalities is less than the latency KPI for the application. Transmission of the coordinated flows (e.g., audio, visual and tactile) translate into a synchronization requirement between the flows. These types of synchronization are typically handled by the application, but awareness of the coordinated flows at the lower layers can also be beneficial to the user experience.

RAN2 discussed variety of solutions how multi-modality (MM) information could be made available to RAN/gNB, how it could be used, and what could be achieved by this information. However, concluding remarks have been few given the solutions depend on other working groups and their decisions on if making such information available from Core Network to the RAN is feasible with intended accuracy. Furthermore, it is not completely clear to what extent the information available at the UE could be exploited (similarly depends on other working groups). Regardless, RAN2 was able to make a working assumption on UE providing the MM information to gNB through UE assistance information procedure:

Working assumption: Regardless of SA2 decision, RAN2 can extend the UAI for multi-modal awareness at least for uplink QoS flows in Rel-19 XR, by having the UE report existence of multi-modality application and association information among QFIs to gNB.

To date, RAN2 couldn’t agree to go forward with any enhancements based on the MM information the UE provides to the gNB that would have impact to Uu interface. Some considerations were agreed what the gNB could do based on this information by implementation, e.g., admission control and QoS flow to DRB mapping.

While the specification impact could be seen limited in defining the UAI signaling support for the UE to provide MM awareness information to gNB, due to uncertainty of this information accuracy at this point in time (i.e., without information by the related working groups, like SA2/SA4) and lack of consensus in exploitation possibilities of this information in RAN2, the feature could be down-prioritized for the time being.

Proposal 1: Down-prioritize the work on multi-modality information and specify the UAI based on the RAN2 working assumption only if time allows.

## 2.2	Scheduling enhancements

For scheduling enhancements RAN2 agreed earlier to support configuration of additional priority for a logical channel which has delay-critical data to transmit while its “default priority” is lower than other logical channels. Additionally, progress on enhanced DSR (Delay Status Report) reporting has been made where network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG in the DSR. Furthermore, given the agreement to out scope the enhancement of LCP restrictions as candidate solution for the LCP enhancements, more detailed scoping of the scheduling enhancements can be made going forward for the normative work.

Proposal 2: Modify the WID wrt. scheduling enhancements as follows:

-	Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

-	Specify additional Logical Channel priority handling using delay/deadline information of packets;

-	Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for a LCG.

## 2.3	Other items in the WID

For other items in the WID that are already in the normative phase, namely RRM measurement gap handling [RAN1, RAN2, RAN4], RLC [RAN2], and NR-NR DC [RAN3] enhancements, the work is already in good pace in the working groups and scope changes seem not required at the present time.

Proposal 3: No scope change for other items in the WID is required.

# 3	Conclusion

This document has made the following observations and proposals:

Proposal 1: Down-prioritize the work on multi-modality information and specify the UAI based on the RAN2 working assumption only if time allows.

Proposal 2: Modify the WID wrt. scheduling enhancements as follows:

-	Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

-	Specify additional Logical Channel priority handling using delay/deadline information of packets;

-	Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for a LCG.

Proposal 3: No scope change for other items in the WID is required.

# References

[1]	RP-240791, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia

[2]	3GPP TS 22.261, Service requirements for 5G system