3GPP TSG RAN meeting #105 	RP-241908

Shanghai, China, June 17-20, 2024

Source: 	T-Mobile USA, Verizon, Telefónica, KT Corp, Orange, Telstra, Nokia, Interdigital Inc.

Title: 	AI/ML UE Sided data collection – September 2024 checkpoint

Agenda Item:	9.3.1.3

Document for:	Discussion/Decision

# 1 Introduction

In the “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” [1], there are several items listed as part of a SI with a checkpoint set for RAN#105 (September 2024).  One of the objectives included in the SI is:

- CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods

38.843 section 7.2.1.3.2 [2] listed several options with a note that “RAN2 did not study or analyze these proposals and did not agree to requirements or recommendations RAN2 couldn’t reach consensus on any of the items. RAN2 developed a table of the options.

# 2 Discussion

This section sets forth a set of data collection requirements and evaluates the options in [4] against these requirements.

## 2.1 Discussion on data collection requirements

R2-2405602 [3] contained a set of principles for AI/ML data collection this was supported by 11 operators and one vendor. These principles are:

1.	Data collected is secured.

2.	Safeguard user data confidentiality.

3.	Ensure data integrity.

4.	Minimize air interface overhead and impact to NW operation especially during high traffic periods.

5.	NW and operator control over and awareness of data collection.

6.	The collected dataset should be accessible to entities inside or outside the MNO network, with an SLA with the MNO

7.	Futureproof and extendable design

Subsequently RAN2 defined two key terms that are very important to operators, one is visibility, and the other is controllability.   The notes in the table below [4] defines “Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer.”

The agreed definition in [1] aligns with the operator requirement for objective 5.

The CR in [4] defines “Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA.”

It is critical from an MNO perspective that the MNO have full visibility which is simply the ability to access, manipulate and comprehend the data without the need of SLA

MNO Full visibility and Full controllability are critical for any standards framework supporting Network or UE side AI/ML data collection.

Observation 1: MNO Full visibility and Full controllability are critical for any standards framework supporting Network or UE side AI/ML data collection

Consolidating the most recent RAN2 discussions on visibility and controllability with the requirements in [3] results in the following data collection principles that the data collection options in [4] need to be evaluated:

1.	Data collected is secured.

2.	Safeguards user data confidentiality.

3.	Ensures data integrity.

4.	Minimizes air interface overhead and impact to NW operation especially during high traffic periods.

5.	The MNO network has full control of the data collection process for the training of a UE-sided model, including   the initiation, termination and management of the data collection and data transfer.

6.	MNO has full visibility as defined in TR38.843

7.	Futureproof and extendable design

Proposal 1: The following data collection requirements are used as the foundation for data collection development and evaluating options:

1.	Data collected is secured.

2.	Safeguards user data confidentiality.

3.	Ensures data integrity.

4.	Minimizes air interface overhead and impact to NW operation especially during high traffic periods.

5.	The MNO network has full control of the data collection process for the training of a UE-sided model, including   the initiation, termination and management of the data collection and data transfer.

6.	MNO has full visibility as defined in TR38.843

7.	Futureproof and extendable design

## 2.1 Results of RAN2 discussion

The following options were discussed in RAN2:

1a.	UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected.

1b.	UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO). FFS whether according to RAN2 there might be 3GPP specification impact.

2.	UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server. FFS whether according to RAN2 there might be 3GPP specification impact.

3.	UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server. FFS whether according to RAN2 there might be 3GPP specification impact.

38.843 Table 7.2.1.3.2-1 [4] is copied below for reference.

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer (Note 1) | No AI/ML specific controllability | FFS: level of controllability (Note 5) | Full controllability | Full controllability |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure or FFS other procedures | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS on level of visibility (Note 5) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |
| Impacted WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. - Note 2: Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content (e.g. UE proprietary information can be included transparently together with the standardized data message). - No standardized visibility (e.g. only UE proprietary information can be included transparently). - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: RAN2 cannot reach consensus on the level of MNO controllability and visibility possible via solution 1b without input from SA groups. - Note 6: RAN2 has not concluded on the need for partial and no visibility options. - Note 7: RAN2 could not reach consensus on the feasibility of UP solution in options 2 and 3. |  |  |  |  |

## 2.1 Discussion on Option 1a

Option 1a has no 3GPP impact and is dependent on proprietary implementations. It fails to meet all of the principles expressed in [3].  Multiple companies will collect the same data, customers will pay for the data, there are privacy concerns, and it doesn’t offer a standardized framework

This clearly shouldn’t be considered as an option for 3GPP.

Proposal 2: Remove option 1a from consideration for AI/ML data collection

## 2.2 Discussion on Option 1b

The difference between Option 1A and Option 1b is clear as mud, proponents state that this is based on TS 26.531 & TS 26.532. The basic architecture from TS 25.531 is shown below for reference:

There are several deficiencies with option 1b, most notably the lack of network control and no MNO control or MNO visibility.  From an MNO perspective option 1b is the same as option 1a only with standards-based interfaces replacing proprietary interfaces.

Observation 2: From an MNO perspective option 1b is the same as option 1a only with standards-based interfaces replacing proprietary interfaces.

Option 1b doesn’t have a clearly defined first termination entity, the termination entity can be inside or outside the MNO network.   this fails many of principles supported by the operators, specifically the controllability requirement. It requires an agreement between the UE vendor and operator for visibility and it also requires the MNO and the UE vendor to agree on the data to be collected. None of this comes close to standardizing data collection.

This solution also lacks any communication between the network and the UE needed for collection, transfer and data management. An example would be if the UE needed additional reference signals during data collection or the ability of the gNB to manage data transfer between the UE and the terminating server.

For the training of a UE-sided model, collecting the data from UEs without network control/awareness is highly inefficient, as a UE may not have the relevant/required information (e.g., UE not configured with the proper measurement configuration).[5]

Collecting large amount of data for the training of a UE-sided model transparently from the RAN/CN may lead to traffic congestion and the operator may not be able to ensure that the required QoS is provided to the UEs in the network.[5]

Proposal 3: Remove option 1b from consideration for AI/ML data collection

## 2.3 Discussion on Option 2

Option 2 leverages the AI/ML release 19 features being developed in SA and the existing functionality of NWDAF, but it requires SA to improve NWDAF’s data collation capability from a network node to support data collection at the UE.

As shown in the table above, option 2 meets all operator requirements, assuming option A for visibility is selected. Option B could be considered once a robust set of standardized metrics is defined for option A. We should avoid using a transparent container to replace option A. The cosigning companies prefer this option over option 3.

Proposal 4: Select Option 2 for normative work in R19

## 2.4 Discussion on Option 3

As shown in the table above, option 3 meets all operator requirements, assuming option A for visibility is selected. Option B could be considered once a robust set of standardized metrics is defined for option A. We should avoid using a transparent container to replace option A. The cosigning companies think this option requires more work over option 2, especially in SA5, and option 2 fits better into an overall RAN / CN framework, therefore option 3 is not the preferred choice.

Proposal 5: Select Option 3 for normative work in R19

## 2.5 Proposed WID objective for [1]

Based on the principles provided only option 2 and 3 meet these requirements, with this there needs to be an objective added to the WID to enable normative work for option 2 and 3.

- CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]:

  - Create solutions for option 2 and 3 as defined in TR 38.843 R19 [4] that meet the following data collection principles

    - Data collected is secured.

    - Safeguards user data confidentiality.

    - Ensures data integrity.

    - Minimizes air interface overhead and impact to NW operation especially during high traffic periods.

    - The MNO network has full control of the data collection process for the training of a UE-sided model, including the initiation, termination and management of the data collection and data transfer.

    - MNO has full visibility as defined in TR38.843

    - Futureproof and extendable design

  - Note: Option 2 is prioritized for R19

Proposal 6: Add the following objective to the AI/ML WID [1]

- CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]:

  - Create solutions for option 2 and 3 as defined in TR 843 R19 [4] that meet the following data collection principles

    - Data collected is secured.

    - Safeguards user data confidentiality.

    - Ensures data integrity.

    - Minimizes air interface overhead and impact to NW operation especially during high traffic periods.

    - The MNO network has full control of the data collection process for the training of a UE-sided model, including the initiation, termination and management of the data collection and data transfer.

    - MNO has full visibility as defined in TR38.843

    - Futureproof and extendable design

  - Note: Option 2 is prioritized for R19

Proposal 7: Send LS to SA and CT informing them of the RAN plenary decision to proceed with option 2 and 3

# 3 Conclusion

Achieving the benefits of AI/ML is impossible without good training data, this requires collecting substantial amounts of data from MNO networks. However, there is little incentive for MNOs to make this data available without a standardized data collection framework.

If 3GPP chooses to do nothing, as proposed in option 1a, it will result in a fragmented ecosystem where each company develops its own AI/ML data collection system that cannot be leveraged by the 3GPP ecosystem.

Based on the data collection principles set forth in proposal 1 it is clear that only option 2 meet all the requirements.

The following proposals are for approval:

Proposal 1: The following data collection requirements are used as the foundation for data collection development and evaluating options:

1.	Data collected is secured.

2.	Safeguards user data confidentiality.

3.	Ensures data integrity.

4.	Minimizes air interface overhead and impact to NW operation especially during high traffic periods.

5.	The MNO network has full control of the data collection process for the training of a UE-sided model, including   the initiation, termination and management of the data collection and data transfer.

6.	MNO has full visibility as defined in TR38.843

7.	Futureproof and extendable design

Proposal 2: Remove option 1a from consideration for AI/ML data collection

Proposal 3: Remove option 1b from consideration for AI/ML data collection

Proposal 4: Select Option 2 for normative work in R19

Proposal 5: Select Option 3 for normative work in R19

Proposal 6: Add the following objective to the AI/ML WID [1]

- CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]:

  - Create solutions for option 2 and 3 as defined in TR 843 R19 [4] that meet the following data collection principles

    - Data collected is secured.

    - Safeguards user data confidentiality.

    - Ensures data integrity.

    - Minimizes air interface overhead and impact to NW operation especially during high traffic periods.

    - The MNO network has full control of the data collection process for the training of a UE-sided model, including the initiation, termination and management of the data collection and data transfer.

    - MNO has full visibility as defined in TR38.843

    - Futureproof and extendable design

  - Note: Option 2 is prioritized for R19

Proposal 7: Send LS to SA and CT informing them of the RAN plenary decision to proceed with option 2 and 3

# References

[1] RP-2400774, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML)  for NR Air Interface”

[2] TR 38.843 R18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML)

for NR air interface”

[3] R2-2405602, “AI/ML Data Collection Requirements”, T-Mobile USA, Verizon, Charter, NTT DOCOMO, Deutsche Telekom, Turkcell, BT, AT&T, Nokia, Telecom Italia, CMCC – May 2024

[4] R2-2407209, “RAN2 inputs to TR 38.843” – August 2024

[5] R2-2407413. “Data collection for the training of a UE-sided model”, August 2024