# 3GPP TSG RAN Meeting #105      	                          RP-2401918
Melbourne, Australia, Sep 9 - 12, 2024

Agenda item:	9.3.1.3

Source:	Apple

Title:	View on Rel-19 UE-sided data collection

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

In RAN2#127 [1], UE-sided data collection was discussed and below table was agreed in [2] without conclusion.

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer | No AI/ML specific controllability | FFS: level of controllability (Note 5) (Note 1) | Full controllability (Note 1) | Full controllability (Note 1) |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure, FFS other procedures | Via RRC procedure |
| Possible Options for data content visibility in MNO (Note 2, Note 3) | No standardized visibility | FFS on level of visibility (Note 5) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |
| Impacted WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. - Note 2: Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content (e.g. UE proprietary information can be included transparently together with the standardized data message). - No standardized visibility (e.g. UE proprietary information can only be included transparently). - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: RAN2 cannot reach consensus on the level of possible MNO controllability and visibility via Option 1b without input from SA groups. - Note 6: RAN2 has not concluded on the need for partial and no visibility options. - Note 7: RAN2 cannot reach consensus on the feasibility of UP tunnel in Options 2 and 3 |  |  |  |  |

In this contribution, we provide our view on normative work of Rel-19 UE-side data collection.

# 2 Discussion

## 2.1 Analysis of Option 1a

How Option 1a works is illustrated in Figure.1.

Figure.1 Illustration of how Option 1a works

Use case

Among all the identified 4 solutions, we think option 1a is feasible for all use cases of Rel-19 because this solution has been widely deployed in industry.

Observation 1: Option 1a (i.e. OTT 3GPP transparent) works for all use cases of Rel-19 and it has been widely deployed in industry.

MNO controllability and visibility

During RAN2 discussion, there were some concerns on whether MNOs have controllability and visibility on Option 1a, which is worth clarification. In existing TR 38.843 [2], it was captured that “no AI/ML specific controllability” and “No standardized visibility” because Service Level Agreement (SLA) is out of RAN2 scope as captured in the TR:

Note:	RAN2 discussed that, except for the case of standardized data content, the data content visibility and any level of controllability could be achieved via SLA (Service Level Agreement). However, SLA is out of RAN2 scope.

Actually, as usual business, the MNOs can interact with chipset vendors in offline engineering and achieve controllability and visibility via SLA.

Observation 2: In TR 38.843, Option 1a was captured that “no AI/ML specific controllability” and “No standardized visibility” because Service Level Agreement (SLA) is out of RAN2 scope. As usual business, the MNOs can interact with chipset vendors in offline engineering and achieve controllability and visibility via SLA.

Privacy risk

As nowadays, the UE itself is responsible to protect data privacy and proprietary implementation. To comply with local regulations, some user consent request/provision procedure is needed at application level, but it is out of 3GPP standardization scope.

Efforts for deployment

RAN2 agreed that option 1a has no specification impacts, and thereby no extra efforts for its deployment

Agreements

1	Solution 1a has no specification impact.

## 2.2 Analysis of Option 1b

Option 1b is illustrated in Figure.2.

Figure.2 Illustration of how Option 1b works

As discussed in RAN2, option 1b is intended to enhance the Event Exposure (EVEX) framework specified in TS 26.531 [3], where the Data Collection Application Function (DCAF) can be considered as “the server for data collection for UE-side model training”.

Use case

In our understanding, the use case of option 1b is mainly targeted to support one chipset vendor collecting UE data from multiple its OEM vendors.

MNO controllability and visibility

In TR 38.843 [2], it was captured that RAN2 didn’t reach consensus on its level of controllability and level of visibility, as illustrated by Note 5:

- Note 5: RAN2 cannot reach consensus on the level of possible MNO controllability and visibility via Option 1b without input from SA groups.

Our understanding is that option 1b can achieve limited MNO controllability and visibility through the OpenAPI specification rather than 3GPP specification. However, full MNO controllability and visibility can’t be achieved if without SLA because the OpenAPI specification typically only provides parameters’ type and value range but without detailed coding rules. Below is an example from TS 29.571 [4]. Let us use PacketDelBudget as example. As we see, OpenAPI specification only provides its data type (i.e. Unsigned integer) and value range (i.e. Minimum = 1). But it doesn’t specify the detailed coding rule, which means the coding rule is vendor specific based on offline engineering. Thus, different MNO may have different coding rule to represent each value of the delay budget parameter. We don’t think it is full MNO visibility on data contents.

5.5.2 Simple Data Types This clause specifies common simple data types. Table 5.5.2-1: Simple Data Types Type Name; Type Definition; Description. Omit something here. PacketDelBudget; integer; Unsigned integer indicating Packet Delay Budget (see clauses 5.7.3.4 and 5.7.4 of 3GPP TS 23.501 [8])), expressed in milliseconds. Minimum = 1.. PacketDelBudgetRm; integer; This data type is defined in the same way as the "PacketDelBudget" data type, but with the OpenAPI "nullable: true" property.. PacketErrRate; string; String representing Packet Error Rate (see clause 5.7.3.5 and 5.7.4 of 3GPP TS 23.501 [8]), expressed as a "scalar x 10-k" where the scalar and the exponent k are each encoded as one decimal digit. Pattern: '^([0-9]E-[0-9])$' Examples: Packer Error Rate 4x10-6 shall be encoded as "4E-6". Packer Error Rate 10-2 shall be encoded as "1E-2".. PacketErrRateRm; string; This data type is defined in the same way as the "PacketErrRate" data type, but with the OpenAPI "nullable: true" property.. PacketLossRate; integer; Unsigned integer indicating Packet Loss Rate (see clauses 5.7.2.8 and 5.7.4 of 3GPP TS 23.501 [8]), expressed in tenth of percent. Minimum = 0. Maximum = 1000.. PacketLossRateRm; integer; This data type is defined in the same way as the "PacketLossRate" data type, but with the OpenAPI "nullable: true" property.. AverWindow; integer; Unsigned integer indicating Averaging Window (see clause 5.7.3.6 and 5.7.4 of 3GPP TS 23.501 [8]), expressed in milliseconds. Minimum = 1. Maximum = 4095. Default = 2000...

Given above analysis, we believe that full MNO controllability and visibility of Option 1b can only be achieved via SLA between chipset vendors and MNOs. From this perspective, we think it is similar to option 1a.

Observation 3: Option 1b can achieve limited MNO controllability and visibility through the OpenAPI specification rather than 3GPP specification. However, the OpenAPI specification typically only provides parameters’ type and value range but without detailed coding rules. Thus, similar to option 1a, full MNO controllability and visibility of Option 1b can only be achieved via SLA between chipset vendors and MNOs.

Privacy risk

Different from option 1a, option 1b brings extra risk of data leakage due to MNO because MNOs have some visibility of data content through the OpenAPI specification.

Efforts for deployment

We think option 1b needs a lot of efforts from both UE and NW before its deployments:

- Specification work: enhance EVEX framework to allow UE to report its data in AS layer via application layer.

- UE side: MNOs’ application client for AS data collection needs to be installed on UEs.

- NW side: CN (e.g. DCAF) needs to be upgraded.

## 2.3 Analysis of Option 2/3

Option 2/3 are illustrated in Figure.3.

Figure.3 Illustration of how Option 2 and Option 3 works

Use case

During RAN2 discussion, below use cases for option 2/3 were proposed, but we are not convinced:

- Proposed use case 1: NW or 3rd party may help train UE-sided model (e.g. CN NF or sever from 3rd party)

  - We don’t think it is a valid use case because which model(s) UE can run is hardware/firmware dependent (e.g. power status, UE memory) which can’t be shared with 3rd party.

  - At least in Rel-19, we believe only UE vendor can ensure the performance of UE-sided model because NW or 3rd party is not aware of one UE’s specific hardware/firmware environment.

- Proposed use case 2: optimize UL radio resource for UE dataset transfer

  - There were some concerns that collecting large amount of data from UE may lead to traffic congestion and MNOs may not satisfy QoS.

  - We think it is a UL BW issue which was widely discussed in 3GPP for a long time rather than AI/ML specific issue. MNOs and NW vendors already have a lot of tools to resolve it. Meanwhile, it is an optimization rather than blocking issue.

MNO controllability and visibility

In TR 38.843 [2], it was captured that RAN2 didn’t reach consensus on the need for partial and no visibility options, as illustrated by Note 6:

- Note 6: RAN2 has not concluded on the need for partial and no visibility options.

We are also not convinced due to below reasons:

- The potential leakage of non-standardized data if it contains proprietary implementation and UE privacy information.

- In 3GPP, all standardized data / measurement is specified with clear RAN4 requirements to ensure the data reported by the UE is valid for the NW. However, we doubt how to specify RAN4 requirements for non- standardized data.

Since 3GPP never discussed how to handle non-standardized data, we don’t think above two issues can be well addressed in Rel-19.

Observation 4: As non-standardized data was never discussed in 3GPP, the concept of “partial visibility” and “no visibility” lead to below concerns:

- The potential leakage of non-standardized data if it contains proprietary implementation and UE privacy information.

- It is not clear how to specify RAN4 requirements for non- standardized data in both solutions to ensure validity for NW.

Meanwhile, the UE can send the non-standardized data with proprietary implementation and UE privacy information over the top. Thus, we propose not to consider partial and no visibility.

Proposal 1: Not consider options of “partial visibility” and “no visibility” for option 2/3. Instead, the UE can send the non-standardized data with proprietary implementation and UE privacy information over the top.

Privacy risk

Different from option 1a, option 2/3 brings extra risk of data leakage due to MNO and Network vendors because MNOs have full visibility of data content.

Meanwhile, please note that RAN2 can’t achieve consensus on whether UP solution is feasible in TR 38.843 [2], as illustrated in Note 7.

Note 7: RAN2 cannot reach consensus on the feasibility of UP tunnel in Options 2 and 3

We are open to considering UP solution. However, we think the UP solution will bring new challenge on UE data privacy because collected data is invisible to intermediate nodes like CN and OAM and may be forwarded to unauthorized 3rd party by mistake. Since if it is UP tunnel, it is possible that it includes UE proprietary implementation information whose exposure to 3rd party will lead to our strong concern. Thus, if UP solution for option 2/3 is supported, we think it is necessary to address the new UE data privacy issue (i.e. how CN/OAM can avoid forwarding UE data to unauthorized 3rd party).

Observation 5: If UP solution for option 2/3 is supported, it will bring new challenge on UE data privacy because collected data is invisible to intermediate nodes like CN and OAM. So, it is necessary to address how CN/OAM can avoid forwarding UE data to unauthorized 3rd party.

Efforts for deployment

We think option 2/3 needs a lot of efforts from both UE and NW before its deployments:

- Option 2

  - Specification work: enhance NAS to allow UE to report its data in AS layer via NAS signalling and may need a new protocol stack for UP solution (if supported).

  - UE side: Upgrade its NAS and may need to implement a new protocol stack for UP solution (if supported).

  - NW side: CN needs to be upgraded to support NAS enhancement and potential new protocol stack for UP solution (if supported).

- Option 3

  - Specification work: enhance MDT framework to allow UE to report its data in AS layer via RRC message.

  - UE side: Upgrade its MDT and may need to implement a new protocol stack for UP solution (if supported).

  - NW side: gNB needs to be upgraded to support MDT enhancement and potential new protocol stack for UP solution (if supported).

## 2.4 Comparison among the identified 4 solutions

Based on the analysis in Section 2.1-2.3, we summarize them in Table.1.

|  | Use case | MNO controllability and visibility | Privacy risk | Efforts for deployment |
| --- | --- | --- | --- | --- |
| Option 1a | Feasible for all Rel-19 use cases and it has been widely deployed in industry. | Can be achieved via SLA | UE is responsible for potential data leakage risk. Specific mechanism is at application level, and thereby out of 3GPP scope. | No extra efforts are required. |
| Option 1b | Support one chipset vendor collecting UE data from multiple its OEM vendors | Full controllability and visibility can only be achieved via SLA. | MNO is responsible for potential data leakage risk. | - Specification work: enhance EVEX framework to allow UE to report its data in AS layer via application layer. - UE side: MNOs’ application client for AS data collection needs to be installed on UEs. - NW side: CN (e.g. DCAF) needs to be upgraded. |
| Option 2 | Not clear - NW and 3rd party can’t train UE-sided model. - UL congestion is system performance optimization instead of blocking issue. | Full controllability and visibility for standardized data. | - MNO and NW vendor are responsible for potential data leakage risk. - Extra risk for UP solution (if supported) due to invisibility of collected data to intermediate nodes. | - Specification work: enhance NAS to allow UE to report its data in AS layer via NAS signalling and may need a new protocol stack for UP solution (if supported). - UE side: Upgrade its NAS and may need to implement a new protocol stack for UP solution (if supported). - NW side: CN needs to be upgraded to support NAS enhancement and potential new protocol stack for UP solution (if supported). |
| Option 3 | Not clear - NW and 3rd party can’t train UE-sided model. - UL congestion is system performance optimization instead of blocking issue. | Full controllability and visibility for standardized data. | - MNO and NW vendor are responsible for potential data leakage risk. - Extra risk for UP solution (if supported) due to invisibility of collected data to intermediate nodes. | - Specification work: enhance MDT framework to allow UE to report its data in AS layer via RRC message. - UE side: Upgrade its MDT and may need to implement a new protocol stack for UP solution (if supported). - NW side: gNB needs to be upgraded to support MDT enhancement and potential new protocol stack for UP solution (if supported). |

Table. 1 comparison between different UE-side data collection options

### 2.4.1 Comparison between option 1a and option 1b

According to Table.1, we don’t think there are much difference between option 1a and option 1b from MNO perspective because both require SLA between MNO and chipset vendor to achieve MNO controllability and visibility.

Observation 6: From MNO perspective, there are not much difference between option 1a and option 1b because both require SLA between MNO and chipset vendor to achieve MNO controllability and visibility.

The main benefit of option 1b is that one chipset vendor can collect UE data from multiple OEM vendors while MNO takes the risk of data leakage. However, option 1b requires specification changes, UE installing MNO’s application client and CN upgradation, which makes its deployment quite difficulty especially in late 5G phase.

Observation 7: The main benefit of option 1b is that one chipset vendor can collect UE data from multiple OEM vendors while MNO takes the risk of data leakage. However, option 1b requires specification changes, UE installing MNO’s application client and CN upgradation, which makes its deployment quite difficulty especially in late 5G phase.

### 2.4.2 Comparison between option 1a and option 2/3

As we discussed in section 2.3, the use case of option 2/3 is not clear. For all the 3 identified AI/ML use cases identified by RAN1 (i.e. beam management, positioning, and CSI enhancement), we don’t see any blocking issue to use solution 1a to implement UE-sided training data collection. Meanwhile, option 2/3 require significant specification changes, UE upgradation and CN/RAN upgradation, which makes its deployment impossible especially in late 5G phase.

Finally, please note that Rel-19 is the first release of applying AI/ML in wireless communication, and it is very late stage of 5G. To avoid AI/ML for PHY becoming a paperwork, we think that it is important to make it easy for deployment with the least 3GPP specification changes. Thus, we think option 1a should be baseline of Rel-19 UE-sided data collection, and other options 1b/2/3 can be considered in 6G.

Proposal 2: Considering Rel-19 is the first release of AI/ML in wireless communication at very late stage of 5G, it is important to make the least 3GPP specification changes for its easy to deployment. Thus, option 1a is sufficient for Rel-19 and no normative work of option 1b/2/3.

# 3 Conclusion

In this contribution, we share our view on normative work of Rel-19 UE-side data collection. Our observations are:

Observation 1: Option 1a (i.e. OTT 3GPP transparent) works for all use cases of Rel-19 and it has been widely deployed in industry.

Observation 2: In TR 38.843, Option 1a was captured that “no AI/ML specific controllability” and “No standardized visibility” because Service Level Agreement (SLA) is out of RAN2 scope. As usual business, the MNOs can interact with chipset vendors in offline engineering and achieve controllability and visibility via SLA.

Observation 3: Option 1b can achieve limited MNO controllability and visibility through the OpenAPI specification rather than 3GPP specification. However, the OpenAPI specification typically only provides parameters’ type and value range but without detailed coding rules. Thus, similar to option 1a, full MNO controllability and visibility of Option 1b can only be achieved via SLA between chipset vendors and MNOs.

Observation 4: As non-standardized data was never discussed in 3GPP, the concept of “partial visibility” and “no visibility” lead to below concerns:

- The potential leakage of non-standardized data if it contains proprietary implementation and UE privacy information.

- It is not clear how to specify RAN4 requirements for non- standardized data in both solutions to ensure validity for NW.

Observation 5: If UP solution for option 2/3 is supported, it will bring new challenge on UE data privacy because collected data is invisible to intermediate nodes like CN and OAM. So, it is necessary to address how CN/OAM can avoid forwarding UE data to unauthorized 3rd party.

Observation 6: From MNO perspective, there are not much difference between option 1a and option 1b because both require SLA between MNO and chipset vendor to achieve MNO controllability and visibility.

Observation 7: The main benefit of option 1b is that one chipset vendor can collect UE data from multiple OEM vendors while MNO takes the risk of data leakage. However, option 1b requires specification changes, UE installing MNO’s application client and CN upgradation, which makes its deployment quite difficulty especially in late 5G phase.

Based on above analysis, we have the following proposals.

Proposal 1: Not consider options of “partial visibility” and “no visibility” for option 2/3. Instead, the UE can send the non-standardized data with proprietary implementation and UE privacy information over the top.

Proposal 2: Considering Rel-19 is the first release of AI/ML in wireless communication at very late stage of 5G, it is important to make the least 3GPP specification changes for its easy to deployment. Thus, option 1a is sufficient for Rel-19 and no normative work of option 1b/2/3.

# 4 References

[1] RAN2#127, Chair Notes.

[2] R2-2407807, CR on RAN2 inputs to TR 38.843, Ericsson, 2024-9.

[3] TS 26.531, Data Collection and Reporting; General Description and Architecture

[4] TS 29.571, 5G System; Common Data Types for Service Based Interfaces; Stage 3