3GPP TSG RAN Meeting # 105      		RP- 241935

# Melbourne, Australia, September 9-12, 2024

Agenda Item:	9.3.4.6

Source: 	CMCC, CATT

Title: 			 Considerations on R19 ATG enhancement

Document for:	Decision

- Introductions

Currently, the scope of R19 ATG enhancement Work Item including DL CA and UL MIMO [1], specifically including:

The core part of the work item includes: - Specify the RF and RRM core requirements for intra-band co-located and inter-band co-located DL CA [RAN4]: - FR1 intra-band contiguous CA - Example band combination: CA_n79C - FR1+FR1 inter-band CA - Example band combination: CA_n3A-n39A - Specify the RF requirements for support of UL MIMO with 2TX for single CC for UE. [RAN4] - Specify signaling and UE capabilities if necessary [RAN2].

Considering the good progress of this WI, in this contribution, we propose to update the work item scope to include DL MIMO, and further give the justification, objective and specification impact.

- Motivation and feasibility for DL MIMO

In R18, two kinds of ATG UE types have been defined, one with omni-directional antenna and the other with antenna array. For the UE with antenna array, if cross polarized antennas are used, both DL-MIMO and UL-MIMO can be supported. However, in current R19 ATG WID, only UL MIMO is included.

Observation 1: From technical feasibility perspective, it is feasible to investigate DL-MIMO for ATG UE with cross polarized antennas.

DL-MIMO can further enhance the DL throughput in both single carrier and CA scenario through multi-layer support. Considering there is quite strong capacity demand for onboard ATG terminal (i.e. ATG UE), which generates UL and DL transmission from all users on one aircraft, we think it is worthwhile to investigate DL-MIMO for DL capacity demand.

Observation 2: From motivation perspective, it is worthwhile to investigate DL-MIMO to meet DL capacity demand.

In order to support DL-MIMO in R19 ATG_enh WI, the RF core requirements and demodulation performance requirements should be specified.

For the RF core part, only the time alignment error (TAE) requirements for BS need to be specified if the DL MIMO was introduced for ATG, and the legacy TAE requirements for MIMO transmission can be reused. In previous RAN4 meeting, some company proposed to add additional restriction in the specification to clarify that the MIMO transmission for ATG BS is not supported. However, in our view, it does not make sense to preclude the support of DL-MIMO especially considering that no additional work is expected. For the demodulation perf part, we think it is sufficient to specify both DL and UL MIMO performance in 3 meetings [2]. From TU perspective, it is feasible to specify DL-MIMO with existing TU allocation.

Observation 3: From TU perspective, it is feasible to specify DL-MIMO with existing TU allocation.

During R18, we have already specified test procedures for ATG BS conformance testing and conformance requirements. In R19 ATG work item, BS conformance testing and requirements need to be updated in order to support new functionalities, e.g. CA. However, BS conformance testing and requirements are missing from the current WID, Hence, we propose to add this missing parts.

Observation 4: Since R19 new functionalities have impact on BS conformance testing and requirements, the WID should be updated to add conformance testing and conformance requirements for BS.

Based on the observations above, we propose to update the WID as follows:

The core part of the work item includes:

- Specify the RF and RRM core requirements for intra-band co-located and inter-band co-located DL CA [RAN4]:

  - FR1 intra-band contiguous CA

    - Example band combination: CA_n79C

  - FR1+FR1 inter-band CA

    - Example band combination: CA_n3A-n39A

- Specify the RF requirements for support of UL MIMO with 2TX for single CC for UE. [RAN4]

- Specify the RF requirements for support of DL MIMO for BS. [RAN4]

- Specify signaling and UE capabilities if necessary [RAN2].

The performance part of the work item includes:

- Specify corresponding RRM performance and demodulation requirements for intra-band co-located and inter-band co-located DL CA.

- Specify corresponding demodulation requirements for support of UL MIMO with 2TX for single CC for UE with dual-polarization antenna.

- Specify corresponding demodulation requirements for support of DL MIMO for UE with dual-polarization antenna.

- Specify conformance testing and conformance requirements for BS.

Proposal 1: For Rel-19 ATG enhancement work item, update the ATG WI as follows: .

The core part of the work item includes:

- Specify the RF and RRM core requirements for intra-band co-located and inter-band co-located DL CA [RAN4]:

  - FR1 intra-band contiguous CA

    - Example band combination: CA_n79C

  - FR1+FR1 inter-band CA

    - Example band combination: CA_n3A-n39A

- Specify the RF requirements for support of UL MIMO with 2TX for single CC for UE. [RAN4]

- Specify the RF requirements for support of DL MIMO for BS. [RAN4]

- Specify signaling and UE capabilities if necessary [RAN2].

The performance part of the work item includes:

- Specify corresponding RRM performance and demodulation requirements for intra-band co-located and inter-band co-located DL CA.

- Specify corresponding demodulation requirements for support of UL MIMO with 2TX for single CC for UE with dual-polarization antenna.

- Specify corresponding demodulation requirements for support of DL MIMO for UE with dual-polarization antenna.

- Specify conformance testing and conformance requirements for BS.

- Conclusion

In this contribution, we discussed the motivation and feasibility for extending the work scope of Rel-19 ATG enhancement WI to introduce the DL MIMO. The following observations and proposals are made.

Observation 1: From technical feasibility perspective, it is feasible to investigate DL-MIMO for ATG UE with cross polarized antennas.

Observation 2: From motivation perspective, it is worthwhile to investigate DL-MIMO to meet DL capacity demand.

Observation 3: From TU perspective, it is feasible to specify DL-MIMO with existing TU allocation.

Observation 4: Since R19 new functionalities have impact on BS conformance testing and requirements, the WID should be updated to add conformance testing and conformance requirements for BS.

Proposal 1: For Rel-19 ATG enhancement work item, update the ATG WI as follows: .

The core part of the work item includes:

- Specify the RF and RRM core requirements for intra-band co-located and inter-band co-located DL CA [RAN4]:

  - FR1 intra-band contiguous CA

    - Example band combination: CA_n79C

  - FR1+FR1 inter-band CA

    - Example band combination: CA_n3A-n39A

- Specify the RF requirements for support of UL MIMO with 2TX for single CC for UE. [RAN4]

- Specify the RF requirements for support of DL MIMO for BS. [RAN4]

- Specify signaling and UE capabilities if necessary [RAN2].

The performance part of the work item includes:

- Specify corresponding RRM performance and demodulation requirements for intra-band co-located and inter-band co-located DL CA.

- Specify corresponding demodulation requirements for support of UL MIMO with 2TX for single CC for UE with dual-polarization antenna.

- Specify corresponding demodulation requirements for support of DL MIMO for UE with dual-polarization antenna.

- Specify conformance testing and conformance requirements for BS.

- Reference

- RP-241213, Revised WID on Enhancements for Air-to-ground network for NR, CMCC.

- R4-2404692, (NR_ATG_enh-Core) Work plan on Rel-19 enhancements for ATG, CMCC.