3GPP TSG RAN Meeting #105			RP-241936

Melbourne, Australia, 9th-12th, September, 2024

Agenda item:	9.3.2.3

Title:	Consideration on Revised Rel-19 XR WID

Source:	CMCC

Document for:	Discussion and decision

Introduction

The original WID for NR Phase 3 XR enhancement has been revised in RAN#103 [1] and the objectives can be found in the following:

The Rel-19 XR Phase 3 objectives are as follows: - Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2]. NOTE: Check in RAN#105 (check also other WG involvement if needed). - Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4] - Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. NOTE: LCP implementation complexity need to be taken into account when evaluating solutions. NOTE: Check in RAN#105 - Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. - Specify Core requirements related to the above objectives as necessary [RAN4] - Extend Release 18 standalone mechanism to support NR-NR dual connectivity as follows [RAN3] - PDU set based handling - ECN marking - Burst Arrival Time reporting, if needed - PSI Discard coordination, if needed - Note: No RAN2 impact from above items NOTE: Whether / to what extent network exposure / RAN awareness / e.g. RAN involved rate control, possibly additional info for DL scheduling, parallel with SA2 work, shall be covered in this WI is TBD.

In this contribution, firstly we provide analysis on whether to introduce RAN based rate control into the Rel-19 XR scope. Subsequently, based on the status of RAN2 progress in previous meeting, we discuss whether R19 XR SI of Multi-modality and Scheduling enhancements can be converted to a WI.

Discussion

  - RAN based rate control

In the way forward of RAN#104 [2], it is suggested to replace current notes in WID in RAN#105, which is,

NOTE: Whether / to what extent network exposure / RAN awareness / e.g. RAN involved rate control, possibly additional info for DL scheduling, parallel with SA2 work, shall be covered in this WI is TBD.

There are perspectives suggesting that the current L4S mechanism is capable of identifying congestion and implementing congestion control, which is applicable to both UE and tethered UE.

There is no doubt that L4S exhibits robust congestion control capabilities. However, there are no definitive conclusions regarding its applicability to Phase 3 XR. As a general-purpose network traffic control technology, L4S typically operates between a pair of sending and receiving hosts (or end nodes). Given the variability in traffic patterns across hosts rather than being homogeneous, it is plausible that L4S might suffer from a too rough granularity.

Especially with the introduction of multi-modality and MMSID in XR, traffic flows between hosts exhibit distinct characteristics, e.g., audio, audio, motion and haptic flows has very different jitter, packet size and periodicity. An overall congestion or rate control mechanism may not perform well for all modalities. As a result, congestion control performed per QoS flow/DRB/PDU set/packet is superior to per IP/MAC address in providing an adequate response to these variations and ensuring effective network resource management. And L4S can generally provide an IP layer granularity, as RFC 9330 shows:

L4S packet identifier (protocol): Once there are at least two treatments in the network, hosts need an identifier at the IP layer to distinguish which treatment they intend to use.

Hence, L4S may not be the best options for at least XR application with multi-modality.

A further concern is that while L4S is applicable to a wide range of scenarios, it does not guarantee excellent performance in all cases. For instance, regarding Round-Trip Time (RTT), L4S necessitates the feedback from the receiving host, instead of any intermediate routing node, to inform the sending host about reducing its congestion windows or other operation, which introduces sensible time latency for traffic/rate control. Particularly with the introduction of haptic applications that are highly sensitive to latency, these applications might not perform optimally enough under L4S.

In contrast, RAN based approaches tend to be more responsive and effective in enhancing user experience compared to L4S, given than RAN and UE are much closer that typically results in fewer network hops and thus potentially lower latency, and RAN could have more assistance information, for example, BSR, BSD and UL grant.

On the other hand, L4S can only apply congestion control after congestion occurs. The data traffic characteristics for multi-modality XR are quite complex, UE or gNB might implement traffic forecasting functionalities based on assistance information, e.g., BAT, TTNB, UL grant and QoS. In this case, L4S may not be able to respond effectively to avoid packet loss.

Proposal 1: Update the revised WID for NR Phase 3 XR enhancement to capture the RAN awareness based rate control in the scope.

  - Multi-modality

Based on the work plan, RAN#105 is a checkpoint for both Multi-modality and Scheduling enhancements.

For the Multi-modality, RAN2 had extensive discussions during RAN2#125-bis, RAN2#126 and RAN2#127 on Multi-modality aspects and had reached some agreements during the meetings.

It appears that the general understanding is that knowledge of Multi-Modality information between flows at the RAN side would be a great help for the gNB in terms of performing scheduling and discarding to avoid leaving a UE to enter a situation where associated flows are out of sync. And the primary conclusion is that RAN2 supports Multi-Modality awareness in RAN side in Rel-19 for UL and DL. As not receiving the SA2 reply in RAN2#127, RAN2 achieved the working assumption that regardless of SA2 decision, RAN2 can extend the UAI for multi-modal awareness at least for uplink QoS flows in Rel-19 XR, by having the UE report existence of multi-modality application and association information among QFIs to gNB. Based on this, RAN2 thinks PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the multi-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4.

This means RAN2 will at least work on the UL Multi-Modality awareness in RAN in Rel-19.

Furthermore, just after the last RAN2#127 meeting, SA2 finally reached the consensus which reflected in the reply LS to RAN2 with the feedback as follows [3]:

SA2 specified policy control enhancements to support multi-modal services in Rel-18, with the use of Multi-modal Service ID (MMSID) in AF and PCF, please see TS 23.501 clause 5.37.2 for details. Forwarding MMSID to RAN via control plane is in principle technically feasible. The NGAP protocol impact needs to be identified by RAN3 later. Also, SA2 would like to first understand the usage RAN2 has envisioned with multi-modality awareness (i.e., the foreseen NG-RAN behaviour for "coordinated handling of the flows at NG-RAN") before SA2 can decide whether to provide the MMSID to RAN as part of Rel-19 alignment work. SA4 is invited to review the feedback from RAN2.

Based on this, it is feasible for RAN to work on the DL Multi-Modality awareness enhancement as well with the information from CN via N3 interface.

Proposal 2: Update the revised WID for NR Phase 3 XR enhancement to include specifying how to  support Multi-Modality awareness in RAN in Rel-19 for both UL and DL.

2.3 Scheduling Enhancements

For scheduling enhancement, some agreements were achieved. RAN2 agreed that delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR. Furthermore, RAN2 considers that usage of  additional priority configured to LCHs in case of these LCHs with delay-critical data is regarded as baseline to address the issue of overriding/adjusting the priority of LCH based on delay/deadline information. Additionally, DSR enhancement with a single triggering threshold will be discussed further, e.g. whether there are any constraints on how the NW configures DSR triggering and reporting thresholds. On the other hand, another candidate for scheduling enhancement, the enhancement of the LCP restriction, will not be considered in Rel-19.

Based on the progress, it can be concluded that at least LCH prioritization techniques and DSR enhancement will be specified, where LCP restriction is out of the scope.

Based on the above agreements, WID can be updated as in Annex.

Proposal 3: Update the revised WID for NR Phase 3 XR enhancement to include specifying how to  support Scheduling Enhancements in RAN in Rel-19.

  - RAN3 scope WID update

Based on the reply LS from SA2, it is noted that forwarding MMSID to RAN via control plane is feasible, and the NGAP protocol impact needs to be identified by RAN3. If Multi-modality is to be supported, RAN3 will work on the potential Multi-modality information provided from CN to NG-RAN node over NG interface (e.g., MMSID and/or synchronization threshold). Thus, the revised WID for NR Phase 3 XR enhancement should include the RAN3 objective to support Multi-modality. Since XR for NR-DC is completed in RAN3 in RAN3#125 meeting, there should be an extra TU allocated for a thorough discussion for Multi-modality in RAN3.

Proposal 4: Update Rel-19 WID to include RAN3 objective to support Multi-modality.

Proposal 5: Allocate an extra TU for a thorough discussion for Multi-modality in RAN3.

Conclusion

This contribution provides the following proposal on WID update.

Proposal 1: Update the revised WID for NR Phase 3 XR enhancement to capture the RAN awareness based rate control in the scope.

Proposal 2: Update the revised WID for NR Phase 3 XR enhancement to include specifying how to  support Multi-Modality awareness in RAN in Rel-19 for both UL and DL.

Proposal 3: Update the revised WID for NR Phase 3 XR enhancement to include specifying how to support Scheduling Enhancements in RAN in Rel-19.

Proposal 4: Update Rel-19 WID to include RAN3 objective to support Multi-modality.

Proposal 5: Allocate an extra TU for a thorough discussion for Multi-modality in RAN3.

References

[1]	RP-240791, “Revised WID on XR (eXtended Reality) for NR Phase 3”, Nokia (Rapporteur), RAN#103

[2]	RP-241651, “Outcome from the XR discussion for NR Phase 3 WI scope” Nokia, RAN#104

[3]	S2-249444, “LS reply on multi-modality awareness at RAN”, SA2#164

Annex

The following is the proposed update revised WID:

The Rel-19 XR Phase 3 objectives are as follows: - Study and if justified, sSpecify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination, PDU set discarding, assistance information (UAI) and QoS flow to DRB mapping. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2, RAN3]. NOTE: Check in RAN#105 (check also other WG involvement if needed). NOTE: Coordination with RAN3 when needed if any. - Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4] - Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. - Delay-aware based LCP prioritization, e.g., use additional priority configured to LCHs in case of these LCHs with delay-critical data. [RAN1, RAN2] - DSR enhancement including at least multiple pairs of remaining time and buffer size for the LCG. [RAN2] NOTE: LCP implementation complexity need to be taken into account when evaluating solutions. NOTE: Check in RAN#105 - Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. - Specify Core requirements related to the above objectives as necessary [RAN4] - Extend Release 18 standalone mechanism to support NR-NR dual connectivity as follows [RAN3] - PDU set based handling - ECN marking - Burst Arrival Time reporting, if needed - PSI Discard coordination, if needed - Note: No RAN2 impact from above items - Specify aspects related to network exposure/RAN awareness for effective rate and/or congestion control and application layer adaptation, e.g., based on information from UE, gNB or CN (with potential coordination with SA2/SA4).[RAN2, RAN3] NOTE: Whether / to what extent network exposure / RAN awareness / e.g. RAN involved rate control, possibly additional info for DL scheduling, parallel with SA2 work, shall be covered in this WI is TBD.