3GPP TSG-RAN Meeting #105                                         RP-241938

Melbourne, Australia, September 9-12, 2024

Agenda Item:	9.3.2.1

Source:	CMCC

Title:	Views on scope of Rel-19 mobility enhancements

Document for:	Discussion and Decision

- Introduction

In RAN#102, the scope of Rel-19 Mobility enhancements WI was revised as in [1], and there are two objectives need to be checked in RAN#105, including event triggered L1 measurement reporting and conditional LTM as following:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1] - Specify support of conditional LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Prioritise intra-CU LTM - Checkpoint to review objective at RAN#105. RAN WG work to not start before this checkpoint

In this contribution, we provide our views on the scope of Rel-19 mobility enhancements.

- Discussion

  - Event triggered L1 measurement reporting

Based on the guideline of Rel-19 Mobility enhancements WID, we need to review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work between MIMO and Mobility. For event triggered L1 measurement reporting, the work may include trigger-event definition, measurement RS for serving cell and candidate cells, UL signaling contents, UL signaling medium/container. In previous meetings, we have good progress on the objectives. The summary of UE-initiated/event-driven beam reporting in MIMO and event triggered L1 measurement reporting in mobility are shown in Table 1. The trigger-event, the measurement RS for new beam or candidate cells, especially the UL signaling medium/container for MIMO and mobility are totally different, we cannot merge the work of MIMO and mobility at current stage. Hence, the modification of objectives for event triggered L1 measurement reporting is not needed for mobility.

Table 1. Summary of UE-initiated/event-driven beam reporting in MIMO and event triggered L1 measurement reporting in mobility

|  | UE-initiated/event-driven beam reporting in MIMO | Event triggered L1 measurement reporting in mobility |
| --- | --- | --- |
| Trigger-event definition | Agree Event-2, working assumption for Event-1 and Event-7 - Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality. | Event LTM2, LTM3, LTM4, LTM5 - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. |
| Measurement RS for serving cell | The RS for current beam is implicitly derived from a QCL RS of indicated TCI state. | Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. |
| Measurement RS for new beam or candidate cells | The RS(s) for new beam(s) are explicitly configured | Any beam in candidate RS configuration can be used for LTM event evaluation. |
| UL signaling contents | L1-RSRP, FFS L1-SINR | L1-RSRP, FFS L1-SINR |
| UL signaling medium/container | UCI | MAC CE |

Proposal 1: The modification of objectives for event triggered L1 measurement reporting is not needed for mobility.

  - Conditional LTM

Conditional mobility including CHO and CPAC is defined as a handover or PsCell addition/change executed by the UE without network command, when one or more handover execution conditions are met based on L3 measurement results. Since the omitted RRC siganalling for handover or PsCell addition/change, the failure of RRC signalling reception can be avoided and the robustness for cell switch or PsCell addition/change can be improved.

If conditional LTM is supported, it means that UE may switch its serving cell to a LTM candidate based on its evaluation of the execution condition. Therefore, the first issue for consideration is about the execution conditions for conditional LTM. Spontaneously, L1-measurement results based events can be used for conditional LTM. But it should be noticed that, LTM L1 measurement capability and LTM capabilities are decoupled, the UE which reports LTM capability without 45-1 may not perform L1 measurement and reporting [3]. Therefore, L3 measurement results based execution condition should also be considered in conditional LTM. Considering that RACH-based LTM is also supported, if L3-measurement based event is configured to the UE, there’s no significant gain for conditional LTM compared with CHO.

The LTM L1 measurement capability and LTM capabilities are decoupled As the LTM L1 measurement capability and LTM capabilities are decoupled: - 1. A UE which reports LTM capability without 45-1 may not perform L1 measurement reporting, and it is up to network implementation how to trigger the LTM execution. - 2. The granularity is FFS, e.g. whether LTM capability is divided into two separate capabilities for inter-frequency and intra-frequency.

Observation 1: The gain of conditional LTM is not clear compared with CHO especially L3-measurement result based execution condition is used.

Currently, LTM supports early synchronization for UE TA acquisition, including the following two ways: UE-based TA measurement and PDCCH order-based TA acquisition. For UE-based TA measurement, it’s UE implementation to acquire TA, while for PDCCH order-based TA acquisition, is done via CFRA triggered by a PDCCH order from the source cell, following which the UE sends preamble towards the indicated candidate cell. In order to minimize the data interruption of the source cell due to CFRA towards the candidate cell(s), the UE does not receive random access response from the network for the purpose of TA value acquisition and the TA value of the candidate cell is indicated in the cell switch command MAC CE. When it comes to conditional LTM, it’s not clear whether early sync is supported. If so, UE-based TA measurement can be inherited without extra enhancements, but the accuracy may not be assured. While for PDCCH-order based TA acquisition, TA value comes with cell switch command does not align with the motivation of conditional mobility, in which signallings to trigger UE’s mobility is avoided. If early TA acquisition is not supported, it seems conditional LTM is not needed since CHO is already introduced.

Observation 2: TA acquisition approaches for early sync adopted in Rel-18 may not meet the requirements of conditional LTM, i.e., PDCCH-order based approach may introduce extra signalling for the network.

Besides, there’s two cases for the UE to identify whether LTM cell switch is successful:

- If the UE has performed a RA procedure in step 7 the UE considers that LTM cell switch execution is successfully completed when the random access procedure is successfully completed.

- For RACH-less LTM, the UE considers that LTM cell switch execution is successfully completed when the UE determines that the network has successfully received its first UL data.

When it comes to conditional LTM, for Case 1), it’s similar to CHO, UE perform RACH procedure and considers that LTM cell switch execution is successfully completed when the random access procedure is successfully completed. While for Case 2), UE needs to transmit UL data, and CG-based RACH-less and DG-based RACH-less procedures are supported, how to provide CG/DG resources should be considered.

Observation 3: It’s not clear on how to provide CG/DG resources in conditional LTM.

Based on the discussion above, we propose that conditional LTM is not supported in Rel-19 mobility.

Proposal 2：Conditional LTM is not supported in Rel-19 mobility enhancements WI.

  - Overall consideration for revised WID

Based on the above discussion for LTM measurement enhancements and conditional LTM, we support to revise the WID as follows:

Proposal 3: Support to revise the WID as follows:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1]

  - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM

  - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1]

    - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs.

  - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

- Conclusions

In this contribution, we discuss the scope of Rel-19 mobility enhancement. Following observations and proposals are made in this contribution:

Observations

Observation 1: The gain of conditional LTM is not clear compared with CHO especially L3-measurement result based execution condition is used.

Observation 2: TA acquisition approaches for early sync adopted in Rel-18 may not meet the requirements of conditional LTM, i.e., PDCCH-order based approach may introduce extra signalling for the network.

Observation 3: It’s not clear on how to provide CG/DG resources in conditional LTM.

Proposals

Proposal 1: The modification of objectives for event triggered L1 measurement reporting is not needed for mobility.

Proposal 2：Conditional LTM is not supported in Rel-19 mobility enhancements WI.

Proposal 3: Support to revise the WID as follows:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1]

  - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM

  - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1]

    - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs.

  - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

- References

- RP-234036, New WID: NR mobility enhancements Phase 4, 3GPP TSG RAN Meeting #102, Edinburgh, Scotland, December 11-15, 2023；

- TS 38.300 NR and NG-RAN Overall Description; Stage 2 (Release 18)；

- R2-2406202 Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan.