3GPP TSG RAN Meeting #105					                                              				RP-241939

Melbourne, Australia, September 9-12, 2024

Title:	Views on scope of Rel-19 NES enhancement

Agenda item:	9.3.1.5

Source:	CMCC

Document for:	Discussion/Decision

Introduction

Plenty of techniques were discussed and evaluated during the work item (WI) of Rel-19 network energy saving (NES) enhancement [1], and some of them were set up for the study phase, including:

2. Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105

In this contribution, we provide our views on the scope of Rel-19 NES enhancement. Including the background for on-demand SIB1 during study phase, discussion on potential cases and consideration for revised WID.

Background for on-demand SIB1 during study phase

In this section, we first introduce the background for on-demand SIB1 during study phase.

The motivation of on-demand SIB1 is to not periodically transmit the NES cell’s SIB1 by defeault, so that the network can significantly save some energy. At the same time, the UE can transmit a UL WUS to request NES cell’s SIB1 when needed, and the configuration of UL WUS can be pre-configured to the UE by network.

During the study phase, RAN1/2/3 had a thoroughly discussion and focused on the following potential cases [2]:

Agreement in RAN1#116-bis: For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 focuses its studies on the following cases: - Case 1: Option 1+A+X - Case 2: Option 1+B+X - Case 3: Option 2+B+Y Where the options 1/2/A/B/X/Y are defined below: - On target cell of UL WUS transmission: - Option 1: UE transmits UL WUS to NES Cell - Option 2: UE transmits UL WUS to Cell A - On configuration provision for UL WUS transmission - Option A: UE obtains the UL WUS configuration from NES Cell - Option B: UE obtains the UL WUS configuration from Cell A - On receiving of SIB1 - Option X: UE receives on-demand SIB1 from NES Cell - Option Y: UE receives on-demand SIB1 from Cell A

Regarding the potential cases for on-demand SIB1:

- Case 1 refers to a pure on-demand SIB1 procedure via NES cell (a cell that may transmit SIB1 transmission in response to UL WUS from a UE). Compare with other potential cases, it does not require the assistance from another cell, so that no additional NES loss on another cell is obtained. Meanwhile, the WUS configuration shall be broadcasted on NES cell, therefore introducing the extra power consumption.

- Case 2 refers a procedure that both Cell A (a cell that is periodically transmitting at least its own SIB1) and NES cell are involved. Compare with other potential cases, it has the minimum spec impact and can achieve the significant NES gain on NES cell.

- Case 3 refers to a pure on-demand SIB1 procedure via Cell A. Compare with other potential cases, it does not require NES cell to transmit any signaling during the procedure, therefore achieving the largest NES gain on NES cell. Meanwhile, both WUS configuration and NES cell’s SIB1 shall be broadcasted on Cell A, thus introducing the extra power consumption.

Discussion on poetential cases for on-demand SIB1

In this section, we discuss whether each potential case for on-demand SIB1 is feasibile and benefical, and whether to support it for normative work.

Discussion on Case 1 for on-demand SIB1

Regarding Case 1 for on-demand SIB1, both RAN1 and RAN2 had a good study and reached the following conclusion [3] and agreement [4]:

Conclusion in RAN1#118: For on-demand SIB1 in idle/inactive mode, RAN1 was not able to achieve consensus to support Case 1 (Option 1+A+X) in Rel-19, while Case 1 is technically possible from RAN1’s perspective. Agreement in RAN2#126 for Scenario 3 (case 1 in RAN1): 16. RAN2 to wait for RAN1’s progress whether to support scenario 3.

Based on the above agreement/conclusion, both RAN1 and RAN2 were not able to achieve consensus to support Case 1 in Rel-19. From our understanding, the main concerns for Case 1 are:

- Marginal NES gain.

- Potential large spec impact.

- Potential coverage hole for legacy UE.

Simultaneously, we also evaluate the NES gain under this potential case, the details can be found in both Appendix A and [5]. Based on our evaluation, the NES gain under Case 1 is:

- 2.7%~16.4% NES gain can be obtained for Cat 1 BS, TDD structure under various SIB1 request rates.

- 2.6%~15.1% NES gain can be obtained for Cat 1 BS, FDD structure under various SIB1 request rates.

- 2.4%~11.3% NES gain can be obtained for Cat 2 BS, TDD structure under various SIB1 request rates.

- 2.2%~9.3% NES gain can be obtained for Cat 2 BS, FDD structure under various SIB1 request rates.

- Note that the SIB1 request rate varies from 0% to 100% in Cat 1 BS scenario, and varies from 0% to 50% in Cat 2 BS scenario. And the assumption for cell load is empty load.

Considering the NES gain is relatively limited, Case 1 for on-demand SIB1 shall not be supported in Rel-19.

Proposal 1: Case 1 for on-demand SIB1 is not supported in Rel-19 due to the marginal NES gain.

Discussion on Case 2 for on-demand SIB1

Regarding Case 1 for on-demand SIB1, RAN1-RAN3 had reached the following agreements [3] [6] [7]:

Agreement in RAN1#118: RAN1 recommends specifying on-demand SIB1 only for Case 2 (Option 1+B+X) in Rel-19. - Note: RAN1 strive to minimize impact to legacy UE. - Note: RAN1 specification impact to support this feature should be minimized. Agreement in RAN2#127 for OD-SIB1 SI conclusion: 8. RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1. Agreement in RAN3#125 for the conclusion report to RAN on on-Demand SIB1 for checkpoint: Case 2 can be supported with the below details: - UL WUS configuration is decided by NES Cell DU. - UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface. - Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). FFS if Cell A broadcasts it right away or not.

Also, during RAN1#117 meeting, companies have provided the evaluation results for Case 2, the detailed observations can be founded in Appendix B and [8]. Through the agreements, it can be observed that RAN1-RAN3 are all support to covert on-demand SIB1 for Case 2 into normative work. Thus, it is recommended for RAN plenary to support Case 2 for on-demand SIB1 for nomartive work.

Proposal 2: Support convert Case 2 for on-demand SIB1 into normative work.

Discussion on Case 3 for on-demand SIB1

Regarding Case 3 for on-demand SIB1, both RAN1 and RAN2 had the good study and reached the following agreement [3] [6]:

Agreement in RAN1#118: For on-demand SIB1 in idle/inactive mode, Case 3 (Option 2+B+Y) is feasible from RAN1 perspective for some scenarios. Case 3 (Option 2+B+Y) is lower priority compared to Case 2 from RAN1 perspective. - RAN1 is inconclusive on whether the required specification support is justified by the observed NES gain for Case 3 Agreement in RAN2#127 for Scenarios: 1. No consensus for RAN1 case 3 in RAN2.

Based on the above agreements, both RAN1 and RAN2 were not able to achieve consensus to support Case 3 in Rel-19. From our understanding, the main concerns for Case 3 are:

- It may impact PRACH capacity and SIB1 transmiossion on Cell A for legacy UE.

- Frequent coordination between Cell A and NES cell on NES cell’s SIB1.

- UE may need to maintain two synchronizations for both NES cell and Cell A.

- UE may need more UL power to transmit UL WUS to Cell A due to coverage.

- Potential impact on legacy procedures (e.g., SIB1 acquisition, SI update, paging).

Simultaneously, we also evaluate the NES gain under this potential case, the details can be found in both Appendix A and [5]. Based on our evaluation result, Case 3 can bring up to 12% extra NES gain in Cat 1 BS scenario, and can bring up to 6% extra NES gain in Cat 2 BS scenario.

Therefore, it can be concluded that Case 3 can bring the significant NES benefit. However, since various RAN subgroups (especially RAN2) are not able to reach the consensus, it is recommended that Case 3 is deprioritized in Rel-19.

Proposal 3: Case 3 for on-demand SIB1 is not supported in Rel-19 due to there is no consensus in both RAN1 and RAN2.

Consideration for revised WID

In this section, we discuss our consideration for revised WID. Based on above discussion for background and potential cases for on-demand SIB1, we support to revise the WID as follows:

Proposal 4: Support to revise the WID as follows:

2. Specify Study procedures and signaling method(s) to support for on-demand SIB1 under Case 2 (Option 1+B+X) for UEs in idle/inactive mode, including: [RAN1/2/3]

  - Triggering method by uplink wake-up-signal using an existing signal/channel.

  - Wake-up-signal configuration provisioning to UE

- Note: No modification of SSB will be discussed under this objective

  - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary.

  - Checkpoint for normative work in RAN#105

  - The options 1/B/X are defined below:

- Option 1: UE transmits UL WUS to NES Cell

- Option B: UE obtains the UL WUS configuration from Cell A

- Option X: UE receives on-demand SIB1 from NES Cell

- Note that Cell A refers to a cell that is periodically transmitting at least its own SIB1, and NES cell refers to a cell that may transmit SIB1 transmission in response to UL WUS from a UE

Conclusion

In this contribution, we discussed the background for on-demand SIB1 during study phase, discussion on potential cases and consideration for revised WID. The following proposals are made.

Proposal 1: Case 1 for on-demand SIB1 is not supported in Rel-19 due to the marginal NES gain.

Proposal 2: Support convert Case 2 for on-demand SIB1 into normative work.

Proposal 3: Case 3 for on-demand SIB1 is not supported in Rel-19 due to there is no consensus in both RAN1 and RAN2.

Proposal 4: Support to revise the WID as follows:

2. Specify Study procedures and signaling method(s) to support for on-demand SIB1 under Case 2 (Option 1+B+X) for UEs in idle/inactive mode, including: [RAN1/2/3]

  - Triggering method by uplink wake-up-signal using an existing signal/channel.

  - Wake-up-signal configuration provisioning to UE

- Note: No modification of SSB will be discussed under this objective

  - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary.

  - Checkpoint for normative work in RAN#105

  - The options 1/B/X are defined below:

- Option 1: UE transmits UL WUS to NES Cell

- Option B: UE obtains the UL WUS configuration from Cell A

- Option X: UE receives on-demand SIB1 from NES Cell

- Note that Cell A refers to a cell that is periodically transmitting at least its own SIB1, and NES cell refers to a cell that may transmit SIB1 transmission in response to UL WUS from a UE

Reference

- RP-234065, New WID: Enhancements of network energy savings for NR.

- R1-2403729, Moderator (MediaTek), FL summary 5 for on-demand SIB1 in idle/inactive mode

- R1-2407217, Moderator (MediaTek), FL summary 5 for on-demand SIB1 in idle/inactive mode

- R2-2405701, Vice Chairman (Samsung), Report from session on V2X/SL, R19 NES and MOB

- R1-2405994, Discussion on on-demand SIB1 for UEs in idle/inactive mode, CMCC

- R2-2407571, Vice Chairman (Samsung), Report from session on V2X/SL, R19 NES and MOB

- R3-244001, RAN3 Chair (ZTE), RAN3#125 Meeting Agenda

- R1-2405658, Moderator (MediaTek), FL summary 6 for on-demand SIB1 in idle/inactive mode

- 3GPP TR 38.864, Study on network energy savings for NR.

- Appendix A: Evaluation results for on-demand SIB1 in Case 1-3

For evaluation assumption, the following agreements are agreed during RAN1#116-bis meeting [2].

Agreement Companies to report at least the following key settings used in the evaluation/simulation of achievable NES gain with on-demand SIB1 in idle/inactive mode - Setting A: SIB1 period (20ms/40ms/160ms) - Setting B1: Cell load (Empty/low/medium) - Setting B2: Traffic model - Setting C: SIB1 PDSCH time domain resource index in 38.214 Table 5.1.2.1.1-2 - Setting D: CORESET0/SSB multiplexing pattern including controlResourceSetZero (index) in 38.213 Table 13-6, and searchSpaceZero (index) in 38.213 Table 13-11 - Setting E: PRACH configurations (including PRACH configuration index in 38.211 Table 6.3.3.2-3) for WUS and initial/random access - Setting F: Cat1/Cat2 BS - Setting G: Number of SSB beams - Setting H: NES gain/loss on Cell A - Setting I: On-demand SIB1 transmission rate (how often UE requests on-demand SIB1) Agreement For further study of the NES gain/loss evaluation assumption on Cell A with on-demand SIB1 on NES cell for idle/inactive mode UE, - Assume the following for network energy evaluation of Cell A in FR1: - Company to report among empty/low/medium cell load as defined in 38.864 - Same Cat BS as the Non-NES cell - 30kHz SCS, DDDSU TDD pattern - Same SSB period as the Non-NES cell and company to report SIB1 period - Same number of SSBs in a SSB burst as the Non-NES cell with SSB pattern case C - 20ms PRACH configuration periodicity for WUS and/or initial access RACH and company to report RACH configuration index in 38.211 Table 6.3.3.2-3 - Same SSB/CORESET0 multiplexing pattern and same SIB1 PDSCH time domain resource allocation as the Non-NES cell - Same traffic model as the Non-NES cell - Companies to report the assumption of WUS configuration provision or UL WUS monitoring or on-demand SIB1 transmission on Cell A if Case 2 (Option 1+B+X) or Case 3 (Option 2+B+Y) is considered

On the basis of this, we evaluate the NES gain on on-demand SIB1 for Case 1, Case 2 and Case 3. Note that the basic parameters for the evaluation result come from the above agreement while other parameters come from the SLS parameter settings in [9]. Figure 1 and Figure 2 show an example for common signal transmission pattern for the baseline in TDD and FDD structure, respectively.

Figure 1. An example for common signal transmission pattern for baseline in TDD structure

Figure 2. An example for common signal transmission pattern for baseline in FDD structure

Evaluation result on NES gain for Case 1

The settings for baseline and on-demand SIB1 mode in Case 1 are shown below:

- Baseline:

- No WUS configuration is transmitted on NES Cell.

- No WUS is transmitted on NES cell.

- SIB1 is transmitted on NES cell periodically with all beam direction (periodicity is 20ms).

- On-demand SIB1 mode:

- WUS configuration is broadcasted on NES cell, WUS configuration is FDM transmitted with NES cell’s SIB1 transmission pattern. The frequency resource (RB number) for WUS configuration transmission and SIB1 are the same. The time duration for each WUS configuration transmission is 3 OFDM symbol.

- WUS is transmitted on NES cell, UL WUS resource is FDM transmitted with legacy PRACH resource. The frequency resource (RB number) and time resource (OFDM symbol number) for WUS resource and legacy resource are the same.

- SIB1 is transmitted on NES cell. The NES cell transmits SIB1 in {1, 2, 4} beams for TDD structure, and {1, 2} beams for FDD structure by demand.

- Note that the assumption for cell load is empty load.

For NES gain on NES cell, Figure 3 shows the evaluation results for different BS type in [9]. It can be observed that:

- 2.7%~16.4% NES gain can be obtained for Cat 1 BS, TDD structure under various SIB1 request rates.

- 2.6%~15.1% NES gain can be obtained for Cat 1 BS, FDD structure under various SIB1 request rates.

- 2.4%~11.3% NES gain can be obtained for Cat 2 BS, TDD structure under various SIB1 request rates.

- 2.2%~9.3% NES gain can be obtained for Cat 2 BS, FDD structure under various SIB1 request rates.

- Note that the SIB1 request rate varies from 0% to 100% in Cat 1 BS scenario, and varies from 0% to 50% in Cat 2 BS scenario.

Figure 3. Evaluation results for NES gain on NES cell for different BS type in [9]

Evaluation result on NES gain for Case 2

The settings for baseline and on-demand SIB1 mode in Case 2 are shown below:

- Baseline:

- No WUS configuration is transmitted on Cell A.

- No WUS is transmitted on NES cell.

- SIB1 is transmitted on NES cell periodically with all beam directions (periodicity is 20ms).

- On-demand SIB1 mode:

- WUS configuration is broadcasted on Cell A, WUS configuration is FDM transmitted with Cell A’s SIB1. The time resource (OFDM symbol number) for WUS configuration and SIB1 are the same. The frequency resource for WUS configuration is 4 RB for TDD structure and 2 RB for FDD structure.

- WUS is transmitted on NES cell, UL WUS resource is FDM transmitted with legacy PRACH resource. The frequency resource (RB number) and time resource (OFDM symbol number) for WUS resource and legacy resource are the same.

- SIB1 is transmitted on NES cell. The NES cell transmits SIB1 in {1, 2, 4} beams for TDD structure, and {1, 2} beams for FDD structure by demand.

- Note that the assumption for cell load is empty load.

For NES gain on NES cell, Figure 4 shows the evaluation results for different BS types in [9]. It can be observed that:

- 16.4%~43.8% NES gain can be obtained for Cat 1 BS, TDD structure under various SIB1 request rates.

- 20.1%~48.9% NES gain can be obtained for Cat 1 BS, FDD structure under various SIB1 request rates.

- 15.1%~30.2% NES gain can be obtained for Cat 2 BS, TDD structure under various SIB1 request rates.

- 16.4%~32.9% NES gain can be obtained for Cat 2 BS, FDD structure under various SIB1 request rates.

Figure 4. Evaluation results for NES gain in NES cell for different BS type in [9]

For NES loss on Cell A, Table 1 shows the evaluation results for different BS types in [9]. It can be observed that 6.3%~13.5% NES loss can be obtained for Cell A in various BS types and TDD/FDD structures.

Table 1. NES loss on Cell A for different BS type in [9]

| BS category and structure | Cat 1 BS, TDD | Cat 1 BS, FDD | Cat 2 BS, TDD | Cat 2 BS, FDD |
| --- | --- | --- | --- | --- |
| NES loss (%) | -0.6% | -0.7% | -0.5% | -1.1% |

For overall NES gain on the network, Figure 5 shows the evaluation results for different BS types in [9]. Note that the assumption here is that each coverage area of Cell A contains 1 NES cell. It can be observed that:

- 7.9%~21.6% NES gain can be obtained for Cat 1 BS, TDD structure under various SIB1 request rates.

- 9.7%~24.1% NES gain can be obtained for Cat 1 BS, FDD structure under various SIB1 request rates.

- 7.3%~14.8% NES gain can be obtained for Cat 2 BS, TDD structure under various SIB1 request rates.

- 7.6%~15.9% NES gain can be obtained for Cat 2 BS, FDD structure under various SIB1 request rates.

Figure 5. Evaluation results for overall NES gain for different BS type in [9]

Evaluation result on NES gain for Case 3

The settings for baseline and on-demand SIB1 mode in Case 3 are shown below:

- Baseline:

- No WUS configuration is transmitted on NES Cell.

- No WUS is transmitted on NES cell.

- SIB1 is transmitted on NES cell periodically with all beam direction (periodicity is 20ms).

- On-demand SIB1 mode:

- WUS configuration is broadcasted on Cell A, WUS configuration is FDM transmitted with Cell A’s SIB1. The time resource (OFDM symbol number) for WUS configuration and SIB1 are the same. The frequency resource for WUS configuration is 4 RB for TDD structure and 2 RB for FDD structure.

- WUS is transmitted on Cell A, UL WUS resource is FDM transmitted with legacy PRACH resource. The frequency resource (RB number) and time resource (OFDM symbol number) for WUS resource and legacy resource are the same.

- SIB1 is transmitted on Cell A, NES cell’s SIB1 is FDM transmitted with Cell A’s SIB1. The NES cell transmits SIB1 in {1, 2, 4} beams for TDD structure, and {1, 2} beams for FDD structure by demand. The frequency resource (RB number) and time resource (OFDM symbol number) for both NES cell’s SIB1 and Cell A’s SIB1 are the same.

- Note that the assumption for cell load is empty load.

For NES gain on NES cell, Table 2 shows the evaluation results for different BS types in [9]. It can be observed that 30.2%~48.9% NES gain can be obtained for NES cell in various BS types and TDD/FDD structures.

Table 2. NES gain on NES cell for different BS type in [9]

| BS category and structure | Cat 1 BS, TDD | Cat 1 BS, FDD | Cat 2 BS, TDD | Cat 2 BS, FDD |
| --- | --- | --- | --- | --- |
| NES gain (%) | 43.8% | 48.9% | 30.2% | 32.9% |

For NES loss on Cell A, Figure 6 shows the evaluation results for different BS type in [9]. It can be observed that:

- -0.6%~-4.0% NES loss can be obtained for Cat 1 BS, TDD structure under various SIB1 request rates.

- -0.7%~-9.0% NES loss can be obtained for Cat 1 BS, FDD structure under various SIB1 request rates.

- -0.5%~-3.7% NES loss can be obtained for Cat 2 BS, TDD structure under various SIB1 request rates.

- -1.1%~-7.9% NES loss can be obtained for Cat 2 BS, FDD structure under various SIB1 request rates.

Figure 6. Evaluation results for NES loss on Cell A for different BS type in [9]

For overall NES gain on the network, Figure 7 shows the evaluation results for different BS types in [9]. Note that the assumption here is that each coverage area of Cell A contains 1 NES cell. It can be observed that:

- 20.0%~21.6% NES gain can be obtained for Cat 1 BS, TDD structure under various SIB1 request rates.

- 20.0%~24.1% NES gain can be obtained for Cat 1 BS, FDD structure under various SIB1 request rates.

- 13.3%~14.8% NES gain can be obtained for Cat 2 BS, TDD structure under various SIB1 request rates.

- 12.5%~15.9% NES gain can be obtained for Cat 2 BS, FDD structure under various SIB1 request rates.

Figure 7. Evaluation results for overall NES gain for different BS type in [9]

- Appendix B: Agreements on observations for on-demand SIB1

The following observations were agreed during RAN1#117 meeting [8]:

Agreement For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 20ms SIB1 period (Case A), FR1, empty load - For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 39.37% - 47.69% from the following 7 sources - R1-2404463, R1-2405595, R1-2405341, R1-2404859, R1-2404507, R1-2405363, R1-2404625 - One source (R1-2405162) reports 25% NES gain with PRACH configuration index = 152 - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98 - 24.6% with one beam of SIB1 PDSCH transmitted in each slot - 17.4% with two beams of SIB1 PDSCH transmitted in each slot - One source (R1-2404122) reports the following with FDMed SSB and PDSCH of SIB1 - 9.27%~18.94% NES gain with UL WUS configuration transmitted in legacy SIB on Cell A - 0%~9.66% NES gain with UL WUS configuration transmitted in separated SIB on Cell A. The separated SIB is TDMed with legacy SIB and transmitted in a 20ms period using dedicated resource. - 0% NES gain with Case 1 (Options 1+A+X) and WUS configuration transmitted in separate SIB on the NES cell - 13.25% NES gain with Case 1 (Options 1+A+X) and pre-configured for fixed WUS configuration - NES cell does not transmit the WUS configuration - For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 22.1% - 42.6% from the following 5 sources - R1-2404408, R1-2405341, R1-2404758, R1-2405363, R1-2404625 - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98 - 16.8% with one beam of SIB1 PDSCH transmitted in each slot - 10.9% with two beams of SIB1 PDSCH transmitted in each slot - For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, - the NES gain is 30.43% - 45.79% from the following 4 sources - R1-2404463, R1-2404859, R1-2405363, R1-2404625 - One source (R1-2405162) reports the following NES gain with PRACH configuration index = 152 - 25% NES gain with Case 3 (Options 2+B+Y) assuming the UE can camp on the NES cell and the SIB1 is only transmitted in a single beam corresponding to the UL WUS beam - 23% with Case 2 (Options 1+B+X) - For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, - the NES gain is 21.79% - 33.11% from the following 2 sources - R1-2405363, R1-2404625 - For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 20.5% - 39.34% from the following 8 sources - R1-2404463, R1-2403942, R1-2405341, R1-2404224, R1-2404859, R1-2404507, R1-2404561, R1-2405595 - One source (R1-2404224) reports 11.9% NES gain with medium cell load on Cell A - For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 19.31 – 19.39% from one source (R1-2405341) - For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, - the NES gain is 29.19% - 36.87% from the following 3 sources - R1-2404463, R1-2403942, R1-2404859 - One source (R1-2404561) reports 15.87%~20.18% NES gain with PRACH configuration index = 148 - For Cat 1 BS, 4/8 beams, 4% < on-demand SIB1 transmission rate < 24%, the NES gain is 17.98%~24.48% from one source (R1-2404408). Agreement For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 160ms SIB1 period (Case C), FR1, empty load - For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 2.21 – 10.6% from the following 10 sources - R1-2405595, R1-2403979, R1-2405341, R1-2404859, R1-2405162, R1-2404122, R1-2404507, R1-2405363, R1-2404625, R1-2404561 - For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 1.7% - 6.23% from the following 5 sources - R1-2403979, R1-2405341, R1-2405363, R1-2404625, R1-2404758 - One source (R1-2404758) reports 28.9% NES gain with Case 3 where no PRACH and paging is activated for the NES cell - For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.75% - 8.48% from the following 6 sources - R1-2405106, R1-2405162, R1-2404859, R1-2405363, R1-2404625, R1-2404561 - For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.01% - 5.8% from the following 3 sources - R1-2405106, R1-2405363, R1-2404625 - For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 3.24% - 7.76% from the following 5 sources - R1-2403942, R1-2405341, R1-2404859, R1-2404507, R1-2404561 - For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 2.96% from one source (R1-2405341) - For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 2.5% - 7.44% from the following 3 sources - R1-2404561, R1-2403942, R1-2404859 Agreement For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 20ms SIB1 period (Case A), FR1, low load - For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 30.56% - 41.8% from the following 5 sources - R1-2405595, R1-2405341, R1-2404859, R1-2405363, R1-2404625 - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98 - 13.9% with one beam of SIB1 PDSCH transmitted in each slot - 7.9% with two beams of SIB1 PDSCH transmitted in each slot - For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 16% - 36.2% from the following 5 sources - R1-2405247, R1-2405341, R1-2404033, R1-2405363, R1-2404625 - One source (R1-2404033) reports 10% NES gain with FDMed SSB and PDSCH of SIB1 - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98 - 5.2% with one beam of SIB1 PDSCH transmitted in each slot - 1.9% with two beams of SIB1 PDSCH transmitted in each slot - For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, - the NES gain is 22.44% - 37.64% from the following 3 sources - R1-2404859, R1-2405363, R1-2404625 - One source (R1-2404122) reports the following with FDMed SSB and PDSCH of SIB1 - 4.73%~9.64% NES gain with UL WUS configuration transmitted in legacy SIB on Cell A - 0%~4.8% NES gain with UL WUS configuration transmitted in separated SIB on Cell A. The separated SIB is TDMed with legacy SIB and transmitted in a 20ms period using dedicated resource. - 0% NES gain with Case 1 (Options 1+A+X) and WUS configuration transmitted in separate SIB on the NES cell - 7% NES gain with Case 1 (Options 1+A+X) and pre-configured for fixed WUS configuration - NES cell does not transmit the WUS configuration - For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, - the NES gain is 13.17% - 36.2% from the following 3 sources - R1-2404625, R1-2405247, R1-2405363 - For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 16.8% - 28.77% from the following 6 sources - R1-2403942, R1-2405341, R1-2404224, R1-2404859, R1-2404561, R1-2405595 - One source (R1-2404224) reports 10.5% NES gain with medium cell load on Cell A - For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 12.71% – 12.88% from one source (R1-2405341) - For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 14.43% - 27.47% from the following 3 sources - R1-2404561, R1-2403942, R1-2404859 Agreement For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 160ms SIB1 period (Case C), FR1, low load - For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 1.2 – 8.3% from the following 7 sources - R1-2405595, R1-2403979, R1-2405341, R1-2404859, R1-2405363, R1-2404625, R1-2404561 - For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.2% - 5.46% from the following 4 sources - R1-2403979, R1-2405341, R1-2405363, R1-2404625 - For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 2.87% - 7.38% from the following 4 sources - R1-2404859, R1-2405363, R1-2404625, R1-2404561 - For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.97% - 4.48% from the following 2 sources - R1-2405363, R1-2404625 - For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 3.04% - 5.57% from the following 4 sources - R1-2403942, R1-2405341, R1-2404859, R1-2404561 - For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 1.79% from one source (R1-2405341) - For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 2.12% - 5.26% from the following 3 sources - R1-2404561, R1-2403942, R1-2404859 Agreement For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 20ms SIB1 period (Case A), FR1, medium load - For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 9.88% - 14.4% from the following 2 sources - R1-2405341, R1-2405363 - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98 - 4.5% with one beam of SIB1 PDSCH transmitted in each slot - 2.3% with two beams of SIB1 PDSCH transmitted in each slot - For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 4.92% - 8.4% from the following 2 sources - R1-2405341, R1-2405363 - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98 - 1.5% with one beam of SIB1 PDSCH transmitted in each slot - 0.5% with two beams of SIB1 PDSCH transmitted in each slot - For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, - the NES gain is 11.5% - 12.88% from one source (R1-2405363) - One source (R1-2404122) reports the following with FDMed SSB and PDSCH of SIB1 - 1.94%~4% NES gain with UL WUS configuration transmitted in legacy SIB on Cell A - 0%~1.94% NES gain with UL WUS configuration transmitted in separated SIB on Cell A. The separated SIB is TDMed with legacy SIB and transmitted in a 20ms period using dedicated resource. - 0% NES gain with Case 1 (Options 1+A+X) and WUS configuration transmitted in separate SIB on the NES cell - 2.64% NES gain with Case 1 (Options 1+A+X) and pre-configured for fixed WUS configuration - NES cell does not transmit the WUS configuration - For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 6%~7.53% from one source (R1-2405363) - For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 8.08% - 15.47% from the following 4 sources - R1-2405341, R1-2404224, R1-2404561, R1-2403942 - For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 4.09% – 4.19% from one source (R1-2405341) - For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 10.78% - 15.3% from the following 2 sources - R1-2404561, R1-2403942 Agreement For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 160ms SIB1 period (Case C), FR1, medium load - For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.3 – 2.44% from the following 4 sources - R1-2403979, R1-2405341, R1-2405363, R1-2404561 - For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.1% - 1.3% from the following 3 sources - R1-2403979, R1-2405341, R1-2405363 - For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.24% - 2.26% from the following 2 sources - R1-2405363, R1-2404561 - For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 0.67% - 1.6% from one source (R1-2405363) - For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.95% - 4.13% from the following 3 sources - R1-2403942, R1-2405341, R1-2404561 - For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.48% from one source (R1-2405341) - For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.44% - 4.02% from the following 2 sources - R1-2404561, R1-2403942 Agreement For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 40ms SIB1 period (Case D), FR1, empty load - For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 24.51% - 31.3% from the following 6 sources - R1-2405595, R1-2405341, R1-2404859, R1-2404507, R1-2405363, R1-2404625 - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98 - 16.3% with one beam of SIB1 PDSCH transmitted in each slot - 10.6% with two beams of SIB1 PDSCH transmitted in each slot - For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 12.4% - 35.5% from the following 5 sources - R1-2404408, R1-2405341, R1-2404758, R1-2405363, R1-2404625 - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98 - 10.1% with one beam of SIB1 PDSCH transmitted in each slot - 6.1% with two beams of SIB1 PDSCH transmitted in each slot - For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 19.44% - 28.81% from the following 3 sources - R1-2404859, R1-2405363, R1-2404625 - For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 12.87% - 20.09% from the following 2 sources - R1-2405363, R1-2404625 - For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 11.77% - 25.46% from the following 5 sources - R1-2403942, R1-2405341, R1-2404859, R1-2404507, R1-2404561 - For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 10.69 – 10.74% from one source (R1-2405341) - For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, - the NES gain is 16.46% - 24.24% from the following 2 sources - R1-2403942, R1-2404859 - One source (R1-2404561) reports 8.92%~10.37% NES gain with PRACH configuration index = 148 Agreement For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 40ms SIB1 period (Case D), FR1, low load - For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 18.38% - 26.4% from the following 5 sources - R1-2405595, R1-2405341, R1-2404859, R1-2405363, R1-2404625 - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98 - 8.1% with one beam of SIB1 PDSCH transmitted in each slot - 4.3% with two beams of SIB1 PDSCH transmitted in each slot - For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 9.84% - 17.41% from the following 3 sources - R1-2405341, R1-2405363, R1-2404625 - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98 - 2.8% with one beam of SIB1 PDSCH transmitted in each slot - 1% with two beams of SIB1 PDSCH transmitted in each slot - For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 13.36% - 23.68% from the following 3 sources - R1-2404859, R1-2405363, R1-2404625 - For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 7.27% - 15.59% from the following 2 sources - R1-2405363, R1-2404625 - For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 10.48% - 17.72% from the following 4 sources - R1-2403942, R1-2405341, R1-2404859, R1-2404561 - For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 7.01% from one source (R1-2405341) - For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 7.7% - 17.05% from the following 3 sources - R1-2403942, R1-2404859, R1-2404561 Agreement For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 40ms SIB1 period (Case D), FR1, medium load - For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 5.07% - 7.8% from the following 2 sources - R1-2405341, R1-2405363 - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98 - 2.4% with one beam of SIB1 PDSCH transmitted in each slot - 1.2% with two beams of SIB1 PDSCH transmitted in each slot - For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate, - the NES gain is 2.46% - 4.4% from the following 2 sources - R1-2405341, R1-2405363 - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98 - 0.7% with one beam of SIB1 PDSCH transmitted in each slot - 0.3% with two beams of SIB1 PDSCH transmitted in each slot - For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 5.62~7.01% from one source (R1-2405363) - For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 2.77% - 3.85% from one source (R1-2405363) - For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 4.08%~10.1% from the following 3 sources - R1-2403942, R1-2405341, R1-2404561 - For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 2.02%~2.09% from one source (R1-2405341) - For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 5.61% - 9.94% from the following 2 sources - R1-2404561, R1-2403942 Agreement For the evaluation of NES loss on cell A, the following is observed - NES loss of 0%~0.98% from the following 15 sources with UL WUS configuration transmitted in legacy SIB on Cell A (with different assumptions on SIB periodicity and cell loading): - R1-2404408, R1-2404463, R1-2403942, R1-2403979, R1-2405341, R1-2404224, R1-2404859, R1-2404758, R1-2405162, R1-2404122, R1-2404507, R1-2404033, R1-2405363, R1-2404625, R1-2404561 (for Case 2) - One source (R1-2405106) reports for the case with UL WUS configuration transmitted in separated SIB on Cell A where the separated SIB is TDMed with legacy SIB and transmitted in a 320ms period using dedicated resource. - 0.62%~2.65% NES loss for Case 2 (Options 1+B+X) with empty load - 0.73%~3.20% NES loss for Case 3 (Options 2+B+Y) with empty load - One source (R1-2404122) reports - 2.95%(medium load)~15.49%(empty load) NES loss with UL WUS configuration transmitted in separated SIB on Cell A - The separated SIB is TDMed with legacy SIB and transmitted in a 20ms period using dedicated resource. - 1.28%(medium load)~3.62%(empty load) NES loss with UL WUS configuration transmitted in legacy SIB on Cell A Agreement For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 20ms SIB1 period (Case A), FR1, 8 beams, on-demand SIB1 transmission rate > 30% - For Cat 1 BS, empty load, the NES gain is 37.77% from one source (R1-2404463) - For Cat 2 BS, empty load, - the NES gain is 8.9% - 28.71% from the following 3 sources - R1-2404224, R1-2404561, R1-2404463 - The NES gain is 1.47% from one source (R1-2404561) with 75% on-demand SIB1 transmission rate - For Cat 2 BS, low load, - the NES gain is 7.9% - 14.65% from the following 2 sources - R1-2404224, R1-2404561 - The NES gain is 0.42% from one source (R1-2404561) with 75% on-demand SIB1 transmission rate - For Cat 2 BS, medium load, - the NES gain is 6.2% - 10.4% from the following 2 sources - R1-2404224, R1-2404561 - The NES gain is 0.12% from one source (R1-2404561) with 75% on-demand SIB1 transmission rate Agreement For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 40ms SIB1 period (Case D), FR1, 8 beams, on-demand SIB1 transmission rate > 30% - For Cat 2 BS, empty/low/medium load, the NES gain is 3.77% - 5.97% from one source (R1-2404561) Agreement For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 160ms SIB1 period (Case C), FR1, 8 beams, on-demand SIB1 transmission rate > 30% - For Cat 1/Cat 2 BS, empty/low/medium load, the NES gain is 0.31%~3.33% from the following 2 sources - R1-2404561, R1-2404122 Agreement For the evaluation of the energy consumption to transmit WUS configuration on NES Cell: - One source (R1-2405595) reports that for broadcasting WUS configuration using DCI occupying 2 symbols per SSB beam) on NES cell every 80 ms on CORESET#0 (48 RBs): - For BS category 1, empty/low load, the energy consumption is increased by 6.26%~8.51% over SSB transmission only. The energy saving gain with Case 1 is 40.52% / 32.41% for empty/low load over the baseline with 8 beams, SIB-1 period 20ms (baseline), 0% SIB-1 transmission rate (for on demand SIB1). - For BS category 2, empty/low load, the energy consumption is increased by 1.55%~2.11% over SSB transmission only. The energy saving gain with Case 1 is 28.25% / 23.03% for empty/low load over the baseline with 8 beams, SIB-1 period 20ms (baseline), 0% SIB-1 transmission rate (for on demand SIB1).