3GPP TSG RAN meeting #105										RP-241960

Melbourne, Australia, September 9-12, 2024

Status Report to TSG

Agenda item:			9.2.3

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_Sensing_NR |  |  |  |  |
| Unique ID | 1020086 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-240799 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 06/2025 | Core part: | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: 35 % | Core part: | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Yingyang Li, Jerome Vogedes |
|  | Company | Xiaomi, AT&T |
|  | Email | liyingyang@xiaomi.com, jerome.vogedes@att.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

RAN1 #118, Maastricht, NL, August 19th – 23th, 2024

ISAC deployment scenarios

Conclusion

RAN1 will consider the recommendations for the physical characteristics (e.g., sizes, shapes, materials, velocities, etc.) of sensing targets and objects provided in 5GAA LS (R1-2405964), along with the relevant characteristics defined in 3GPP TRs, within the scope of the Rel-19 study item.

- No LS response from RAN1 to 5GAA is necessary.

- R1-2405964 is proposed to be NOTED.

Agreement

General principles for all sensing target deployment scenarios should consider the following:

- “Sensing mode” is removed in the scenario tables, but may be included in the evaluation/calibration phase. Per the SI, all sensing modes are possible for the deployment scenarios.

- “Sensing area” may be addressed as part of the sensing target distribution and/or Tx/Rx characteristics and/or cell layout.

Agreement

For UAV sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#117 as a baseline.

Note: Additional parameters, value/value ranges are not precluded.

The detailed scenario description in this clause can be used for channel model calibration.

ISAC-UAV

Details on ISAC-UAV scenarios are listed in Table x.

Table x. Evaluation parameters for UAV sensing scenarios

| Parameters |  |  | Value |
| --- | --- | --- | --- |
| Applicable communication scenarios |  |  | UMi, UMa, RMa [38.901] UMi-AV, UMa-AV, RMa-AV |
| Sensing transmitters and receivers properties | Rx/Tx Locations |  | Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenarios. Note1: This may include aerial UEs for UMi-AV, UMa-AV, RMa-AV communication scenarios. In this case, other Rx/Tx properties (e.g. mobility) are also taken from the corresponding communication scenario. |
| Sensing target |  | Outdoor/indoor | Outdoor |
|  |  | 3D mobility | Horizontal velocity: uniform distribution between 0 and 180km/h, if horizontal velocity is not fixed to 0. Vertical velocity: 0km/h, optional {20, 40} km/h NOTE2: 3D mobility can be horizontal only or vertical only or a combination for each sensing target FFS: time-varying velocity. |
|  |  | 3D distribution | Horizontal plane: Option A: N targets uniformly distributed within one cell. Option B: N targets uniformly distributed per cell. Option C: N targets uniformly distributed within an area not necessarily determined by cell boundaries. FFS: Value of N, defined area, and other distributions Vertical plane: Option A: Uniform between 1.5m and 300m. Option B: Fixed height value chosen from {25, 50, 100, 200, 300} m assuming vertical velocity is equal to 0. FFS Other options are not precluded. Note2: target(s) are outside the minimum distance to the Tx/Rx |
|  |  | Orientation | Random in horizontal domain |
|  |  | Physical characteristics (e.g., size) | Size: - Option 1: 1.6m x 1.5m x 0.7m - Option 2: 0.3m x 0.4m x 0.2m FFS: Material(s), Structure, Other size(s) |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  |  | Option B: Min distances based on min. TRP/UE distances defined in TR36.777 as a starting point. Option C: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  |  | Option 1: At least larger than the physical size of a target Option 2: 10 meters |
| [Unintended/Environment objects, e.g., types, characteristics, mobility, distribution, etc.] |  |  | FFS |

Note: further down-selection between the options in the table is not precluded.

ISAC channel modelling

Agreement

if RCS related coefficient of a scattering point is included in small scale, the RCS related coefficients are separately determined for different pairs of incident/scattered ray(s) at the scattering point.

Agreement

For radio propagation Case 1, for modelling the target channel of a target with single scattering point,

- To model a direct path, a single LOS ray from Tx to target and a single LOS ray from target to Rx are generated

  - AoA/ZoA of the direct path at Rx, AoD/ZoD of the direct path at target are generated at least based on the 3D location of target and Rx in the global coordinate system

  - AoD/ZoD of the direct path at Tx, AoA/ZoA of the direct path at target are generated at least based on the 3D location of Tx and target in the global coordinate system

  - The Delay of the direct path = (d3D_tx_target + d3D_target_rx)/c

  - The Doppler of the direct path is generated by spherical unit vectors by AoD/ZoD at Tx, by spherical unit vectors by AoA/ZoA at Rx, and velocity of Tx, target and Rx

  - The power of the direct path is generated as the product of the power of the LOS ray from Tx to target, the power of the LOS ray from target to Rx, and the effect of RCS

  - FFS initial phase

  - FFS how to model RCS, polarization of target

- FFS number of direct paths

- FFS on detailed modelling of indirect path(s)

- FFS applicability of direct path generation to each scattering point when the target is modelled as multiple scattering points

Agreement

For the target channel of a target with single scattering point, when stochastic cluster is used to model an indirect path in the target channel,

- An indirect path in small scale is modelled by concatenation of path(s) from Tx to target and from target to Rx, i.e., Option 1 in the agreement of RAN1 #117

  - AoD/ZoD/AoA/ZoA from Tx to target or from target to Rx are

    - generated for a LOS ray at least based on the 3D location of Tx/Rx and target in the global coordinate system

    - stochastically generated for a NLOS ray using section 7, 38.901 as starting point

  - Delay is sum of delay of LOS/NLOS ray from Tx to target and the LOS/NLOS ray from target to Rx

  - Doppler is generated by spherical unit vector by AoD/ZoD at Tx and velocity of Tx, by spherical unit vector by AoA/ZoA at Rx and velocity of Rx, and by spherical unit vectors by AoA/ZoA/AoD/ZoD at target and velocity of target

    - FFS The mobility of stochastic clutter

  - The power of the indirect path is generated as the product of the power of the LOS/NLOS ray from Tx to target, the power of the LOS/NLOS ray from target to Rx and the effect of RCS

  - FFS initial phase

  - FFS how to model effect of RCS at target

  - FFS whether/how to model polarization at target

  - FFS How to reduce complexity

  - FFS applicability of the indirect path generation to each/a single scattering point when the target is modelled as multiple scattering points

Working assumption

The RCS related coefficient of a scattering point can be modelled with two components, i.e., linear value RCS = A*B

- A first RCS component A (m2) is included in large scale

  - FFS the first RCS component is deterministic or stochastic

  - FFS The first component is dependent on incident and scattered directions at target

- A second RCS component B (unit ratio) is included in small scale

  - FFS The second component is dependent on incident and scattered directions at target

  - FFS the second RCS component is deterministic or stochastic or combination

- Note: RCS component A or B can be disabled by setting its linear value to 1

  - Whether to disable a component can be discussed per target type

- FFS how to determine A and B for each target

- FFS whether/how to model polarization impact at target

- FFS whether/how to normalize power accounting for target channel and background channel

Agreement

The impact of a scattering point of the target in the target channel is modelled by a scalar RCS value RCS times a complex-valued 2x2 polarization matrix CPMsp, i.e., RCS∙CPMsp

  - FFS whether CPMsp is angular/ray-dependent or independent

  - FFS whether polarization matrix CPMsp is modelled assuming specular reflection or random coefficient for diffraction or scattering

  - FFS whether polarization matrix CPMsp is explicitly modelled or merged with other polarization matrixes from Tx to target and/or from target to Rx

Agreement

For modeling stochastic cluster in background channel, in order to define the background channel for TRP-UE and UE-TRP bistatic sensing mode,

- The large scale and small scale parameters defined in TR 38.901, TR 37.885, TR 36.777 are used as start point

In order to define the background channel for TRP-TRP and UE-UE bistatic sensing mode,

- RAN1 to study how to model background channel

  - Option 1: The large scale and small scale parameters defined in TR 38.901, TR 38.858, 37.885, 38.859 are used as start point

  - Option 2: New channel model based on measurement results or ray-tracing model validated by experimental results

FFS whether/how to do power normalization between target channel and background channel

Agreement

In order to define the background channel for TRP and UE mono-static sensing mode,

- RAN1 to study how to model the background channel

  - Option 1: randomly drop at least one virtual Rx, and then the background channel is generated based on the channel generated as TR 38.901 between the real Tx and each virtual Rx for a scenario

    - FFS EO is modelled in the background channel

  - Option 2: New channel model based on measurement results or ray-tracing model validated by experimental results or radar literatures

    - FFS EO is modelled in the background channel

  - Option 3: the locations of clusters in the target channel are deterministically generated, then the background channel is generated using the clusters with the determined locations.

  - Other options are not precluded

Agreement

When EO type-2 is modelled, specular reflection is considered to model EO type-2 using section 7.6.8 of TR 38.901 as reference

- As starting point, the effect of type-2 EO (i.e., in the path node1-EO-node2) is modelled as bR∥specular00-R⊥specular, b is a scaling factor (e.g., c equals to d3DdGR relative to LOS ray in section 7.6.8 in TR 38.901)

  - FFS any update to b,R∥specular,R⊥specular

  - FFS any update taking EO orientation into account

- FFS any changes based on section 7.6.8 of TR 38.901 if EO type-2 has finite size

- FFS whether diffraction and scattering can be considered in addition to specular reflection

- EO type-2 is an optional modelling component if supported in a sensing scenario

  - FFS which deployment scenario(s) EO type-2 will apply

2.1.2	Remaining Open issues

The following open issues need to be addressed:

- ISAC deployment scenarios

  - Channel model calibration/evaluation parameters for sensing target scenarios other than UAV, i.e., Automotive, Humans (indoor/outdoor), AGVs, and objects creating hazards on roads/railways.

  - Whether/how to parameterize unintended/environment objects in the deployment scenarios.

- ISAC channel modelling

  - Physical object modelling

    - Exact RCS value/pattern for each target type

    - RCS for bistatic

    - Details on modelling object with multiple scattering points

  - Target channel

    - Details on concatenation of Tx-target link and target-Rx link, and complexity reduction

    - Polarization matrix of direct/indirect path

    - Details on generating power of direct/indirect path

    - Number of direct paths for a target with single scattering point

    - Target channel modelling for target with multiple scattering point

    - Remaining details on EO type-2

    - Whether to model interaction between two targets

    - Details on Doppler modelling

  - Background channel

    - Remaining details on background channel for bistatic/monostatic sensing modes

  - Combining target channel and background channel

    - With or without power normalization

  - Further details on spatial consistency

  - Calibration of the ISAC channel model

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1 #118

R1-2405922	Discussion on ISAC deployment scenarios	Spreadtrum Communications

R1-2405999	Discussion on ISAC deployment scenarios	CMCC, China Southern Power Grid

R1-2406009	Deployment scenarios for ISAC channel modeling	Intel Corporation

R1-2406047	Discussion on ISAC deployment scenarios and requirements	EURECOM

R1-2406067	Discussion on deployment scenarios for ISAC	Tejas Network Limited

R1-2406075	Discussion on ISAC deployment scenarios	KPN N.V.

R1-2406084	Discussion on ISAC deployment scenarios	Tiami Networks

R1-2406098	Discussion on ISAC deployment scenarios	China Telecom

R1-2406137	Discussion on ISAC deployment scenarios	Nokia, Nokia Shanghai Bell

R1-2406196	Views on Rel-19 ISAC deployment scenarios	vivo

R1-2406207	Discussion on ISAC deployment scenarios	InterDigital, Inc.

R1-2406249	Discussion on ISAC deployment scenarios	OPPO

R1-2406298	Deployment scenarios and evaluation assumptions for ISAC channel model	Xiaomi

R1-2406382	Discussion on ISAC deployment scenarios	CATT, CICTCI

R1-2406450	Discussion on ISAC deployment scenarios	LG Electronics

R1-2406483	Discussion on ISAC deployment scenarios	Sony

R1-2406488	Deployment scenarios for integrated sensing and communication with NR	NVIDIA

R1-2406528	Discussion on ISAC deployment scenarios	Panasonic

R1-2406529	Discussion on deployment scenarios for ISAC	Hanbat National University

R1-2406664	Discussion on  ISAC deployment scenarios	Samsung

R1-2406715	Discussion on ISAC deployment scenarios	Lenovo

R1-2406767	Discussion on ISAC deployment scenario	MediaTek Inc.

R1-2406792	Discussion on ISAC deployment scenarios	TOYOTA InfoTechnology Center

R1-2406856	Discussion on ISAC deployment scenarios	Apple

R1-2406867	Deployment Scenarios for ISAC Channel Modeling	AT&T, FirstNet

R1-2406896	Considerations on ISCA deployment scenarios	CAICT

R1-2406905	Discussion on ISAC Deployment Scenarios	Ericsson

R1-2406944	Study on deployment scenarios for ISAC channel modelling	NTT DOCOMO, INC.

R1-2406959	Discussion on ISAC deployment scenarios	ZTE Corporation, Sanechips

R1-2406974	Deployment scenarios for ISAC channel model	Huawei, HiSilicon

R1-2407043	Discussion on ISAC deployment scenarios	Qualcomm Incorporated

R1-2406873	FL Summary #1 on ISAC Deployment Scenarios	Moderator (AT&T)

R1-2406874	FL Summary #2 on ISAC Deployment Scenarios	Moderator (AT&T)

R1-2406875	FL Summary #3 on ISAC Deployment Scenarios	Moderator (AT&T)

R1-2405923	Discussion on ISAC channel modeling	Spreadtrum Communications

R1-2406000	Discussion on channel modeling methodology for ISAC	CMCC,BUPT,SEU, PML

R1-2406010	Discussion on ISAC channel modeling	Intel Corporation

R1-2406048	Discussion on ISAC channel modeling	EURECOM

R1-2406066	Discussion on ISAC channel modelling	Tejas Network Limited

R1-2406085	Discussion on ISAC Channel Modeling	Tiami Networks

R1-2406099	Discussion on ISAC channel modelling	China Telecom

R1-2406107	ISAC Channel Measurements and Modeling	BUPT, CMCC, VIVO

R1-2406138	Discussion on ISAC channel modelling	Nokia, Nokia Shanghai Bell

R1-2406197	Views on Rel-19 ISAC channel modelling	vivo,BUPT

R1-2406208	Discussion on ISAC channel modeling	InterDigital, Inc.

R1-2406211	Measurement-Based EO Study and Multi-Dimensional Target Modeling	NIST

R1-2406250	Study on ISAC channel modelling	OPPO

R1-2406383	Discussion on ISAC channel modelling	CATT, CICTCI

R1-2406451	Discussion on ISAC channel modelling	LG Electronics

R1-2406484	Discussion on Channel Modelling for ISAC	Sony

R1-2406489	Channel modeling for integrated sensing and communication with NR	NVIDIA

R1-2406530	Discussion on channel modelling for ISAC	Hanbat National University

R1-2406665	Discussion on ISAC channel modelling	Samsung

R1-2406698	Channel modeling methodology updates for ISAC	Keysight Technologies UK Ltd

R1-2406710	Discussion on ISAC channel model	Xiaomi, BJTU

R1-2406713	Discussion on ISAC Channel Modelling	Panasonic

R1-2406714	Discussion on Channel Modelling for ISAC	Lenovo

R1-2406768	Discussion on ISAC channel modelling	MediaTek Inc.

R1-2406793	Discussion on ISAC channel modelling	TOYOTA InfoTechnology Center

R1-2406799	General Requirements for Automotive ISAC	DENSO CORPORATION

R1-2406857	Discussion on ISAC channel modelling	Apple

R1-2406868	Discussions on ISAC Channel Modeling	AT&T

R1-2406897	Considerations on ISAC channel modelling	CAICT

R1-2406906	Discussion on ISAC Channel Modelling	Ericsson

R1-2406945	Discussion on ISAC channel modeling	NTT DOCOMO, INC.

R1-2406960	Discussion on channel modelling for ISAC	ZTE Corporation, Sanechips

R1-2406975	Channel modelling for ISAC	Huawei, HiSilicon

R1-2407044	Discussion on ISAC channel modelling	Qualcomm Incorporated

R1-2407092	Discussion on ISAC Channel Modelling	CEWiT

R1-2406299	Summary #1 on ISAC channel modelling	Moderator (Xiaomi)

R1-2406300	Summary #2 on ISAC channel modelling	Moderator (Xiaomi)

R1-2406301	Summary #3 on ISAC channel modelling	Moderator (Xiaomi)

R1-2406302	Summary #4 on ISAC channel modelling	Moderator (Xiaomi)

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template