3GPP TSG RAN Meeting #105                                                               RP- 241969

# Melbourne, Australia, Sep 9 - 12, 2024

Agenda Item:	9.3.1.3

Source:	Apple Inc.

Title:	Discussion on AI/ML for air interface for R19

Document for:	Discussion/Decision

# Introduction

In WID [1], part of the study scope on CSI prediction, CSI compression and other aspects of data and model have a check point in RAN 105 meeting.   In this contribution, we provide our views on the R19 AI/ML for air interface scope of work.

# Discussion

## CSI prediction

In [1], the following study scope is listed for CSI prediction use case.

- For CSI prediction (UE-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843 (e.g., cell/site specific model could be considered to improve performance gain).

In RAN1 #118, the following summary on performance gain are captured. and conclusion are agreed for CSI prediction.

Based on the evaluation for CSI prediction, the following high-level observations are provided:

- From the perspective of basic performance gain over benchmark of non-AI/ML based CSI prediction, under the same UE speed for training and inference

  - for AI/ML based CSI prediction over non-AI/ML based CSI prediction, 0%~7.8% gain depending on traffic model, in terms of mean UPT, is observed by 7 sources

  - for AI/ML based CSI prediction over non-AI/ML based CSI prediction, 3.8%~20.7% gain depending on traffic model, in terms of 5% UE UPT, is observed by 7 sources.

- From a perspective of AI/ML complexity, 19 sources adopt the model subject to the computational complexity in units of FLOPs from 0.05M to 3000M. For complexity of non-AI/ML benchmark, 16 sources adopt the algorithm (e.g., Kalman filter, Auto-regression, Wiener filter) subject to the computational complexity in units of FLOPs from 0.14M to 107M. The ratio of FLOPs (AI/ML over benchmark 2) ranges from 1 to 35, which is used by majority sources. For non-AI/ML benchmark, main computation complexity is dominated by filter updates, which may not be need to be updated per inference at the expense of performance loss.

- From the perspective of performance impact on channel estimation error and phase discontinuity, compared to non-AI/ML CSI prediction, higher gain is observed by 10 sources for AI/ML based CSI prediction in the presence of channel estimation error or phase discontinuity.

- From the perspective of generalization over various scenarios/configurations (e.g., various UE speed, deployment scenario, carrier frequency) that have been evaluated, compared to generalization Case 1 where the AI/ML model is trained with dataset subject to a certain scenario#B/configuration#B and applied for inference with a same scenario#B/configuration#B

  - For generalization Case 2 where the AI/ML model is trained with dataset from a different scenario#A/configuration#A, generalized performance may be achieved for some certain combinations of scenario#A/configuration#A and scenario#B/configuration#B but not for others.

  - For generalization Case 3 where the training dataset is constructed with data samples subject to multiple scenarios/configurations including scenario#B/configuration#B, generalized performance of the AI/ML model can be achieved.

The following conclusion are agreed for CSI prediction in RAN1 115.

Agreement

From RAN1 perspective, study of CSI prediction has been completed and performance improvement is observed with increased complexity.

Comparing to benchmark 2, we observe that AI based solution is more robust to impairment and achieve reasonable performance gain particular for cell edge UEs. The complexity has a wide range due to different methods of filter design and AI model design. It is an implementation choice to optimize the model for complexity and performance tradeoff.

Based on this study, we recommend normative work for CSI prediction, focusing on data collection and performance monitoring.

Proposal 1: Start normative work for CSI prediction sub-use case.

Proposal 2: For R19 AI/ML normative work on CSI prediction with UE side model, support the following:

- Specify necessary signaling/mechanism(s) to facilitate LCM operations including data collection and performance monitoring.

- Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE.

## CSI Compression

In [1], the following study scope is captured.

- For CSI compression (two-sided model), further study ways to:

  - Improve trade-off between performance and complexity/overhead

    - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc.

  - Alleviate/resolve issues related to inter-vendor training collaboration.

while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.

SI compression focus on two aspects: alleviate/resolve inter-vendor training complexity and improve performance and complexity tradeoff. Five different options and sub-options are studied to alleviate/resolve inter-vendor training collaboration. However, the feasibility and performance of each option/sub-option need further study.

For trade-off between performance and complexity, both time-frequency-spatial domain CSI compression and localized model are studied, and results are captured in TR 38.843. In general, higher performance gain is observed with similar complexity compared to frequency-spatial domain CSI compression.

In RAN1 115, RAN1 conclude that

RAN1 is recommending extending the study of the AI/ML CSI compression based on RAN1 understanding the study on CSI compression is not completed.

For case 2 and case 4 of time-frequency-spatial domain CSI compression, potential specification discussion are limited due to lack of time. In the remaining R19 study for CSI compression use case, we recommend focusing on inter-vendor collaboration options and potential specification impact.

Proposal 3: Continue study of CSI compression sub-use case.

Proposal 4: For R19 AI/ML study of CSI compression using two-sided model, further study ways to

- Alleviate/resolve issues related to inter-vendor training collaboration.

- Potential specification on signaling/mechanism(s) to facilitate LCM operations for spatial-frequency domain CSI compression, and case 2 and case 3 of time-frequency-spatial domain CSI compression.

## Other aspect of model and data

In [1], the following aspects are captured on model identification, model transfer and data collection for UE side model training.

- Necessity and details of model Identification concept and procedure in the context of LCM [RAN2/RAN1]

- CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]:

  - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection

  - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods

- Model transfer/delivery [RAN2/RAN1]:

  - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

During R19, RAN1 did not discuss anything on CN/OAM/OTT collection of UE-sided model training data.

On model identification, 3 different options are further discussed. MI-option 1 is used for one sided model with association ID. Further details on the association ID and corresponding procedure will be discussed in beam management WI. MI-option 2 and MI-option 3 are used with two-sided model with training collaboration type 1 and type 3 (inter-vendor collaboration option 3/5 and 4 respectively). As further discussion is tightly coupled with CSI compression use case, it is more efficient to discuss it with CSI compression use case.

Proposal 5: Remove RAN1 from CN/OAM/OTT collection of UE-sided model training data

Observation: MI-option 2, MI-option 3 and model transfer corresponding to different training collaboration types/inter-vendor collaboration options of CSI compression. The necessary of model identification and model transfer depends on the CSI compression inter-vendor collaboration conclusion.

# Conclusion

In this contribution, we discussed aspects on R19 AI based enhancement WI scope. Based on the discussion, the following proposals have been proposed.

Proposal 1: Start normative work for CSI prediction sub-use case.

Proposal 2: For R19 AI/ML normative work on CSI prediction with UE side model, support the following:

- Specify necessary signaling/mechanism(s) to facilitate LCM operations including data collection and performance monitoring.

- Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE.

Proposal 3: Continue study of CSI compression sub-use case.

Proposal 4: For R19 AI/ML study of CSI compression using two-sided model, further study ways to

- Alleviate/resolve issues related to inter-vendor training collaboration.

- Potential specification on signaling/mechanism(s) to facilitate LCM operations for spatial-frequency domain CSI compression, and case 2 and case 3 of time-frequency-spatial domain CSI compression.

Proposal 5: Remove RAN1 from CN/OAM/OTT collection of UE-sided model training data

Observation: MI-option 2, MI-option 3 and model transfer corresponding to different training collaboration types/inter-vendor collaboration options of CSI compression. The necessary of model identification and model transfer depends on the CSI compression inter-vendor collaboration conclusion.

# Reference

[1] RP-240774, Revised WID on artificial intelligence (AI)/machine learning (ML) for NR air interface, March 18-21, 2024

[2] TR 38.843, Study on AI/ML for NR air interface, Nov 2023