3GPP TSG RAN WG Meeting #105	RP-241974

Melbourne, AU,  9 September – 12 September, 2024

Agenda Item:	9.3.2.1

Source: CEWiT

Title:	Discussion on Conditional LTM

Document for:	Discussion

# Introduction

Based on 3GPP TSG RAN meeting #104, the below are the agreements on WID for NR mobility enhancements Phase 4

• Specify support of conditional LTM [RAN2, RAN3, RAN1] ◦ Specify UE evaluated conditions for triggering LTM ◦ Aim to support conditional LTM including subsequent LTM ◦ Prioritise intra-CU LTM ◦ Checkpoint to review objective at RAN#105. RAN WG work to not start before this checkpoint

We will be discussing the importance and feasibility of Conditional LTM, and the challenges that comes with it.

- Discussion

In this section we will be discussing on Rel-18 LTM procedure for Intra-CU mobility, and also Rel-16 procedure for CHO. With this background we will compare and state the improvements that are required for Conditional LTM (CLTM).

## Introduction

Given below is the flow of events from Rel-18 Intra-CU LTM for reference :

Given below is the flow of events in CHO (Rel-16) for reference :

Design of CLTM

With Rel-18 LTM and CHO as the baseline, CLTM could be designed for intra-CU mobility. The primary goals are to be defined in facilitating the UE to make the LTM decisions and acquiring specific information which were previously provided by the source gNB.

Observation 1:  Similar to CHO, CLTM for Intra-CU mobility would have legacy intra-CU LTM as the baseline

Proposal 1: Rel-18 LTM to be the baseline for Rel-19 CLTM

## Detailed Discussion on CLTM phases

We will discuss CLTM by citing all the challenges and scope for work that comes in each of the handover phases.

### HO Preparation

During this phase, along with the legacy candidate cell configurations, extra configurations are to come from source gNB conveying the conditions that are to be evaluated by the UE to arrive CLTM decisions. This could be different for different candidate cells. The security key is assumed to not change during intra-CU handover. If there is TA available at the source gNB towards a particular candidate cell, this should also be included.

Observation 2:  UE needs the conditions to be evaluated in order to make the decisions

Proposal 2: Source to configure the conditions to do a CLTM in RRC Reconfiguration. Details to be studied.

Early Synchronization and LTM decision

If early synchronization is required, UE should be able to perform it without involving source gNB (including PDCCH order in some cases) as much as possible. For this, all the related information required for early RACH and specific triggers and timer configurations need to be provided in prior, by the source gNB. Since LTM cell switch is not expected from source, TA information must come in this phase itself. It could be from source, directly from candidates/target cell. This must be facilitated.

Observation 3:  Source may not send PDCCH order to trigger the early synchronisation

Proposal 3: Methods to perform early synchronization without source gNB’s involvement is to be studied

Observation 4:  UE to receive TA information before the LTM decision is made

Proposal 4: Support transfer of TA for each candidate cell either through source and/or through candidate/target

Observation 5: Source may not be sending Candidate Cell TCI state activation/deactivation MAC CE

Proposal 5: Mechanism to activate/deactivate TCI states is to be studied

Observation 6: UE will not receive LTM Cell Switch Command

Proposal 6: Decision to be  made at the UE regarding the target cell, and acquiring all the information that were previously coming in LTM cell switch command.

Target Preparation and HO execution

In this phase since source gNB is not involved in the decision, target must be informed of the arrival of the UE. In case of RACH-based, preamble transmission would serve as an indication. In RACH-less LTM, target could be notified of UE’s arrival by sending specific small sized indication (FFS existing or new) with required flags (if required), towards target. Target could, upon receiving the indication, process to send any further required information to the UE.

Observation 7: Source is not involved in the HO execution process

Proposal 7: Target gNB to be indicated about the arrival of the UE so that it can further send the necessary information to the UE

HO completion

This phase can be reused as such from Rel-18 LTM because notification to the source, as well as the source notifying other candidates about the successful completion shall remain unchanged in CLTM.

Subsequent CLTM

To support CLTM or Subsequent CLTM, each candidate might choose different conditions to be configured. Example, a CLTM session can be followed by a S-LTM; but a S-LTM might not have sufficient information to carry out a C-LTM. This is because an indication from network typically overrides any conditional decisions taken at the UE. This must be facilitated in the specification.

Observation 8: Subsequent CLTM (SCLTM) is to be supported considering the nature of LTM

Proposal  8: Validity of preambles, resources, TA etc are to be studied to ensure supporting SCLTM

Proposal 9: Inter-operability of LTM, SLTM, CLTM and SCLTM to be studied

## Conclusion

Given below is the summary of our observations, and corresponding proposals:

- Proposal 1: Rel-18 LTM to be the baseline for Rel-19 CLTM

- Proposal 2: Source gNB to configure the conditions to do a CLTM in RRC Reconfiguration. Details to be studied.

- Proposal 3: Methods to perform early synchronization without source gNB’s involvement is to be studied

- Proposal 4: Support transfer of TA for each candidate cell either through source and/or through candidate/target

- Proposal 5: Mechanism to activate/deactivate TCI states is to be studied

- Proposal 6: Decision to be  made at the UE regarding the target cell, and acquiring all the information that were previously coming in LTM cell switch command.

- Proposal 7: Target gNB to be indicated about the arrival of the UE so that it can further send the necessary information to the UE

- Proposal  8: Validity of preambles, resources, TA etc are to be studied to ensure supporting SCLTM

- Proposal 9: Inter-operability of LTM, SLTM, CLTM and SCLTM to be studied