3GPP TSG RAN Meeting #105	RP-241990

Melbourne, Australia, September 9-12, 2024

Agenda Item:		9.2.5

Source:		Nokia

Title:				Confirmation of RAN4 Scope for the Study on AI/ML for Mobility

Document for:		Discussion

Introduction

The following RAN4 related objective is captured in the SID on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR [1]:

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - NOTE 1: RAN1/3 work can be triggered via LS - NOTE 2: RAN4 scope/work can be defined and confirmed by RAN#105 after some RAN2 discussions (within the RAN4 pre-allocated TUs) NOTE 3: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 NOTE 4: Two-sided model is not included

Since it is noted that RAN4 scope/work can be defined and confirmed by RAN#105, in this paper consider the RAN2 objectives and status of the study and discuss whether there is a need to clarify further RAN4 objective.

Discussion

Overview of AI/ML for mobility SI

The FS_NR_IAML_Mob focuses on mobility enhancement in RRC_CONNECTED mode over air interface by following existing mobility framework, i.e., handover decision is always made at the network side. Mobility use cases focus on standalone NR PCell change. UE-side and network-side AI/ML model can be both considered, respectively.

The following objectives are listed in the SID [1]:

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2] - The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2] - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2 - Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID - Potential specification impacts of AI/ML aided mobility [RAN2] - Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - NOTE 1: RAN1/3 work can be triggered via LS - NOTE 2: RAN4 scope/work can be defined and confirmed by RAN#105 after some RAN2 discussions (within the RAN4 pre-allocated TUs) NOTE 3: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 NOTE 4: Two-sided model is not included

The RAN2/RAN4 TU allocation of this SID is as following [2]:

Table 1: TU allocation

| RAN2#125bis | #126 | #127 | #127bis | #128 | #129 | #129bis | #130 | #131 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.5 | 1.5 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
|  |  |  | RAN4#112bis | #113 | #114 | #114bis | #115 | #116 |
|  |  |  | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |

The work plan for the study item was discussed at RAN2#126 [3]. The work in RAN2 starts with the discussion of the simulation assumptions.

Figure 1: Possible evaluation plan in RAN2.

At the first two meetings, the focus was mostly on RRM measurement prediction use-case. At the RAN2#127 meeting, the discussion of the RLF use-case has also started. However, it seems that Measurement even prediction and HOF discussion will take place only at a later stage.

Additionally, mobility-related LCM discussions in RAN2 are also planned for the second half of the study item.

- RAN2 work has started with the discussion of simulation assumptions and RRM measurement prediction use-case. RLF use-case was also considered. Evaluation of measurement event prediction use-case, generalization and LCM aspects for mobility are expected to take place at a later stage.

RRM Measurement Prediction

In RAN#125bis the following sub-use cases were agreed for RRM measurement prediction:

- Sub-Case 1: To predict beam level results, then generate cell level results based on the predicted beam results.

- Sub-Case 2: To directly predict cell level results based on cell level results.

- Sub-Case 3: To directly predict cell level results based on beam level results.

In the draft of TR 38.744 [4], the two main identified study goals are:

- Measurement reduction. Achieve the same mobility performance, but with reduced DL radio measurements that the UE needs to take and report, which improves UE energy efficiency and reduces the required signalling.

- Mobility optimization. Predict future UE radio measurements and use them to optimize HO performance and mobility decisions.

Moreover, the following prioritization of the scenarios was captured:

Table 2: Prioritization of evaluation scenarios (Table 5.2.1-1 in TR 38.744 draft)

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

      - AI/ML Measurement Prediction Based Measurement Reduction (UE-Side, Study goal 1)

To avoid additional complexity, i.e., in the design of new reporting mechanisms, in the 1st goal of measurement reduction, the network is expected to use the predicted measurements reported by a UE, or measurements derived from predicted and measured values, as it uses the current measurements reports. Requirements for the accuracy of predicted measurements may follow the requirements for baseline measurements, but RAN4 should study if there are potentially new or updated requirements when AI/ML-based predicted measurements are reported for measurement reduction purpose.

Figure 2: 3GPP TS 38.300 Measurement Model

As shown in Figure 2, the reported predicted measurements or measurements derived from predictions may include:

- Beam-level layer-1 filtered measurements (A1)

- Cell-level layer-3 filtered measurements (D)

- Beam-level layer-3 filtered measurements (F)

RAN4 will need to study if there are potentially new or impacted requirements for AI/ML-based predicted beam level L1/L3-filtered or cell-level L3-filtered measurements for measurement reduction purpose.

Two alternative methods for measurement reduction have been identified:

- Spatial: when UE is not measuring every cell and/or beam

- Temporal: when UE may skip some of the measurement time steps

In both approaches, either the skipped measurements or measurements derived both from skipped and actually taken measurements are predicted with AI/ML. Temporal and spatial measurement reduction may be also combined. RAN2 signaling may be required to coordinate the measurement reduction patterns between the UE and the network.

RAN4 will need to study the impacts of the measurement reduction patterns, and configurations on the existing requirements following RAN2 agreements.

An additional use case in the inter-cell mobility measurement reduction is inter-frequency measurement prediction, in which the set of measured or predicted cells or beams may include cell or beams from different frequency layers. It can be used to reduce the required inter-frequency measurements a UE needs to take, which in turn can reduce the number of measurement gaps that the UE needs to be configured with. Reducing measurement gaps can improve the data throughput and reduce the latency. The required accuracy of the inter-frequency measurement prediction needs to be studied based on how the predictions are used. Additionally, RAN4 to study any impacts due to coordinating the measurement gaps between the network and the UE.

RAN4 will need to study the required accuracy of inter-frequency measurement prediction and any impacts due to coordinating the measurement gaps between the network and the UE.

      - Future Measurement Prediction for Mobility Optimization (UE-Side, study goal 2)

The other aspect of measurement prediction is the enhancement of mobility/HO performance in the case when the time/spatial density of measurements is kept the same, i.e., there is not measurement reduction. In this use-case, future measurements predicted with AI/ML are used to optimize mobility decisions. The measurements may be predicted by the UE and either reported to the network or used in the UE to derive further predictions, for example predicting handover or radio link failures or measurement events. The exact use case and decisions, where the predicted measurements are used to optimize mobility performance, and their signaling impact are still subject to further study in RAN2.

Depending on the use cases considered by RAN2, RAN4 may study requirements for reported predicted future measurements or for other predictions derived from those measurements with the goal to optimize mobility performance/HO without reduction of measurements.

Handover and Radio Link Failure Prediction

Handover and radio link failure prediction was agreed to be studied for UE-side AI/ML. In RAN2#126 the following agreements were made:

- Study Indirect: RLF prediction based on the temporal domain serving cell measurement predictions (e.g. SINR). - Study Direct: Directly RLF prediction by AI/ML models. - FR2 study will be prioritized for RLF prediction - The study should focus on RLF due to T310 expiry (i.e. in-synch/out-of-synch case) as the representative RLF case for direct and indirect prediction. - HOF prediction is downprioritized in our study. NO simulations/evaluations should be done/submitted - RLF prediction result is the RLF probability within a time window or at time instance, at least for direct case. FFS on expected RLF time and indirect case.

Additionally, further progress on the RLF evaluation was achieved at RAN2#127:

- Both direct and indirect are allowed. Companies should indicate what they used and what inputs they are using - Output for indirect: predicted SINR. Based on predicted SINR the time instance the RLF occurs can be determined without further AI/ML models. - Output for direct: probability of RLF within an window - Companies should report the prediction time window they have used in the simulations

Depending on the RAN2 study outcome and proposed use cases using the predicted RLFs, RAN4 will need to study requirements for the reported RLF predictions, e.g., the required accuracy and interoperability of those. RLF prediction might also have an impact on the interruption time associated with re-establishment procedures.

Depending on the RAN2 study outcome and proposed reporting formats and utilization of the predicted RLFs, RAN4 will need to study requirements for the reported RLF predictions, e.g., accuracy and interoperability of reports and latency of related procedures.

Measurement Event Prediction

Predicted measurement events may be used to optimize mobility performance by more pro-active reaction to handover situations. Depending on how the predicted measurement events are used, different accuracy is required for the predictions. RAN4 will need to study the required accuracy metrics and required accuracy in different measurement event prediction scenarios.

RAN4 will need to study the required accuracy metrics and core requirements for measurement event prediction depending on the progress of this use-case in RAN2.

Other RAN4 aspects of AI/ML-Based Mobility

Looking specifically at the existing RAN4 objective we can break it into the following components for evaluation:

- Testability,

- Interoperability,

- Impacts on RRM requirements and performance

In our view, all these sub-objectives are relevant and require RAN4 work and evaluation for the considered RAN2 use-cases and scenarios.

Introduction of new AIML-based features poses new Testability challenges. Similarly to beam management (BM) use-case in NR_AIML_Air WI, prediction of measurements will require the presence of multiple beams in the testing setup so that could measure those and use as an input for inference. This is assumption is very different from the current RRM tests where just a few beams are sufficient. Moreover, the ongoing BM work focuses purely on single-FR2 carrier FR2, where the new mobility scenarios also consider FR1 and inter-carrier and inter-cell predictions.

Interoperability of procedures and reporting can be ensured in two ways:

- Via standardisation of control messages and report formats what is done in RAN2, i.e., so that the same information and formats are used be different UE vendors and can be interpreted in the same way by different NW vendors;

- Via RAN4 testing of the content of the messages and reports, i.e., that UEs of different vendors are reporting the same values in the same conditions and with expected latency so that NW can interpret these reports timely and in the same manner.

For example, RAN4 will need to discuss how to ensure that “reported probability of RLF within a window” will have the same meaning for different UE vendors.

We already discussed potential impacts on RRM requirements and performance in the previous sections (Observations 2-7). Additionally, it is important to highlight that RAN4 may have different requirement KPIs (metrics) than RAN2, e.g., because of the availability of the ground truth/reference values and their accuracy in the tests. Evaluation of new requirements can be associated with a need to perform new simulations, that will take into account RAN4 specific aspects, such as errors/accuracy of the inputs/measurements, latencies of the procedures, new metrics, etc. On the other hand, as it was also emphasized by some companies, such evaluation will be challenging to the very limited number of RAN4 TUs in the SI. Therefore, the reasonable approach would be to, firstly, evaluate the assumptions considered in RAN2, and if a strong need is identified, to consider further simulation of a specific scenario in RAN4.

It is important that RAN4 ensures that assumptions are realistic....

Finally, testing an AI/ML feature implemented using AI/ML may have new requirements for its testing compared to a feature that does not include AI/ML, mainly depending on the required life-cycle management (LCM). However, LCM procedures.

Durgin the offline discussion that took place in the end of RAN4#112 meeting, no strong views were identified about whether the current RAN4 objective has any missing or demanding components:

Informal summary: - No concrete proposals about enhancements of the RAN4 part of the SID were identified in this offline - Options for the beginning of RAN4 work: - RRM measurement prediction use-case analysis in RAN4 to be discussed - Start with the analysis of RAN2 assumptions from RAN4 perspective - Then decide how to use these outcomes, e.g., internal RAN4 dedicated study or capture those in TR. - Consider new testability aspects, e.g., FR1, multi/inter-cell - Other discussions are not precluded

- RAN4 objective in its current form already covers the main aspects that need to be analyzed in the study.

RAN4 objective for Rel-19 FS_NR_AIML_Mob can be left without changes.

At the same time, we believe that RAN4 work will require focus due to the limited number of allocated TUs. For example,

- RAN4 to follow the priorities and timeline of scenarios identified by RAN2 that reflect the interest and expected gains of the companies per use-case.

  - Start with RRM measurement prediction use-case (including, FR1 to FR1 intracell and inter-cell/inter-carrier, FR2 to FR2 intra-cell), and add other use-cases following RAN2 maturity.

- Avoid the discussion of general aspects that should be primarily covered by the ongoing Rel19 WI and focus on use-case/scenario specific aspects.

- RAN4 to base the analysis on RAN2 simulations assumptions and outcomes, however, evaluation of new metrics and assumptions are not prohibited.

- LCM aspects in application to the use-cases considered in the SI can be considered at later stage.

On the practical side, I think that the work should be arranged in the following tracks demonstrated in Figure 3:

- Start with RRM measurement prediction use-cases, then add RLF, and then maybe mobility even prediction, depending on the progress in RAN2.

- Evaluate the use-cases from RAN4 perspective:

  - Discuss for RAN4 metrics and ground truth and their feasibility

  - Review RAN2 simulations assumptions and conclude whether RAN4 aspects are considered or what needs to be considered

  - If need agreed, simulate very limited case(s) using updated assumptions

- New aspects of testability and testing setup, e.g.,

  - Testing in FR1

  - Testing in multi-cell, inter-carrier

  - Avoid direct overlap with/reuse WI testing setup discussion

- Identification of relevant RAN4 requirements, how they can be applied and/or modified and whether new requirements might be needed

  - Discussion of the requirement values and concrete formulations should be left to the WI phase.  The goal is to list the options.

- In the second half of the SI consider impacts of test coverage, generalization, monitoring, LCM, and other AI/ML specific aspects in application to considered use-cases.

Figure 3: Possible evaluation plan in RAN4.

RAN4 work needs to be focused due to the limited number of allocated TUs.

The RAN4 work should follow RAN2 progress, priorities of the use-cases, and avoid overlaps with ongoing discussions on testing setups and LCM in ongoing Rel-19 NR_AIML_Air WI. This can be clarified in RAN4 work plan.

Conclusion

In this paper we overview the RAN2 driven study on AIML mobility from RAN4 perspective and elaborate whether RAN4 SID objectives need to be changed.

The following Observations were made:

Observation 1: RAN2 work has started with the discussion of simulation assumptions and RRM measurement prediction use-case. RLF use-case was also considered. Evaluation of measurement event prediction use-case, generalization and LCM aspects for mobility are expected to take place at a later stage.

Observation 2: RAN4 will need to study if there are potentially new or impacted requirements for AI/ML-based predicted beam level L1/L3-filtered or cell-level L3-filtered measurements for measurement reduction purpose.

Observation 3: RAN4 will need to study the impacts of the measurement reduction patterns, and configurations on the existing requirements following RAN2 agreements.

Observation 4: RAN4 will need to study the required accuracy of inter-frequency measurement prediction and any impacts due to coordinating the measurement gaps between the network and the UE.

Observation 5: Depending on the use cases considered by RAN2, RAN4 may study requirements for reported predicted future measurements or for other predictions derived from those measurements with the goal to optimize mobility performance/HO without reduction of measurements.

Observation 7: RAN4 will need to study the required accuracy metrics and core requirements for measurement event prediction depending on the progress of this use-case in RAN2.

Observation 8: RAN4 objective in its current form already covers the main aspects that needs to be analyzed in the study.

Observation 9: RAN4 work needs to be focused due to the limited number of allocated TUs.

Based on the Observations above, the following Proposals are made:

Proposal 1: RAN4 objective for Rel-19 FS_NR_AIML_Mob can be left without changes.

Proposal 2: The RAN4 work should follow RAN2 progress, priorities of the use-cases, and avoid overlaps with ongoing discussions on testing setups and LCM in ongoing Rel-19 NR_AIML_Air WI. This can be clarified in RAN4 work plan.

- References

- RP-240082, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, OPPO, RAN#103.

- R2-2402167, Discussion on work plan of AI mobility SI, OPPO, MediaTek, Nokia, RAN2#125-bis, Changsha.

- R2-2404711, Discussion on work plan of AI mobility SI, OPPO, Nokia, MediaTek, RAN2#126, Fukuoka.

- R2-2406309, Text proposal of 38.744, OPPO, RAN2#127, Maastricht.