3GPP TSG RAN Meeting #105	RP-241996

Melbourne, Australia, September 9-12, 2024

Status Report to TSG

Agenda item:			9.3.4.10

| WI / SI Name | NR demodulation performance: Phase 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: No | Performance part: Yes |  | Testing part: No |
| Acronym | NR_demod_Ph5-Perf |  |  |  |  |
| Unique ID | 1032080 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-241297 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: NA | Performance part: 09/2025 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: NA | Performance Part: 14% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Jingzhou Wu, Hidekazu Shimodaira |
|  | Company | China Telecom, NTT DOCOMO |
|  | Email | wujingzhou@chinatelecom.cn, hidekazu.shimodaira.sa@nttdocomo.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

- The Work plan for NR demodulation performance: Phase 5 is approved in R4-2413569.

- The Topic summary for [112][328] NR_demod_Ph5 is provided in R4-2413428

- The Offline meeting minutes for [112][328] NR_demod_Ph5 is provided in R4-2413565

- The Way Forward for [112][328] NR_demod_Ph5 is approved in R4-2413570 with the following agreements:

  - UE performance requirements for 8Rx:

Test scope - RAN4 to introduce UE demodulation requirements for both TDD and FDD - Not to cover PDSCH absolute physical layer throughput requirements for 8Rx Channel model - Only consider TDL channel model Test parameters for different devices with 8Rx - Not to define different set of requirements for FWA, vehicular and industrial devices Receiver type for PDSCH requirements with intra cell inter user interference - Only consider MMSE-IRC

  - BS demodulation performance requirements for MMSE-IRC:

MMSE-IRC Receiver assumption - Reuse the same receiver model for MMSE-IRC in TR36.884 as below: MMSE-IRC receiver The MMSE-IRC receiver weight matrix is usually defined as where and denote the estimated channel matrix and covariance matrix, respectively. To obtain the MMSE-IRC receiver weight matrix, the covariance matrix including the sources of inter-cell interference needs to be estimated. The covariance matrix is estimated at DM-RS REs by following equations: , , where the DM-RS symbols are used to estimate the covariance matrix, and Nsp is the number of sampling DM-RS REs. The estimation of interference covariance matrix is performed at per PRB and per TTI basis.. Baseline receiver assumption (for performance gain study only) - MMSE receiver as defined in TR36.884: Baseline receiver: MMSE receiver The MMSE receiver weight matrix is expressed as follow: , where and denote the estimated channel matrix and noise power, respectively. P1 is the transmission power of UE scheduled by the serving cell and is equal to .. Signalling model - Reuse the same signalling model in TR36.884 with agnostic to the waveform 4.1 Signal model For PUSCH, the receiver equations are provided as the reference receiver for evaluation purpose. Assume that is the frequency domain representation of the k-th subcarrier in the -th DFT-OFDM symbol for the -th UE scheduled by its serving BS. is obtained with DFT operation of the time-domain modulating symbol, denoted as , where , with DFT size of . The NRx-dimensional received signal vector of the k-th subcarrier and the l-th DFT-OFDM symbol by the serving BS of the 1st UE is assumed to be expressed as a sum of the signal from the 1st UE, the interference signals (j>1) from other UEs and the white noise , as where , represent the (NRx x NStream) effective channel matrix between the -th UE and the serving BS of the 1st UE, containing the contribution from all receiver branches. is the total number of UEs scheduled by their corresponding BSs in the network. The recovered NStream x 1 signal vector for the 1st UE at its serving BS is detected as follows: where the (NStream x NRx) receiver weight matrix is determined according to the type of PUSCH receiver, as discussed in the following sections. The detected time-domain modulating symbol, denoted as , can be obtained with a IDFT operation for . There will be two types of PUSCH receiver, as MMSE receiver and MMSE-IRC receiver. The equalization is assumed to be performed in the frequency domain.. Network type - Only synchronous scenario

2.4.2	Remaining Open issues

- Further discussion on the test scope, test parameters and simulation assumptions for UE and BS requirements.

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

4.	References

| R4-2411026 | Discussion on UE requirements with 8Rx MMSE-IRC receiver | MediaTek inc. |
| --- | --- | --- |
| R4-2411027 | Discussion on 8Rx MMSE-IRC PDSCH requirements for scenario with intra-cell inter-user interference | MediaTek inc. |
| R4-2411045 | Discussion on 8Rx UE Demodulation with MMSE-IRC receiver | Nokia |
| R4-2411118 | Discussion on BS demodulation performance for MMSE-IRC | CATT |
| R4-2411183 | Discussion on UE demodulation performance requirements for 8Rx with MMSE-IRC | China Telecom |
| R4-2411184 | Discussion on BS demodulation performance requirements for MMSE-IRC | China Telecom |
| R4-2411391 | On UE demodulation performance requirements with 8RX | Apple |
| R4-2411517 | Views on BS demodulation requirements for MMSE-IRC | Qualcomm Germany |
| R4-2411759 | (NR_demod_Ph5-Perf) Discussion on interference suppressing performance for 8Rx UE | CMCC |
| R4-2411760 | (NR_demod_Ph5-Perf) Discussion on BS demodulation requirements for interference suppressing with MMSE-IRC receiver | CMCC |
| R4-2412143 | On general issues for demodulation requirement with interference | Ericsson |
| R4-2412144 | On MMSE-IRC receiver for interference mitigation with 8Rx | Ericsson |
| R4-2412319 | Discussion on MMSE-IRC BS demodulation requirements | Ericsson |
| R4-2412333 | BS demodulation performance requirements for MMSE-IRC | Intel Corporation |
| R4-2412464 | Views on 8Rx UE demodulation and CSI requirements with inter-cell and intra-cell interference | QUALCOMM Europe Inc. - Spain |
| R4-2412763 | Overviews on UE 8Rx IRC performance requirements with interference | Huawei,HiSilicon |
| R4-2412764 | Overviews on BS IRC performance requirements with inter cell interference | Huawei,HiSilicon |
| R4-2412790 | Discussion on UE demodulation performance requirements for 8Rx with MMSE-IRC | ZTE Corporation, Sanechips |
| R4-2412791 | Discussion on BS demodulation performance requirements MMSE-IRC | ZTE Corporation, Sanechips |
| R4-2412878 | Discussion on UE demodulation performance requirements for 8Rx with MMSE-IRC | Samsung |
| R4-2412905 | NR Demodulation Performance Phase 5: BS Demodulation Performance Requirements for MMSE-IRC | Nokia |
| R4-2413428 | Topic summary for [112][328] NR_demod_Ph5 | Moderator (China Telecom) |
| R4-2413444 | Initial discussion on BS demodulation requirement with MMSE-IRC receiver | Samsung |
| R4-2413565 | Offline meeting minutes for [112][328] NR_demod_Ph5 | China Telecom |
| R4-2413569 | Work plan for NR demodulation performance: Phase 5 | China Telecom, NTT DOCOMO |
| R4-2413570 | Way Forward for [112][328] NR_demod_Ph5 | China Telecom |