3GPP TSG RAN Meeting #105	RP-242013

Melbourne, Australia, September 9-12, 2024

Agenda Item:	15

Source: 	CEPRI

Title: 	Discussion on Efficient OCDM System Design with Underloaded Factor

Document for: 	Discussion and Decision

Introduction

In high-mobility wireless environments, the channel response changes rapidly due to fast movement between the transmitter and receiver through a multi-path environment, commonly known as a doubly selective channel. This type of fading channel can substantially impair the performance of traditional orthogonal frequency-division multiplexing (OFDM) wireless communication systems, in which a single-tap equalizer is usually employed for the sake of system simplicity. However, this approach comes at the price of high sensitivity to Doppler spread, which breaks the orthogonality of subcarriers in OFDM system, and a sophisticated Doppler compensation algorithm has to be deployed to mitigate the doubly selective channel effects.

Orthogonal chirp division multiplexing (OCDM) is a novel multi-carrier modulation scheme first proprosed in [1]. Different chirped waveforms occupy the same bandwidth and time duration, achieving the maximum spectrum efficiency of chirp spread spectrum (CSS) [2-4]. OCDM systems have been proved to outperform OFDM systems in many aspects, especially in doubly selective scenarios. The compact infrastructure of OCDM transceivers is achieved through phase rotation similar to OFDM systems. Consequently, OCDM systems are increasingly recognized in various communication scenarios, such as optical communications [5][6], underwater acoustic channels [7], and joint radar sensing and communication [8][9].

Discussion

Discussion

However, existing studies primarily focus on fully loaded communication systems, where each subcarrier transmits a constellation point. In practical communication systems, it is possible to configure empty subcarriers, thereby designing an underloaded communication system. In an underloaded OCDM system, some chirp subcarriers do not transmit symbols, a structure that is equally applicable to OFDM systems. Compared to fully loaded systems, underloaded systems can achieve a signal-to-noise ratio (SNR) gain, though at the expense of a reduced data rate, allowing for a trade-off based on practical requirements. In underwater acoustic communications, the underloaded OCDM system has been demonstrated to outperform the underloaded OFDM system, particularly in terms of BER performance. Additionally, some researchers have pointed out that frequency domain spatial modulation systems can be considered a specific type of underloaded MIMO-OFDM system, as they distribute transmitted symbols across multiple antennas, thereby reducing the power demand on high-power amplifiers.

Observation 1: For various multicarrier systems, introducing a overloaded factor a by not utilizing all subcarriers can enhance both the bit error rate (BER) performance and system capacity.

Proposal 1: Study how to introduce a underloaded factor a to modulate system capacity and enhance BER performance for OCDM system.

Conclusion

In this contribution, underloaded OCDM system with different underloaded factors was discussed with giving corresponding proposals:

Proposal 1: Study how to introduce a underloaded factor a to modulate system capacity and enhance BER performance for OCDM system.

Reference

[1] X. Ouyang and J. Zhao, “Orthogonal chirp division multiplexing,” IEEE Trans. Commun., vol. 64, no. 9, pp. 3946–3957, 2016.

[2] M. S. Omar and X. Ma, “Designing OCDM-based multi-user transmissions,” in 2019 IEEE Global Commun. Conf. (GLOBECOM), 2019, pp.1–6.

[3] M. S. Omar and X. Ma, “Performance analysis of OCDM for wireless communications,” IEEE Trans. Wireless Commun., vol. 20, no. 7, pp.4032–4043, 2021.

[4] M. A. Khan, R. K. Rao, and X. Wang, “Performance of quadratic and exponential multiuser chirp spread spectrum communication systems,” in 2013 Int. Symp. on Performa. Eval. of Comput. and Telecommun. Sys. (SPECTS). IEEE, 2013, pp. 58–63.

[5] X. Ouyang and J. Zhao, “Orthogonal chirp division multiplexing for coherent optical fiber communications,” J. of Lightw. Tech., vol. 34, no. 18, pp. 4376–4386, 2016.

[6] C. Browning, D. Dass, P. Townsend, and X. Ouyang, “Orthogonal chirp division multiplexing for future converged optical/millimeter-wave radioaccess networks,” IEEE Access, vol. 10, pp. 3571–3579, 2021.

[7] P. Zhu, X. Xu, X. Tu, Y. Chen, and Y. Tao, “Anti-multipath orthogonal chirp division multiplexing for underwater acoustic communication,” IEEE Access, vol. 8, pp. 13 305–13 314, 2020.

[8] L. G. de Oliveira, B. Nuss, M. B. Alabd, Y. Li, L. Yu, and T. Zwick,“MIMO-OCDM-based joint radar sensing and communication,” in 2021 15th Eur. Conf. on Antennas and Propag. (EuCAP), 2021, pp. 1–5.

[9] V. Savaux, “Flexible communication system for 6G based on orthogonal chirp division multiplexing,” in 2022 1st Int. Conf. on 6G Netw. (6GNet). IEEE, 2022, pp. 1–5