3GPP TSG RAN Meeting #105							RP-242028

Melbourne, Australia, September 9-12, 2024

Agenda Item:	9.3.1.1

Source:	Ericsson

Title:	On extending Rel-19 MIMO scope

Document for:	Discussion

# 1	Introduction and proposal

The Rel-19 MIMO Ph5 WI has progressed very well during the initial meetings and is ahead of the predicted time schedule. This leaves room for a discussion on extending the scope of the ongoing MIMO WI. In our view, such an extension should be limited to extension, completion and small improvements of the existing features in scope of the ongoing WI. We shall not extend the Rel.19 MIMO WI scope to new features.

Therefore, we have this proposal with the detailed motivations in Section 2.

Modify the Rel-19 MIMO Ph5 scope with the following extension of the current features:

- For the asymmetric DL sTRP/UL mTRP feature, extend the scope to include two time advance (2TA)

- For the 3Tx UE feature, extend the scope to support partial-coherent UL codebook and non-codebook based operation.

# 2	Motivation and further consideration of the extension

## 2.1 Asymmetric DL/UL

The current feature work on “UL only TRP” or “asymmetric UL/DL” is restricted to a single time advance (TA) command. This severely limits the usefulness of the feature since signals needs to be received within a CP, and an extension to 2 TA is very straightforward since the 2TA feature already exists in NR specifications for multi-DCI based multi-TRP (Rel.18).

When the propagation difference is larger than 703 meters for the asymmetric deployment in a typical FR1 network with 30kHz SCS, as shown in the table below, the timing difference of the receiving TRPs will be larger than a CP. It means without 2TA being supported for asymmetric cell, even assuming ideal LOS propagation and zero synchronization error among the TRPs, the UL-only TRPs can only be allocated close to the anchor TRP. For 30kHz SCS that is mostly used for FR1 TDD and assumably the major configuration for asymmetric deployment, the distances between UL-only TRP and anchor TRP is maximum 703 meters.

Table 2: Maximum inter-TRP Distance of Normal CP unless scope is extended to 2TA

| SCS (kHz) | Normal CP (μs) | Distance (meter) |
| --- | --- | --- |
| 15 | 4.69 | 1407 |
| 30 | 2.34 | 703 |
| 60 | 1.17 | 351 |
| 120 | 0.59 | 175 |
| 240 | 0.29 | 87 |

The cell radiuses in real world are typically 3km for 15kHz SCS and 1km for 30kHz SCS, the largest distances listed in the table is the 1407 meters, only half of the cell range, If same TA is maintained for anchor TRP and UL-only TRP, the UL-only TRP must be put close to the cell center, otherwise the UE accesses UL-only TRP and is far from anchor TRP will suffer timing issue. Such performance and cost drawbacks would significantly limit the use of asymmetric deployment.

One straightforward option to support this could be to adapt the Rel-18 multi-DCI based two-TA solutions to single-DCI (sDCI) operation. To do this, only minor changes for the Rel-18 two-TA solution is needed, e.g., removing the association between a TAG and CORESETPooIndex, and using a single DL timing reference signal (from the Anchor TRP) for both TAs, instead of using two DL reference signals as in the Rel-18 multi-DCI two-TA solution.

As UL-only TRP do not transmit any SSB, UE may have to rely on Anchor TRP SSB to derive the DL reference timing. When slot boundaries of Anchor TRP and UL-only TRP are perfectly aligned in time (i.e., synchronization error is 0µs), UE may use the DL reference timing of Anchor TRP to perform random access procedure and obtain TA command to handle the asymmetric propagation delay difference between Anchor TRP to UE and UL-only TRP to UE.

However, in the practical deployments (such asymmetric deployments considered in this WI) it may not be always possible to maintain perfect synchronization between different TRP (even for intra-DU deployments as most of the NW deployments use something called switched fronthaul).  Figure 1 illustrate synchronization scenario where the timing of UL-only TRP is ahead of Anchor TRP. When UL-only TRP is ahead of Anchor TRP slot timing, when UE acquires DL timing from Anchor TRP and transmits PUSCH using the same DL reference timing, it may result in inter-slot interference at UL-only TRP.

Figure 1: Synchronization scenario UL-only TRP ahead of Anchor TRP slot timing

Based on the above discussion, we have the following proposal to consider practical synchronization error between Anchor TRP and UL-only TRP for UL and DL timing control in an asymmetric deployment.

The extended work should consider practical synchronization error between Anchor TRP and UL-only TRP for UL and DL timing control in asymmetric DL sTRP UL mTRP deployment.

## 2.2 2.2 Support implementation of partially coherent 3Tx UE

Current scope only supports Non-coherent (NC) transmission from the 3 Tx UE and associated precoders. The NC have mainly two disadvantages:

- Firstly, a 3 Tx UE transmitting with NC precoders for rank<3 and with Rel-15 power scaling (PS), cannot transmit with full power. For example, for rank 1 and 2 PUSCH, a 3 Tx UE operating with Rel-15 PS can transmit with ~1.8 dBm lower power compared to a 2 Tx UE. This can adversely affect the performance of the coverage limited UEs. The lower transmit power can affect the PUSCH channel estimation (especially for the coverage limited UEs).

- Secondly, for antenna configurations where at least two antennas in the same polarization are arranged in a ULA, the NC precoders fail to achieve a precoding gain of 3 dB. As was analysed in [3], these disadvantages can result in loss in performance, such that 3 Tx UEs without full power Mode 0 actually perform worse that 2 Tx fully coherent UEs, which implies that the current non-coherent-only design forces UEs to have full power PAs on each Tx chain. On the other hand, if a UE with 1/3 power PAs supports coherence over just two of 3 of its Tx chains, there can be substantial gain over 2 Tx operation.

Maintaining the coherence over a pair of Tx chains (of existing UEs supporting 2 coherent Tx chains) while adding a non-coherent branch can allow a 3 Tx UE to avoid the need for full power PAs.

Now, the discussion so far in RAN1 on this topic reveals many different options. Design of a new 3 Tx codebook from scratch would take significant time and effort in RAN1.  However, zeroing out elements of Rel-15 4 Tx partially coherent precoders requires minimal design effort and provides a good performing codebook.  Such a design requires at most 8 partially coherent precoders for ranks 1 and 2 in addition to the 7 non-coherent precoders currently used in the 3 Tx codebook [3].

For rank 3, no additional precoder is needed, i.e, the non-coherent (identity matrix) precoder is sufficient and would simplify the standardization process. Therefore, our view is to grab the low hanging fruit and introduce the basic, simplest scheme by port blanking:

Creating a partial coherent codebook for 3 Tx can be simply done by adding 8 precoders to the non-coherent codebook constructed by setting a same row of all 4 Tx precoders to zero (‘port blanking’), and results in a good performing codebook that can be completed in the limited time available in Rel-19.

With this approach and scope captured in the updated WID, the additional work to support this is very limited.

Lastly, we observe that introducing non-codebook based precoding for 3Tx is merely a UE capability discussion and needs zero time in the MIMO session. Hence, this should be added as well, for completeness, in this scope extension.

# Conclusion

We made the following observations:

Observation 1	The extended work should consider practical synchronization error between Anchor TRP and UL-only TRP for UL and DL timing control in asymmetric DL sTRP UL mTRP deployment.

Observation 2	Maintaining the coherence over a pair of Tx chains (of existing UEs supporting 2 coherent Tx chains) while adding a non-coherent branch can allow a 3 Tx UE to avoid the need for full power PAs.

Observation 3	Creating a partial coherent codebook for 3 Tx can be simply done by adding 8 precoders to the non-coherent codebook constructed by setting a same row of all 4 Tx precoders to zero (‘port blanking’), and results in a good performing codebook that can be completed in the limited time available in Rel-19.

Given the observations, we propose the following:

Proposal 1	Modify the Rel-19 MIMO Ph5 scope with the following extension of the current features:

	For the asymmetric DL sTRP/UL mTRP feature, extend the scope to include two time advance (2TA)

	For the 3Tx UE feature, extend the scope to support partial-coherent UL codebook and non-codebook based operation.

# References

R1-2403834 “LS on 3T6R and 4T6R antenna switching SRS”, RAN4(Huawei)

RP-240087 “WID revision: NR MIMO Phase 5”, Moderator(Samsung)

R1-2401077 “Support for 3 Tx UL transmissions”, Ericsson

R1-2405302 “Discussion on RAN4 LS on 3T6R and 4T6R SRS antenna switching”, Ericsson

RP-240828 “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN4 chair (Huawei)