3GPP TSG RAN meeting #105 	RP-242039

Melbourne, Australia, Sep. 9-12, 2024

Source: 	Huawei, HiSilicon, Samsung, NTT DoCoMo

Title: 	Discussion on SRS Port Grouping in Rel-19 MIMO

Agenda Item:	9.3.1.1

Document for:	Discussion/Decision

# 1 Introduction

In RAN1#117 and RANP#104 meeting, there was a discussion on SRS port grouping for supporting up to 6/8 layers for low complexity 6Rx/8Rx receiver in Rel-19 MIMO. It was concluded to be further discussed in RANP#105 meeting.

Postpone discussions on the following topics until RAN#105. These topics will not be discussed in RAN1 before the decision in RAN#105. - Support of two TAs as part of enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios - SRS port grouping as part of CSI enhancement - Partial-coherent UL codebook for 3Tx multi-panel - Open loop-based UL precoder cycling

In this contribution, we discuss the SRS port grouping for 6/8Rx receivers, and provide the analysis on spec impact as well.

# 2 Discussion

Why introduce SRS port grouping:

With increasing number of ports in gNB side, e.g., 128 ports or more, it is high probability for downlink high rank data transmission for a UE to enhance the performance, such as up to rank-6 or rank-8 for 6Rx and 8Rx UEs, respectively. However, due to the complexity of demodulation processing for high ranks, it is difficult to have more than rank-4 data transmission in the practical cases, especially for smart phones. On other words, due to the complexity issue, the transmission will be restricted as no more than rank-4 for 6Rx or 8Rx. From network perspective, the complexity issue restricts the high layers transmission and also restricts the network performance.

The effective solution to reduce the complexity for higher rank is to divide UE receive antennas into two low dimensional antenna groups, where each antenna group corresponds to a low dimensional CSI acquisition and PDSCH reception. As an example, 8Rx could be divided into two 4Rx receivers instead. With the introduction of low complexity 8R receivers, the complexity to support max Rank-8 could be reduced by 60%.

Then, the antenna groups information needs to be aligned between gNB and UE, otherwise the precoding for data transmission will be mixed for different antenna groups. To align the antenna group information between gNB and UE, SRS ports need to be grouped (mapped to two UE antenna groups), and corresponding to the two CWs for supporting more than rank-4.

As discussed above, to enable high rank transmission for 6Rx and 8Rx, reduce the complexity for high rank and support SRS port grouping is required. Therefore, we have following proposal:

Proposal 1: Support SRS port grouping and CW association to receive antenna groups to enable high rank transmission for 6/8Rx with low complexity.

We also show the performance evaluation for the low complexity receiver with SRS port grouping in the following figure. It can be observed that 43% performance gain can be achieved by PDSCH reception with SRS port grouping compared to max Rank-4 restriction. If without SRS port grouping, due to the misalignment of precoding and receiver antennas, the performance is very bad shown in the figure. By the way, the performance of 8Rx with up to rank-8 is also shown in the figure as an upper bound, which is difficult to be implemented due to the high complexity as discussed above. The simulation assumptions are listed in Appendix, where the inter cell interference, practical SRS channel estimation and channel aging are assumed in the evaluation.

Spec Impact of SRS port grouping:

In the following, we provide the detailed analysis of Spec impacts for the SRS port grouping.

In RAN1, the issue is already discussed, as it is a part of CSI enhancement in Rel-19. To align the understanding between gNB and UE, the only Spec change is to define the SRS port grouping and the CW association. E.g., for 8Rx, SRS ports in resources are numbering as (0,1), (2,3), (4,5), (6,8), then the SRS ports could be defined as two groups (0,1,2,3), and (4,5,6,7), and each group associated with a CW for two CW case. Alternatively, the SRS ports also could be grouped as (0,2,4,6), (1,3,5,7). The detailed proposal for SRS grouping could be found in RAN1#117 Chairman’s notes and also in Feature lead summary R1-2404108, there is no additional TU required.

For RAN4, there is no new REFSENS required, since the existing 4Rx could be reused. Then, for the demodulation requirement, it already has definition of demodulation performance requirement for “simplified 8Rx receiver” with rank-2 and rank-4 in Section 5.2.4 and Section 5.2A.4 of TS38.101-4, the only work is to extend the requirement for low complexity receiver to rank-6 and rank-8, while the performance standard work in RAN4 could be handled in the reserved MIMO RAN4 TUs during performance stage, no additional TU required.

Proposal 2: The standard work for SRS ports grouping is very limited, no additional TU required for RAN1 and RAN4.

SRS port grouping is a part of TDD CSI enhancement. If needed, the WID for MIMO in Rel-19 could be clarified accordingly as follows:

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - Specify SRS port grouping and codeword association to the two segments of receive antennas for the 6/8Rx low complexity receiver supporting up to 6/8-layers, with legacy codebook and legacy codeword-to-layer mapping

# 3 Conclusion

Base on the above discussion, we have the following proposal:

Proposal 1: Support SRS port grouping and CW association to receive antenna groups to enable high rank transmission for 6/8Rx with low complexity.

Proposal 2: There is very limited standard work for SRS ports grouping, no additional TU required for RAN1 and RAN4.

# References

[1] RP-240087, “WID revision: NR MIMO Phase 5”, Rapporteur (Samsung)

[2] Draft Report of 3GPP TSG RAN WG1 #117 v0.3.0

[3] R1-2404108	“Moderator summary on Rel-19 CSI enhancements” Moderator (Samsung)

Appendix:

Table 1. Evaluation assumptions for SRS port grouping for up to 128 CSI-RS ports

| Parameters | Evaluation assumptions |
| --- | --- |
| Duplex, Waveform | TDD, OFDM |
| Carrier Frequency | 3.5 GHz |
| Channel Model | CDL-C channel model in TR 38.901 |
| Delay Spread | 100ns |
| BS antenna configuration | (M, N, P, Mg, Ng; Mp, Np) = (8, 16, 2, 1, 1; 4, 16), (dH, dV) = (0.5, 0.8) |
| UE antenna configuration | (M, N, P, Mg, Ng; Mp, Np) = (2, 2, 2, 1, 1; 2, 2), (dH, dV) = (0.5, 0.5) |
| MCS | Link adaptation (256QAM MCS Table) |
| SRS Bandwidth | 48RB |
| Numerology | 14 OFDM symbol per slot, 30kHz SCS |
| UE speed | 3km/h |
| Precoding granularity | 2RB |
| SRS periodicity | 5ms |
| Receiver Type | MMSE-IRC |
| SRS Channel Estimation | Real channel estimation |
| Inter cell interference | Exist |