3GPP TSG-RAN Meeting #105                                              	RP-242056

Melbourne, Australia, September 9-12, 2024

Source:	MediaTek Inc.

Title:	Rel-19 MIMO: Scope Expansion

Agenda Item:	9.3.1.1

Document for:	Discussion and decision

# Introduction

In RAN#102, it was agreed that the scope expansion of the RAN#102 approved RAN1/2/3-led projects could be further discussed and approved at a later stage of Rel-19, with the earliest opportunity being September 2024 [1]. Given that the progress of NR MIMO Phase 5 [2] is on track or even ahead of schedule, scope expansion should be both feasible and reasonable to utilize the remaining TU available in later RAN1 meetings. In this contribution, we will share our views on the potential topics for the scope expansion of NR MIMO Phase 5.

# Discussion

First of all, in our view, the potential scope expansion is considered due to the good progress and utilizing the remaining TU of NR MIMO Phase 5, we therefore don’t see additional TU is needed for scope expansion. However, we should also be careful on the workload for selection of new topics and strive to reuse the enhancements agreed till RAN1#118 as much as possible.

Proposal 1: On scope expansion of NR MIMO Phase 5, no additional TUs are introduced.

In RAN#104, the following topics have been brought up as candidate topics of scope expansion:

- SRS port grouping for 6/8RX receiver

- Partial-coherent codebook-based 3-port transmission

- SRS antenna switching for 3T6R

- 2 TAs for asymmetric DL sTRP/UL mTRP deployment scenarios

- Open-loop UL precoder cycling

Our views on above 5 candidate topics are provided in the followings:

- SRS port grouping for low complexity 8RX receiver

As commercial NR deployments move to higher frequency TDD bands, SRS based DL channel acquisition becomes more important. SRS based channel acquisition can benefit from higher rank transmissions due to full channel information availability at gNB. Correspondingly, UE receivers also need to be ready to receive more data transmission layers. SRS port grouping aims to ease UE receiver complexity for 8-layer reception with SRS antenna switching based DL precoding. The assumption is that gNB acquires the DL channel corresponding to two disjoint UE antenna groups and controls the inter-group interference so that the UE calculates the CQI separately for the two groups. Such feature can be considered as a part of CSI enhancement for a potential future UE implementation. However, whether any network functionality should be assumed to mitigate such mutual interference should also be considered.

- Partial-coherent codebook-based 3-port transmissions

As 3Tx is being targeted for handset UEs and aim to reap the benefits of increased Tx ports, partial-coherent codebooks could be one such feature which can benefit by combining a pair of coherent ports. We should aim considering at least partial-coherent codebooks if not full coherent codebooks for UEs with one pair of coherent ports. Also, the antenna layout with one pair of antennas with a synthesizer that ensure coherency between the ports can be a use case of foldable phones which are gaining traction. The partial-coherent codebooks for 3Tx can reuse the design principles from 2Tx codebooks in view of low specification impact. In summary, we think such feature can be considered as a part of 3Tx UE enhancement for a potential future UE implementation.

- SRS antenna switching for 3T6R

Whether ‘3TnR’ SRS antenna switching is supported in NR MIMO Phase 5 was discussed in RAN1#116, and the following conclusion was reached to preclude ‘3TnR’ SRS antenna switching in Rel-19.

Conclusion There is no consensus in RAN1 to support antenna switching for 3TX UE in Rel-19

However, a new WID for UE RF enhancements was agreed that includes specifying requirements for 6 Rx UE [3]. In RAN#104, it was decided that for this WID, RAN1 will study and, if necessary, specify SRS antenna switching to support 3T6R based on UE capability. Given that the original scope of 3Tx is nearly complete, we believe that SRS antenna switching for 3T6R can be revisited within the scope of NR MIMO Phase 5.

- 2 TAs for asymmetric DL sTRP/UL mTRP deployment scenarios

In Rel-18, 2 TAs can be supported within one serving cell for M-DCI based MTRP operation, with the association with coresetPoolIndex values, by the followings:

- Two TA groups (TAGs) can be configured on a serving cell, and two DL reference timings are assumed for the two TAGs on a serving cell, respectively

- Two TA offset values can be provided for a serving cell

For asymmetric DL STRP/UL MTRP deployment scenarios, supporting 2TA could be necessary to promote the deployment of asymmetric deployment in practical network, and this can be done by removing the dependence of coresetPoolIndex values from 2 TAs specification supported in Rel-18. Considering the specification effort could be quite small, 2 TAs can be considered as a part of the enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios.

- Open-loop UL precoder cycling

To our understanding, frequency-selective precoding-based schemes can significantly impact channel estimation and interference estimation. Additionally, such a scheme represents a substantial deviation from Rel-15, as it would necessitate the definition of uplink PRGs, which could have a considerable impact on UE implementation, despite the relatively small specification change of introducing UL PRGs. On the other hand, UL precoder cycling has the potential to increase UE PAPR, thereby decreasing coverage. Consequently, we believe that further study is necessary for UL precoder cycling, including an evaluation of performance gains over other UL robustness schemes. Then, we do not think it is appropriate to include such an extensive study within the scope expansion of NR MIMO Phase 5.

In summary, even we do not see a strong commercial need to up-scope for above candidate topics at all, if companies are desired to the scope expansion, we are open to those candidate topics have been the most justified.

Proposal 2: Limit consideration of the scope expansion of NR MIMO Phase 5 to the following candidate topics, with the following corresponding constraints:

- SRS port grouping for low complexity 8RX receiver (as a part of Objective 2)

- Apart from grouping SRS ports into disjoint groups, no other specification enhancement is introduced, e.g., new CW-to-layer mapping, DL resource allocation, DCI format.

- No specification optimization to assist the DL receiver for DL reception is introduced, e.g., association among the SRS port groups during PDSCH reception.

- RAN4 assumptions and corresponding workload for this feature would need to be discussed further.

- Partial-coherent codebook-based 3-port transmission (as a part of Objective 4)

- Without further SRS and PT-RS enhancements, i.e., reusing the enhancements agreed for CB-based 3-port transmission.

- Without enhancement on UL full power transmission, i.e., UL full power transmission mode 1 and 2 are not supported.

- SRS antenna switching for 3T6R (as a part of Objective 4)

- If needed, revise NR_ENDC_RF_Ph4 WID correspondingly as well.

- This is independent of the feasibility of 6 layers for handheld UE being studied by RAN4

- 2 TAs for asymmetric DL sTRP/UL mTRP deployment scenarios (as a part of Objective 5)

- Rel-18 2TA specification is reused by removing coresetPoolIndex association, and no further enhancement on PDCCH-order PRACH transmission towards an UL TRP in FR2

# Conclusion

According to the discussion in Section 2, we have the following proposals:

Proposal 1: On scope expansion of NR MIMO Phase 5, no additional TUs are introduced.

Proposal 2: Limit consideration of the scope expansion of NR MIMO Phase 5 to the following candidate topics, with the following corresponding constraints:

- SRS port grouping for low complexity 8RX receiver (as a part of Objective 2)

- Apart from grouping SRS ports into disjoint groups, no other specification enhancement is introduced, e.g., new CW-to-layer mapping, DL resource allocation, DCI format.

- No specification optimization to assist the DL receiver for DL reception is introduced, e.g., association among the SRS port groups during PDSCH reception.

- RAN4 assumptions and corresponding workload for this feature would need to be discussed further.

- Partial-coherent codebook-based 3-port transmission (as a part of Objective 4)

- Without further SRS and PT-RS enhancements, i.e., reusing the enhancements agreed for CB-based 3-port transmission.

- Without enhancement on UL full power transmission, i.e., UL full power transmission mode 1 and 2 are not supported.

- SRS antenna switching for 3T6R (as a part of Objective 4)

- If needed, revise NR_ENDC_RF_Ph4 WID correspondingly as well.

- This is independent of the feasibility of 6 layers for handheld UE being studied by RAN4

- 2 TAs for asymmetric DL sTRP/UL mTRP deployment scenarios (as a part of Objective 5)

- Rel-18 2TA specification is reused by removing coresetPoolIndex association, and no further enhancement on PDCCH-order PRACH transmission towards an UL TRP in FR2

# Reference

- 3GPP RAN#102, “Summary for RAN Rel-19 Package: RAN1/2/3-led”, RP-232745, December 2023

- 3GPP RAN#102, “New WID: NR MIMO Phase 5”, RP-234007, December 2023

- RP-240828, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, 3GPP TSG RAN Meeting #103, March 2024