3GPP TSG RAN Meeting #105									RP-242081

Melbourne, Australia, September 9-12, 2024

Agenda item:	9.3.2.3

Source: 	Huawei, HiSilicon

Title: 	Checkpoints on Rel-19 XR

Document for:	Discussion/Decision

- Introduction

The latest WID for Rel-19 XR enhancements is excerpted as follows [1]:

The Rel-19 XR Phase 3 objectives are as follows: - Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2]. NOTE: Check in RAN#105 (check also other WG involvement if needed). - Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4] - Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. NOTE: LCP implementation complexity need to be taken into account when evaluating solutions. NOTE: Check in RAN#105 - Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. - Specify Core requirements related to the above objectives as necessary [RAN4] - Extend Release 18 standalone mechanism to support NR-NR dual connectivity as follows [RAN3] - PDU set based handling - ECN marking - Burst Arrival Time reporting, if needed - PSI Discard coordination, if needed - Note: No RAN2 impact from above items NOTE: Whether / to what extent network exposure / RAN awareness / e.g. RAN involved rate control, possibly additional info for DL scheduling, parallel with SA2 work, shall be covered in this WI is TBD.

Several checkpoints/option issues still exist in the WID, which should be reviewed in RAN#105.

In this contribution, we focus on those checkpoints marked in the Notes within the WID, and provide our views based on recent progresses achieved in RAN and SA. In addition, we provide an updated WID accordingly.

- Discussion

Aspects of multi-modality

As mentioned in the justification of the WID [1], potential impacts of multi-modality may include RAN awareness enhancements, user plane enhancements and multiple DRX configurations enhancements.

This WI aims to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination, see also TR 22.847, TR 23.700 60. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. Related Potential Impacts has been proposed as follows: a) Enhanced RAN Awareness, by signaling from Core Network and/or indication by UE, b) Enhancements User Plane, e.g. Scheduling, LCP, Resource allocation, Discard. c) Support for multiple DRX configurations, without the potentially problematic restrictions of Pre-Rel-19 2nd DRX.

Next, we will review RAN2 progress for these potential impacts one by one.

First of all, for Multi-Modality awareness in RAN, RAN2 has made the following agreements:

RAN2#126: Support Multi-Modality awareness in RAN in Rel-19 for UL and DL. RAN2#127: Working assumption: Regardless of SA2 decision, RAN2 can extend the UAI for multi-modal awareness at least for uplink QoS flows in Rel-19 XR, by having the UE report existence of multi-modality application and association information among QFIs to gNB. FFS whether this can be applied to DL. RAN2 considers that based on multi-modal information: - The gNB may perform joint admission control. Details can be left up to RAN3 in potential WI phase. FFS if MMSID can be used for this purpose. - The gNB may consider this information during QoS flow to DRB mapping (up to gNB implementation)

RAN2 has concluded to support multi-modality awareness in RAN in Rel-19 for both UL and DL. In RAN2#126, one LS [2] has been sent to SA2 to query if SA2 can provide multi-modality information to RAN. In the LS, RAN2 has listed MMSID and/or synchronization thresholds as examples of multi-modality information, which are seen beneficial from RAN2 perspective. Actually, MMSID has already been introduced in Rel-18 XR by SA2, while it is only used in core network for policy control and is unknown by RAN. Currently, some companies in SA2 still have some concerns about whether CN should provide MMSID to RAN. As for synchronization thresholds, after careful check of SA2’s discussion, it seems SA2 has no interests to introduce such information in Rel-19, no matter in QoS flow level or packet level.

Besides, in RAN2#127, RAN2 has assumed that regardless of SA2 decision, RAN2 can obtain multi-modality information at least for uplink QoS flows from the UE. For downlink, considering that the UE is the receiver of multi-modal XR traffic, we think the UE also has the capability to obtain multi-modality association information from the application layer and report the information to gNB.

Based on the above progress, we propose to specify RAN awareness of multi-modality association information in the WI phase, i.e. via UAI report from the UE, and/or by signaling from CN. RAN usage of the multi-modality association information which has RAN2 consensus is joint admission control and QoS flow to DRB mapping. While the latter is purely up to gNB implementation, the signaling of multi-modal information from CN and specification of joint admission control by using multi-modality association information requires RAN3 WG involvement in WI phase .

Proposal 1: For the study for multi-modality, specify RAN awareness of multi-modality association information as agreed by RAN2 in study phase, i.e. via UAI report from the UE and/or by signaling from CN. RAN3 is involved in WI phase to specify required CN signaling and joint admission control.

Secondly, for user plane enhancements, especially enhancements for traffic scheduling, LCP and discard, the following agreements were made in RAN2#127:

For UL, RAN2 does not intend to perform LCP enhancements due to complexity vs gains concerns. For DL, whether traffic synchronization (on a per packet basis) can be achieved depends on whether packet level synchronization information can be provided from CN to RAN. RAN2 thinks PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the multi-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4. RAN2 thinks in case this is feasible, it should be limited to intra-DRB case.

It is noted that RAN2 has precluded LCP enhancements for multi-modality.

Observation 1: RAN2 has agreed to not support LCP enhancements for multi-modality in Rel-19 XR.

As for traffic synchronization and PDU set discard across QoS flows, RAN2 agrees that they can be achieved if CN can provide synchronization information to RAN, e.g. packet level synchronization information. In our opinion, it will incur considerable work efforts in both SA2 and SA4 to support packet level synchronization awareness at UPF. Furthermore, as mentioned in previous paragraph, SA2 has no interests to introduce synchronization information in Rel-19. Based on these considerations, we propose to not support user plane enhancements based on synchronization information in Rel-19 XR, e.g. traffic synchronization guarantee at RAN, or PDU set discard across QoS flows.

Lastly, for multiple DRX configurations, RAN2#127 has concluded to not support such direction. The relevant agreements are listed as follows:

Not support multiple active DRX configurations Not support independent configuration of drx-StartOffset for the secondary DRX group

Observation 2: RAN2 has agreed to not support DRX enhancements in Rel-19 XR.

Aspects of scheduling enhancements using delay information

For scheduling enhancements using delay information, two potential enhancements have been discussed in study phase:

- Delay-aware LCP enhancement to avoid delay critical data being delayed by non-delay critical data from LCH with higher priority.

- DSR enhancement to report more information than Rel-18 DSR, to assist RAN’s efficient scheduling.

For delay-aware LCP enhancement, RAN2 has made the following agreements:

RAN2#125: - RAN2 will study whether/how to resolve the issue of data with low remaining time being delayed due to other data from LCHs with higher LCH priority when using the existing LCP procedure. At least the following alternatives will be studied: - Alternative 1: Enhance LCP restrictions/LCH selection. - Alternative 2: Enhance LCH prioritization. RAN2#126: - Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR. - The solution should consider impact on UE complexity (as already indicated in SI objective description) - For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - - Use additional priority configured to LCHs in case of these LCHs with delay-critical data RAN2#127: RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR.

RAN2 concluded to support delay-aware LCP enhancement to resolve the issue of data with short remaining delay budget being delayed due to data from other LCHs with no delay critical data. Two candidate solutions have been proposed and discussed. One is to override/adjust the priority of LCH based on delay/deadline information, which has been agreed as the baseline solution. The other is to enhance LCP restriction, which was precluded for further consideration in RAN2#127. Thus, we propose to specify delay-aware LCP enhancement by adjusting the priority of LCH based on delay/deadline information in the WI phase.

For DSR enhancement, RAN2 achieved the following agreements:

RAN2#125: - RAN2 will study enhancing existing DSR with additional information, e.g. multiple pairs of remaining time/buffer information, importance. FFS whether this only includes more information on delay-critical data or also information about non-delay critical data. RAN2#126: - Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. - FFS whether DSR triggering is impacted - FFS whether PDU set importance needs to be included RAN2#127: Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

RAN2 has concluded to enhance DSR to report multiple pairs of remaining time and buffer size per LCG. Even though several FFSs are still on the table, those issues are stage-3 details which can be discussed and resolved in WI phase.

We propose to specify DSR enhancements to report multiple pairs of remaining time and buffer size per LCG in WI phase.

Proposal 2: For the study for UL scheduling, specify delay-aware LCP enhancement by adjusting the priority of LCH based on delay/deadline information, and DSR enhancements to report multiple pairs of remaining time and buffer size per LCG, as agreed by RAN2 in the study phase.

Aspects of network exposure for uplink rate adaptation

In RAN#104, it was endorsed to replace the following note in the WID with the objective to address uplink rate control from RAN, which is complementary to upper layer solutions (e.g. L4S) [3].

NOTE: Whether / to what extent network exposure / RAN awareness / e.g. RAN involved rate control, possibly additional info for DL scheduling, parallel with SA2 work, shall be covered in this WI is TBD.

Currently, RAN is able to provide a recommended bit rate for a specific LCH for uplink to the UE via the Recommended bit rate MAC CE. To effectively support uplink rate control, we think the following issues can be further considered for such MAC CE signaling:

- Granularity of recommended bit rates.

In NR Rel-15 Recommended bit rate MAC CE, 56 codepoints are used to represent the recommended bit rate values from 0 to 8000kbps. In NR Rel-16 E_FLUS [4], in order to support uplink streaming services with extremely high data rates, a multiplier factor which can take a value from 40/70/100/200 was introduced to scale up the recommended bit rate values. With such a multiplier, RAN can indicate a recommended bit rate up to 1.6Gbps, which should be sufficient for XR services.

However, 56 codepoints combined with only 1 configured multiplier may not be able to reflect a bit rate in sufficient granularity. For example, when RAN estimates an available data rate for an LCH is 23Mbps, RAN may indicate a recommended bit rate 20Mbps by using codepoint 30, i.e. 500kbps if the multiplier 40 is configured for the LCH. This incurs some inaccuracy which may affect the service’s experience. RAN2 can consider how to indicate a recommended bit rate in a much finer granularity.

- QoS flow level or LCH level recommendation.

Currently, the MAC CE indicates the recommended bit rate in LCH level. If RAN tries to indicate recommended bit rates for a specific QoS flow, RAN may have to map the QoS flow to a dedicated LCH. If there are quite a few QoS flows co-exist, it may not be suitable to map each QoS flow to a separate LCH. In such case, we think RAN2 can discuss whether to support recommended bit rate indication in QoS flow level.

- Different multipliers for uplink and downlink.

In the current specification, the bit rate multiplier is configured per LCH. It means for an LCH, the uplink and downlink directions are configured with the same multiplier value. Considering for future XR services, there may exist significant differences between downlink bitrate and uplink bitrate. The same multiplier for both directions may not be appropriate for such XR traffic. We think RAN2 can discuss whether to configure different bit rate multipliers for uplink and downlink directions.

To align with the endorsement in RAN#104, we propose to replace the note in the WID for network exposure with the objective to address rate control from RAN, including RAN signaling to address uplink rate control.

We propose such uplink rate control can be based on existing ANBR solution, not only because it has already a mature framework in 3GPP specifications across different WGs, but also because it is already commercialized for IMS services and its required APIs to application layer is already industrialized.

Observation 3: ANBR already has a mature framework in 3GPP specifications across TSGs/WGs, and ANBR is already commercialized and its required APIs to application layer is already in use for IMS services.

Proposal 3: For the TBD, to replace the NOTE in the WID for network exposure with the objective to specify uplink rate control from RAN based on Recommended Bit Rate MAC CE (ANBR).

RAN impacts of SA2’s features

Besides the above checkpoint issues, SA2 has concluded to support the following features in Rel-19 XRM:

- in SA2 [5], it was concluded that RAN reports the UL and/or DL available data rate to the UPF via the GTP-U header and the UPF can further report the available data rate to the AF, to assist the data rate control in server side. As stated in the note, network exposure should be parallel with SA2 work in the WI.

- Dynamic indication of burst size via user plane.

In the new Rel-19 WID on XRM [6], it contains the following objective:

WT#2.2 KI#5 Dynamic traffic characteristics update. - Support dynamic change (via user plane) in traffic characteristics (i.e., burst size) provided by the application in the DN.

This feature will involve RAN3 work to add burst size information in GTP-U header.

- Time to Next Burst (TTNB) indication via user plane.

As stated in [7], based on SMF instruction, the UPF identifies and marks the Time to Next Burst to NG-RAN via DL GTP-U header. TTNB can be used to assist RAN’s DL scheduling. This feature will involve RAN3 work to add TTNB indication in GTP-U header.

- FEC ratio-based enhancements.

As stated in [8], the SMF may provide FEC ratio to NG-RAN when establishing/modifying a QoS flow. RAN node can use the static FEC ratio information to perform PDU discard during congestion. In SA2’s discussion, the FEC ratio is applied to DL only. We think how to perform DL PDU discard in congestion by using the static FEC ratio information can be up to RAN’s implementation, e.g. to proactively discard some PDUs when a certain level of DL congestion is detected. Thus, if such feature is finally approved by SA2, it will involve RAN3 work to support semi-static FEC ratio awareness at RAN.

- AQP enhancements to include PDU set QoS information.

In [9], SA2 concluded to enhance AQP to include PDU set QoS information in the QoS profile. The work will involve RAN3 work in the WI phase.

In order to well capture RAN impacts of those agreed SA2’s features, we propose to add a general objective to specify RAN impacts resulted by SA2 enhancements in Rel-19 XRM. The stage-3 impacts should be for RAN3 only.

Proposal 4: Add a general objective to support above SA2 enhancements in Rel-19 XRM (i.e. exposure of available data rate, indication of burst size, TTNB, FEC ratio-based enhancements and PDU Set QoS parameters in AQP).

- Conclusion

In this paper, we provide our views on the checkpoints/option issues in the XR WID objectives, and make the following observations and proposals:

Observation 1: RAN2 has agreed to not support LCP enhancements for multi-modality in Rel-19 XR.

Observation 2: RAN2 has agreed to not support DRX enhancements in Rel-19 XR.

Observation 3: ANBR already has a mature framework in 3GPP specifications across TSGs/WGs, and ANBR is already commercialized and its required APIs to application layer is already in use for IMS services.

Proposal 1: For the study for multi-modality, specify RAN awareness of multi-modality association information as agreed by RAN2 in study phase, i.e. via UAI report from the UE and/or by signaling from CN. RAN3 is involved in WI phase to specify required CN signaling and joint admission control.

Proposal 2: For the study for UL scheduling, specify delay-aware LCP enhancement by adjusting the priority of LCH based on delay/deadline information, and DSR enhancements to report multiple pairs of remaining time and buffer size per LCG, as agreed by RAN2 in the study phase.

Proposal 3: For the TBD, to replace the NOTE in the WID for network exposure with the objective to specify uplink rate control from RAN based on Recommended Bit Rate MAC CE (ANBR).

Proposal 4: Add a general objective to support above SA2 enhancements in Rel-19 XRM (i.e. exposure of available data rate, indication of burst size, TTNB, FEC ratio-based enhancements and PDU Set QoS parameters in AQP).

- Updated WID objectives

- 4.1 Objective of SI or Core part WI or Testing part WI The Rel-19 XR Phase 3 objectives are as follows: - Specify signalling for RAN awareness of multi-modality association information, i.e. via UAI report from the UE, and by signalling from CN. [RAN2, RAN3] - Specify joint admission control enabled by awareness of multi-modality association information. [RAN3] - Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4] - Specify Enhancements for Scheduling, as follows: - Specify delay-aware LCP enhancement to adjust the priority of LCH based on delay/deadline information. [RAN2]. - Specify DSR enhancements to report multiple pairs of remaining time and buffer size per LCG. [RAN2]. NOTE: LCP implementation complexity need to be taken into account when evaluating solutions. - Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. - Specify Core requirements related to the above objectives as necessary [RAN4] - Extend Release 18 standalone mechanism to support NR-NR dual connectivity as follows [RAN3] - PDU set based handling - ECN marking - Burst Arrival Time reporting, if needed - PSI Discard coordination, if needed - Note: No RAN2 impact from above items - Specify uplink rate control from RAN based on Recommended Bit Rate MAC CE (ANBR) [RAN2] - Specify required RAN impacts for SA2 related enhancements in Rel-19 XRM, including supports on exposure of available data rate, indication of burst size, TTNB, FEC ratio-based enhancements and PDU Set QoS parameters in AQP. [RAN3]

- References

RP-240791, Revised WID on XR (eXtended Reality) for NR Phase 3, 3GPP TSG RAN Meeting #103.

R2-2405782 LS on multi-modality awareness at RAN	TSG RAN WG2 LS out	Rel-19	NR_XR_Ph3-Core	To:SA2 Cc: TSG RAN WG3, TSG SA WG4.

RP-241651, Moderator's summary for offline discussion on REL-19 XR, Nokia.

R2-1911499, LS on Recommended Bit Rate/Query for FLUS and MTSI, 3GPP TSG RAN WG2#107.

S2-2409429, Update the Conclusion for KI#9, 3GPP TSG-SA WG2 Meeting #164.

SP-240990, New WID on Extended Reality and Media service (XRM) Phase 2, TSG SA Meeting #104.

S2-2409548, Updated Conclusions for KI#5 to include TTNB, 3GPP SA WG2 Meeting #164.

S2-2409424, Conclusion for KI#1: AL-FEC Awareness at RAN, 3GPP TSG-SA WG2#164.

S2-2409425, KI#1, conclusion update to fix the EN, SA WG2 Meeting #164.