3GPP TSG RAN Meeting #105	RP-242096

Melbourne, Australia, September 9-12, 2024

Agenda item:	9.3.2.3

Source:	Nokia

Title:	Views on XR for NR Phase 3 WI

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we present our view on Rel-19 WID objectives for XR [RP-240791]. As part of this discussion, we highlight the need for WID objective updates and revision of the RAN3/RAN4 TU allocation.

Specifically, we have a two check points on multi-modality and scheduling enhancement to be discussed in RAN#105:

- Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2]. NOTE: Check in RAN#105 (check also other WG involvement if needed). - Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. NOTE: LCP implementation complexity need to be taken into account when evaluating solutions. NOTE: Check in RAN#105

In addition, we also have another topic on uplink congestion indication for XR. In this contribution, we present our views on these topics.

# 2	[RAN2] Multi-modality

## 2.1	Overall RAN2 progress

In RAN2, the study phase of multi-modality aimed at finding good justification of having it in WID objective. RAN2 has discussed multi-modality for the past three meetings:

RAN2#125bis: The first meeting to start RAN2 discussion on multi-modality. RAN2 had high-level assumptions for further study, e.g., the UE/gNB assumption on having some MM information, DRB mapping, etc. No discussion on the potential enhancement in detail.

RAN2#126: An LS was sent to SA2, asking if any MM information can be provided to RAN. At that time, RAN2 has not yet discussed or concluded on the potential enhancement or its gain/complexity. The reason why we sent LS, even amidst such uncertainty, was to have clearer understanding of what RAN can do or cannot do based on the available information. No discussion on the potential enhancement in detail.

RAN2#127: RAN2 has not received the reply LS from SA2. RAN2 discussed potential mechanism in terms of necessity, complexity, and feasibility. RAN2 excluded LCP, DRX, and haptic-specific scheduling enhancement.

RAN2 spent much time on discussing the availability of multi-modal information in RAN while leaving the detailed discussion on each mechanism to the last one meeting, which was not sufficient to identify the gains and necessity of potential enhancement.

Observation 1: During past three meetings, RAN2 has not sufficiently analysed the potential mechanism especially in terms of gain and benefit.

## 2.2	Views on the potential solutions

In RAN2#127, RAN2 had some discussion and WA/Agreements on 1) UAI enhancement, 2) Joint admission control, 3) DL synchronous delivery, and 4) PDU set discard within DRB.

1) UAI enhancement for multi-modal awareness

Working assumption:

-	Regardless of SA2 decision, RAN2 can extend the UAI for multi-modal awareness at least for uplink QoS flows in Rel-19 XR, by having the UE report existence of multi-modality application and association information among QFIs to gNB.

-	FFS whether this can be applied to DL.

RAN2 has a WA on UAI enhancement. The original plan was to revisit the WA after discussing the potential enhancement because the need of UAI enhancement depends on whether RAN2 agreed on a specific solution that requires MM information.

For UL, RAN2 discussed LCP enhancement and PDU Set discard.

-	LCP enhancement was excluded due to concerns about its complexity

-	PDU set discard was considered achievable only when synchronization information is available at RAN.

Given two facts below, it would be difficult to get synchronous information for PDU set discard:

-	SA2 discussion is currently focused on MMSID.

-	Synchronous information for PDU set discard would require UPF to provide time stamp information per packet, which is currently not possible.

In addition, WA assumes MM information, to be reported by the UE, only indicates association information among QFIs and does not assume packet level synchronization information to be reported, which is then not useful for PDU set discard.

Therefore, we see no use case that would utilize UE reporting of MM information.

Observation 2: For UAI enhancement, RAN2 identified no use case or potential enhancement that requires MM information reported from the UE via UAI.

2) Joint admission control

Agreements:

-	The gNB may perform joint admission control. Details can be left up to RAN3 in potential WI phase. FFS if MMSID can be used for this purpose.

-	The gNB may consider this information during QoS flow to DRB mapping (up to gNB implementation)

RAN2 simply determines that gNB may perform admission control but didn’t see how much capacity improvement can be shown by introducing joint admission control. Furthermore, it is left up to RAN3 whether to do it and/or how to do it. Given that the scope of study on multi-modality was limited to RAN2, it would be out of scope to have such objective requiring further work from other WG, i.e., RAN3.

Observation 3: For joint admission control, RAN2 identifies no technical aspect that RAN2 should be involved.

3) DL synchronous delivery

Agreements:

-	For DL, whether traffic synchronization (on a per packet basis) can be achieved depends on whether packet level synchronization information can be provided from CN to RAN

The question is whether and how packet level synchronization information can be provided to RAN. To our understanding, it is currently not possible because UPF needs to provide a kind of time stamp information per packet to RAN, which is expected to have a big impact on SA2.

Observation 4: For DL synchronous delivery, it is currently not possible to have a packet-level synchronization information due to lack of time stamp information provided per packet level.

4) PDU set discard

Agreements:

-	RAN2 thinks PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the multi-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4.

-	RAN2 thinks in case this is feasible, it should be limited to intra-DRB case.

Similar to DL synchronous delivery, PDU set discard requires synchronous information per e.g., packet level, and it is not currently possible.

Observation 5: For PDU set discard, it is currently not possible to have proper synchronization information due to lack of time stamp information provided per packet level.

With the observations above, considering the limited time unit available and the already wide scope of the WI, we propose that:

Proposal 1: Multi-modality is not included in the WI objective of Rel-19 XR.

# 3	[RAN2] Scheduling enhancements

The following agreements have been made during the study phase of discussion RAN2 on scheduling enhancements:

-	For LCP enhancements, LCP Prioritisation:

-	Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR;

-	The solution should consider impact on UE complexity (as already indicated in SI objective description);

-	For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline:

-	Use additional priority configured to LCHs in case of these LCHs with delay-critical data.

-	For LCP enhancements, Granularity:

-	LCP prioritization within a logical channel will not be considered in RAN2 discussions;

-	FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

-	For LCP enhancements, LCH restriction

-	RAN2 to no longer consider the enhancement of the LCP restriction as one of the candidate solutions for LCP enhancements in Rel-19 XR

-	For DSR enhancements:

-	Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG;

-	Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG.

-	There will be a single triggering threshold, as in Rel-18.

Based on those, the study phase could be concluded with the following two aspects added as WI objectives:

-	Specify LCP enhancement with priority adjustment for the LCHs with delay critical data;

-	Specify DSR enhancement with multiple pairs of buffer status and remaining time for an LCG.

Corresponding WID update can be found in [RP-241771].

Proposal 2: The study phase for scheduling enhancement is concluded with the following added to the WID:

-	Specify the following Enhancements for Scheduling [RAN2]:

-	Specify enhancements that supports priority adjustment during LCH prioritization for the LCHs with delay critical data;

-	Specify DSR enhancement with multiple pairs of buffer status and remaining time for an LCG.

# 4	[RAN2] Uplink congestion indication

Another topic being discussed in TSG RAN level is the uplink congestion notification. Assuming we don’t have further work needed on the multi-modality, it should be possible to accommodate further work in the area of uplink congestion notification.

While use of L4S based on Release 18 covers many scenarios (RP-241498), there are cases when use of uplink congestion notification from gNB to the UE could add value, especially when the service is such a point to point communication cases, which requires large data rate and end points can be far from each other, even in different countries.

Thus, we propose to add simple objective to address the uplink congestion notification as RAN2 lead objective. While ideally, we could already define which layer to cover that but it would be better to have such a discussion to take place in RAN2 and hopefully to get quick convergence whether one is looking at MAC layer or RLC layer solution for this.

The proposed XR Release 19 objective would be then as

-	Specify, if needed, the uplink rate control solution from RAN, complementing the upper layer solutions in place [RAN2]

Proposal 3: WID is updated to include an objective on the uplink rate control solution e.g., uplink congestion indication.

# 5	[RAN3] New objective and TU allocation

In RAN3#125 meeting, RAN3 concluded the XR for NR-DC is completed in RAN3. RAN3 will not have any TU for XR in Rel-19.

Following SA2 SI conclusions have impacts to RAN3, that may be not appropriate to be handled as TEI19.

- For Key Issue #1: Support of PDU set based QoS handling enhancement

  - the 5GC sends NG-RAN an indicator indicating the DL PDU Set Information Marking is supported. Based on the indicator, , the NG RAN may indicate the 5GC to perform the PDU Set Information marking.

  - FEC Content ratio agreed to be provided over NGAP.

  - Enhance Alternative UL/DL PDU Set QoS and Notification control for PDU Set QoS.

- For Key Issue #5: QoS Handling when Traffic Characteristics Change Dynamically

  - TTNB and Burst size to be provided over GTP-U

NOTE: TTNB and Burst Size may also have impact to RAN2.

- For Key Issue #9: Enhancement for XR related network information exposure

  - Support the report available data rate value for GBR QoS Flow

WID Update may be needed to add the new RAN3 objectives to support SA2 work, and request additional TUs for RAN3#125bis, RAN3#126, and RAN3#127 meetings (0.5 TU for each meeting).

Proposal 4: WID Update is needed to add the following RAN3 objectives to support SA2 work, and request additional TUs for RAN3#125bis, RAN3#126, and RAN3#127 meetings (0.5 TU for each meeting)

-	Support of PDU set based QoS handling enhancement [RAN3]

-	the 5GC sends NG-RAN an indicator indicating the DL PDU Set Information Marking is supported. Based on the indicator, , the NG RAN may indicate the 5GC to perform the PDU Set Information marking.

-	FEC Content ratio agreed to be provided over NGAP.

-	Enhance Alternative UL/DL PDU Set QoS and Notification control for PDU Set QoS.

-	QoS Handling when Traffic Characteristics Change Dynamically [RAN3]

-	TTNB and Burst size to be provided over GTP-U

-	Enhancement for XR related network information exposure [RAN3]

-	Support the report available data rate value for GBR QoS Flow

# 6	[RAN4] TU allocation for RRM work

RAN4 started discussion during RAN4 #112 on the enhancements regarding the following objective:

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

During RAN4 #112, the discussion focused on proposals from companies regarding which types of measurement gaps should be considered during the work. Additionally, RAN4 discussed which of multiple types of RRM measurements to be considered for measurement skipping.

From the 17 open issues listed in the moderator topic summary (R4-2411820) 2 issues were agreed. Considering the interest among companies, the input from companies, meeting discussion (and participation) and the solution space the companies want to include, the work needed to develop the UE requirements will be larger than initially anticipated. Hence, we believe RAN4 would need additional time for online discussion on the objective. Therefore, we propose to increase the TU allocation for RAN4 RRM work from 0.5 to 1 TU, to reflect the expected work amount in RAN4 RRM.

Proposal 5: Increase RAN4 TU allocation for RRM from 0.5 TU to 1 TU starting from RAN4 #112bis.

# 7	Conclusion

In this contribution, we discussed the necessary update of the WID objectives and TU allocations for RAN2/RAN3/RAN4. With the observations below, we propose:

Observation 1: During past three meetings, RAN2 has not sufficiently analysed the potential mechanism especially in terms of gain and benefit.

Observation 2: For UAI enhancement, RAN2 identifies no use case or potential enhancement that requires MM information reported from the UE via UAI.

Observation 3: For joint admission control, RAN2 identifies no technical aspect that RAN2 should be involved.

Observation 4: For DL synchronous delivery, it is currently not possible to have a packet-level synchronization information due to lack of time stamp information provided per packet level.

Observation 5: For PDU set discard, it is currently not possible to have proper synchronization information due to lack of time stamp information provided per packet level.

Proposal 1: Multi-modality is not included in the WI objective of Rel-19 XR.

Proposal 2: The study phase for scheduling enhancement is concluded with the following added to the WID:

-	Specify the following Enhancements for Scheduling [RAN2]:

-	Specify enhancements that support priority adjustment during LCH prioritization for the LCHs with delay critical data;

-	Specify DSR enhancement with multiple pairs of buffer status and remaining time for an LCG.

Proposal 3: WID is updated to include an objective on the uplink rate control solution e.g., uplink congestion indication.

Proposal 4: WID Update is needed to add the following RAN3 objectives to support SA2 work, and request additional TUs for RAN3#125bis, RAN3#126, and RAN3#127 meetings (0.5 TU for each meeting)

-	Support of PDU set based QoS handling enhancement [RAN3]

-	the 5GC sends NG-RAN an indicator indicating the DL PDU Set Information Marking is supported. Based on the indicator, , the NG RAN may indicate the 5GC to perform the PDU Set Information marking.

-	FEC Content ratio agreed to be provided over NGAP.

-	Enhance Alternative UL/DL PDU Set QoS and Notification control for PDU Set QoS.

-	QoS Handling when Traffic Characteristics Change Dynamically [RAN3]

-	TTNB and Burst size to be provided over GTP-U

-	Enhancement for XR related network information exposure [RAN3]

-	Support the report available data rate value for GBR QoS Flow

Proposal 5: Increase RAN4 TU allocation for RRM from 0.5 TU to 1 TU starting from RAN4 #112bis.