3GPP TSG RAN Meeting #105	                                                                       RP-242097

Melbourne, Australia, September 9-12, 2024

Agenda Item:	9.2.3

Source:	Lenovo

Title:	On the scope for study of ISAC in Rel-19

Document for:	Discussion

- Introduction

The study on channel modelling for Integrated sensing and communication (ISAC) for NR, i.e., FS_ISAC_NR, has been approved in RAN#102 with the following objectives:

#### Objectives of SI FS_ISAC_NR (RP-234069) The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: - UAVs - Humans indoors and outdoors - Automotive vehicles (at least outdoors) - Automated guided vehicles (e.g. in indoor factories) - Objects creating hazards on roads/railways, with a minimum size dependent on frequency All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). Frequencies from 0.5 to 52.6 GHz are the primary focus, with the assumption that the modelling approach should scale to 100 GHz. (If significant problems are identified with scaling above 52.6 GHz, the range above 52.6 GHz can be deprioritized.) For the above use cases, sensing modes and frequencies: - Identify details of the deployment scenarios corresponding to the above use cases. - Define channel modelling details for sensing using 38.901 as a starting point, and taking into account relevant measurements, including: - modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns; - spatial consistency. It will be discussed at RAN#105 whether to include additional study beyond channel modelling for ISAC.

Apart from the stated objectives on the study of channel modelling and deployment scenarios for object detection and tracking of the exemplified target objects, it is agreed that the further scope of the ISAC study in Rel-19 to be re-visited in RAN#105.

In this document, we provide our view on the scope of ISAC study in Rel-19 given the current progress, remaining/open issues and the initial SI objectives.

- Discussion

The study of channel modelling methodologies and related deployment scenarios for ISAC have been conducted in RAN WG1#116-118, from the aspects of defining deployment scenario for channel calibration (AI 9.7.1) and channel modelling methodologies (AI 9.7.2), addressing the objectives stated in RP-234069.

From the aspects of deployment scenarios, it is agreed to define dedicated set of evaluation parameters (for channel calibration) for each of the example target object types, including UAVs, Humans indoors and outdoors, automative vehicles (outdoor), automated guided vehicles (indoor) as well as objects creating hazards on roads. This is further agreed that the deployment scenarios shall include description of the Tx/Rx (e.g., Tx/Rx node height and position) and target properties (e.g., size/velocity, distribution).

For the scenarios of UAV sensing, the evaluation parameters for channel calibration are agreed in RAN1#118. Nevertheless, further discussions are needed to reach a common set of assumptions among the remaining multiple options in different fields. Moreover, the deployment scenarios and calibration assumptions shall be continued to cover other target types/scenarios (human, vehicle indoor/outdoor), according to the stated list in the SID objectives. It is worth mentioning that the related parameters of deployment scenarios for different target types are not expected to be highly correlated (e.g., vehicle size, speed, and related sensing Tx/Rx nodes for outdoor are not related to the same properties derived for the UAVs) and require detailed separate discussions.

Observation  1. Additional effort is required to further refine the assumptions of the UAV sensing scenario and agree on deployment scenarios and calibration assumptions for other target types (e.g., human, vehicle indoor/outdoor), according to the stated SID objectives.

Furthermore, in case of expanding the scope of the ISAC study to beyond channel modelling, the defined deployment scenarios for channel calibration need to be further developed to define assumptions for evaluation of sensing performance. Such extension requires additional efforts to agree on assumptions for performance evaluation, e.g., on the operating frequency band, signal type/SCS, bandwidth, performance metrics/KPIs as well as the applicable (potentially down-selected) sensing modes adjusted to different target type and evaluation scenarios. As such, any expansion of the scope of the study shall take into account the additional efforts required to agree on the detailed evaluation assumptions beyond the current deployment scenarios for channel calibration.

Observation  2. In case of expanding the scope of the ISAC study to beyond channel modelling, the defined deployment scenarios for channel calibration need to be further developed to define assumptions on performance evaluation for different sensing target/scenarios.

The aspects of channel modelling methodologies, for different target types and related scenarios to the SID objectives, have been studied in RAN WG1#116-118. In RAN1#116 it is agreed that the ISAC channel can be decomposed into separate “target channel” and “background channel” components, thereby facilitating a framework for utilizing 3GPP TR 38.901 for generating the background channel while generating the target channel, at least in part, via separately characterized objects (e.g., target or target-type objects). As such, related elements of the above framework have been developed on RAN1#116bis-118, including a high-level RCS & CPM target characterization, LOS/NLOS status for Tx/Rx-target path, target channel components including direct and indirect paths, the background channel modelling via stochastic clutter, concatenation-based construction of a channel including multiple scattering/reflections, and discussion of environment objects type/modelling options. Nevertheless, the developed modelling elements require further study before it can be used for ISAC channel generation and evaluations, at least considering following aspects.

Firstly, while the majority of the modelling agreements are focused around targets modelled via “a single scattering point”, this assumption (and hence the subsequent modelling procedure) is not applicable to majority of the defined target objects within the SID objective. In particular, a target may be observed via multiple scattering points (e.g., delay taps) depending on the target size as well as the processing resolution (e.g., time/angle) of a sensing receiver node. Moreover, artificial reduction of the target scattering effect to a single scattering point leads to an unrealistic focus of the target energy around a single scattering point and mislead the achievable sensing performance. As such, it is expected that for the target types of vehicles (indoor/outdoor), human (indoor/outdoor) and animals on the road (outdoor) the single scattering point modelling to be expanded, at least as a modelling option, into multiple scattering point modelling. This necessitates further discussions on the RCS modelling, path concatenation, and scattering point interaction/self-blockage for objects with multiple scattering points before the channel modelling study can be concluded.

Observation  3. For the target types of vehicle (indoor/outdoor), human (indoor/outdoor) and animals on the road (outdoor), the single scattering point modelling needs to be expanded, at least as a modelling option, into multiple scattering point modelling, and this necessitates further discussions on the RCS modelling, path concatenation, and scattering point interaction/self-blockage for objects with multiple scattering points before the channel modelling study can be concluded.

Secondly, in addition to the extensions for targets with multiple scattering points, several issues are pending and need to be addressed during the upcoming meetings, including, among others, modelling consideration for mono-static sensing background channel, modelling details for environment objects (type 1/2), convergence on RCS distribution and values for different target types, special consistency procedure for target mobility. In our view, it is important to conclude on the abovementioned pending issues before channel modelling study is concluded and prior to a scope expansion for the ISAC study.

Observation  4. Multiple channel modelling issues are pending and need to be addressed during the upcoming meetings, including modelling details for mono-static sensing background channel, modelling details for environment objects (type 1/2), convergence on RCS distribution and values for different target types, special consistency procedure for target mobility, etc.

Apart from the progress of the channel modelling study it is worth mentioning that other studies of ISAC are dependent on the outcome of the channel modelling study, at least for evaluating the feasibility and functional requirements of the radio nodes participating in a sensing operation for a required performance KPI. This includes, e.g., impact of a sensing target path quality/strength on the required resources for sensing, interference between communication and sensing, as well as the needed coordination level of the involved radio nodes.

Observation  5. Any other study of ISAC needs to be dependent on the outcome of the channel modelling, at least for evaluating the feasibility and functional requirements of the radio nodes participating in a sensing operation given a required performance KPI.

Given the remaining gap between the stated scope of the FS_ISAC_NR objectives and the developed modelling methodologies and deployment scenarios in RAN1#116-118, additional study effort is required to develop a modelling and evaluation assumption to be used in evaluation of radio sensing methodologies. As such, it seems reasonable to focus the remaining study efforts of Rel-19 on developing the outstanding channel modelling issues. The future ISAC studies in RAN WGs can be started once the channel modelling study, in view of the stated objectives of FS_ISAC_NR, is concluded.

Proposal 1. Focus the study efforts on developing the remaining channel modelling issues without any additional ISAC study in the Rel-19 time frame.

- Conclusion

Observation  1. Additional effort is required to further refine the assumptions of the UAV sensing scenario and agree on deployment scenarios and calibration assumptions for other target types (e.g., human, vehicle indoor/outdoor), according to the stated SID objectives.

Observation  2. In case of expanding the scope of the ISAC study to beyond channel modelling, the defined deployment scenarios and assumptions for channel calibration need to be further developed to define assumptions on performance evaluation for different sensing target/scenarios.

Observation  3. For the target types of vehicle (indoor/outdoor), human (indoor/outdoor) and animals on the road (outdoor), the single scattering point modelling needs to be expanded, at least as a modelling option, into multiple scattering point modelling, and this necessitates further discussions on the RCS modelling, path concatenation, and scattering point interaction/self-blockage for objects with multiple scattering points before the channel modelling study can be concluded.

Observation  4. Multiple channel modelling issues are pending and need to be addressed during the upcoming meetings, including modelling details for mono-static sensing background channel, modelling details for environment objects (type 1/2), convergence on RCS distribution and values for different target types, special consistency procedure for target mobility, etc.

Observation  5. Any other study of ISAC needs to be dependent on the outcome of the channel modelling, at least for evaluating the feasibility and functional requirements of the radio nodes participating in a sensing operation given a required performance KPI.

Proposal 1. Focus the study efforts on developing the remaining channel modelling issues without any additional ISAC study in the Rel-19 time frame.

- References

- 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz, V17.1.0 (2024-01)”

- 3GPP TR 22.837, “Feasibility Study on Integrated Sensing and Communication (Release 19)”, V19.2.0 (2023-12).

- 3GPP TS 22.137, “Service requirements for Integrated Sensing and Communication; Stage 1 (Release 19)”, V1.1.0 (2023-12).

- RP-234069, “New SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR”, Nokia, Nokia Shanghai Bell

Appendix

Agreement of RAN1#118 on UAV sensing scenarios For UAV sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#117 as a baseline. Note: Additional parameters, value/value ranges are not precluded. The detailed scenario description in this clause can be used for channel model calibration. Table x. Evaluation parameters for UAV sensing scenarios Parameters; Value. Applicable communication scenarios; UMi, UMa, RMa [38.901] UMi-AV, UMa-AV, RMa-AV. Sensing transmitters and receivers properties; Rx/Tx Locations; Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenarios. Note1: This may include aerial UEs for UMi-AV, UMa-AV, RMa-AV communication scenarios. In this case, other Rx/Tx properties (e.g. mobility) are also taken from the corresponding communication scenario.. Sensing target; Outdoor/indoor; Outdoor. 3D mobility; Horizontal velocity: uniform distribution between 0 and 180km/h, if horizontal velocity is not fixed to 0. Vertical velocity: 0km/h, optional {20, 40} km/h NOTE2: 3D mobility can be horizontal only or vertical only or a combination for each sensing target FFS: time-varying velocity.. 3D distribution; Horizontal plane: Option A: N targets uniformly distributed within one cell. Option B: N targets uniformly distributed per cell. Option C: N targets uniformly distributed within an area not necessarily determined by cell boundaries. FFS: Value of N, defined area, and other distributions Vertical plane: Option A: Uniform between 1.5m and 300m. Option B: Fixed height value chosen from {25, 50, 100, 200, 300} m assuming vertical velocity is equal to 0. FFS Other options are not precluded. Note2: target(s) are outside the minimum distance to the Tx/Rx. Orientation; Random in horizontal domain. Physical characteristics (e.g., size); Size: - Option 1: 1.6m x 1.5m x 0.7m - Option 2: 0.3m x 0.4m x 0.2m FFS: Material(s), Structure, Other size(s). Minimum 3D distances between pairs of Tx/Rx and sensing target; Option B: Min distances based on min. TRP/UE distances defined in TR36.777 as a starting point. Option C: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx. Minimum 3D distance between sensing targets; Option 1: At least larger than the physical size of a target Option 2: 10 meters. [Unintended/Environment objects, e.g., types, characteristics, mobility, distribution, etc.]; FFS. Note: further down-selection between the options in the table is not precluded.