- 3GPP TSG RAN Meeting #105		          			                       	    RP-242100

- Melbourne, Australia, September 9-12, 2024

Source:	ZTE Corporation, Sanechips

Title:	Discussion on Rel-19 ISAC

Agenda Item:	9.2.3

Document for: Discussion

- Introduction

The objective of Rel-19 ISAC channel modeling is copied below with a checkpoint to decide whether to include additional study beyond channel modelling [1].

Objective of SI The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: - UAVs - Humans indoors and outdoors - Automotive vehicles (at least outdoors) - Automated guided vehicles (e.g. in indoor factories) - Objects creating hazards on roads/railways, with a minimum size dependent on frequency All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). Frequencies from 0.5 to 52.6 GHz are the primary focus, with the assumption that the modelling approach should scale to 100 GHz. (If significant problems are identified with scaling above 52.6 GHz, the range above 52.6 GHz can be deprioritized.) For the above use cases, sensing modes and frequencies: - Identify details of the deployment scenarios corresponding to the above use cases. - Define channel modelling details for sensing using 38.901 as a starting point, and taking into account relevant measurements, including: - modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns; - spatial consistency. It will be discussed at RAN#105 whether to include additional study beyond channel modelling for ISAC.

In this contribution, the overview on the future plan of the ISAC SI is elaborated based on the reviewing of RAN1 progress.

- Discussion on the scope extension of ISAC

2.1 Importance of UAV sensing

Ground traffic is becoming increasingly congested, and the evolution of transportation from 2D to 3D, moving towards the lower airspace, is becoming a technological and market trend.  Data from the Civil Aviation Administration of China indicates that the low-altitude economy market is anticipated to reach 1.5 trillion yuan by 2025 and then 3.5 trillion yuan by 2035, emerging as a significant business opportunity. [2].

Low altitude economy is a comprehensive economic form based on manned and unmanned aerial vehicles. UAVs are one of the main drivers of low altitude economy. There are numerous applications for UAVs, including but not limited to power line inspections, UAV delivery, intrusion detection, agricultural crop protection, and more. Some use cases are as shown in Figure 2-1.

- Power line inspection                    (b) UAV delivery                             (c) UAV intrusion detection

Figure 2-1 Use cases for UAV sensing

In short, sensing will play a crucial role in these applications, and sensing can be used to monitor illegal intrusions by low, slow, and small UAVs. Sensing can also be used for trajectory tracking and for collision avoidance. Base stations in 5G system are already used to provide 5G coverage for communication, and the radio signals can also be used to sense UAVs. In [3], some field test results for UAV intrusion detection and tracking were provided. It shows good sensing accuracy with implementation even based on 5G-A gNB.

Observation 1: Sensing for UAV case is an essential aspect to enable the low altitude economy with justified feasibility and performance.

2.2 Review on the RAN1 discussion

2.2.1 Progress for evaluation scenario

In RAN1 agenda for ISAC deployment scenarios, good progress has been achieved for Evaluation parameters for UAV sensing scenarios as shown in the following table [4].

Table x. Evaluation parameters for UAV sensing scenarios

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios |  | UMi, UMa, RMa [38.901] UMi-AV, UMa-AV, RMa-AV |
| Sensing transmitters and receivers properties | Rx/Tx Locations | Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenarios. Note1: This may include aerial UEs for UMi-AV, UMa-AV, RMa-AV communication scenarios. In this case, other Rx/Tx properties (e.g. mobility) are also taken from the corresponding communication scenario. |
| Sensing target | Outdoor/indoor | Outdoor |
|  | 3D mobility | Horizontal velocity: uniform distribution between 0 and 180km/h, if horizontal velocity is not fixed to 0. Vertical velocity: 0km/h, optional {20, 40} km/h NOTE2: 3D mobility can be horizontal only or vertical only or a combination for each sensing target FFS: time-varying velocity. |
|  | 3D distribution | Horizontal plane: Option A: N targets uniformly distributed within one cell. Option B: N targets uniformly distributed per cell. Option C: N targets uniformly distributed within an area not necessarily determined by cell boundaries. FFS: Value of N, defined area, and other distributions Vertical plane: Option A: Uniform between 1.5m and 300m. Option B: Fixed height value chosen from {25, 50, 100, 200, 300} m assuming vertical velocity is equal to 0. FFS Other options are not precluded. Note2: target(s) are outside the minimum distance to the Tx/Rx |
|  | Orientation | Random in horizontal domain |
|  | Physical characteristics (e.g., size) | Size: - Option 1: 1.6m x 1.5m x 0.7m - Option 2: 0.3m x 0.4m x 0.2m FFS: Material(s), Structure, Other size(s) |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option B: Min distances based on min. TRP/UE distances defined in TR36.777 as a starting point. Option C: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a target Option 2: 10 meters |
| [Unintended/Environment objects, e.g., types, characteristics, mobility, distribution, etc.] |  | FFS |

It can be seen that the only remaining action is for potential down-selection on the options and to fix a few FFS bullets. One or two meetings is expected to be sufficient to finish the deployment scenarios for UAV use cases. Additionally, these issues can also be fixed during the evaluation stage based on the inputs and observations.

Observation 2: The assumption for UAV case is almost stable with limited remaining issues.

2.2.2 Progress for channel modelling

In RAN1 agenda for ISAC channel modeling, there is also good progress for stochastic channel modeling in the case when sensing target is modeled as a single scattering point. For example, the basic modelling methodology is concluded that the target channel modeling is agreed as the concatenation of the links from Tx to target and from target to Rx including both large scale and small-scale fading. The detailed agreements can be found in [2].

Although there are still some remaining issues as highlighted in SR[5], for the UAV use cases, it is expected to be finished soon with the following reasons:

- It is straightforward to model UAV as a single scattering point based on RAN1 discussion, the open issues on multiple scattering modeling do not impact UAV use cases.

- EO-type 2 is introduced mainly for urban grid scenarios and indoor scenarios probably, which are different from UAV applicable scenarios.

- Super majority supports random distribution for UAV RCS modeling. The consensus can be achieved soon. It is noted that the most complicated RCS modeling is only on Vehicle use cases.

- The candidate solutions have been agreed for background channel modeling where the existing TR 38.901, TR 38.858, etc. as starting points are supported by most companies. It is expected to be addressed by the end of this year.

Observation 3: RAN1 has good progress for UAV use case including both deployment scenario and channel modeling.

- It is expected to be sufficient to address the remaining issues in one or two meetings.

2.3 Future plan on ISAC

Given the above analysis for UAV related ISAC channel modeling in RAN1 along with the emerging needs for application, we suggest to start channel modeling calibration work on UAV use cases from 2025 Q1, and then start performance evaluation work in the remaining time of Rel-19 without extra TUs. The performance evaluation can help to check the feasibility of UAC sensing use cases, and further provide useful information on whether/how much normative work is needed for ISAC in 5G-A.

Proposal 1: It’s recommended to extend the scope of ISAC SI without extra TUs:

- Start channel modeling calibration on UAV use cases from 2025 Q1

- Start performance evaluation in RAN1 from Q1 2025 till the end of Rel-19.

- Conclusions

In this contribution, we propose to slightly extend the scope of Rel-19 ISAC SI with the following observation and proposals:

Observation 1: Sensing for UAV case is an essential aspect to enable the low altitude economy with justified feasibility and performance.

Observation 2: The assumption for UAV case is almost stable with limited remaining issues.

Observation 3: RAN1 has good progress for UAV use case including both deployment scenario and channel modeling.

- It is expected to be sufficient to address the remaining issues in one or two meetings.

Proposal 1: It’s recommended to extend the scope of ISAC SI without extra TUs:

- Start channel modeling calibration on UAV use cases from 2025 Q1

- Start performance evaluation in RAN1 from Q1 2025 till the end of Rel-19.

- Reference

- RP-240799 Revised SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR.

- https://equalocean.com/analysis/2024043020831

- RP-232276, Demonstration of RIS and ISAC techniques in Hangzhou Asian Games, China Telecom, ZTE, RAN#101.

- RAN1 Chair’s notes of RAN1#118 meeting

- RP-241960, Status report for SI: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR, RAN#105