3GPP TSG RAN WG meeting #105	                                          RP-242104

Melbourne, Australia, September 9-12, 2024

Title: 	Discussion on XR enhancements

Source: 	ZTE Corporation, Sanechips

Agenda item:	9.3.2.3

Document for:	Discussion and decision

# Introduction

The WI for XR enhancements in Rel-19 has the following objectives:

The Rel-19 XR Phase 3 objectives are as follows: - Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2]. NOTE: Check in RAN#105 (check also other WG involvement if needed). - Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4] - Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. NOTE: LCP implementation complexity need to be taken into account when evaluating solutions. NOTE: Check in RAN#105 - Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. - Specify Core requirements related to the above objectives as necessary [RAN4] - Extend Release 18 standalone mechanism to support NR-NR dual connectivity as follows [RAN3] - PDU set based handling - ECN marking - Burst Arrival Time reporting, if needed - PSI Discard coordination, if needed - Note: No RAN2 impact from above items NOTE: Whether / to what extent network exposure / RAN awareness / e.g. RAN involved rate control, possibly additional info for DL scheduling, parallel with SA2 work, shall be covered in this WI is TBD.

In this paper, we discuss the conclusions made in RAN2 for the multi-modality related objectives and propose the updated objective for the WI. Further, regarding RAN awareness and RAN involved rate control, there was further discussion at RAN#104 and it was agreed that this objective will also be discussed at RAN#105 along with other items related to the check point at plenary. We also propose the way forward for this objective.

# Multi-modality

With regards to multi-modality, RAN2 has made a number of agreements:

RAN2#125-bis For the purpose of study, RAN2 assumes that UE and gNB have some kind of multi-modal information. FFS what information is needed/useful, e.g. just mulit-0modal ID, association between the flow, synchronization requirement etc. RAN2 will study both UL and DL directions based on the assumption of multi-modality association knowledge at RAN/UE RAN2 will focus on analysing potential usage and benefits (e.g. in terms of capacity and power saving) of multi-modal association knowledge Areas to study include: synchronization between the flows, FFS impact on QoS insurance and other areas RAN2 assumes that traffic of different modals having different QoS requirements is mapped to different QoS flows For different XR traffic flows belonging to the same Multi-modal service and having different QoS requirements, it should be possible to provide differentiated QoS handling over the air. RAN2 should study if that is possible with current mechanism or new ones are needed Existing QoS flow to DRB mapping framework is used as a baseline, i.e. up to gNB how to map QoS flows to DRBs RAN2#126 Support Multi-Modality awareness in RAN in Rel-19 for UL and DL. Ask SA2 whether/what information could be provided to RAN Clarify in the LS how RAN2 considers to use this information, e.g. coordinated handling of multi-modal flows. RAN2#127 Working assumption: Regardless of SA2 decision, RAN2 can extend the UAI for multi-modal awareness at least for uplink QoS flows in Rel-19 XR, by having the UE report existence of multi-modality application and association information among QFIs to gNB. FFS whether this can be applied to DL. RAN2 considers that based on multi-modal information: - The gNB may perform joint admission control. Details can be left up to RAN3 in potential WI phase. FFS if MMSID can be used for this purpose. - The gNB may consider this information during QoS flow to DRB mapping (up to gNB implementation) For UL, RAN2 does not intend to perform LCP enhancements due to complexity vs gains concerns. For DL, whether traffic synchronization (on a per packet basis) can be achieved depends on whether packet level synchronization information can be provided from CN to RAN. RAN2 thinks PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the multi-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4. RAN2 thinks in case this is feasible, it should be limited to intra-DRB case. Not support multiple active DRX configurations Not support independent configuration of drx-StartOffset for the secondary DRX group

Based on the above agreements, it can be observed that RAN2 made significant progress for multi-modality study and identified a number of promising options for normative work. There were agreements both on which items to pursue further, but also regarding which items are not further considered. Furthermore, there is still some dependency with SA2, especially regarding the level of assistance information that could be provided by CN to RAN. Thus, the enhancements for the normative phase of XR Rel-19 can be categorized as follows:

To be included in Rel-19 normative phase:

- Enhancements to UAI for multi-modal awareness for UL and if feasible also for DL

  - These enhancements relate inclusion of static/semi-static dependencies and multi-modality related association information among QFIs in UAI (e.g. MMSID and other such information)

- Enhancements to PDI set discard across QoS flows for intra-DRB case

  - This objective is dependent on SA2/SA4 support

To be excluded fromRel-19 normative phase:

- LCP enhancements for multi-modality

- DRX enhancements for multi-modality

Thus, the following objectives are proposed for multi-modality support in Rel-19:

Proposal 1: Add the following objectives for normative work on multi-modality in Rel-19:

- Specify enhancements to extend the UE Assistance Information framework to enable UE to provide multi-modality related assistance information [RAN2]

- Specify enhancements to the PDU set based discard to enable joint discarding across QoS flows of the same multi-modal service for the intra-DRB case [RAN2]

- Specify enhancements to support CN assistance information and packet level synchronization information for the multi-modal traffic in UL and DL [RAN2]

NOTE: Dependencies with SA2/SA4 for the above objective in RAN3 should be further checked at RAN#106

# RAN exposure/RAN involved rate control for XR

At RAN#102, a note was added to specify enhancements for network exposure/RAN involved rate control for Rel-19 WI. For XR it is useful to specify some mechanism to let the upper layers throttle the throughput based on network conditions. A similar feature has already been specified for NR using the Recommended bit rate MAC CE. However, this feature is not tailored for XR and mainly targets VoIP traffic. Specifically, the following shortcomings for the recommended bit rate MAC CE needs to be addressed for this mechanism to be reused for XR applications:

- Support of XR data rates:

Recommended bit rate MAC CE covers only data rates up to 8000 kb/s. The XR applications on the other hand needs much higher data rates (going up to tens of Mbps or even higher). The upper bound for the recommended bit rate MAC CE should hence be adapted to cover the higher range for the XR applications. Although, the network can configure a multiplication factor for the signalled recommended bit rate, only multipliers of 40, 70, 100 and 200 are supported which don’t provide enough granularity for the XR applications in question.

- Granularity of signalled recommended bit rates:

Currently the Recommended bit rate MAC CE uses only 6 bit field and hence there are only a total of 64 code points available. The granularity of this field is not sufficient to cover the wide range of traffic profiles and data rates that might be generated by various codec modes for XR applications. It is hence better to define either a new MAC CE to cover the suitable range of traffic with sufficient granularity for XR applications or define finer values for the bitRateMultiplier to enable better granularity and peak data rates for XR.

- LCID specific indication:

Currently the Recommended bit rate MAC CE indicates the recommended bit rate per LCID. However, in case of XR, different QoS flows may be mapped to the same radio bearer and in order to better control the upper layer throughput it may be better to extend this mechanism to indicate the throughput on a per QoS flow basis rather than LCID.

Based on the above, we propose to enhance the recommended bit rate MAC procedure to cover XR applications in Rel-19.

Proposal 2: Add the following objective for RAN exposure/RAN involved rate control for XR:

- Specify the signaling for network exposure / RAN awareness on the UL congestion information per QoS flow/per DRB by enhancing the recommended bit rate MAC CE [RAN2]

# Scheduling enhancements

For scheduling enhancements, RAN2 study phase also has a check point in RAN’105. So, far, the RAN2 study concluded that the LCP enhancements are not considered further due to complexity. However, a number of DSR related enhancements have been identified as possible enhancements for normative work and these can be further considered during the WI phase. These include enhancements to configure multiple remaining time thresholds for reporting for each LCG to report pairs of remaining time and buffer sizes per LCH and other DSR enhancements for triggering the DSR etc. Based on this the following updated objective is proposed for scheduling enhancements:

Proposal 3: Add the following objective for scheduling enhancements for XR:

- Specify enhancements to the Delay Status Reporting mechanism to support UL scheduling to enable high XR capacity while meeting delay requirements and avoiding delayed PDUs [RAN2]

# Time unit discussion

The new objectives in RAN2 are essentially continuation of the existing study phase within RAN2 for multi-modality. The only exception is the addition of the RAN exposure for RAN controlled bit rate enhancement. However, if the proposal to reuse the existing recommended bit rate MAC CE is acceptable, then RAN can narrow down the scope of this objective too and hence all the work in RAN2 can fit within the existing time units already allocated for RAN2. For RAN3, the work depends on the progress in SA2. So, work in RAN3 can be progressed based on the input from SA2 and the dependencies can be further checked after the LS replies from SA2.

Proposal 4: No additional TUs are needed in RAN2 to handle the updated objectives for the XR WI and dependencies with SA2/SA4 for the above objectives in RAN3 should be further checked at RAN#106

# Conclusion

Proposal 1: Add the following objectives for normative work on multi-modality in Rel-19:

- Specify enhancements to extend the UE Assistance Information framework to enable UE to provide multi-modality related assistance information [RAN2]

- Specify enhancements to the PDU set based discard to enable joint discarding across QoS flows of the same multi-modal service for the intra-DRB case [RAN2]

- Specify enhancements to support CN assistance information and packet level synchronization information for the multi-modal traffic in UL and DL [RAN2]

NOTE: Dependencies with SA2/SA4 for the above objective in RAN3 should be further checked at RAN#106

Proposal 2: Add the following objective for RAN exposure/RAN involved rate control for XR:

- Specify the signaling for network exposure / RAN awareness on the UL congestion information per QoS flow/per DRB by enhancing the recommended bit rate MAC CE [RAN2]

Proposal 3: Add the following objective for scheduling enhancements for XR:

- Specify enhancements to the Delay Status Reporting mechanism to support UL scheduling to enable high XR capacity while meeting delay requirements and avoiding delayed PDUs [RAN2]

Proposal 4: No additional TUs are needed in RAN2 to handle the updated objectives for the XR WI and dependencies with SA2/SA4 for the above objectives in RAN3 should be further checked at RAN#106

The following updates to the objectives are proposed as a result:

The Rel-19 XR Phase 3 objectives are as follows: - Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2]. - Specify enhancements to extend the UE Assistance Information framework to enable UE to provide multi-modality related assistance information [RAN2] - Specify enhancements to the PDU set based discard to enable joint discarding across QoS flows of the same multi-modal service for the intra-DRB case [RAN2] - Specify enhancements to support CN assistance information and packet level synchronization information for the multi-modal traffic in UL and DL [RAN2] NOTE: Dependencies with SA2/SA4 for the above objectives in RAN3 should be further checked at RAN#106 NOTE: Check in RAN#105 (check also other WG involvement if needed). - Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4] - Specify Enhancements for Scheduling, as follows: - Enhancements to the Delay Status Reporting mechanism to support UL scheduling to enable high XR capacity while meeting delay requirements and avoiding delayed PDUs [RAN2] For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. NOTE: No enhancements to LCP procedure are considered for the above NOTE: LCP implementation complexity need to be taken into account when evaluating solutions. NOTE: Check in RAN#105 - Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. - Specify Core requirements related to the above objectives as necessary [RAN4] - Extend Release 18 standalone mechanism to support NR-NR dual connectivity as follows [RAN3] - PDU set based handling - ECN marking - Burst Arrival Time reporting, if needed - PSI Discard coordination, if needed - Note: No RAN2 impact from above items - Specify the signaling for network exposure / RAN awareness on the UL congestion information per QoS flow/per DRB by enhancing the recommended bit rate MAC CE [RAN2] NOTE: Whether / to what extent network exposure / RAN awareness / e.g. RAN involved rate control, possibly additional info for DL scheduling, parallel with SA2 work, shall be covered in this WI is TBD.

# References

- RP-240791 Revised WID on XR (eXtended Reality) for NR Phase 3	RAN#103