3GPP TSG RAN #105	 RP-242146

Melbourne, Australia, September 9th – September 12th, 2024

Agenda item:       9.2.2

Source:	IIT Kanpur, CEWiT, Indian Institute of Tech. Madras

Title:	Views on Release 19 Ambient IoT Study

Document for:     Discussion and Decision

# Introduction

In the last RAN#103, the following agreements in [1] were reached guiding working groups on the Release-19 study on Ambient IoT.

Proposal 3v2 - Regarding the objective in the SID: Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. - This objective allows studying CW waveform characteristics which would need control of the CW node(s), e.g. waveform characteristics that impact interference such as when CW is transmitted or not transmitted, power, bandwidth, spectrum, etc. - No SID revision is necessary

In the last RAN1#118 meeting consensus on the following agreements and observations on carrier wave characteristics was reached [2].

Agreement For the cases that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1, capture the following table in the TR. - Further observations (if any) on these CW transmission cases are not precluded CW Transmission case; Observations. Case 1-1; - NO need for BS to support full-duplex capability in DL spectrum for scenario ‘A1’ - Spatial isolation is possible for scenario ‘A1’, reducing the received CW interference power at BS side. - Cross-link interference handing for CW at BS side for scenario ‘A1’. - BS needs to support full-duplex capability (including self-interference suppression for CW) in DL spectrum for scenario ‘A2’. - Higher CW transmission power can be assumed in the DL spectrum than that of in the UL spectrum.. Case 1-2; - NO need for BS to support full-duplex capability in UL spectrum for scenario ‘A1’ - Spatial isolation is possible for scenario ‘A1’, reducing the received CW interference power at BS side. - Cross-link interference handing for CW at BS side for scenario ‘A1’. - BS needs to support full-duplex capability (including self-interference suppression for CW) in UL spectrum for scenario ‘A2’. - Lower CW transmission power can be assumed in the UL spectrum than that of in the DL spectrum.. Case 1-4; - NO need for BS to support full-duplex capability in UL spectrum - Spatial isolation is possible, reducing the received CW interference power at BS side. - Cross-link interference handing for CW at BS side. - Estimation of CW interference may be needed for successful D2R reception (i.e., at BS). - Lower CW transmission power can be assumed in the UL spectrum than that of in the DL spectrum.. Agreement For the cases that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, capture the following table in the TR. - Further observations (if any) on these CW transmission cases are not precluded CW Transmission case; Observations. Case 2-2; - NO need for intermediate UE to support full-duplex capability in UL spectrum for scenario ‘A1’ - Spatial isolation is possible for scenario ‘A1’, reducing the received CW interference power at intermediate UE side. - Cross-link interference handling for CW at intermediate UE side for scenario ‘A1’. - Intermediate UE needs to support full-duplex capability (including self-interference suppression for CW) in UL spectrum for scenario ‘A2’. - Lower CW transmission power can be assumed in the UL spectrum than that of in the DL spectrum.. Case 2-3; - NO need for intermediate UE to support full-duplex capability in DL spectrum - Spatial isolation is possible, reducing the received CW interference power at intermediate UE side. - Cross-link interference handling for CW at intermediate UE side. - Estimation of CW interference may be needed for successful D2R reception (i.e., at intermediate UE). - Higher CW transmission power can be assumed in the DL spectrum than that of in the UL spectrum.. Case 2-4; - NO need for intermediate UE to support full-duplex capability in UL spectrum - Spatial isolation is possible, reducing the received CW interference power at intermediate UE side. - Cross-link interference handling for CW at intermediate UE side. - Estimation of CW interference may be needed for successful D2R reception (i.e., at intermediate UE). - Lower CW transmission power can be assumed in the UL spectrum than that of in the DL spectrum.. Observation With respect to the following RAN guidance: - Regarding the objective in the SID: Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. - This objective allows studying CW waveform characteristics which would need control of the CW node(s), e.g. waveform characteristics that impact interference such as when CW is transmitted or not transmitted, power, bandwidth, spectrum, etc. - From RAN1 perspective, the following CW characteristics are identified: - When CW is transmitted or not transmitted - Transmission Power - Frequency resources - FFS other characteristics

# Discussion

In this contribution, we want to discuss the scope of control for the carrier wave node in the case of the bistatic mode of operation.

# Carrier Wave related aspects

In the Release-19 study on Ambient IoT, we are investigating three device types: device 1, device 2a, and device 2b. Device types 1 and 2a rely on an externally provided carrier wave for their device-to-reader (D2R) transmission. The carrier wave can be provided either in monostatic mode, where the carrier is emitted by the reader node, or in bistatic mode, where it is supplied by a secondary node other than the AIoT reader.

For working group studies on the bistatic mode of operation, we are considering two scenarios. The first scenario involves the carrier wave being provided by another base station within the same topology as the reader base station. The second scenario considers the carrier wave being provided by a node outside the topology. In the latter case, currently, there is no consensus among companies on whether the carrier wave node is a 3GPP or a non-3GPP device or could be both.

Proposal 1: RAN should guide RAN 1 to reach a common understanding of the definition of the CW outside the topology.

- Whether a CW node outside the topology a 3GPP node or a non-3GPP node?

It is unclear how to provide control to a non-3GPP CW (carrier wave) node. A CW node without proper control or with an always-on behavior may cause interference in the R2D receptions of the AIoT device and other systems, particularly in cases of in-band operation. Agreements and observations regarding interference caused by the CW node at the reader were discussed in the previous RAN1#118 meeting [2].

Observation 1: A non-3GPP CW node without proper control may negatively impact system performance by causing interference with AIoT and NR links during in-band operations.

Proposal 2: In the case of a non-3GPP CW node, RAN should guide RAN 1 to clarify how to control such a node.

# Conclusion

Proposal 1: RAN should guide RAN 1 to reach a common understanding of the definition of the CW outside the topology.

- Whether a CW node outside the topology a 3GPP node or a non-3GPP node?

Observation 1: A non-3GPP CW node without proper control may negatively impact system performance by causing interference with AIoT and NR links during in-band operations.

Proposal 2: In the case of a non-3GPP CW node, RAN should guide RAN 1 to clarify how to control such a node.

# References

- RP-240854 “Moderator's summary on R19 Ambient IoT”, RAN#103

- 3GPP RAN1 Chairman’s Notes, 3GPP TSG RAN1 meeting #118