3GPP TSG RAN Meeting #105	RP-242147

Melbourne, Australia, September 9-12, 2024

Agenda item:	9.1.3

Source:	Nokia

Title:	Additional topological enhancements in Rel-19

Document for:	Discussion

# 1	Introduction

Additional topological enhancements have been studied in Rel-19 with overall scope as defined in the SID [1]. The study included objectives for two distinct topics:

1)	5G Femto

2)	Enhancements for the support of WAB (Wireless Access Backhaul)

The study was completed at RAN3#125 in August, with recommendations for follow-on normative work captured in TR 38.799 [2]. In this paper, we provide our views on potential work item objectives to facilitate approval of the follow-on WID.

# 2	Discussion

## 2.1	5G Femto

Numerous operators have expressed the need to enable 5G Femto use cases to satisfy commercial deployment needs:

-	5G Femto offers a cost-effective way to improve 5G indoor coverage, offload macro gNB network traffic, enable better voice quality and provide better support for Enterprise mobility.

-	5G Femto extends coverage using higher frequency bands (e.g., FR2), leading to efficient and effective usage of higher frequency spectrum.

-	A high number of mobile sessions are indoor but indoor coverage with 5G mid/high-bands can be limited; therefore, there is a need for a solution that enables simple end user plug and play and allowing for customized access control.

-	High bandwidth and throughput with 5G are required at home and at campus locations to enable new immersive applications such as AR/VR/MR gaming, e-sports, UHD 8K video, telepresence, etc.

Support for large numbers of 5G Femtos should be possible in a scalable manner. Access control for 5G Femto can leverage the CAG concept defined for PNI-NPN.

The study item was agreed with the following scope for 5G Femto:

- Study the overall RAN architecture and required functional and procedural impacts for supporting 5G Femto deployments. - Study how to define the 5G access control mechanism by (re-)using the existing CAG functionality and identify needed enhancements (if any). - Study how to enable access to local services from the 5G Femto via collocated local UPF. Note: The study involves a gap analysis of existing 5G functionality with HomeNB functionality.

The study item was then completed at RAN3#125 with the following conclusions and recommendations captured in the TR 38.799:

Architecture for NG interface: - Option 1, namely direct connection of an NR Femto node to the 5GC via the NG interface, is already possible. - In order to maintain the existing infrastructure for an operator who has deployed LTE HeNBs, Option 2 with an optional NR Femto GW is recommended for a normative phase. Architecture for Xn interface: - Option A, namely direct connection of an NR Femto node to other NR Femto nodes / gNBs via the Xn interface, is already possible. Access Control: - For Access control, NR Femto reuses the existing CAG functionalities, no stage3 impact has been identified. Access to Local Services: - In order to support access to local services, NR Femto nodes reuse LADN and edge computing functionality as specified in TS 23.501 [6] and TS 23.548 [8]. - Aspects out of RAN3 scope, described in Sec. 5.4, have been identified and may require further analysis.

For the architecture part, the TR 38.799 concluded with a recommendation to specify support of NR Femtos in NG-RAN with an architecture including an optional NR Femto Gateway (GW) for NG interface in a similar way as HeNBs are supported in 4G including an optional HeNB GW which requires both stage 2 and stage 3 work. For Xn interface, only direct connections between NR Femtos to neighbour gNBs/NR Femtos is envisioned at this stage.

Observation-1:	For NG interface architecture, support for an optional NR Femto GW is recommended for the normative phase.

For the access control part, TR 38.799 concluded that NR Femtos can reuse the existing CAG functionalities for access control and that no stage 3 impact was identified. It was also concluded that all modes of open, hybrid and closed Femto can be supported by NR Femto by reusing existing CAG mechanisms as follows:

-	Open mode: the NR Femto node activates a PLMN cell, which can be accessed by legacy UE without access control of CAG.

-	Hybrid mode: the NR Femto activates a cell shared by both PLMN and CAG, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse.

-	Closed mode: the NR Femto node activates an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”.

The above access control conclusions should therefore be specified with only stage 2 work expected.

Observation-2:	For access control, existing CAG functionalities are reused (only stage 2 impact is expected).

For the access to local services part, TR 38.799 concluded that NR Femtos can access to local services reusing the existing LADN or mobile edge computing mechanisms. However, RAN3 identified a few open issues which can only be addressed externally to RAN3, e.g. in SA2 or CT4: scalability of local UPF discovery e.g. when using mobile edge computing solutions, N4 interface switch on/off, N4 interface aggregation, etc. Therefore, the specification of this access to local services should be done in SA2 and SA2 further involving RAN3 and CT4 if/when needed. At this point in time, no related objective seems needed in the RAN3 WI.

## 2.2	Wireless Access Backhaul

The study item scope for WAB was as follows [1]:

- Study the support of WAB including [RAN3, RAN2]: - Study the architecture and protocol stack of supporting a gNB with MT function providing PDU session backhaul. - Study impact of WAB mobility within an existing RAN (e.g., inter-gNB neighbour relations). - Identify necessary inter-gNB- and gNB-to-CN signalling to address the support of WAB. - Study signalling enhancements on resource multiplexing for WAB. NOTE 1: No impact on the UE. NOTE 2: Coordination with other WGs (e.g. SA2) when needed. The WAB study does not preclude any backhaul scenario (e.g. NTN or TN).

The study item was completed at RAN3#125 with conclusions and recommendations captured in TR 38.799 [2]. One of the conclusions to note is given below.

- The normative work for WAB should be based on the functionalities, terminology and requirements captured in the present TR. Addition of further details during normative phase is not precluded.

We are supportive of all conclusions listed in the TR and note that the overall outcome of the RAN3 study is aligned with the SA2 study on architectural enhancements for VMR [3].

Observation-3:	The conclusions and recommendations for WAB captured in TR 38.799 can be used as the basis for the work item objectives.

Although the defined objectives for WAB are generally fine, some clarification may be beneficial to have a common understanding of the underlying assumptions and correct interpretation of certain definitions. For example, “the architecture and protocol stack” makes the assumption that the architecture agreed in study phase will be used as reference.

Observation-4:	Some aspects in the SID objectives are also relevant to the WID objectives, i.e. specifying the architecture and protocol stack of the WAB-node, and not precluding any backhaul scenario (e.g. NTN or TN).

To achieve “avoidance of multi-hop WAB”, alignment with SA2 is necessary. Based on the agreed architecture in the study phase, for the signaling enhancements for the “Authorization of WAB nodes”, the TR captures “Authorization procedures for the WAB-MT are out of RAN3 scope and are expected to be handled by SA2. RAN3 should define the WAB-node behaviour in case the authorization status of the WAB-MT and/or WAB-gNB changes.” The conclusion is generally fine, but it is to be noted that WAB-MT's authorization is based on the dedicated slice for WAB being present in the Allowed S-NSSAI list. The OAM of Broadcasted PLMN takes care of the WAB-gNBs authorization. So, no signaling enhancement is needed for authorization itself as captured in the conclusion. It is also worth mentioning that graceful shutdown of WAB node is discussed and captured in the SA2 TR, thus there is no need for new solutions to handle mismatch of authorization status of WAB-MT and WAB-gNB. Furthermore, only textual (Stage-2) description of WAB-node behaviour in case the authorization status change is to be captured in normative phase. Thus, no Stage-3 impact is needed for authorization, unless requested by SA2.

In specifying the handling of “radio-resource coordination between access and backhaul links”, it should be noted that only the necessary changes required for WAB functioning are allowed and further improvements/enhancements are not in the scope, e.g. for the out-of-band deployment of WAB, there will not be any essential resource multiplexing enhancements, whereas for in-band deployment WAB operation, something(s) might need to be specified.

SA2 has already agreed on solutions pertaining to “QoS support on WAB backhaul”, and RAN3 needs to align with the same. Addition of further details for completeness during normative phase is not precluded. The QoS related discussions such as PDB, PER should be specified, if necessary.

Observation-5:	It may be beneficial to include a general bullet in the WID to align with SA2 as needed, for aspects that are not otherwise explicitly mentioned in the RAN3-led WID (e.g. QoS support on WAB backhaul).

# 3	Proposed work item objectives

The objectives for the work item on additional topological enhancements for NR will be categorized into a set of objectives for Wireless Access Backhaul (WAB) and a set of objectives for 5G Femto.

For Wireless Access Backhaul (WAB):

-	The normative work for WAB should be based on the functionalities, terminology and requirements captured in the present TR. Addition of further details during normative phase is not precluded.

-	Specify support of a WAB-node including a WAB-gNB and WAB-MT as defined in TR 38.799 including [RAN3, RAN2]:

-	the architecture and protocol stack of the WAB-node;

NOTE 1: Split architecture of the WAB-gNB is out-of-scope.

-	backhauling of the WAB-gNB’s NG, Xn and OAM traffic over the WAB-MT’s PDU session(s);

-	establishing, by the WAB-gNB, Xn interface(s) with the WAB-MT’s serving BH RAN node and with other surrounding gNBs;

-	WAB-node behaviour in case the authorization status of the WAB-MT and/or WAB-gNB changes;

-	integration procedure for WAB nodes following the description in TR 38.799;

-	handling of WAB-gNB’s traffic (including Xn, NG and OAM traffic) during WAB-node mobility, including the case where the WAB-MT’s BH PDU session changes;

-	procedure to support the UE’s AMF change for UEs connected to, or camped on, a WAB-gNB;

-	enhancements to the UE’s ULI that reflect the WAB node’s location;

-	handling of the following:

-	PCI collision;

-	reconfiguration of TAC and RANAC on WAB-gNBs;

-	avoidance of multi-hop WAB topology;

-	radio-resource coordination between access and backhaul links;

-	the following enhancements to the support of WAB, if needed:

-	Handling of backhaul link degradation by the backhaul network and the WAB-gNB;

-	Xn connection management (e.g., potential avoidance of setting up Xn between WAB-gNBs, dynamic establishment/removal of Xn connectivity with BH-RAN-node and surrounding NG-RAN-nodes);

-	NG connection management (e.g., procedures for NG connection removal/suspension);

-	QoS (e.g., PDB, PER);

-	Handling of RRC_INACTIVE UEs.

-	alignment with SA2 as needed.

NOTE 1: No impact on the UE.

NOTE 2: Coordination with other WGs (e.g. SA2) when needed.

NOTE 3: Access link for WAB backhaul can be TN or NTN.

NOTE 4: Mobility procedures to be used for the UEs served by a WAB-gNB are legacy UE mobility procedures. Mobility of the WAB-MTs is based on legacy UE mobility procedures.

NOTE 5: The interface between the WAB-MT and the co-located WAB-gNB is out-of-scope.

NOTE 6: Split architecture of the WAB-gNB is out-of-scope.

For NR Femto:

-	Specify the overall RAN architecture and required functional and procedural impacts for supporting NR Femto deployments using an optional NR Femto Gateway for NG interface similar to the 4G HeNB GW and direct Xn connections between NR Femtos and neighbour gNBs/NR Femtos [RAN3].

-	Specify the 5G access control mechanism for NR Femtos at connection and at mobility by (re-)using the existing CAG functionality [RAN3].

NOTE 7: Only stage 2 impact is expected.

# 4	Conclusions

In this contribution, we discussed the outcome of the Rel-19 Study on additional topological enhancements and made the following observations:

NR Femto:

Observation-1:	For NG interface architecture, support for an optional NR Femto GW is recommended for the normative phase.

Observation-2:	For access control, existing CAG functionalities are reused (only stage 2 impact is expected).

WAB:

Observation-3:	The conclusions and recommendations for WAB captured in TR 38.799 can be used as the basis for the work item objectives.

Observation-4:	Some aspects in the SID objectives are also relevant to the WID objectives, i.e. specifying the architecture and protocol stack of the WAB-node, and not precluding any backhaul scenario (e.g. NTN or TN).

Observation-5:	It may be beneficial to include a general bullet in the WID to align with SA2 as needed, for aspects that are not otherwise explicitly mentioned in the RAN3-led WID (e.g. QoS support on WAB backhaul).

Based on the above observations, a set of objectives for a follow-on work item are captured in section 3 which are aligned with the conclusions and recommendations in TR 38.799.

The following is proposed:

Proposal:	Adopt the objectives in section 3 for the Rel-19 Additional Topological Enhancements WID.

The RAN3 TU plan for Rel-19 already anticipated 1.5 TUs in RAN3 for the work item phase, which we believe is fine for the WI scope proposed above.

# References

- RP-241264 Revised SID on Study on additional topological enhancements for NR, NTT DOCOMO, INC., AT&T

- TR 38.799 Study on additional topological enhancements for NR

- S2-2407385 New WID on Vehicle Mounted Relays (VMR) Phase 2