3GPP TSG-RAN4 Meeting #112	R4-2412642

# Maastricht, The Netherlands, 19 – 23 August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4857 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on HO TCs for option C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BWP_wor-Perf |  |  |  |  |  | Date: |  |  | 2024-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The NCD-SSB configuration used in HO TCs for option C are not referring to the SSB RMC in clause A.3.10, and not aligned with other TCs for option C. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update NCD-SSB configuration in HO TCs for option C by referring to the SSB RMC in clause A.3.10. The SMTC configurations and test requirements are updated accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | HO TCs for option C are incorrect and not aligned with other option C TCs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.3.1.15, A.6.3.1.16, A.7.3.1.12, A.7.3.1.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### A.6.3.1.15	Intra-frequency handover from FR1 to FR1; known target cell configured with NCD-SSB

##### A.6.3.1.15.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 intra-frequency handover requirements specified in clause 6.1.1.2, when the target cell is configured with NCD-SSB.

##### A.6.3.1.15.2	Test Parameters

Supported test configurations are shown in table A.6.3.1.15.2-1. Both handover delay and interruption length are tested by using the parameters in table A.6.3.1.15.2-2, and A.6.3.1.15.2-3.

The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2.

Before the test starts,

-	UE is connected to Cell 1 with active DL BWP and active UL BWP;

-	UE is configured with nonCellDefiningSSB-r17 under BWP-DownlinkDedicated, and NCD-SSB serves as the reference SSB for the serving cell, and is contained in the active DL BWP.

During T2, Cell 2 is switched ON, and transmits two SSBs, i.e. CD-SSB at SSB frequency 1 and NCD-SSB at SSB frequency 2. Before the test, UE is configured to measure SSB frequency 2. The test equipment shall send an RRC message implying handover to Cell 2. The RRC message implying handover shall be sent to the UE during period T2, after the UE has reported Event A3.

The start of T3 is defined as the end of the last TTI containing the RRC message implying handover. The handover command indicates the UE to handover to Cell 2 with firstActiveDownlinkBWP-Id configured to BWP-1. The UE then performs handover from Cell 1’s active DL-BWP associated with the NCD-SSB of Cell 1 to Cell 2’s BWP-1 which is associated with NCD-SSB of Cell 2.

Table A.6.3.1.15.2-1: Intra-frequency handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.15.2-2: General test parameters Intra-frequency handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| SMTC configuration |  |  | SMTC.11 | For SSB frequency 2. |
| Measurement gap configuration |  |  | MG pattern #0, offset = 39 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

Table A.6.3.1.15.2-3: Cell specific test parameters for NR FR1-FR1 Intra frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  |  |  | 1 |  |  | 1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |
| measurement channel |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| CD-SSB Configuration |  | Config 1 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | Config 2 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  | SSB.2 FR1 |  |  |
| NCD-SSB Configuration |  | Config 1 |  | SSB.9 FR1 |  |  | SSB.9 FR1 |  |  |
|  |  | Config 2 |  | SSB.9 FR1 |  |  | SSB.9 FR1 |  |  |
|  |  | Config 3 |  | SSB.10 FR1 |  |  | SSB.10 FR1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.2 Note 4 |  |  | DLBWP.1.2 Note 4 |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.2 |  |  | ULBWP.1.2 |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |  |
|  |  |  | dB | 8 | -3.3 | -3.3 | -Infinity | 2.36 | 2.36 |
|  |  |  | dB | 8 | 8 | 8 | -Infinity | 11 | 11 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -90 | -90 | -90 | -Infinity | -87 | -87 |
|  | Config 3 |  | dBm/SCS | -87 | -87 | -87 | -Infinity | -84 | -84 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -61.41 | -57.06 | -57.06 | -61.41 | -57.06 | -57.06 |
|  | Config 3 |  | dBm/ 38.16MHz | -55.31 | -50.96 | -50.96 | -55.31 | -50.96 | -50.96 |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The starting PRB index for dedicated DL BWP is selected such that NCD-SSB is within the BWP BW. |  |  |  |  |  |  |  |  |  |

##### A.6.3.1.15.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 132 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = 122 ms in the test. Tinterrupt is defined in clause 6.1.1.2.2.

This gives a total of 132 ms.

#### A.6.3.1.16	Inter-frequency handover from FR1 to FR1; known target cell configured with NCD-SSB

##### A.6.3.1.16.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 inter-frequency handover requirements specified in clause 6.1.1.2, when the target cell is configured with NCD-SSB.

##### A.6.3.1.16.2	Test Parameters

Supported test configurations are shown in table A.6.3.1.16.2-1. Both handover delay and interruption length are tested by using the parameters in table A.6.3.1.16.2-2 and A.6.3.1.16.2-3.

The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2.

Before the test starts,

-	UE is connected to Cell 1 with active DL BWP and active UL BWP;

-	UE is not configured with nonCellDefiningSSB-r17 under BWP-DownlinkDedicated, and CD-SSB serves as the reference SSB for the serving cell, and is contained in the active DL BWP.

During T2, Cell 2 is switched ON, and transmits two SSBs, i.e. CD-SSB at SSB frequency 1 and NCD-SSB at SSB frequency 2. Before the test, UE is configured to measure SSB frequency 1. The test equipment shall send an RRC message implying handover to Cell 2. The RRC message implying handover shall be sent to the UE during period T2, after the UE has reported Event A3.

The start of T3 is defined as the end of the last TTI containing the RRC message implying handover. The handover command indicates the UE to handover to Cell 2 with firstActiveDownlinkBWP-Id configured to BWP-1. The UE then performs handover from Cell 1’s active DL-BWP associated with the CD-SSB of Cell 1 to Cell 2’s BWP-1 which is associated with NCD-SSB of Cell 2.

Table A.6.3.1.16.2-1: Inter-frequency handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.16.2-2: General test parameters Inter-frequency handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| SMTC configuration |  |  | SMTC.1 | For SSB frequency 1. |
|  |  |  | SMTC.11 | For SSB frequency 2. |
| Measurement gap configuration |  |  | MG pattern #0, offset = 39 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

Table A.6.3.1.16.2-3: Cell specific test parameters for NR FR1-FR1 Inter frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |  | T1 | T2 | T3 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 2 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |
| measurement channel |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |
| CD-SSB Configuration |  | Config 1 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |  |
|  |  | Config 2 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |  |
|  |  | Config 3 |  | SSB.2 FR2 |  |  | SSB.2 FR1 |  |  |  |
| NCD-SSB Configuration |  | Config 1 |  | N/A |  |  | SSB.9 FR1 |  |  |  |
|  |  | Config 2 |  | N/A |  |  | SSB.9 FR1 |  |  |  |
|  |  | Config 3 |  | N/A |  |  | SSB.10 FR2 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.3 |  |  | DLBWP.1.2 Note 4 |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.3 |  |  | ULBWP.1.2 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |  |  |
|  |  |  | dB | 4 | 4 | 4 |  | -Infinity | 5 | 5 |
|  |  |  | dB | 4 | 4 | 4 |  | -Infinity | 5 | 5 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -94 | -94 | -94 |  | -Infinity | -93 | -93 |
|  | Config 3 |  | dBm/SCS | -91 | -91 | -91 |  | -Infinity | -90 | -90 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59 | -64.59 | -64.59 |  | -70.05 | -63.85 | -63.85 |
|  | Config 3 |  | dBm/ 38.16MHz | -58.49 | -58.49 | -58.49 |  | -63.94 | -57.75 | -57.75 |
| Propagation condition |  |  | - | AWGN |  |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The starting PRB index for dedicated DL BWP is selected such that NCD-SSB is within the BWP BW. |  |  |  |  |  |  |  |  |  |  |

##### A.6.3.1.16.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 212 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = 202 ms in the test. Tinterrupt is defined in clause 6.1.1.2.2.

This gives a total of 212 ms.

<End of Change 1>

<Start of Change 2>

#### A.7.3.1.12	Intra-frequency handover from FR2 to FR2; known target cell configured with NCD-SSB

##### A.7.3.1.12.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR2-NR FR2 intra-frequency handover requirements specified in clause 6.1.1.4, when the target cell is configured with NCD-SSB.

##### A.7.3.1.12.2	Test Parameters

Supported test configurations are shown in table A.7.3.1.12.2-1. Both handover delay and interruption length are tested by using the parameters in table A.7.3.1.12.2-2 and A.7.3.1.12.2-3.

The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2.

Before the test starts,

-	UE is connected to Cell 1 with active DL BWP and active UL BWP;

-	UE is configured with nonCellDefiningSSB-r17 under BWP-DownlinkDedicated, and NCD-SSB serves as the reference SSB for the serving cell, and is contained in the active DL BWP.

During T2, Cell 2 is switched ON, and transmits two SSBs, i.e. CD-SSB at SSB frequency 1 and NCD-SSB at SSB frequency 2. Before the test, UE is configured to measure SSB frequency 2. The test equipment shall send an RRC message implying handover to Cell 2. The RRC message implying handover shall be sent to the UE during period T2, after the UE has reported Event A3.

The start of T3 is defined as the end of the last TTI containing the RRC message implying handover. The handover command indicates the UE to handover to Cell 2 with firstActiveDownlinkBWP-Id configured to BWP-1. The UE then performs handover from Cell 1’s active DL-BWP associated with the NCD-SSB of Cell 1 to Cell 2’s BWP-1 which is associated with NCD-SSB of Cell 2.

Table A.7.3.1.12.2-1: Intra-frequency handover from FR2 to FR2 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.1.12.2-2: General test parameters Intra-frequency handover from FR2 to FR2

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| SMTC configuration |  |  | SMTC.1 RedCap | For SSB frequency 2. |
| Measurement gap configuration |  |  | MG pattern #13, offset = 39 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

Table A.7.3.1.12.2-3: Cell specific test parameters for NR FR2-FR2 Intra frequency handover test case

| Parameter |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |  | T1 | T2 | T3 |
| Assumption for UE beamsNote 6 |  |  | Rough |  |  |  | Rough |  |  |
| AoA setup |  |  | Setup 1 as defined in A.3.15 |  |  |  |  |  |  |
| NR RF Channel Number |  |  | 1 |  |  |  | 1 |  |  |
| Duplex mode |  |  | TDD |  |  |  |  |  |  |
| TDD configuration |  |  | TDDConf.3.1 |  |  |  |  |  |  |
| BWchannel |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |
| BWP BW |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |
| Data RBs allocated |  |  | 66 |  |  |  |  |  |  |
| DRx Cycle |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  | SR3.1 TDD |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | CR3.1 TDD |  |  |  |  |  |  |
| Control Channel RMC |  |  | CCR.3.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  | OP. 1 |  |  |  |  |  |  |
| CD-SSB Configuration |  |  | SSB.3 FR2 |  |  | SSB.3 FR2 |  |  |  |
| NCD-SSB Configuration |  |  | SSB.21 FR2 |  |  | SSB.21 FR2 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | kHz | 120 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  | FR2 PRACH configuration 1 |  |  |  |  |  |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |  |  |  |  |  |
| PDSCH/PDCCH TCI state |  |  | TCI.State.2 |  |  |  |  |  |  |
| BWP configuraiton | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.2 Note 7 |  |  | DLBWP.1.2 Note 7 |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.2 |  |  | ULBWP.1.2 |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -104.7 |  |  |  | -104.7 |  |  |
| Note2 |  | dBm/SCS | -95.7 |  |  |  | -95.7 |  |  |
|  |  | dB | 6 | -1.8 | -1.8 |  | -Infinity | 0 | 0 |
|  |  | dB | 6 | 6 | 6 |  | -Infinity | 7 | 7 |
| IoNote3 | Config 1,2 | dBm/ BW | -59.7 | -56.7 | -56.7 |  | -59.7 | -56.7 | -56.7 |
| Propagation condition |  | - | AWGN |  |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 7: The starting PRB index for dedicated DL BWP is selected such that NCD-SSB is within the BWP BW. |  |  |  |  |  |  |  |  |  |

##### A.7.3.1.12.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 92 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = 82 ms in the test. Tinterrupt is defined in clause 6.1.1.4.2.

This gives a total of 92 ms.

#### A.7.3.1.13	Inter-frequency handover from FR2 to FR2; known target cell configured with NCD-SSB

##### A.7.3.1.13.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR2-NR FR2 inter-frequency handover requirements specified in clause 6.1.1.4, when the target cell is configured with NCD-SSB.

##### A.7.3.1.13.2	Test Parameters

Supported test configurations are shown in table A.7.3.1.13.2-1. Both handover delay and interruption length are tested by using the parameters in table A.7.3.1.13.2-2 and A.7.3.1.13.2-3.

The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2.

Before the test starts,

-	UE is connected to Cell 1 with active DL BWP and active UL BWP;

-	UE is not configured with nonCellDefiningSSB-r17 under BWP-DownlinkDedicated, and CD-SSB serves as the reference SSB for the serving cell, and is contained in the active DL BWP.

During T2, Cell 2 is switched ON, and transmits two SSBs, i.e. CD-SSB at SSB frequency 1 and NCD-SSB at SSB frequency 2. Before the test, UE is configured to measure SSB frequency 1. The test equipment shall send an RRC message implying handover to Cell 2. The RRC message implying handover shall be sent to the UE during period T2, after the UE has reported Event A3.

The start of T3 is defined as the end of the last TTI containing the RRC message implying handover. The handover command indicates the UE to handover to Cell 2 with firstActiveDownlinkBWP-Id configured to BWP-1. The UE then performs handover from Cell 1’s active DL-BWP associated with the CD-SSB of Cell 1 to Cell 2’s BWP-1 which is associated with NCD-SSB of Cell 2.

Table A.7.3.1.13.2-1: Inter-frequency handover from FR2 to FR2 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.1.13.2-2: General test parameters Inter-frequency handover from FR2 to FR2

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| SMTC configuration |  |  | SMTC.1 | For SSB frequency 1. |
|  |  |  | SMTC.1 RedCap | For SSB frequency 2. |
| Measurement gap configuration |  |  | MG pattern #13, offset = 39 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

Table A.7.3.1.13.2-3: Cell specific test parameters for NR FR2-FR2 Inter frequency handover test case

| Parameter |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |  | T1 | T2 | T3 |
| Assumption for UE beamsNote 6 |  |  | Rough |  |  |  | Rough |  |  |
| AoA setup |  |  | Setup 1as defined in A.3.15 |  |  |  |  |  |  |
| NR RF Channel Number |  |  | 1 |  |  |  | 2 |  |  |
| Duplex mode |  |  | TDD |  |  |  |  |  |  |
| TDD configuration |  |  | TDDConf.3.1 |  |  |  |  |  |  |
| BWchannel |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |
| BWP BW |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |
| Data RBs allocated |  |  | 66 |  |  |  |  |  |  |
| DRx Cycle |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  | SR3.1 TDD |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | CR3.1 TDD |  |  |  |  |  |  |
| Control Channel RMC |  |  | CCR.3.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  | OP. 1 |  |  |  |  |  |  |
| CD-SSB Configuration |  |  | SSB.3 FR2 |  |  | SSB.3 FR2 |  |  |  |
| NCD-SSB Configuration |  |  | N/A |  |  | SSB.21 FR2 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | kHz | 120 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  | FR2 PRACH configuration 1 |  |  |  |  |  |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |  |  |  |  |  |
| PDSCH/PDCCH TCI state |  |  | TCI.State.2 |  |  |  |  |  |  |
| BWP configuraiton | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.3 |  |  | DLBWP.1.2 Note 7 |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.3 |  |  | ULBWP.1.2 |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -104.7 |  |  |  | -104.7 |  |  |
| Note2 |  | dBm/SCS | -95.7 |  |  |  | -95.7 |  |  |
|  |  | dB | 5 | 5 | 5 |  | -Infinity | 5 | 5 |
|  |  | dB | 5 | 5 | 5 |  | -Infinity | 5 | 5 |
| IoNote3 | Config 1,2 | dBm/ BW | -60.5 | -60.5 | -60.5 |  | -66.7 | -60.5 | -60.5 |
| Propagation condition |  | - | AWGN |  |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 7: The starting PRB index for dedicated DL BWP is selected such that NCD-SSB is within the BWP BW. |  |  |  |  |  |  |  |  |  |

##### A.7.3.1.13.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 132 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = 122 ms in the test. Tinterrupt is defined in clause 6.1.1.4.2.

This gives a total of 132 ms.

<End of Change 2>