3GPP TSG-RAN WG4 Meeting #112	R4-2412054

Maastricht, Netherlands, 19th – 23rd August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.161 | CR | 0015 | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to Rel-17 TS 38.161 on preliminary MU alignment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_TRP_TRS |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The MU assessment in Rel-18 TS 38.161 has been updated, the corresponding correction should also be captured in Rel-17 spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating the MU assessment table. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MU value is not aligned in different release. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

A.4	Preliminary example MU budget

## A.4.1	General

## A.4.2	Test system of Anechoic Chamber method

The uncertainty contributions related to TRP are described in Annex B in [2]. A preliminary example uncertainty budget for TRP hand only test case using Anechoic Chamber test system is presented in Table A.4.2-1.

Table A.4.2-1 Preliminary example of uncertainty budget for TRP hand only (browsing mode) measurement for anechoic chamber method for NR FR1 bands

| UID | Uncertainty Source | Comment | Uncertainty Value [dB] |  | Prob Distr | Div | ci | Standard Uncertainty [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Below 3GHz | Above 3GHz |  |  |  | Below 3GHz | Above 3GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |
| 1 | Mismatch of receiver chain | Гreceiver < 0.33 Гmeasurement antenna < 0.5 Cable attenuation > 3dB | 0.26 | 0.26 | U-shaped | 1.4 | 1 | 0.18 | 0.18 |
| 2 | Insertion loss of receiver chain | Systematic with Stage 1 (=> cancels) | 0 | 0 | Rectangular | 1.7 | 1 | 0.00 | 0.00 |
| 3 | Influence of the measurement antenna cable | Systematic with Stage 1 (=> cancels) | 0 | 0 | Rectangular | 1.7 | 1 | 0.00 | 0.00 |
| 4 | Measurement Receiver: uncertainty of the absolute level | Spectrum Analyzer | 0.42 | 0.54 | Normal | 2 | 1 | 0.21 | 0.27 |
| 5 | Measurement distance | DUT is not offset for hand-only phantom testing | 0 | 0 | Rectangular | 1.7 | 1 | 0.00 | 0.00 |
| 6 | Quality of quiet zone | Surface standard deviation of power measurements in ripple test | 0.5 | 0.5 | Actual | 1 | 1 | 0.50 | 0.50 |
| 7 | DUT Tx-power drift | Drift | 0.2 | 0.2 | Rectangular | 1.7 | 1 | 0.12 | 0.12 |
| 8 | Uncertainty related to the use of phantoms | Material Dielectric Constant, Material Conductivity, Geometry/Shape (incl. spacer), Data Mode Fixture | 0.64 | 0.64 | Rectangular | 1.7 | 1 | 0.37 | 0.37 |
| 9 | Coarse sampling grid | Sampling grids per Table B.2.12-1 in [2] | 0 | 0 | Actual | 1 | 1 | 0.00 | 0.00 |
| 10 | Random Uncertainty | Fixed MU to account for all the unknown, unquantifiable, etc. uncertainties | 0.25 | 0.25 | Normal | 2 | 1 | 0.13 | 0.13 |
| 11 | Frequency Response | Average path loss corrected | 0 | 0 | Rectangular | 1.7 | 1 | 0.00 | 0.00 |
| Stage 1: Calibration measurement, network analyzer method |  |  |  |  |  |  |  |  |  |
| 12 | Uncertainty of network analyzer | From datasheet of VNA with assessed transmission coefficients | 0.2 | 0.5 | Normal | 2 | 1 | 0.10 | 0.25 |
| 13 | Mismatch of receiver chain | Taken into account in VNA uncertainty term | 0 | 0 | U-shaped | 1.4 | 1 | 0.00 | 0.00 |
| 14 | Insertion loss of receiver chain | Systematic with Stage 2 (=> cancels) | 0 | 0 | Rectangular | 1.7 | 1 | 0.00 | 0.00 |
| 15 | Mismatch in the connection of calibration antenna | Taken in to account in VNA setup uncertainty | 0 | 0 | U-shaped | 1.4 | 1 | 0.00 | 0.00 |
| 16 | Influence of the calibration antenna feed cable | Gain calibration with a dipole | 0.3 | 0.3 | Rectangular | 1.7 | 1 | 0.17 | 0.17 |
| 17 | Influence of the measurement antenna cable | Systematic with Stage 2 (=> cancels) | 0 | 0 | Rectangular | 1.7 | 1 | 0.00 | 0.00 |
| 18 | Uncertainty of the absolute gain/ radiation efficiency of the calibration antenna | Calibration report with traceability to a National Metrology Institute | 0.58 | 0.58 | Normal | 2 | 1 | 0.29 | 0.29 |
| 19 | Measurement distance | Dipole: aligned with phase center | 0 | 0 | Rectangular | 1.7 | 1 | 0.00 | 0.00 |
| 20 | Quality of the Quiet Zone | Peak-to-null ripple | 0.5 | 0.5 | Rectangular | 1.7 | 1 | 0.29 | 0.29 |
| Combined standard uncertainty [dB] |  |  |  |  |  |  |  | 0.84 | 0.88 |
| Expanded uncertainty [dB] (Confidence interval of 95 %) |  |  |  |  |  |  |  | 1.64 | 1.73 |
| 21 | Systematic Error related to TRP grids | mean error | 0 | 0 | Actual | 1 | 1 | 0.00 | 0.00 |
| Total Expanded uncertainty [dB] (Confidence interval of 95 %) |  |  |  |  |  |  |  | 1.64 | 1.73 |

The uncertainty contributions related to TRS are described in Annex B in [2]. A preliminary example uncertainty budget TRS hand only test case using Anechoic Chamber test system is presented in Table A.4.2-2.

Table A.4.2-2: Preliminary example of uncertainty budget for TRS hand only (browsing mode) measurement for anechoic chamber method for NR FR1 bands

| UID |  | Uncertainty Source | Comment |  | Uncertainty Value [dB] |  |  | Prob Distr |  |  | Div |  | ci |  |  | Standard Uncertainty [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Below 3GHz | Above 3GHz |  |  |  |  |  |  |  |  |  | Below 3GHz |  |  | Above 3GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | Mismatch of receiver chain | Гcomm tester < 0.29 Гmeasurement antenna < 0.5 Cable attenuation > 3dB |  | 0.22 | 0.22 |  | U-shaped |  |  | 1.4 |  | 1 |  |  | 0.16 |  |  | 0.16 |
| 2 |  | Insertion loss of receiver chain | Systematic with Stage 1 (=> cancels) |  | 0 | 0 |  | Rectangular |  |  | 1.7 |  | 1 |  |  | 0.00 |  |  | 0.00 |
| 3 |  | Influence of the measurement antenna cable | Systematic with Stage 1 (=> cancels) |  | 0 | 0 |  | Rectangular |  |  | 1.7 |  | 1 |  |  | 0.00 |  |  | 0.00 |
| 4 |  | Communication Tester: uncertainty of the absolute output level | Manufacturer’s data sheet |  | 1.3 | 1.3 |  | Normal |  |  | 2 |  | 1 |  |  | 0.65 |  |  | 0.65 |
| 5 |  | Sensitivity measurement: output level step resolution | Systematic error that can be corrected |  | 0 | 0 |  | Rectangular |  |  | 1.73 |  | 1 |  |  | 0.00 |  |  | 0.00 |
| 6 |  | Measurement distance | DUT is not offset for hand-only phantom testing |  | 0 | 0 |  | Rectangular |  |  | 1.7 |  | 1 |  |  | 0.00 |  |  | 0.00 |
| 7 |  | Quality of quiet zone | Surface standard deviation of power measurements in ripple test |  | 0.5 | 0.5 |  | Actual |  |  | 1 |  | 1 |  |  | 0.50 |  |  | 0.50 |
| 8 |  | DUT sensitivity drift | Drift |  | 0.2 | 0.2 |  | Rectangular |  |  | 1.7 |  | 1 |  |  | 0.12 |  |  | 0.12 |
| 9 |  | Uncertainty related to the use of phantoms | Material Dielectric Constant, Material Conductivity, Geometry/Shape (incl. spacer), Data Mode Fixture |  | 0.64 | 0.64 |  | Rectangular |  |  | 1.7 |  | 1 |  |  | 0.37 |  |  | 0.37 |
| 10 |  | Coarse sampling grid | Sampling grids per Table B.2.12-1 in [2] |  | 0.10 | 0.18 |  | Actual |  |  | 1 |  | 1 |  |  | 0.10 |  |  | 0.18 |
| 11 |  | Random Uncertainty | Fixed MU to account for all the unknown, unquantifiable, etc. uncertainties including digital error rate |  | 0.4 | 0.4 |  | Normal |  |  | 2 |  | 1 |  |  | 0.20 |  |  | 0.20 |
| 12 |  | Frequency Response | Included in the output level step resolution |  | 0 | 0 |  | Rectangular |  |  | 1.7 |  | 1 |  |  | 0.00 |  |  | 0.00 |
| Stage 1: Calibration measurement, network analyzer method |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  | Uncertainty of network analyzer | From datasheet of VNA with assessed transmission coefficients |  | 0.2 | 0.5 |  | Normal |  |  | 2 |  | 1 |  |  | 0.10 |  |  | 0.25 |
| 14 |  | Mismatch of receiver chain | Taken into account in VNA uncertainty term |  | 0 | 0 |  | U-shaped |  |  | 1.4 |  | 1 |  |  | 0.00 |  |  | 0.00 |
| 15 |  | Insertion loss of receiver chain | Systematic with Stage 2 (=> cancels) |  | 0 | 0 |  | Rectangular |  |  | 1.7 |  | 1 |  |  | 0.00 |  |  | 0.00 |
| 16 |  | Mismatch in the connection of calibration antenna | Taken in to account in VNA setup uncertainty |  | 0 | 0 |  | U-shaped |  |  | 1.4 |  | 1 |  |  | 0.00 |  |  | 0.00 |
| 17 |  | Influence of the calibration antenna feed cable | Gain calibration with a dipole |  | 0.3 | 0.3 |  | Rectangular |  |  | 1.7 |  | 1 |  |  | 0.17 |  |  | 0.17 |
| 18 |  | Influence of the measurement antenna cable | Systematic with Stage 2 (=> cancels) |  | 0 | 0 |  | Rectangular |  |  | 1.7 |  | 1 |  |  | 0.00 |  |  | 0.00 |
| 19 |  | Uncertainty of the absolute gain/ radiation efficiency of the calibration antenna | Calibration report with traceability to a National Metrology Institute |  | 0.58 | 0.58 |  | Normal |  |  | 2 |  | 1 |  |  | 0.29 |  |  | 0.29 |
| 20 |  | Measurement distance | Dipole: aligned with phase center |  | 0 | 0 |  | Rectangular |  |  | 1.7 |  | 1 |  |  | 0.00 |  |  | 0.00 |
| 21 |  | Quality of the Quiet Zone | Peak-to-null ripple |  | 0.5 | 0.5 |  | Rectangular |  |  | 1.7 |  | 1 |  |  | 0.29 |  |  | 0.29 |
| Combined standard uncertainty [dB] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.05 |  |  | 1.09 |
| Expanded uncertainty [dB] (Confidence interval of 95 %) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.06 |  |  | 2.13 |
| 22 |  | Systematic Error related to TRP grids | mean error |  | 0 | 0 |  |  | Actual |  |  | 1 |  | 1 |  | 0.00 |  |  | 0.08 |
| Total Expanded uncertainty [dB] (Confidence interval of 95 %) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.06 |  |  | 2.13 |

< End of changes >