3GPP RAN TS Meeting #105	RP-242202

Melbourne, Australia, Sept 9-13,

Title:	NTN for NR Phase 3 – Satellite stakeholders’ view on SSB Periodicity Extension

Source: 	EchoStar, Thales, Eutelsat Group, Airbus, ESA, Novamint

Type:	discussion

Document for:	agreement

Agenda Item:	9.3.2.2

# 1.0 Introduction

## 1.1 RAN#104

RAN#104 meeting has updated the WID for NR NTN phase 3 to make clear that SSB periodicity extension is within the scope, as shown following.

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency, and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

## 1.2 WF from RAN1#118

Agreement As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider: - Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. o Default value[s] with extended periodicity assumed by NTN UE for initial access can be: o One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms} - Potential enhancements for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint - Link-level enhancements for the following channels: - PDCCH - PDSCH with Msg 4 - PDSCH with SIB1/SIB19. Note: link-level enhancements for PDSCH with SIB1/SIB19 may be applicable to other SIBs, without additional specification impact. Note: the above does not imply that all the channels above will be enhanced, but all of them should be considered based on this agreement

# 2.0 Discussion

## 2.1 SSB Periodicity Extension

Regarding the benefits of extending the SSB Periodicity the RAN1#117 and RAN1#118 made the following observations:

Based on the results of DL coverage ratio evaluation at system level collected from 7 sources for all the three LEO 600km satellite parameter sets where the beam footprint diameter is 50 km:

- For Set 1-1/1-3, the coverage ratio can be improved from 10% to 100% if the SSB periodicity is increased from 20ms to 80ms and beam hopping is applied

- For Set 1-2, the coverage ratio can be improved from 1.5% to 96.8% if the SSB periodicity is increased from 20ms to 320ms and beam hopping is applied.

- Note: coverage ratio is N2+N3/ total beam footprints

- Note: the baseline assumes no beam hopping. TDM between SIB1 and SIB19 is assumed in those results, following current specs.

Based on the results of DL coverage evaluation at system level collected from different sources, it is observed that extending the default value of SSB periodicity (different from 20ms) in NTN with LEO600km satellite parameter sets where the beam footprint diameter is 50 km, is beneficial in terms of reduction of common control channel overhead, when targeting a full coverage of 1058 beam footprints:

- With Set 1-1 FR1 and Set 1-3 FR1, the common messages (SSB, SIB1) overhead is around 40% assuming 5 MHz BW when SSB/SIB1 periodicity of 20ms is in use, this overhead ratio could be reduced to less than 14% when 160ms SSB/SIB1 periodicity is used.

- With Set 1-2 FR1, the common message (SSB, SIB1) overhead is greater than 100% assuming 5 MHz BW when SSB/SIB1 periodicity of 20ms is in use, this overhead could be reduced to around 25.8% when 640ms SSB/SIB1 periodicity is used.

- Note: the overhead of SIB19 was included in some of the results

Observation 1

Many companies have brought proposals to the last RAN1 [1] and highlighted the importance and benefits of extending SSB periodicity for NTN. In principle, the feature can optimize satellite communication systems, particularly in beam-hopping scenarios, by enhancing resource efficiency, reducing power consumption, and supporting diverse use cases.

- Satellite systems are power-limited, and beam-hopping can improve downlink coverage ratios, a longer SSB periodicity is essential to ensure that a limited number of active beams maximize the efficiency of beam-hopping.

- By extending the SSB periodicity for NTN, the frequency of transmitting downlink common channel is reduced, which lowers the downlink signalling overhead. This frees up time and frequency domain resources, allowing more capacity for user data transmission and hence improved system capacity and cell throughput.

- Beam-hopping if used in efficient manner can minimize interference with adjacent satellite beams or terrestrial networks, leading to better coexistence and more efficient use of the spectrum. For beam-hopping to operate SSB periodicity extension is essential

In our view, extending the periodicity of the SS/PBCH blocks assumed by UE during initial access is an essential enhancement to improve downlink coverage ratio at system level in NR NTN. The supported extended SSB periodicity value can be decided based on the achievable coverage ratio improvement, the common channel overhead reduction, while minimizing the impacts on UE modem complexity, cell search latency and on other specifications. This value can be defined during the normative phase.

Based on RAN1#118 [1]: a majority of companies support SSB Periodicity Extension.

Proposal 1:  As part of the NTN DL coverage enhancements at both system level and link level, RAN shall prioritize SSB Periodicity Extension

## 2.2 Wider Beam Approach

Satellite operators invest in larger satellite antennas to create sharper beams, ensuring sufficient EIRP and G/T to deliver data at speeds acceptable to users. Widening the beam universally would undermine and compromise this objective. Using wider beams for SSB would therefore require at least two differently sized beams, adding complexity to both upper-layer software and operational considerations. The wide beam approach is not ideal from a satellite power perspective, since total satellite downlink power is a critical resource directly tied to the satellite capacity.

Even though increasing the SSB beam size by a factor of n may seem to reduce the number of beams used for SSB transmission by the same factor n to achieve the same threshold performance as before for each SSB, this would adversely cause a significant degradation of SINR causing performance loss and coverage issues, as reported by a number of sources [1], and the approach would result in little or no benefit.

Observation 2

Based on RAN1#118 [1], it was observed that:

- Based on different sources that wider beam has significant degradation on the SINR distribution, causing performance loss and coverage issue.

- Wider beam requires additional signaling to identify UE’s position, which will result in additional standard specification and may introduce privacy and security issues. Given the limited TU we need to focus on the SSB extension.

- Several companies mentioned that using wider beams can be an implementation matter.

Proposal 2: The support of wider beam for downlink common channel shall be considered with low priority compared to the extension of the SSB periodicity.

# 3.0 Conclusion

Proposal 1: As part of the NTN DL coverage enhancements at both system level and link level, RAN shall prioritize SSB Periodicity Extension.

Proposal 2: The support of wider beam for downlink common channel shall be considered with low priority compared to the extension of the SSB periodicity.

# 4.0 Reference

[1] R1-2406437 FL Summary #3: NR-NTN downlink coverage enhancements (RAN WG1 #118 - Maastricht August 19th – 23rd, 2024)