3GPP TSG-RAN WG1 Meeting #118	R1-2407577

# Maastricht, NL, August 19th – 23rd, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0627 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 editorial corrections for TS 38.214 mirrored to Rel-18 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |
| Work item code: | TEI17, NR_2step_RACH-Core, NR_redcap-Core, NR_SmallData_INACTIVE-Core, NR_cov_enh-Core |  |  |  | Date: |  | 2024-08-30 |
| Category: | A |  |  |  | Release: |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NR_2step_RACH-Core In clause 5.1.6.3: there is a square bracket around MSGB-RNTI for UE assumption on whether PT-RS is present in clause 5.1.6.3, which leads to unclear UE behavior. NR_redcap-Core, NR_SmallData_INACTIVE-Core, NR_cov_enh-Core In clause 6.1.3, it is unclear the PUSCH configuration w.r.t BWP and RACH configurations. |  |  |  |  |  |
| Summary of change: |  | NR_2step_RACH-Core In clause 5.1.6.3: removed the square bracket around MSGB-RNTI in clause 5.1.6.3. NR_redcap-Core, NR_SmallData_INACTIVE-Core, NR_cov_enh-Core In clause 6.1.3, clarified the PUSCH configuration w.r.t BWP and RACH configurations. |  |  |  |  |  |

| Consequences if not approved: | Unclear specification NR_2step_RACH-Core In clause 5.1.6.3: unclear UE behavior on whether UE shall assume PT-RS is not present for MsgB PDSCH. NR_redcap-Core, NR_SmallData_INACTIVE-Core, NR_cov_enh-Core In clause 6.1.3, unclear PUSCH configuration. |  |  |  |
| --- | --- | --- | --- | --- |
| Clauses affected: | 5.1.6.3, 6.1.3 |  |  |  |
|  | Y | N |  |  |
| Other specs |  | X | Other core specifications | TS/TR ... CR ... |
| affected: |  | X | Test specifications | TS/TR ... CR ... |
| (show related CRs) |  | X | O&M Specifications | TS/TR ... CR ... |
| Other comments: |  |  |  |  |
| This CR's revision history: |  |  |  |  |

<omitted text>

#### 5.1.6.3	PT-RS reception procedure

The procedures on PT-RS reception described in this clause apply to a UE receiving PDSCH scheduled by DCI format 1_2 configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 or dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 and to a UE receiving PDSCH scheduled by DCI format 1_0, 1_1 or 1_3 configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA or dmrs-DownlinkForPDSCH-MappingTypeB. The procedures on PT-RS reception described in this clause apply to a UE receiving PDSCH scheduled by DCI format 4_1 or 4_2 configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA or dmrs-DownlinkForPDSCH-MappingTypeB in pdsch-ConfigMulticast.

A UE shall report the preferred MCS and bandwidth thresholds based on the UE capability at a given carrier frequency, for each subcarrier spacing applicable to data channel at this carrier frequency, assuming the MCS table with the maximum Modulation Order as it reported to support.

If a UE is configured with the higher layer parameter phaseTrackingRS in DMRS-DownlinkConfig,

-	the higher layer parameters timeDensity and frequencyDensity in PTRS-DownlinkConfig indicate the threshold values ptrs-MCSi, i=1,2,3 and NRB,i , i=0,1, as shown in Table 5.1.6.3-1 and Table 5.1.6.3-2, respectively.

-	if either or both of the additional higher layer parameters timeDensity and frequencyDensity are configured, and the RNTI equals MCS-C-RNTI, C-RNTI or CS-RNTI, the UE shall assume the PT-RS antenna port' presence and pattern is a function of the corresponding scheduled MCS of the corresponding codeword and scheduled bandwidth in corresponding bandwidth part as shown in Table 5.1.6.3-1 and Table 5.1.6.3-2,

-	if the higher layer parameter timeDensity given by PTRS-DownlinkConfig is not configured, the UE shall assume LPT-RS = 1.

-	if the higher layer parameter frequencyDensity given by PTRS-DownlinkConfig is not configured, the UE shall assume KPT-RS = 2.

-	otherwise, if neither of the additional higher layer parameters timeDensity and frequencyDensity are configured and the RNTI equals MCS-C-RNTI, C-RNTI or CS-RNTI, the UE shall assume the PT-RS is present with LPT-RS = 1, KPT-RS = 2, and the UE shall assume PT-RS is not present when

-	the scheduled MCS from Table 5.1.3.1-1 is smaller than 10, or

-	the scheduled MCS from Table 5.1.3.1-2 is smaller than 5, or

-	the scheduled MCS from Table 5.1.3.1-3 is smaller than 15, or

-	the scheduled MCS from Table 5.1.3.1-4 is smaller than 3, or

-	the number of scheduled RBs is smaller than 3, or

-	otherwise, if the RNTI equals RA-RNTI, MSGB-RNTI, SI-RNTI, or P-RNTI, the UE shall assume PT-RS is not present

Table 5.1.6.3-1: Time density of PT-RS as a function of scheduled MCS

| Scheduled MCS | Time density () |
| --- | --- |
| IMCS < ptrs-MCS1 | PT-RS is not present |
| ptrs-MCS1 IMCS < ptrs-MCS2 | 4 |
| ptrs-MCS2 IMCS < ptrs-MCS3 | 2 |
| ptrs-MCS3 IMCS < ptrs-MCS4 | 1 |

Table 5.1.6.3-2: Frequency density of PT-RS as a function of scheduled bandwidth

| Scheduled bandwidth | Frequency density () |
| --- | --- |
| NRB < NRB0 | PT-RS is not present |
| NRB0 NRB < NRB1 | 2 |
| NRB1 NRB | 4 |

If a UE is not configured with the higher layer parameter phaseTrackingRS in DMRS-DownlinkConfig, the UE assumes PT-RS is not present.

The higher layer parameter PTRS-DownlinkConfig provides the parameters ptrs-MCSi, i=1,2,3 and with values in range 0-29 when MCS Table 5.1.3.1-1 or MCS Table 5.1.3.1-3 is used and 0-28 when MCS Table 5.1.3.1-2 is used, and 0-27 when MCS Table 5.1.3.1-4 is used, respectively. ptrs-MCS4 is not explicitly configured by higher layers but assumed 29 when MCS Table 5.1.3.1-1 or MCS Table 5.1.3.1-3 is used and 28 when MCS Table 5.1.3.1-2 is used and 27 when MCS Table 5.1.3.1-4 is used, respectively. The higher layer parameter frequencyDensity in PTRS-DownlinkConfig provides the parameters NRBi i=0,1 with values in range 1-276.

If the higher layer parameter PTRS-DownlinkConfig indicates that the time density thresholds ptrs-MCSi = ptrs-MCSi+1, then the time density LPT-RS of the associated row where both these thresholds appear in Table 5.1.6.3-1 is disabled. If the higher layer parameter PTRS-DownlinkConfig indicates that the frequency density thresholds NRBi = NRBi +1, then the frequency density KPTRS of the associated row where both these thresholds appear in Table 5.1.6.3-2 is disabled.

If either or both of the parameters PT-RS time density (LPT-RS) and PT-RS frequency density (KPT-RS), shown in Table 5.1.6.3-1 and Table 5.1.6.3-2, indicates that 'PT-RS not present', the UE shall assume that PT-RS is not present.

When the UE is receiving a PDSCH with allocation duration of 2 symbols as defined in Clause 7.4.1.1.2 of [4, TS 38.211] and if LPT-RS is set to 2 or 4, the UE shall assume PT-RS is not transmitted.

When the UE is receiving a PDSCH with allocation duration of 4 symbols and if LPT-RS is set to 4, the UE shall assume PT-RS is not transmitted.

When a UE is receiving PDSCH for retransmission, if the UE is scheduled with an MCS index greater than V, where V=28 for MCS Table 5.1.3.1-1 and Table 5.1.3.1-3, and V=27 for MCS Table 5.1.3.1-2, and V=26 for MCS Table 5.1.3.1-4 respectively, the MCS for the PT-RS time-density determination is obtained from the DCI received for the same transport block in the initial transmission, which is smaller than or equal to V.

The DL DM-RS port(s) associated with a PT-RS port are assumed to be quasi co-located with respect to 'typeA' and 'typeD'. If a UE is scheduled with one codeword, the PT-RS antenna port is associated with the lowest indexed DM-RS antenna port among the DM-RS antenna ports assigned for the PDSCH.

If a UE is scheduled with two codewords, the PT-RS antenna port is associated with the lowest indexed DM-RS antenna port among the DM-RS antenna ports assigned for the codeword with the higher MCS. If the MCS indices of the two codewords are the same, the PT-RS antenna port is associated with the lowest indexed DM-RS antenna port assigned for codeword 0.

When a UE is not indicated with a DCI that DCI field 'Time domain resource assignment' indicating an entry which contains repetitionNumber in PDSCH-TimeDomainResourceAllocation, and if the UE is configured with the higher layer parameter maxNrofPorts equal to n2, the UE is not configured with sfnSchemePDSCH and if the UE not configured with dl-OrJointTCI-StateList is indicated with two TCI states by the codepoints of the DCI field 'Transmission Configuration Indication' or when the UE configured with dl-OrJointTCI-StateList and having two indicated TCI States to be applied to PDSCH, and indicated DM-RS port(s) within two CDM groups in the DCI field 'Antenna Port(s)', the UE shall receive two PT-RS ports which are associated to the lowest indexed DM-RS port among the DM-RS ports corresponding to the first/second indicated TCI state, respectively.

When a UE configured by the higher layer parameter repetitionScheme set to 'fdmSchemeA' or  'fdmSchemeB', and the UE not configured with dl-OrJointTCI-StateList is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' or when the UE configured with dl-OrJointTCI-StateList and having two indicated TCI States to be applied to PDSCH, and indicated DM-RS port(s) within one CDM group in the DCI field 'Antenna Port(s)', the UE shall receive a single PT-RS port which is associated with the lowest indexed DM-RS antenna port among the DM-RS antenna ports assigned for the PDSCH, a PT-RS frequency density is determined by the number of PRBs associated to each TCI state, and a PT-RS resource element mapping is associated to the allocated PRBs for each TCI state.

<omitted text>

### 6.1.3	UE procedure for applying transform precoding on PUSCH

For a PUSCH scheduled by RAR UL grant, or for a PUSCH scheduled by fallbackRAR UL grant, or for a PUSCH scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, the UE shall consider the transform precoding either 'enabled' or 'disabled' according to the higher layer configured parameter msg3-transformPrecoder.

For a MsgA PUSCH, the UE shall consider the transform precoding either 'enabled' or 'disabled' according to the higher layer configured parameter msgA-TransformPrecoder. If higher layer parameter msgA-TransformPrecoder is not configured, the UE shall consider the transform precoding either 'enabled' or 'disabled' according to the higher layer configured parameter msg3-transformPrecoder.

For PUSCH transmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, C-RNTI, or MCS-C-RNTI or SP-CSI-RNTI:

-	If the DCI with the scheduling grant was received with DCI format 0_0, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder from rach-ConfigCommon included directly within BWP configuration.

-	If the DCI with the scheduling grant was not received with DCI format 0_0

-	If the DCI with the scheduling grant was received with DCI format 0_1 or 0_2 with CRC scrambled by C-RNTI, MCS-RNTI, or CS-RNTI with NDI=1 and if the UE is configured with a higher layer parameter [dynamicTransformPrecoderIndicationDCI-0-1] in pusch-Config for DCI format 0_1 or [dynamicTransformPrecoderIndicationDCI-0-2] in pusch-Config for DCI format 0_2 and the higher layer parameter is set to 'enabled',

-	the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the Transform precoder indicator field in the DCI with the scheduling grant.

-	For pusch-TimeDomainAllocationListForMultiPUSCH in pusch-Config, the UE shall, for all PUSCH transmissions, consider the transform precoding either enabled or disabled according to Transform precoder indicator field in the DCI format 0_1 with the scheduling grant.

-	If resourceAllocation in pusch-Config for DCI format 0_1 or resourceAllocationDCI-0-2 in pusch-Config for DCI format 0_2 is set to resourceAllocationType0, or if the resource allocation is set to resource allocation type 0 according to the DCI configuration as described in clauses 7.3.1.1.2 and 7.3.1.1.3 of [6, TS 38.212], or if dmrs-Type in DMRS-UplinkConfig is set to ‘type 2’ for this PUSCH transmission, the UE does not expect that the Transform precoder indicator field in the DCI with the scheduling grant indicates that transform precoding is enabled.

-	If the UE is configured with the higher layer parameter dmrs-TypeEnh in DMRS-UplinkConfig, and if the scheduling grant indicates that transform precoding is enabled for the scheduled PUSCH transmission, the UE ignores the higher layer parameters dmrs-TypeEnh in DMRS-UplinkConfig, if configured, for the DM-RS transmission of the scheduled PUSCH transmission.

-	Otherwise,

-	If the UE is configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter.

-	If the UE is not configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder.

For PUSCH transmission with a configured grant

-	If the UE is configured with the higher layer parameter transformPrecoder in configuredGrantConfig, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter.

-	If the UE is not configured with the higher layer parameter transformPrecoder in configuredGrantConfig, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder.

<omitted text>