3GPP TSG-RAN WG1 Meeting #118	R1-2407359

Maastricht, NL, August 19th – 23rd, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0600 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on CBGTI Field Size Determination for 2CW Transmission by 8TX UL MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (InterDigital), Ericsson, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When 8 Tx UL MIMO was introduced, it was agreed to update the CBGTI field as captured below to support two codeword transmission. While 38.212 subclause 7.3.1.1.2 specifies that 38.214 subclause 6.1.5 does so, 38.214 does not presently have how the CBGTI field size is calculated for 8 Tx. Agreement (RAN1#113) To support CBG-based transmission for dual CW PUSCH operation, the range N=2, 4, 6, or 8 is confirmed for the CBGTI bit field Agreement (RAN1#112bis) - To support dual CW PUSCH operation by an 8TX UE, if CBG-based transmission is configured, the DL principle for CBGTI DCI field is reused where, - The first half of CBGTI field bits is used to indicate the transmission state of CBGs of the first transport block, while the second half of CBGTI field bits is used to indicate the transmission state of CBGs of the second transport block. - The bit field may be configured to have a length of N bits that can support operation of N/2 CBGs , where N=[2, 4, 6 or 8]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Calculate the CBGTI field size for two codewords in the same way as the downlink: when the DCI carries two codewords (i.e. for PUSCH when maxRank or maxMIMO-Layers is larger than 4), there are two sets of bits in the CBGTI field, one for each codeword, and otherwise one set of bits for the one codeword carried in the DCI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The agreed functionality of two codeword operation with code block groups and 8 Tx UL MIMO is not supported, since the two sets of bits in CBGTI for the two codewords are not defined for PUSCH. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | N | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.1.5.1	UE procedure for grouping of code blocks to code block groups

If a UE is configured to transmit code block group (CBG) based transmissions by receiving the higher layer parameter codeBlockGroupTransmission in PUSCH-ServingCellConfig, the UE shall determine the number of CBGs for a transport block transmission as

,

where N is the maximum number of CBGs per transport block as configured by maxCodeBlockGroupsPerTransportBlock in PUSCH-ServingCellConfig, and C is the number of code blocks in the transport block according to the procedure defined in Clause 6.2.3 of [5, TS 38.212].

Define , , and .

If , CBG m, , consists of code blocks with indices . CBG m, , consists of code blocks with indices .

#### 6.1.5.2	UE procedure for transmitting code block group based transmissions

If a UE is configured to transmit code block group-based transmissions by receiving the higher layer parameter codeBlockGroupTransmission in PUSCH-ServingCellConfig,

-	The 'CBG transmission information' (CBGTI) field of DCI format 0_1 is of length 2∙N  bits if maxRank or maxMIMO-Layers is larger than 4, and N bits otherwise.  If maxRank or maxMIMO-Layers is larger than 4, the CBGTI field bits are mapped such that the first set of N bits starting from the MSB corresponds to the first TB while the second set of N  bits corresponds to a second TB, if scheduled. The first M bits of each set of N  bits in the CBGTI field have an in-order one-to-one mapping with the M CBGs of the TB, with the MSB mapped to CBG#0.

-	For an initial transmission of a TB as indicated by the 'New Data Indicator' field of the scheduling DCI, the UE may expect that the CBGTI field indicates all the CBGs of the TB are to be transmitted, and the UE shall include all the code block groups of the TB.

-	For a retransmission of a TB as indicated by the 'New Data Indicator' field of the scheduling DCI, the UE shall include only the CBGs indicated by the CBGTI field of the scheduling DCI.

A bit value of '0' in the CBGTI field indicates that the corresponding CBG is not to be transmitted and '1' indicates that it is to be transmitted. The order of CBGTI field bits is such that the CBGs are mapped in order from CBG#0 onwards starting from the MSB.