3GPP TSG RAN Meeting #105								RP-242220

Melbourne, Australia, September 9-12, 2024

Agenda Item:	9.3.2.3

Source:	Ericsson

Title:	Discussion on scope of Rel-19 XR WI

Document for:	Discussion, Decision

# 1	Introduction

The two Rel-19 XR objectives that were in the study phase (Multi-Modality enhancements and UL Scheduling enhancements) have reached checkpoints and it is time to decide on the scope for the normative phase.

Furthermore, it was decided in the previous plenary RAN#104 that an extension of the Rel-19 XR WID scope for UL congestion signaling enhancements is intended to be included in the scope Rel-19 XR work, with further details to be discussed in RAN#105 [1]:

According to the discussion in RAN#104, we aim to replace the current note with the WID in the RAN#105 (see below the note), with the objective to address uplink rate control from RAN, which is complementary to upper layer solutions (e.g. L4S).”. For the RAN#105 we should aim to have also agreement on the which layer the gNb would provide the information in the downlink direction The RAN2 in August is expected to make conclusions on the other aspects as per current WID, such that we are able to have scope fitting to the time available for XR work going forward after RAN#105. NOTE from the current WID: “Whether / to what extent network exposure / RAN awareness / e.g. RAN involved rate control, possibly additional info for DL scheduling, parallel with SA2 work, shall be covered in this WI is TBD. “

# 2	Discussion

## 2.1 Multi-modality enhancements

There were multiple agreements in RAN2 on multi-modality during the study phase. Agreements can be found in Appendix.

Based on those agreements a way forward for the normative phase can be proposed. Two areas have been identified as possibly requiring solutions: the UE providing multi-modal information in the UAI and RAN using the multi-modal information.

2.1.1 Multi-modality UAI solution

The current WID objective for multi-modality is:

Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2].

RAN2 has concluded that it would be possible for RAN to extend the UAI with multi modal information at least for UL (cf. agreements in the Appendix). No conclusion could be made if this could also be applied for DL.

Based on the discussion it remains unclear what is the information that the UE can provide and how it would be useful for RAN. It should be noted that SA2 is still discussing the DL part of the multi-modal information based on the LS RAN2 sent earlier. Providing the DL information from the CN would be a straightforward approach and it is up to SA2 to decide if the information will be provided to RAN. Thus, it remains best for RAN to wait for the SA2 reply before specifying anything for the DL.

Furthermore, RAN has not identified any clear use for static UL information to be provided in the UAI. The discussed use cases are focused on utilizing possible DL multi-modal information, whose details are not clear at the moment. Even if one could argue the possible specification work is not much, that is, if UAI is to be specified, it is not clear what exact information would be included in UAI and further discussions would be required in the WGs. Also, there are no clear conclusions from RAN2 regarding enhanced efficiency in terms of capacity or power consumption, even if UAI for multi modality is specified.

Therefore, we propose to not standardize any multi-modal information in the UAI and wait for SA2 to conclude on the topic of providing information from CN side to RAN.

- RAN does not specify any new multi-modal information to be provided by the UE to RAN in the UAI, instead wait for SA2 reply regarding information from CN.

2.1.2 Multi-modal information usage in RAN

The only thing that RAN2 has concluded could be standardized is joint admission control, everything else is left to network implementation. RAN2 indicated that it can be left to RAN3 to decide on any details on a joint admission control solution and in fact that domain is for RAN3 to decide on, however RAN3 has no more TUs assigned to work on multi-modality in Rel19. Furthermore, SA2 has already discussed joint admission control in Rel18 and concluded that such enhancement was not needed since it is better handled by the application. In our view also the admission control topic can be left to network implementation, and we don’t deem it necessary to add any work for RAN3 on this. It should be noted that many of the other use cases that has been proposed in RAN2 (e.g. traffic synchronization, QoS flow mapping) has already been discussed and studied by SA2 during Rel18 and concluded that there is no need for any specification work.

- If provided to the RAN, usage of the multi-modal information is unspecified.

## 2.2 UL Scheduling enhancements

For the UL scheduling enhancements there were relevant agreements for the conclusion of the study phase both in RAN2 #126 and #127. The agreements can be found in the Appendix.

Based on these agreements it can be concluded that there are two ways forward identified for the normative phase of the UL scheduling objective; LCP enhancements and DSR enhancements.

2.2.1 LCP enhancements

RAN2 has found that the LCP enhancements should be solving the issue of data with low remaining time being delayed due to other non-delay critical data using the transmission resources. This should be solved by changing the priorities of the LCH containing delay critical data. The solution has been limited in scope to prioritization between LCHs, no RAN1 impacts, no dynamic indications to trigger the functionality and no special usage of the UL grants. RAN2 also agreed that no LCP restrictions is in the scope of Rel-19 XR work.

- RAN2 to specify an LCP solution that prioritizes the delay critical data.

2.2.2 DSR enhancements

RAN2 has agreed that to solve the issue with DSR having low granularity, the DSR needs enhancements to support reporting multiple information pairs of buffer size and remaining time values, which will be done by introducing multiple remaining time thresholds. Only one threshold will be used for triggering of the DSR.

- RAN2 to specify an enhanced DSR solution with reporting multiple buffer size and remaining time values based on multiple configured remaining time thresholds.

In the latest RAN2 #127 meeting long discussions emerged on the DSR reporting when the data in the queue is not ordered based on the remaining time. This also highlighted the issue with delay critical data being delayed if non delay critical data will be transmitted first. This created multiple FFSs on how to handle the situation. It can be noted that UE vendors during the LCP discussions claim that the transmission order can be solved by UE implementation, as it is the UE that selects the data for transmission from one DRB. RAN2 also concluded that intra LCH prioritization is something the UE can already do and thus was not needed to be further specified. The problems brought up about non ordered queues and DSR reporting is only a problem if the UE will not transmit the most delay critical data from a LCH first, something that has been clarified can already be done today by UEs. The knowledge of such operation is however transparent to the network and a solution to solve this issue in the DSR is thus needed for UEs not following such transmission principle.

- RAN2 to solve the issue when a UE is not transmitting the most delay critical data first from a LCH by enhancing the reporting in the DSR for those scenarios.

## 2.3 UL Congestion signaling enhancements

The following was the way forward at the last plenary meeting, RAN#104:

According to the discussion in RAN#104, we aim to replace the current note with the WID in the RAN#105 (see below the note), with the objective to address uplink rate control from RAN, which is complementary to upper layer solutions (e.g. L4S).”. For the RAN#105 we should aim to have also agreement on the which layer the gNb would provide the information in the downlink direction The RAN2 in August is expected to make conclusions on the other aspects as per current WID, such that we are able to have scope fitting to the time available for XR work going forward after RAN#105. NOTE from the current WID: “Whether / to what extent network exposure / RAN awareness / e.g. RAN involved rate control, possibly additional info for DL scheduling, parallel with SA2 work, shall be covered in this WI is TBD. “

For the highlighted part on which layer the gNB would provide the information in the downlink direction, we believe it would be sufficient to reuse the recommended bit rate MAC CE based approach i.e., indication on the MAC-layer

- Reuse the recommended bit rate MAC CE to indicate UL congestion information in the downlink direction.

In addition, on the granularity of indication, RAN2 made the following agreement:

It is feasible for RAN to estimate the congestion information per-QoS flow and per-DRB in downlink and uplink directions. It is feasible for RAN to estimate the congestion information per-QoS flow and per-DRB in uplink without UE impacts

As it is possible for RAN to estimate the congestion on a per-DRB level in the downlink and uplink direction, it should be sufficient to reuse the per-DRB level indication i.e., per-LCID level indication. This also gives UE the flexibility to adapt the required flows.

- Indication of UL congestion information in the downlink direction can be on a per-DRB level i.e., per-LCID level.

# Conclusion

We have made the following proposals regarding scope of the Rel-19 XR WI:

Proposal 1	RAN does not specify any new multi-modal information to be provided by the UE to RAN in the UAI, instead wait for SA2 reply regarding information from CN.

Proposal 2	If provided to the RAN, usage of the multi-modal information is unspecified.

Proposal 3	RAN2 to specify an LCP solution that prioritizes the delay critical data.

Proposal 4	RAN2 to specify an enhanced DSR solution with reporting multiple buffer size and remaining time values based on multiple configured remaining time thresholds.

Proposal 5	RAN2 to solve the issue when a UE is not transmitting the most delay critical data first from a LCH by enhancing the reporting in the DSR for those scenarios.

Proposal 6	Reuse the recommended bit rate MAC CE to indicate UL congestion information in the downlink direction.

Proposal 7	Indication of UL congestion information in the downlink direction can be on a per-DRB level i.e., per-LCID level.

# References

RP-241651, “Outcome from the XR discussion for NR Phase 3 WI scope”, 3GPP TSG RAN Meeting #104, Shanghai, China, June 17-20, 2024.

# Appendix

Multi-modality agreements

- Working assumption: Regardless of SA2 decision, RAN2 can extend the UAI for multi-modal awareness at least for uplink QoS flows in Rel-19 XR, by having the UE report existence of multi-modality application and association information among QFIs to gNB.

- FFS whether this can be applied to DL.

- RAN2 considers that based on multi-modal information:

    - The gNB may perform joint admission control. Details can be left up to RAN3 in potential WI phase. FFS if MMSID can be used for this purpose.

    - The gNB may consider this information during QoS flow to DRB mapping (up to gNB implementation)

- For UL, RAN2 does not intend to perform LCP enhancements due to complexity vs gains concerns.

- For DL, whether traffic synchronization (on a per packet basis) can be achieved depends on whether packet level synchronization information can be provided from CN to RAN.

- RAN2 thinks PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the multi-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4.

- RAN2 thinks in case this is feasible, it should be limited to intra-DRB case.

UL Scheduling agreements

Agreements from RAN2 #126:

- Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR.

- The solution should consider impact on UE complexity (as already indicated in SI objective description)

- For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline:

- - Use additional priority configured to LCHs in case of these LCHs with delay-critical data

- FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH

- We try to avoid RAN1 impacts.

- RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way.

- For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial

- The solutions should not disallow non-delay critical data from using an UL grant.

- LCP prioritization within a logical channel will not be considered in RAN2 discussions

- FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

- Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG.

- FFS whether DSR triggering is impacted

- FFS whether PDU set importance needs to be included

Agreements from RAN2 #127:

- RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR.

- Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG.

- For enhanced DSR:

    - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds

    - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR

    - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

- FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3)