3GPP TSG RAN Meeting #105								    RP-242221

Melbourne, Australia, September 9-12, 2024

Agenda Item:	9.3.2.1

Source:	Ericsson

Title:	Discussion on scope of Rel-19 NR mobility enhancements Phase 4 WI

Document for:	Discussion, Decision

# 1	Introduction

The WID [1] for Rel-19 NR mobility enhancements Phase 4 WI includes the following objectives:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1] - Specify support of conditional LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Prioritise intra-CU LTM - Checkpoint to review objective at RAN#105. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

The third objective about conditional LTM mentions a checkpoint at RAN#105 to review this objective. This contribution proposes a reasonable ambition level for the objective enabling it to be completed in time. We also discuss some clarifications in other objectives.

# 2	Discussion

## 2.1	Conditional LTM

As indicated in the WID [1] the work on Conditional LTM has not yet been started in the RAN WGs.

The assumption is that the UE evaluated conditions for triggering LTM are to be based on the event triggered L1 events that are being introduced in Rel-19. There is thus a dependency to this work, which is being done in parallel. This will make it difficult to finalize conditional LTM while the objective on measurements related enhancements, to which there is a dependency, needs to be finalized at the same time.

For the supported scenarios it is indicated that intra-CU cases are prioritized. However, inter-CU cases have not been excluded and support of inter-CU cases are being introduced in Rel-19 for normal LTM. This includes support for solutions to perform security key changes at inter-CU LTM cell switches. If inter-CU cases are to be supported for conditional LTM there will then be a dependency to the work that is done in parallel for inter-CU LTM, with quite some impact mainly on RAN3.

Given that the amount of time that is available for conditional LTM in Rel-19 is quite limited based on the TU allocation, and that there are dependencies to the other work that is taking place in the same release for normal LTM, we think that it can be challenging to finalize the work in time with sufficient quality. We therefore propose that the introduction of conditional LTM is postponed until Rel-20. That way there would be a baseline from Rel-19 that conditional LTM can be built on.

Due to the high workload, postpone the introduction of conditional LTM to Rel-20.

In case proposal 1 cannot be agreed, the scope of conditional LTM in Rel-19 should however be limited and clarified, so that the specification work in affected groups can fit into the allocated TUs and be finalized in time.

## 2.2	Measurement related enhancements for LTM

The second WID objective about measurement related enhancements states the following:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

We have a check point as highlighted above. We provide the some of the differences we observed for events introduced in MIMO and mobility WI to address the highlighted check point.

| Mobility | MIMO |
| --- | --- |
| Agreed to introduce MAC CE for the event triggered L1 measurement report and the events are evaluated at MAC layer | Agreed to carry the report over UCI and the layer which evaluates events is FFS. |
| Agreed on Time To Trigger (TTT) in the event evaluation | Agreed counter-based event evaluation |
| Report on leave is agreed. | No report on leave? |
| Agreed on configurable reportQuantity and reportInterval? | Single report once the event is met? |

Also, the number of events and the purpose the events agreed for mobility WI are different than the MIMO WI. Based on the above analysis we think the event triggered reporting design in MIMO and Mobility WI have multiple key differences and there is not much overlap between the WIs. We think both the work items can independently progress and introduce event triggered reporting.

Hence, we don’t think that any modification of the objectives are required to avoid/manage any overlap in the work. Naturally, both RAN1 and RAN2 can reuse relevant designs from the MIMO WI in the Mobility WI, and vice versa. If there is room for alignment and this comes naturally then this is of course good, but the Mobility WI objective should not be formulated so that the work is conditional to what happen in another WI, as usual, the WIDs should be self-contained.

RAN1 and RAN2 can reuse relevant designs from the MIMO WI in the Mobility WI, and vice versa.

No modification of the objectives are required to avoid/manage overlap in the MIMO WI and the Mobility WI.

At RAN1#118, companies proposed several enhancements with reference to the formulation about "other necessary physical layer operations on candidate cells" in the WID objective.

However, no agreement was reached regarding these enhancement proposals, instead the RAN1 chairman suggested to first clarify the WID scope to avoid a situation where work is done on too many enhancements or enhancements that are only loosely connected to the objective.

RAN1 has only limited TUs allocated for Release 19 Mobility Enhancements Phase 4. However, the decision by RAN2 to use MAC CE as the container for event triggered LTM reports instead of UCI significantly reduces the workload on RAN1 and frees up time that can be spent on other enhancements.

In legacy LTM, the UE starts estimating the link quality of the target after the LTM execution. This leads to a delay before accurate link quality reports reach the NW, and link adaptation can be performed. If the UE can measure the link quality for candidate cells already before LTM execution and report to the network, then the network can perform link adaptation for PDSCH and PDCCH directly after the LTM execution. Thus, the benefit of the proposal is that it gives faster link adaptation, which is an important tool to improve the spectral efficiency in NR.

We propose to clarify the WID by replacing the current formulation about "other necessary physical layer operations on candidate cells" with the objective to specify CSI acquisition and reporting for candidate cells.

In the second bullet of the second WID objective, remove “other necessary physical 	layer operations on candidate cells”.

Add a third bullet for the second WID objective: Specify (early) CSI acquisition and 	reporting for candidate cell(s).

- Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

- Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1]

# Conclusion

We propose the following:

Proposal 1	Due to the high workload, postpone the introduction of conditional LTM to Rel-20.

Proposal 2	No modification of the objectives are required to avoid/manage overlap in the MIMO WI and the Mobility WI.

Proposal 3	In the second bullet of the second WID objective, remove “other necessary physical  layer operations on candidate cells”.

Proposal 4	Add a third bullet for the second WID objective: Specify (early) CSI acquisition and  reporting for candidate cell(s).

# References

[1]	RP-241515, Revised Work Item: NR mobility enhancements Phase 4

# Annex

## Annex A.1

Below is a Text Proposal to update the objectives in the WID for Rel-19 NR mobility enhancements Phase 4 in [1].

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1]

  - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM

  - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1]

  - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management[RAN1]

  - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1]